VC921

(GD5430)

User Manual

Notice

The information in this document is subject to change in order to improve the reliability, design, or function of this product without prior notice and does not represent a commitment on the part of this company.

In no event will the manufacturer be liable for direct, indirect, special, or consequential damages arising out of the use the product or documentation, even if advised of the possibility of such damages.

No part of this reference manual may be reproduced or transmitted in any form or by any means without the prior written permission of this company.

Trademark Acknowledgments

*	IBM® AT, XT, PC/AT, PS/2, and OS/2 are trademarks of International Business Machines Corp.

*	Windows and MS-DOS are trademarks of Microsoft Corporation.

*	AutoCAD is a registered trademark, and AutoShade and AutoSketch are trademarks of AutoDesk , Inc.

*	Display List Driver-VGA, TurboDLDClassic, CustomColors, are trademarks of Panacea Inc.

All other tradenames and trademarks belong to their respective owners.

Version  1.1

FCC Compliance Statement

This device complies with part 15 of the FCC Rules. Operation is subject to the following two conditions:

(1) this device may not cause harmful interference, and

(2) this  device  must  accept  any  interference  received,  including

interference that may cause undesired operation.

FCC WARNING

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation.

This equipment generates, uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.

However, there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

*	Reorient or relocate the receiving antenna.

*	Increase the separation between the equipment and receiver.

*	Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

*	Consult the dealer or an experienced radio/TV technician for help.

Shielded cables and I/O cords must be used for this equipment to comply with relevant FCC regulations.

Table of Contents

1.0	Features	1

Compatibility	1

Resolution and Color Selection	1

Extended Text Display	1

Software Drivers Supported	2

2.0	Installation	3

Installation Procedures	3

3.0	Utility software	5

3.1	CLMode	5

3.1.1	Using CLMode’s graphic interface	5

3.1.2	Using CLMode's command line options	9

3.2	TSRFONT driver	10

4.0	Driver Installation	11

4.1	Microsoft Windows 3.x	11

4.1.1	Using Install	12

4.1.2	Reconfiguring Windows 3.x	16

4.1.3	To turn on/off DDC detection support	17

4.2	Microsoft  Windows NT Driver	18

4.2.1	Windows NT 3.1	18

4.2.2	Windows NT 3.5	19

4.3	Power management screen saver	20

4.3.1	About the screen saver	20

4.3.2	Special considerations	22

4.3.3	Using the screen saver	22

4.3.4	Screen saver mode	23

4.3.5	Screen saver animation speed	23

4.3.6	Password options	24

4.3.7	The Energy Star program	24

4.4	Microsoft Windows 95	25

Section 1. Feature

# 1.0  Features

Thank you for purchasing this Graphical User Interface (GUI) accelerator adapter. The adapter works with your 486 or Pentium Peripheral Component Interconnect (PCI) Local Bus system to bring you high resolution, true color capability, high performance, and compatibility with most software and hardware.

## Compatibility

	486SX/DX and Pentium PCI systems

	Register compatible with EGA and VGA

	Non-interlaced or interlaced monitor support

	Compatible with Multi-Scanning and PS/2 monitors

	1024x768 75Hz refresh

## Resolution and Color Selection

	Supports 640x480 in 16, 256, 32K, 64K, and 16M colors

	Supports 800x600 in 16, 256, 32K, and 64K colors

	Supports 1024x768 in 16 and 256 colors (interlaced and non-interlaced)

	Supports 1280xl024 in 16 and 256 colors interlaced

	Supports 1600xl200 in 16 colors interlaced

## Extended Text Display

	80 column text modes in 30, 43, and 60 rows

132 column text modes in 25, 30, 43 and 60 rows

Section 1. Feature

## Software Drivers Supported

|  CLMODE |  |
| --- | --- |
|  TSRFONT |  |
|  Windows NT 3.1 |  |
|  Windows NT 3.5 |  |
|  MS Windowsä 3.x |  |

Section 2. Installation

# 2.0  Installation

## Installation Procedures

To install the adapter into your system, follow these steps:

1.	Turn OFF all power to your system, including any peripherals (printer, external drives, modem, etc.).

2.	Unfasten the cover mounting screws on your system. Reference your system user manual to determine the location of the mounting screws.

3.	Remove the system cover. Reference your system user manual for instructions on how to remove the system cover.

4.	Select an open expansion slot. Select an appropriate PCI Bus expansion slot for the adapter. Please refer to your computer system manual for the location of the PCI Bus expansion slot. Remove the retaining screw that holds the slot cover in place. Slide the slot cover out and put the screw aside (you will need it to secure the adapter).

5.	Install the adapter. To install the adapter in the selected PCI Bus expansion slot, place the gold-fingered edge-connector of the adapter directly above an expansion slot on the motherboard.

6.	Attach the adapter. Use the screw you removed from the expansion slot in Step 4 to secure the adapter in place.

7.	Replace the computer cover. Secure the cover with the mounting screws you removed in Step 2.

You have now completed the installation of your new graphics adapter on your system.

Section 2. Installation

Table 1. Extended Video Mode Requirements

| Mode No. | No. of Colors | Screen Fomat | Pixel Freq. MHz | Vert. Freq. Hz | 1 Mbyte 32 bits | 2 Mbyte 64 bits |
| --- | --- | --- | --- | --- | --- | --- |
| 5C | 256 | 800x600 | 36 | 56 | 45 MHz | 45 MHz |
| 5C | 256 | 800x600 | 40 | 60 | 45 MHz | 45 MHz |
| 5C | 256 | 800x600 | 50 | 72 | 45 MHz | 45 MHz |
| 5C | 256 | 800x600 | 49.5 | 75 | 45 MHz | 45 MHz |
| 60 | 256 | 1024x768 | 44.9 | 87 | 45 MHz | 45 MHz |
| 60 | 256 | 1024x768 | 65 | 60 | 45 MHz | 45 MHz |
| 60 | 256 | 1024x768 | 75 | 70 | 45 MHz | 45 MHz |
| 60 | 256 | 1024x768 | 77 | 72 | 45 MHz | 45 MHz |
| 60 | 256 | 1024x768 | 78.7 | 75 | 45 MHz | 45 MHz |
| 64 | 64K | 640x480 | 25 | 60 | 45 MHz | 45 MHz |
| 64 | 64K | 640x480 | 31.5 | 72 | 45 MHz | 45 MHz |
| 64 | 64K | 640x480 | 31.5 | 75 | 45 MHz | 45 MHz |
| 65 | 64K | 800x600 | 36 | 56 | 45 MHz | 45 MHz |
| 65 | 64K | 800x600 | 40 | 60 | 48 MHz | 48 MHz |
| 65 | 64K | 800x600 | 50 | 72 | 60 MHz | 60 MHz |
| 66 | 32K | 640x480 | 25 | 60 | 45 MHz | 45 MHz |
| 66 | 32K | 640x480 | 31.5 | 72 | 45 MHz | 45 MHz |
| 66 | 32K | 640x480 | 31.5 | 75 | 45 MHz | 45 MHz |
| 67 | 32K | 800x600 | 36 | 56 | 45 MHz | 45 MHz |
| 67 | 32K | 800x600 | 40 | 60 | 48 MHz | 48 MHz |
| 67 | 32K | 800x600 | 50 | 72 | 60 MHz | 60 MHz |
| 68 | 32K | 1024x768 | 44.9 | 87 | NEM | 60 MHz |
| 6C | 16 | 1280x1024 | 75 | 87 | 45 MHz | 45 MHz |
| 6D | 256 | 1280x1024 | 75 | 87 | NEM | 48 MHz |
| 71 | 16M | 640x480 | 25 | 60 | 45 MHz | 45 MHz |
| 74 | 64K | 1024x768 | 44.9 | 87 | NEM | 55 MHz |

Notes:	1) "  " character indicates 32K Direct-color/256-color Mixed  mode.

2) "  " character indicates Interlaced mode.

3) NEM indicates 'Not Enough Memory'.

Section 3. Utility software

# 3.0 Utility software

There are utility programs which are supplied with your VC921 VGA. The following section describes these utilities and how to use them.

CLMode is a program supplied with your VC921 VGA to configure it's various options. The TSRFONT utility will make a full 8x14 size character set available to programs that bypass the BIOS and directly draw characters to the screen.

## 3.1  CLMode

The CLMode utility allows the user to define the type of monitor attached, set the video mode timings supported by the VC921 VGA and preview them.

Note that to use a mouse, a mouse driver should be installed prior to running the CLMode utility.

### 3.1.1  Using CLMode’s graphic interface

CLMode requires that the computer have 300KB of standard memory and 10MB of extended or expanded memory available for it to run. "CLMode" can be used in two ways:

1.	"CLMode" is in Utility/Drivers CD Title. You have to run README.BAT in Utility/Drivers CD Title, type “G” ENTER for CIRRUS GD5430, and type “C” ENTER, then type “y” to change to subdirectory UTIL.

Section 3. Utility software

2.When Windows driver has been intalled. "CLMode" is in C:\Window\VGAUTIL directory.

At the DOS prompt type:    CLMODE  [Enter]

When CLMode starts, it will blank the screen for a couple of seconds. During this time, CLMode is attempting to identify the monitors capabilities using the VESA Display Data Channel (DDC). In its graphics mode, CLMode looks and works just like a Windows application.

The main window is used to select and configure the monitor that you have connected to the graphics adapter. Select the brand and model of the current monitor using the two dropdown boxes. To get a scrollable list of available choices, click on the down arrow with the mouse. For keyboard only, hit the [Tab] key until the Monitor Brand field is highlighted, and use the cursor keys to move through the choices. If the correct brand is not available select ‘Other Brand’. Move to the ‘Monitor Model’ field and select from the list provided. If ‘Other Brand’ was selected then the only choice will be ‘Other Monitor’. If the monitor is not on the list select “Other Model’.

If the graphics adapter and the monitor both support VESA Display Data Channel, this will be the default choice. This means that CLMode was able to identify the monitor and knows what resolutions and refresh rates are available for the monitor. Select the refresh rate for each resolution. If the monitor was selected by name, then selecting the highest refresh for each resolution will give the highest quality display. If ‘Other Brand’ or ‘Other Model’ was chosen, consult the manual that came with the monitor to determine the proper refresh rates.

At the bottom of the screen are six buttons. Each button represents a

Section 3. Utility software

different option or window. The underlined letter of a button name specifies the hot key combination for that item. For example, press the [Alt] and the underlined letter keys simultaneously.

In the lower right corner of the main window is configuration information about the graphics system. The information details the VGA BIOS version, the total display memory of the graphics controller, whether or not an EEPROM is present, and whether or not a centering TSR is loaded. This information is useful when installing software or getting technical support.

#### Preview video modes

Select the Preview button. The Video Modes Preview window displays a list of all of the modes supported based on the currently selected monitor timings and the amount of video memory present. This list of video modes will tell you which are available in your current configuration for use with extended resolution drivers. To see what different video modes look like on your monitor, select the Show button. After each test screen is displayed, press [Enter]. If you don't want to see any more video modes pressing [Esc] will return you to the Video Modes Preview window.

#### Centering adjust

Centering is available while in Video Modes Preview (only for extended Graphics modes 58h and greater). While previewing a video mode, you have the option to use the Right/Left/Up/Down arrow keys to adjust your screen accordingly. The screen adjustments have predefined limits, and in some cases, the limit for the right side adjustment for some modes will have already been reached when entering

Section 3. Utility software

the Preview mode.  In that case, no right side adjustment will be allowed. Once a mode has been adjusted, the new values for horizontal and/or vertical positioning are saved within CLMode.

When exiting from CLMode, you are prompted on whether you want to save these new parameters to the TSR (CENTER.COM). If you choose to do this, the values to be used for the adjusted modes are stored in CENTER.COM on exit. CENTER.COM can then be loaded outside of CLMode.  Once this is accomplished, all of the modes which you have adjusted will be the active modes set when that mode request    is made. To unload CENTER.COM, you must change the monitor type currently set in CLMode. This will unload CENTER.COM and uninstall all centering adjustments previously redefined.

An additional adjustment for monitor SYNC POLARITY is also available while centering is active. To adjust the polarity of the current mode, simply press the PgUp or PgDn keys (while in Preview) to cycle through the available options, (Vertical Sync Polarity/Horizontal Sync Polarity):

+/+, +/-, -/+, -/-

Note: Centering will not be allowed if CENTER.COM has been loaded. You must unload CENTER.COM before attempting to save new adjustments.

Limitations: You may only adjust and save 16 modes at a time to CENTER.COM.

The information in the main window displays the VGA controller type, the BIOS version number, and the amount of video memory present.

Section 3. Utility software

#### Getting help

Selecting the Help button from the main window will display instructions on using CLMode.

#### Information about CLMode

Selecting the About button will display version and copyright information about CLMode.

#### Undoing changes

To return all settings to their state when CLMode was started, select the Undo button.

#### Exiting the CLMode

To exit CLMode at any time, press the [Alt] and [F4] keys simultaneously, or click the left mouse button on the system button of the main window (i.e., the top left corner button of the window which is shown as a dot), or select the Exit button.

CLMode will ask if the changes should be saved in the AUTOEXEC.BAT. In most cases, the end user should answer yes. Answering no means that the next time the computer is booted, all of CLMode's configuration information will be lost.

When the CLMode utility exits, the currently selected monitor brand, model, timings, and centering information will be saved. The current monitor timings will be displayed.

### 3.1.2  Using CLMode's command line options

When command line options for CLMODE.EXE are given at the

Section 3. Utility software

DOS prompt, the menu-driven windows will not be displayed.  Instead, configuration, monitor type, video mode and refresh rate will

be set at the DOS prompt.  To display the available command line syntax for CLMode, type:

CLMODE /?[Enter]

Typing an invalid option will display the command line help text. Typing [S] as a command line option will display the current CLMode settings.

## 3.2  TSRFONT driver

Some DOS application programs bypass the BIOS and directly draw characters to the screen. Programs which directly draw characters using the 8x14 font from the BIOS will appear to be writing incorrect data to the screen. Typically this can occur in programs which offer a selection to use a graphics 25 or 34 line display mode.  Other programs may appear to cut off the descenders of characters like "y" and "j".

Running the TSRFONT driver will make a full 8x14 size character set available to these programs, and should correct display errors discussed above which were observed in these programs.

To run the TSRFONT driver, type TSRFONT [Enter]. If you want to run it automatically when you turn on your computer, add it to your AUTOEXEC.BAT file.

Section 4. Driver Installation

# 4.0 Driver Installation

## 4.1  Microsoft Windows 3.x

The Windows 3.x driver installation utility copies all of the graphics driver and utility files to your hard disk. It also allows you to configure your graphics system for Windows 3.x in either DOS or original OS/2 v2.1. You can change the monitor refresh rates, resolution, number of available colors, large or normal size fonts and font cache size.

After new options have been selected, you can either immediately restart Windows, in which case the new configuration will take effect immediately, or you can continue working in the current resolution, in which case the new resolution will take effect the next time Windows is started. In some configurations the AUTOEXEC.BAT file needs to be modified to make the changes permanent.

The display drivers support both Windows versions 3.1 and 3.11 and also Windows for Workgroups version 3.11. Because these drivers were written and optimized for performance, they were written to use a linear address for the display buffer and run in protected mode. To load the display drivers with Windows 3.11 or Windows for Workgroups 3.11, you must install the VGA driver first before installing these display drivers. The display drivers will only run in enhanced mode and not in standard mode. Original OS/2 2.x is non-upgraded to support high color and true color display drivers.

Section 4. Driver Installation

### 4.1.1  Using Install

To run the installation program, start Windows 3.x. Insert the Utility/Drivers CD Title into your CD-ROM drive. From the Windows Program Manager select Run from the File menu. Type the letter of the CD-ROM drive that the Utility/Drivers  CD Title is in, followed by the word install. For instance, if the Utility/Drivers CD Title is in drive D:, type D:\VGA\CIRRUS\5430\INSTALL. Click on the OK button.

In the first dialog box that is displayed, you can set the path where you would like the utility programs to be installed. If you want to use the default directory, click on Continue. To change to another directory, you can either type the path name or you can click on the down arrow and select a directory for installation. The drop down box works just like the Directory field in a file open dialog box. After you select the directory, press the Continue button.

After the drivers have been copied to your hard disk, the install program will execute the WinMode utility.

After the extended resolution drivers and utilities have been installed, they can be reconfigured from within Windows 3.x using Windows Setup, WinMode, or from the DOS prompt.

#### Resolution

These buttons let you choose the resolution that WinMode will use to

Section 4. Driver Installation

run Windows 3.x after Windows is restarted. Some of these buttons may be unavailable because of the amount of video memory in your

system or if some of the monitor refresh rates have been set to Unavailable.

#### Colors

These buttons let you choose the number of colors that will be available to Windows 3.x after Windows is restarted. Some of these choices may be unavailable. This is because of the amount of video memory in your system and the resolution that you have selected. If you have chosen OS/2 v2.1 in the Operating System field, then the only possible choices are for 16 or 256 colors. Generally, 256 color mode will be the fastest choice. If you need more colors then there will be some slowdown in graphics performance.

#### Font Size

The small fonts are intended for lower resolutions and higher resolutions on large monitors. With smaller monitors, the large fonts are more readable at higher resolutions.

#### Monitor brand

Select the brand of your monitor from the list provided in the drop down box. This will change the list of monitors in the Monitor Model drop down box to display only the models available under the selected brand. If your monitor brand is not listed, select Other Brand.

If your graphics adapter and monitor both support VESA Display Data Channel, this will be the default choice. This means that

Section 4. Driver Installation

WinMode was able to identify the monitor and knows what resolutions and refresh rates are available for the monitor.

#### Monitor model

Select the current model from the list first presented. If your monitor

is not listed select Other Monitor. If your graphics adapter and monitor both support VESA Display Data Channel, this will be the default choice. This means that WinMode was able to identify the monitor and knows what resolutions and refresh rates are available for the monitor.

#### Monitor Refresh Rates

The drop down list boxes let you select the monitor refresh rates for each resolution. If you select Unavailable for any screen resolution, any higher resolutions will also be unavailable. This will also turn off the corresponding choices in the Resolution box.

In general, the higher the refresh rate, the better the display quality and the lower the performance. This is because the graphics system can only do a fixed number of operations per second. The more time it spends redrawing the screen, the less time it has available to perform other operations. Consult the manual that came with your monitor to determine the proper settings for these fields.

#### Font Cache Size

The Font Cache Size lets you set the amount of system memory that will be available for font caching. Next to the font cache size is an up arrow and a down arrow. Click on the up arrow to increase the cache size. Click on the down arrow to decrease the cache size.

Section 4. Driver Installation

Font caching is a technique to increase the performance of Windows by saving the bitmaps of frequently used characters. Normally, when a character is displayed on the screen it first is created from the Truetype outline then it is copied to the screen. A cached character has already been created and stored and so it is just copied as needed.

WinMode has tried to determine the correct setting for this field for you, but you may change it. Just remember that memory set aside for font caching will not be available for  Windows program and system usage.

#### Operating System

The Operating System buttons let you set the operating system in which you are running Windows.

This is very important because the configuration and capabilities of the drivers are very different in OS/2 v2.1 and DOS. The most obvious difference is that the drivers will only work in 16 and 256 color modes in OS/2. There are a number of other differences in the configuration of the drivers.

Prior to installing OS/2 For Windows, you will need to run WinMode and select the OS/2 v2.1 option.

#### OK

The OK button closes the dialog box and accepts the choices that you have made.

After clicking OK, your computer will be reconfigured to use the choices that you have made. These changes may need to be added to

Section 4. Driver Installation

your AUTOEXEC.BAT file for them to be permanent. If this is necessary, you will be prompted by the program.

If WinMode detects that you have changed your Windows 3.x configuration, you will be asked if Windows should be restarted. If you answer yes, Windows will be restarted immediately. If you

answer no, you can continue to use Windows, however the changes will not take effect until Windows is restarted.

#### Cancel

The Cancel button closes the dialog box and exits the program without making any changes. Selecting Close from the control menu or double clicking on the control menu box will have the same effect.

### About

The About button will display a dialog box with the version number of WinMode and a copyright notice.

#### About

The About button will display a dialog box with the version number of WinMode and a copyright notice.

#### Help

For help on WinMode, select the Help button. The main help screen contains an image of the WinMode screen. Place the cursor over the field(s) that you need help with. When the mouse pointer changes to a hand, click to display the help text for that field. Click again to make the pop-up text disappear.

After the extended resolution drivers and utilities have been installed, they can be reconfigured either from within the Windows 3.x Control Panel or from the DOS prompt.

Section 4. Driver Installation

### 4.1.2  Reconfiguring Windows 3.x

If you are in Windows, run WinMode to reconfigure the Windows 3.x drivers. Please refer to the Driver Installation section for information on Microsoft Windows 3.x. If you are in DOS, proceed as follows to reconfigure the Windows 3.x drivers.

1.	Ensure that Windows 3.x and the extended resolution drivers are already installed on your computer.

2. From your Windows directory, at the DOS prompt, type SETUP[Enter] to run the Windows SETUP.EXE program. Follow the instructions on the screen. When you come to the screen which lists the hardware and software components such as display adapter (e.g. VGA, CGA, etc.), keyboard type, mouse type, etc., go to the Display selection by using cursor keys to move the highlighted bar and press [Enter].

3.	You will see the list of drivers and their associated resolutions.

4.	Highlight the desired choice by moving the cursor to the correct display driver, and then press [Enter].

5.	Setup will prompt you that the driver is already in your Windows directory and give you a chance to replace it. Use the existing driver.

6.	Continue with the remainder of the setup procedure.

### 4.1.3  To turn on/off DDC detection support

To turn the DDC detection support on or off, the following changes need to be made to the WINMODE.INI file.

To turn on DDC detection support:   	MakeDdcCall=on

Section 4. Driver Installation

To turn off DDC detection support:   	MakeDdcCall=off

To turn the resolution-change-on-the-fly support on or off, the

following changes need to be made to the WINMODE.INI file.

To turn on resolution-change-on-the-fly support, locate the following line in the WINMODE.INI file and change the last word in the line

to 'on':

system.ini,CL_WinAccel,changeres:lin=$winmode.ini,Configuration,changeres,on

To turn off resolution-change-on-the-fly support, locate the following line in the WINMODE.INI file and change the last word in the line to 'off':

system.ini,CL_WinAccel,changeres:lin=$winmode.ini,Configuration,changres,off

## 4.2  Microsoft  Windows NT Driver

### 4.2.1  Windows NT 3.1

Windows NT 3.1 display drivers installation:

decompress it to any directory you made.

The Windows NT 3.1 display drivers is in Utility/Drivers CD Title. Before using it, You have to run README.BAT in Utility/Drivers CD Title, and type “G” for CIRRUS GD5430, then you can see which subdirectory the Windows NT 3.1 drivers is in.

1.	Start Windows NT.

Section 4. Driver Installation

2.	From the Main window of the Program Manager run the Windows NT Setup program.

3.	Select Change Systems Settings from the Options menu of Setup.

4.	Click on the down arrow at the right side of the DISPLAY: line. Scroll to the end of the list of available display drivers and select Other display (requires disk from hardware manufacturer).

5. Type the pathname where the Windows NT drivers is in your CD-ROM drive( e.g. D:\VGA\CIRRUS\5430\NT31), then click on OK.

6.	You will see the list of available drivers, their associated resolution and monitor refresh rates.

7.	Highlight by moving the cursor to the desired display driver, click on OK, and then click on Close.

8.	Continue with the remainder of the setup procedure. The changes will not take effect until the computer is restarted.

9.	After you have installed the Windows NT driver, to select another display resolution, follow steps 2 and 3 above, then click on the down arrow on the right side of the DISPLAY: line to select the desired resolution from the list of available display drivers.

These Windows NT drivers only support systems or video adapters with 1MB of DRAM or more.

### 4.2.2  Windows NT 3.5

Windows NT 3.5 display drivers installation:

decompress it to any directory you made.

The Windows NT 3.5 display drivers is in Utility/Drivers CD Title.

Section 4. Driver Installation

Before using it, You have to run README.BAT in Utility/Drivers CD Title, and type “G” for CIRRUS GD5430, then you can see which subdirectory the Windows NT 3.5 is in.

1.	Select Control Panel from the Main group.

2.	Select the Display icon.

3.	Select Change Display Type.

4.	Select Change from the Adapter Type area.

5.	Select Other.

6.	Select Install and click "Yes" when the Installing Driver dialog box appears.

7.	When the Windows NT Setup dialog box appears, type the pathname where the Windows NT drivers is in your CD-ROM drive(e.g. D:\VGA\CIRRUS\5430\NT35), and click "Continue".

8.	A message will appear stating that the drivers were successfully installed. Click "OK".  You must now restart Windows NT 3.5.

9.	Selecting resolution and color depth.

10.	Select Control Panel from the Main group.

11.	Select Display icon.

12.	Select Color Palette to change between 16 colors, 256 colors and 65536 colors.

13.	To select desktop resolution size, go to the Desktop area and use the slide bar to change resolution from 640x480, 800x600, 1024x768, and 1280x1024.

14.Select Test to test the resolution. If the display test screen was

Section 4. Driver Installation

good then select "Yes" when the Test Mode dialog box appears. If the display test screen was bad then select "No". Windows NT will give you an error message.

If the display screen was good and you selected "Yes", Windows NT 3.5 will prompt you to restart Windows NT 3.5.

## 4.3  Power management screen saver

### 4.3.1 About the screen saver

The Display Power Manager for Windows 3.x provides a mechanism to control the amount of power used by a computer's monitor.  It provides a total of 5 levels of power savings.

The first level is a conventional screen saver that turns the screen black and bounces a logo around. The black screen provides substantial power savings and the animation lets you know that the computer is still active.

The other four levels of power savings are...

Reduced on mode - Reduced on mode is an optional power saving state that allows the computer to use some power savings while still fully operational. Currently this is limited to some portable computers that can lower the power usage on their LCD displays. This causes some degradation of display quality.

Stand by mode - Stand by mode is an optional power saving state that gives minimal power savings, but provides the fastest recovery time.

Section 4. Driver Installation

Suspend mode - Suspend mode is a mandatory state in whichsubstantial power savings are achieved by the display. The trade off is that recovery times are longer than from stand by mode.

Off mode - Off mode provides the highest level of power savings and the longest recovery times. With this mode the display is actually turned off so that recovery times are equivalent to switching the monitor on.

They match the power saving modes defined by the VESA VBE/PM (see below) standard. As you move down from one mode to the next, the amount of power being saved is greater, but so is the amount of time that it takes for a monitor to recover and be ready to display data.

The screen saver will cycle through all of the selected power saving modes one after another until the maximum selected power saving mode has been reached. The number of minutes that is specified for each level is the number of minutes after the previous mode has been enabled. It is not the total time before entering that mode.

Using the Windows Control Panel, select Desktop. In the group Screen Saver within the Desktop dialog box, select the Screen Saver named Display Power Manager. Select the desired delay before entering a power saving mode. Select Setup to further configure the power saving options. The following sections describe these options. For further information on setting up Windows screen savers, please refer to the Windows User Guide.

Section 4. Driver Installation

### 4.3.2  Special considerations

Some of these power saving modes are intended for specific display types, so not all of them are available on all video controllers. On some controllers, only the animated logo will be available. For the power saving modes to have any effect beyond just blanking the screen, the monitor being used must have specific power saving features.

Note that if the screen saver is in any of the power saving modes, moving the mouse will not wake it up. This is different from normal screen savers and is used to keep the monitor from waking up because

of accidents such as bumping the desk that the computer is sitting on. This is especially important if the password option is enabled, since once the password dialog box pops up, it stays there until the user turns it off. This is a limitation of the Windows 3.x screen saver interface.

### 4.3.3  Using the screen saver

When the screen saver is started, it will initially display a bouncing logo on a black background. To enable additional levels of power savings, select them from the Screen Saver Mode section of the Setup dialog box.

### 4.3.4  Screen saver mode

The Screen Saver Mode section has four check boxes in it. They are ReducedOn, StandBy, Suspend and Off. The screen saver detects the type of graphics controller that is being used and the types of power savings that it is capable of.

If the text immediately to the right of any of the check boxes is

Section 4. Driver Installation

unavailable, that option is not available. If a power saving mode is available, then the text immediately to the right of the check box is black. If the box is selected with the mouse or the keyboard, the rest of the fields on the line will also turn black. If the check box is deselected, the other fields will turn gray.

The minutes field for each line can be changed only if the check box for that line is selected. The amount of time can be set for any number between 1 and 60. The number can either be typed directly, or by clicking on the up or down arrow next to the number. Holding the mouse button down on the arrow will quickly increase or decrease the minutes field.

The number of minutes that is specified for each level is the number of minutes after the previous mode has been enabled. It is not the total time before entering that mode.

### 4.3.5  Screen saver animation speed

The scroll bar controls how fast the animation moves. The checkbox controls whether or not a bouncing sound will be played whenever the logo hits an edge of the screen.

### 4.3.6  Password options

If the password checkbox is enabled, then the screen saver will prompt the user for a password before it quits. The password is the same one that is used for other Windows 3.x screen savers.

### 4.3.7  The Energy Star program

Energy Star is a program created by the US Environmental Protection

Section 4. Driver Installation

Agency (EPA) to promote energy efficiency. The goal of this program is to lower electricity usage by making computers and related hardware more energy efficient.

Computers currently use an estimated 5% of commercial electricity consumption. If no actions are taken, this could rise to 10% by the year 2000. Ironically, much of this electricity is wasted. Research shows that the vast majority of time personal computers are on, they are not actively in use. Additionally 30% - 40% are left running at night and on weekends.

Electricity generation accounts for 35% of all U.S. emissions of carbon dioxide - the most prevalent greenhouse gas. It also accounts for 75% and 38% of all U.S. emissions of sulfur dioxide of nitrogen oxides respectively - the two pollutants most responsible for acid rain.

By using more energy-efficient equipment in our homes, offices and factories, we can reduce this pollution - while saving money.

The Energy Star logo is used to mark computers, peripherals and software that have adopted the EPAs power saving guidelines.

## 4.4  Microsoft Windows 95

MS Windows 95 provides lots of VGA drivers, and Cirrus Logic GD5430/34 driver is included.

If you want to use extended mode, please see Windows 95 user's guide refer to VGA drivers installation.