TABLE OF CONTENTS

0001 ahi.device/AHI_AllocAudioA

0002 ahi.device/AHI_AllocAudioRequestA

0003 ahi.device/AHI_AudioRequestA

0004 ahi.device/AHI_BestAudioIDA

0005 ahi.device/AHI_ControlAudioA

0006 ahi.device/AHI_FreeAudio

0007 ahi.device/AHI_FreeAudioRequest

0008 ahi.device/AHI_GetAudioAttrsA

0009 ahi.device/AHI_LoadSound

0010 ahi.device/AHI_NextAudioID

0011 ahi.device/AHI_PlayA

0012 ahi.device/AHI_SampleFrameSize

0013 ahi.device/AHI_UnloadSound

0014 ahi.device/CMD_FLUSH

0015 ahi.device/CMD_READ

0016 ahi.device/CMD_RESET

0017 ahi.device/CMD_START

0018 ahi.device/CMD_STOP

0019 ahi.device/CMD_WRITE

0020 ahi.device/CloseDevice

0021 ahi.device/NSCMD_DEVICEQUERY

0022 ahi.device/OpenDevice

ahi.device/AHI_AllocAudioA

NAME

AHI_AllocAudioA -- allocates and initializes the audio hardware

AHI_AllocAudio -- varargs stub for AHI_AllocAudioA()

SYNOPSIS

audioctrl = AHI_AllocAudioA( tags );

D0                           A1

struct AHIAudioCtrl *AHI_AllocAudioA( struct TagItem * );

audioctrl = AHI_AllocAudio( tag1, ... );

struct AHIAudioCtrl *AHI_AllocAudio( Tag, ... );

FUNCTION

Allocates and initializes the audio hardware, selects the best

mixing routine (if necessary) according to the supplied tags.

To start playing you first need to call AHI_ControlAudioA().

INPUTS

tags - A pointer to a tag list.

TAGS

AHIA_AudioID (ULONG) - The audio mode to use. Default is

AHI_DEFAULT_ID. (AHI_DEFAULT_ID is the ID the user has selected

in the preferences program. It's a good value to use the first

time she starts your application.)

AHIA_MixFreq (ULONG) - Desired mixing frequency. The actual

mixing rate may or may not be exactly what you asked for.

Default is AHI_DEFAULT_FREQ. (AHI_DEFAULT_FREQ is the user's

prefered frequency.)

AHIA_Channels (UWORD) - Number of channel to use. The actual

number of channels used will be equal or grater than the

requested. If too many channels were requested, this function

will fail. This tag must be supplied.

AHIA_Sounds (UWORD) - Number of sounds to use. This tag must be

supplied.

AHIA_SoundFunc (struct Hook *) - A function to call each time

when a sound has been started. The function receives the

following parameters:

A0 - (struct Hook *)

A2 - (struct AHIAudioCtrl *)

A1 - (struct AHISoundMessage *)

The hook may be called from an interrupt, so normal interrupt

restrictions apply.

The called function should follow normal register conventions,

which means that d2-d7 and a2-a6 must be preserved.

Default is NULL.

AHIA_PlayerFunc (struct Hook *) - A function to be called at regular

intervals. By using this hook there is no need for music players

to use other timing, such as VBLANK or CIA timers. But the real

reason it's present is that it makes it possible to do non-

realtime mixing to disk.

Using this interrupt source is currently the only supported way

to ensure that no mixing occurs between calls to AHI_SetVol(),

AHI_SetFreq() or AHI_SetSound().

If the sound playback is done without mixing, 'realtime.library'

is used to provide timing. The function receives the following

parameters:

A0 - (struct Hook *)

A2 - (struct AHIAudioCtrl *)

A1 - Undefined.

Do not assume A1 contains any particular value!

The hook may be called from an interrupt, so normal interrupt

restrictions apply.

The called function should follow normal register conventions,

which means that d2-d7 and a2-a6 must be preserved.

Default is NULL.

AHIA_PlayerFreq (Fixed) - If non-zero, enables timing and specifies

how many times per second PlayerFunc will be called. This must

be specified if AHIA_PlayerFunc is! Do not use any extreme

frequencies. The result of MixFreq/PlayerFreq must fit an UWORD,

ie it must be less or equal to 65535. It is also suggested that

you keep the result over 80. For normal use this should not be a

problem. Note that the data type is Fixed, not integer (see BUGS

below). 50 Hz is 50<<16, which is a reasonable lower limit.

Default is a reasonable value. Don't depend on it.

AHIA_MinPlayerFreq (Fixed) - The minimum frequency (AHIA_PlayerFreq)

you will use. You MUST supply this if you are using the device's

interrupt feature!

AHIA_MaxPlayerFreq (Fixed) - The maximum frequency (AHIA_PlayerFreq)

you will use. You MUST supply this if you are using the device's

interrupt feature!

AHIA_RecordFunc (struct Hook *) - This function will be called

regularly when sampling is turned on (see AHI_ControlAudioA())

with the following parameters:

A0 - (struct Hook *)

A2 - (struct AHIAudioCtrl *)

A1 - (struct AHIRecordMessage *)

The message (AHIRecordMessage) is filled as follows:

ahirm_Buffer - Pointer to the samples. The buffer is valid

until next time the Hook is called.

ahirm_Length - Number of sample FRAMES in buffer.

To get the size in bytes, multiply by 4 if ahiim_Type is

AHIST_S16S.

ahirm_Type - Always AHIST_S16S at the moment, but you *must*

check this, since it may change in the future!

The hook may be called from an interrupt, so normal interrupt

restrictions apply. Signal a process if you wish to save the

buffer to disk. The called function should follow normal register

conventions, which means that d2-d7 and a2-a6 must be preserved.

NOTE: The function MUST return NULL (in d0). This was previously

not documented. Now you know.

Default is NULL.

AHIA_UserData (APTR) - Can be used to initialize the ahiac_UserData

field. Default is 0.

RESULT

A pointer to an AHIAudioCtrl structure or NULL if an error occured.

EXAMPLE

NOTES

SoundFunc will be called in the same manner as Paula interrupts

occur; when the device has updated its internal variables and can

accept new commands.

BUGS

For compability reasons with some really old applications,

AHIA_PlayerFreq, AHIA_MinPlayerFreq and AHIA_MaxPlayerFreq

interpret values lower than 0x10000 as integers, not Fixed.

This means that the lowest frequency possible is 1 Hz. However,

you should *never* use a value than say 10-20 Hz anyway, because

of the high latency and the impact on multitasking.

This kludge will be removed some day. Always use Fixed!

SEE ALSO

AHI_FreeAudio(), AHI_ControlAudioA()

ahi.device/AHI_AllocAudioRequestA

NAME

AHI_AllocAudioRequestA -- allocate an audio mode requester.

AHI_AllocAudioRequest -- varargs stub for AHI_AllocAudioRequestA()

SYNOPSIS

requester = AHI_AllocAudioRequestA( tags );

D0                                  A0

struct AHIAudioModeRequester *AHI_AllocAudioRequestA(

struct TagItem * );

requester = AHI_AllocAudioRequest( tag1, ... );

struct AHIAudioModeRequester *AHI_AllocAudioRequest( Tag, ... );

FUNCTION

Allocates an audio mode requester data structure.

INPUTS

tags - A pointer to an optional tag list specifying how to initialize

the data structure returned by this function. See the

documentation for AHI_AudioRequestA() for an explanation of how

to use the currently defined tags.

RESULT

requester - An initialized requester data structure, or NULL on

failure.

EXAMPLE

NOTES

The requester data structure is READ-ONLY and can only be modified

by using tags!

BUGS

SEE ALSO

AHI_AudioRequestA(), AHI_FreeAudioRequest()

ahi.device/AHI_AudioRequestA

NAME

AHI_AudioRequestA -- get an audio mode from user using an requester.

AHI_AudioRequest -- varargs stub for AHI_AudioRequestA()

SYNOPSIS

success = AHI_AudioRequestA( requester, tags );

D0                           A0         A1

BOOL AHI_AudioRequestA( struct AHIAudioModeRequester *,

struct TagItem * );

result = AHI_AudioRequest( requester, tag1, ... );

BOOL AHI_AudioRequest( struct AHIAudioModeRequester *, Tag, ... );

FUNCTION

Prompts the user for an audio mode, based on the modifying tags.

If the user cancels or the system aborts the request, FALSE is

returned, otherwise the requester's data structure reflects the

user input.

Note that tag values stay in effect for each use of the requester

until they are cleared or modified by passing the same tag with a

new value.

INPUTS

requester - Requester structure allocated with

AHI_AllocAudioRequestA(). If this parameter is NULL, this

function will always return FALSE with a dos.library/IoErr()

result of ERROR_NO_FREE_STORE.

tags - Pointer to an optional tag list which may be used to control

features of the requester.

TAGS

Tags used for the requester (they look remarkable similar to the

screen mode requester in ASL, don't they? ;-) )

AHIR_Window (struct Window *) - Parent window of requester. If no

AHIR_Screen tag is specified, the window structure is used to

determine on which screen to open the requesting window.

AHIR_PubScreenName (STRPTR) - Name of a public screen to open on.

This overrides the screen used by AHIR_Window.

AHIR_Screen (struct Screen *) - Screen on which to open the

requester. This overrides the screen used by AHIR_Window or by

AHIR_PubScreenName.

AHIR_PrivateIDCMP (BOOL) - When set to TRUE, this tells AHI to

allocate a new IDCMP port for the requesting window. If not

specified or set to FALSE, and if AHIR_Window is provided, the

requesting window will share AHIR_Window's IDCMP port.

AHIR_IntuiMsgFunc (struct Hook *) - A function to call whenever an

unknown Intuition message arrives at the message port being used

by the requesting window. The function receives the following

parameters:

A0 - (struct Hook *)

A1 - (struct IntuiMessage *)

A2 - (struct AHIAudioModeRequester *)

AHIR_SleepWindow (BOOL) - When set to TRUE, this tag will cause the

window specified by AHIR_Window to be "put to sleep". That is, a

busy pointer will be displayed in the parent window, and no

gadget or menu activity will be allowed. This is done by opening

an invisible Intuition Requester in the parent window.

AHIR_UserData (APTR) - A 32-bit value that is simply copied in the

ahiam_UserData field of the requester structure.

AHIR_TextAttr (struct TextAttr *) - Font to be used for the

requesting window's gadgets and menus. If this tag is not

provided or its value is NULL, the default font of the screen

on which the requesting window opens will be used. This font

must already be in memory as AHI calls OpenFont() and not

OpenDiskFont().

AHIR_Locale (struct Locale *) - Locale to use for the requesting

window. This determines the language used for the requester's

gadgets and menus. If this tag is not provided or its value is

NULL, the system's current default locale will be used.

AHIR_TitleText (STRPTR) - Title to use for the requesting window.

Default is no title.

AHIR_PositiveText (STRPTR) - Label of the positive gadget in the

requester. English default is "OK".

AHIR_NegativeText (STRPTR) - Label of the negative gadget in the

requester. English default is "Cancel".

AHIR_InitialLeftEdge (WORD) - Suggested left edge of requesting

window.

AHIR_InitialTopEdge (WORD) - Suggested top edge of requesting

window.

AHIR_InitialWidth (WORD) - Suggested width of requesting window.

AHIR_InitialHeight (WORD) - Suggested height of requesting window.

AHIR_InitialAudioID (ULONG) - Initial setting of the Mode list view

gadget (ahiam_AudioID). Default is ~0 (AHI_INVALID_ID), which

means that no mode will be selected.

AHIR_InitialMixFreq (ULONG) - Initial setting of the frequency

slider. Default is the lowest frequency supported.

AHIR_InitialInfoOpened (BOOL) - Whether to open the property

information window automatically. Default is FALSE.

AHIR_InitialInfoLeftEdge (WORD) - Initial left edge of information

window.

AHIR_InitialInfoTopEdge (WORD) - Initial top edge of information

window.

AHIR_DoMixFreq (BOOL) - Set this tag to TRUE to cause the requester

to display the frequency slider gadget. Default is FALSE.

AHIR_DoDefaultMode (BOOL) - Set this tag to TRUE to let the user

select the mode she has set in the preferences program. If she

selects this mode,  ahiam_AudioID will be AHI_DEFAULT_ID and

ahiam_MixFreq will be AHI_DEFAULT_FREQ. Note that if you filter

the mode list (see below), you must also check the mode (with

AHI_BestAudioIDA()) before you use it since the user may change

the meaning of AHI_DEFAULT_MODE anytime, without your knowledge.

Default is FALSE. (V4)

AHIR_FilterFunc (struct Hook *) - A function to call for each mode

encountered. If the function returns TRUE, the mode is included

in the file list, otherwise it is rejected and not displayed. The

function receives the following parameters:

A0 - (struct Hook *)

A1 - (ULONG) mode id

A2 - (struct AHIAudioModeRequester *)

AHIR_FilterTags (struct TagItem *) - A pointer to a tag list used to

filter modes away, like AHIR_FilterFunc does. The tags are the

same as AHI_BestAudioIDA() takes as arguments. See that function

for an explanation of each tag.

RESULT

result - FALSE if the user cancelled the requester or if something

prevented the requester from opening. If TRUE, values in the

requester structure will be set.

If the return value is FALSE, you can look at the result from the

dos.library/IoErr() function to determine whether the requester

was cancelled or simply failed to open. If dos.library/IoErr()

returns 0, then the requester was cancelled, any other value

indicates a failure to open. Current possible failure codes are

ERROR_NO_FREE_STORE which indicates there was not enough memory,

and ERROR_NO_MORE_ENTRIES which indicates no modes were available

(usually because the application filter hook filtered them all

away).

EXAMPLE

NOTES

The requester data structure is READ-ONLY and can only be modified

by using tags!

The mixing/recording frequencies that are presented to the user

may not be the only ones a driver supports, but just a selection.

BUGS

SEE ALSO

AHI_AllocAudioRequestA(), AHI_FreeAudioRequest()

ahi.device/AHI_BestAudioIDA

NAME

AHI_BestAudioIDA -- calculate the best ModeID with given parameters

AHI_BestAudioID -- varargs stub for AHI_BestAudioIDA()

SYNOPSIS

ID = AHI_BestAudioIDA( tags );

D0                     A1

ULONG AHI_BestAudioIDA( struct TagItem * );

ID = AHI_BestAudioID( tag1, ... );

ULONG AHI_BestAudioID( Tag, ... );

FUNCTION

Determines the best AudioID to fit the parameters set in the tag

list.

INPUTS

tags - A pointer to a tag list. Only the tags present matter.

TAGS

Many combinations are probably stupid to ask for, like not supporting

panning or recording.

AHIDB_AudioID (ULONG) - The mode must use the same audio hardware

as this mode does.

AHIDB_Volume (BOOL) - If TRUE: mode must support volume changes.

If FALSE: mode must not support volume changes.

AHIDB_Stereo (BOOL) - If TRUE: mode must have stereo output.

If FALSE: mode must not have stereo output (=mono).

AHIDB_MultiChannel (BOOL) - If TRUE: mode must have 7.1 channel output.

If FALSE: mode must not have 7.1 channel output (=mono or stereo).

AHIDB_Panning (BOOL) - If TRUE: mode must support volume panning.

If FALSE: mode must not support volume panning.

AHIDB_HiFi (BOOL) - If TRUE: mode must have HiFi output.

If FALSE: mode must not have HiFi output.

AHIDB_PingPong (BOOL) - If TRUE: mode must support playing samples

backwards. If FALSE: mode must not support playing samples

backwards.

AHIDB_Record (BOOL) - If TRUE: mode must support recording. If FALSE:

mode must not support recording.

AHIDB_Realtime (BOOL) - If TRUE: mode must be realtime. If FALSE:

take a wild guess.

AHIDB_FullDuplex (BOOL) - If TRUE: mode must be able to record and

play at the same time.

AHIDB_Bits (UBYTE) - Mode must have greater or equal number of bits.

AHIDB_MaxChannels (UWORD) - Mode must have greater or equal number

of channels.

AHIDB_MinMixFreq (ULONG) - Lowest mixing frequency supported must be

less or equal.

AHIDB_MaxMixFreq (ULONG) - Highest mixing frequency must be greater

or equal.

AHIB_Dizzy (struct TagItem *) - This tag points to a second tag list.

After all other tags has been tested, the mode that matches these

tags best is returned, i.e. the one that has most of the features

you ask for, and least of the ones you don't want. Without this

second tag list, this function hardly does what its name

suggests. (V4)

RESULT

ID - The best AudioID to use or AHI_INVALID_ID if none of the modes

in the audio database could meet the requirements.

EXAMPLE

NOTES

BUGS

Due to a bug in the code that compared the boolean tag values in

version 4.158 and earlier, TRUE must be equal to 1. The bug is not

present in later revisions.

SEE ALSO

AHI_NextAudioID(), AHI_GetAudioAttrsA()

ahi.device/AHI_ControlAudioA

NAME

AHI_ControlAudioA -- change audio attributes

AHI_ControlAudio -- varargs stub for AHI_ControlAudioA()

SYNOPSIS

error = AHI_ControlAudioA( audioctrl, tags );

D0                         A2         A1

ULONG AHI_ControlAudioA( struct AHIAudioCtrl *, struct TagItem * );

error = AHI_ControlAudio( AudioCtrl, tag1, ...);

ULONG AHI_ControlAudio( struct AHIAudioCtrl *, Tag, ... );

FUNCTION

This function should be used to change attributes for a given

AHIAudioCtrl structure. It is also used to start and stop playback,

and to control special hardware found on some sound cards.

INPUTS

audioctrl - A pointer to an AHIAudioCtrl structure.

tags - A pointer to a tag list.

TAGS

AHIC_Play (BOOL) - Starts (TRUE) and stops (FALSE) playback and

PlayerFunc. NOTE: If the audio hardware cannot play at the same

time as recording samples, the recording will be stopped.

AHIC_Record (BOOL) - Starts (TRUE) and stops (FALSE) sampling and

RecordFunc. NOTE: If the audio hardware cannot record at the same

time as playing samples, the playback will be stopped.

AHIC_MonitorVolume (Fixed) - Sets the input monitor volume, i.e. how

much of the input signal is mixed with the output signal while

recording. Use AHI_GetAudioAttrsA() to find the available range.

AHIC_MonitorVolume_Query (Fixed *) - Get the current input monitor

volume. ti_Data is a pointer to a Fixed variable, where the result

will be stored.

AHIC_MixFreq_Query (ULONG *) - Get the current mixing frequency.

ti_Data is a pointer to an ULONG variable, where the result will

be stored.

AHIC_InputGain (Fixed) - Set the input gain. Use AHI_GetAudioAttrsA()

to find the available range. (V2)

AHIC_InputGain_Query (Fixed *) - Get current input gain. (V2)

AHIC_OutputVolume (Fixed) - Set the output volume. Use

AHI_GetAudioAttrsA() to find the available range. (V2)

AHIC_OutputVolume_Query (Fixed *) - Get current output volume. (V2)

AHIC_Input (ULONG) - Select input source. See AHI_GetAudioAttrsA().

(V2)

AHIC_Input_Query (ULONG *) - Get current input source. (V2)

AHIC_Output (ULONG) - Select destination for output. See

AHI_GetAudioAttrsA(). (V2)

AHIC_Output_Query (ULONG *) - Get destination for output. (V2)

The following tags are also recognized by AHI_ControlAudioA(). See

AHI_AllocAudioA() for what they do. They may be used from interrupts.

AHIA_SoundFunc (struct Hook *)

AHIA_PlayerFunc (struct Hook *)

AHIA_PlayerFreq (Fixed)

AHIA_RecordFunc (struct Hook *)

AHIA_UserData (APTR)

Note that AHIA_PlayerFreq must never be outside the limits specified

with AHIA_MinPlayerFreq and AHIA_MaxPlayerFreq! Also note that the

timing feature is designed to be used for music. When you change the

frequency, be reasonable. Using 50 Hz one moment and 5 the other is

to ask for trouble.

RESULT

An error code, defined in <devices/ahi.h>.

EXAMPLE

NOTES

The AHIC_Play and AHIC_Record tags *must not* be used from

interrupts.

BUGS

SEE ALSO

AHI_AllocAudioA(), AHI_GetAudioAttrsA(), <devices/ahi.h>

ahi.device/AHI_FreeAudio

NAME

AHI_FreeAudio -- deallocates the audio hardware

SYNOPSIS

AHI_FreeAudio( audioctrl );

A2

void AHI_FreeAudio( struct AHIAudioCtrl * );

FUNCTION

Deallocates the AHIAudioCtrl structure and any other resources

allocated by AHI_AllocAudioA(). After this call it must not be used

by any other functions anymore. AHI_UnloadSound() is automatically

called for every sound.

INPUTS

audioctrl - A pointer to an AHIAudioCtrl structure obtained from

AHI_AllocAudioA(). If NULL, this function does nothing.

EXAMPLE

NOTES

BUGS

SEE ALSO

AHI_AllocAudioA(), AHI_UnloadSound()

ahi.device/AHI_FreeAudioRequest

NAME

AHI_FreeAudioRequest -- frees requester resources

SYNOPSIS

AHI_FreeAudioRequest( requester );

A0

void AHI_FreeAudioRequest( struct AHIAudioModeRequester * );

FUNCTION

Frees any resources allocated by AHI_AllocAudioRequestA(). Once a

requester has been freed, it can no longer be used with other calls to

AHI_AudioRequestA().

INPUTS

requester - Requester obtained from AHI_AllocAudioRequestA(), or NULL

in which case this function does nothing.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AHI_AllocAudioRequestA()

ahi.device/AHI_GetAudioAttrsA

NAME

AHI_GetAudioAttrsA -- examine an audio mode via a tag list

AHI_GetAudioAttrs -- varargs stub for AHI_GetAudioAttrsA()

SYNOPSIS

success = AHI_GetAudioAttrsA( ID, [audioctrl], tags );

D0                            D0  A2           A1

BOOL AHI_GetAudioAttrsA( ULONG, struct AHIAudioCtrl *,

struct TagItem * );

success = AHI_GetAudioAttrs( ID, [audioctrl], attr1, &result1, ...);

BOOL AHI_GetAudioAttrs( ULONG, struct AHIAudioCtrl *, Tag, ... );

FUNCTION

Retrieve information about an audio mode specified by ID or audioctrl

according to the tags in the tag list. For each entry in the tag

list, ti_Tag identifies the attribute, and ti_Data is mostly a

pointer to a LONG (4 bytes) variable where you wish the result to be

stored.

INPUTS

ID - An audio mode identifier, AHI_DEFAULT_ID (V4) or AHI_INVALID_ID.

audioctrl - A pointer to an AHIAudioCtrl structure, only used if

ID equals AHI_INVALID_ID. Set to NULL if not used. If set to

NULL when used, this function returns immediately. Always set

ID to AHI_INVALID_ID and use audioctrl if you have allocated

a valid AHIAudioCtrl structure. Some of the tags return incorrect

values otherwise.

tags - A pointer to a tag list.

TAGS

AHIDB_Volume (ULONG *) - TRUE if this mode supports volume changes.

AHIDB_Stereo (ULONG *) - TRUE if output is in stereo. Unless

AHIDB_Panning (see below) is TRUE, all even channels are played

to the left and all odd to the right.

AHIDB_MultiChannel (ULONG *) - TRUE if output is in 7.1 channels.

AHIDB_Panning (ULONG *) - TRUE if this mode supports stereo panning.

AHIDB_HiFi (ULONG *) - TRUE if no shortcuts, like pre-division, is

used by the mixing routines.

AHIDB_PingPong (ULONG *) - TRUE if this mode can play samples backwards.

AHIDB_Record (ULONG *) - TRUE if this mode can record samples.

AHIDB_FullDuplex (ULONG *) - TRUE if this mode can record and play at

the same time.

AHIDB_Realtime (ULONG *) - Modes which return TRUE for this fulfills

two criteria:

1) Calls to AHI_SetVol(), AHI_SetFreq() or AHI_SetSound() will be

performed within (about) 10 ms if called from a PlayFunc Hook.

2) The PlayFunc Hook will be called at the specified frequency.

If you don't use AHI's PlayFunc Hook, you must not use modes that

are not realtime. (Criterium 2 is not that obvious if you consider

a mode that renders the output to disk as a sample.)

AHIDB_Bits (ULONG *) - The number of output bits (8, 12, 14, 16 etc).

AHIDB_MaxChannels (ULONG *) - The maximum number of channels this mode

can handle.

AHIDB_MinMixFreq (ULONG *) - The minimum mixing frequency supported.

AHIDB_MaxMixFreq (ULONG *) - The maximum mixing frequency supported.

AHIDB_Frequencies (ULONG *) - The number of different sample rates

available.

AHIDB_FrequencyArg (ULONG) - Specifies which frequency

AHIDB_Frequency should return (see below). Range is 0 to

AHIDB_Frequencies-1 (including).

NOTE: ti_Data is NOT a pointer, but an ULONG.

AHIDB_Frequency (ULONG *) - Return the frequency associated with the

index number specified with AHIDB_FrequencyArg (see above).

AHIDB_IndexArg (ULONG) - AHIDB_Index will return the index which

gives the closest frequency to AHIDB_IndexArg

NOTE: ti_Data is NOT a pointer, but an ULONG.

AHIDB_Index (ULONG *) - Return the index associated with the frequency

specified with AHIDB_IndexArg (see above).

AHIDB_MaxPlaySamples (ULONG *) - Return the lowest number of sample

frames that must be present in memory when AHIST_DYNAMICSAMPLE

sounds are used. This number must then be scaled by Fs/Fm, where

Fs is the frequency of the sound and Fm is the mixing frequency.

AHIDB_MaxRecordSamples (ULONG *) - Return the number of sample frames

you will receive each time the RecordFunc is called.

AHIDB_BufferLen (ULONG) - Specifies how many characters will be

copied when requesting text attributes. Default is 0, which

means that AHIDB_Driver, AHIDB_Name, AHIDB_Author,

AHIDB_Copyright, AHIDB_Version and AHIDB_Annotation,

AHIDB_Input and AHIDB_Output will do nothing.

AHIDB_Driver (STRPTR) - Name of driver (excluding path and

extension).

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen.

AHIDB_Name (STRPTR) - Human readable name of this mode.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen.

AHIDB_Author (STRPTR) - Name of driver author.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen.

AHIDB_Copyright (STRPTR) - Driver copyright notice.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen

AHIDB_Version (STRPTR) - Driver version string.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen.

AHIDB_Annotation (STRPTR) - Annotation by driver author.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen.

AHIDB_MinMonitorVolume (Fixed *)

AHIDB_MaxMonitorVolume (Fixed *) - Lower/upper limit for input

monitor volume, see AHI_ControlAudioA(). If both are 0.0,

the sound hardware does not have an input monitor feature.

If both are same, but not 0.0, the hardware always sends the

recorded sound to the outputs (at the given volume). (V2)

AHIDB_MinInputGain (Fixed *)

AHIDB_MaxInputGain (Fixed *) - Lower/upper limit for input gain,

see AHI_ControlAudioA(). If both are same, there is no input

gain hardware. (V2)

AHIDB_MinOutputVolume (Fixed *)

AHIDB_MaxOutputVolume (Fixed *) - Lower/upper limit for output

volume, see AHI_ControlAudioA(). If both are same, the sound

card does not have volume control. (V2)

AHIDB_Inputs (ULONG *) - The number of inputs the sound card has.

(V2)

AHIDB_InputArg (ULONG) - Specifies what AHIDB_Input should return

(see below). Range is 0 to AHIDB_Inputs-1 (including).

NOTE: ti_Data is NOT a pointer, but an ULONG. (V2)

AHIDB_Input (STRPTR) - Gives a human readable string describing the

input associated with the index specified with AHIDB_InputArg

(see above). See AHI_ControlAudioA() for how to select one.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen. (V2)

AHIDB_Outputs (ULONG *) - The number of outputs the sound card

has. (V2)

AHIDB_OutputArg (ULONG) - Specifies what AHIDB_Output should return

(see below). Range is 0 to AHIDB_Outputs-1 (including)

NOTE: ti_Data is NOT a pointer, but an ULONG. (V2)

AHIDB_Output (STRPTR) - Gives a human readable string describing the

output associated with the index specified with AHIDB_OutputArg

(see above). See AHI_ControlAudioA() for how to select one.

NOTE: ti_Data is a pointer to an UBYTE array where the name

will be stored. See AHIDB_BufferLen. (V2)

AHIDB_AudioID (ULONG *) - The ID for this mode. (V4)

If the requested information cannot be found, the variable will be not

be touched.

RESULT

TRUE if everything went well.

EXAMPLE

NOTES

BUGS

In versions earlier than 3, the tags that filled a string buffer would

not NULL-terminate the string on buffer overflows.

SEE ALSO

AHI_NextAudioID(), AHI_BestAudioIDA()

ahi.device/AHI_LoadSound

NAME

AHI_LoadSound -- prepare a sound for playback

SYNOPSIS

error = AHI_LoadSound( sound, type, info, audioctrl );

D0                     D0:16  D1    A0    A2

ULONG AHI_LoadSound( UWORD, ULONG, APTR, struct AHIAudioCtrl * );

FUNCTION

Defines an ID number for the sound and prepares it for playback.

INPUTS

sound - The numeric ID to be used as a reference to this sound.

The ID is a number greater or equal to 0 and less than what you

specified with AHIA_Sounds when you called AHI_AllocAudioA().

type - The type of the sound. Currently four types are supported:

AHIST_SAMPLE - array of 8 or 16 bit samples. Note that the

portion of memory where the sample is stored must NOT be

altered until AHI_UnloadSound() has been called! This is

because some audio drivers may wish to upload the samples

to local RAM. It is OK to read, though.

AHIST_DYNAMICSAMPLE - array of 8 or 16 bit samples, which can be

updated dynamically. Typically used to play data that is

loaded from disk or calculated realtime.

Avoid using this sound type as much as possible; it will

use much more CPU power than AHIST_SAMPLE on a DMA/DSP

sound card.

info - Depends on type:

AHIST_SAMPLE - A pointer to a struct AHISampleInfo, filled with:

ahisi_Type - Format of samples (four formats are supported).

AHIST_M8S: Mono, 8 bit signed (BYTEs).

AHIST_S8S: Stereo, 8 bit signed (2WORDs) (V4).

AHIST_M32S: Mono, 32 bit signed (LONGs). (V6)

AHIST_S32S: Stereo, 32 bit signed (2LONGs) (V6).

ahisi_Address - Address to the sample array.

ahisi_Length - The size of the array, in samples.

Don't even think of setting ahisi_Address to 0 and

ahisi_Length to 0xffffffff as you can do with

AHIST_DYNAMICSAMPLE! Very few DMA/DSP cards have 4 GB onboard

RAM...

AHIST_DYNAMICSAMPLE A pointer to a struct AHISampleInfo, filled

as described above (AHIST_SAMPLE).

If ahisi_Address is 0 and ahisi_Length is 0xffffffff

AHI_SetSound() can take the real address of an 8 bit sample

to be played as offset argument. Unfortunately, this does not

work for 16 bit samples.

audioctrl - A pointer to an AHIAudioCtrl structure.

RESULT

An error code, defined in <devices/ahi.h>.

EXAMPLE

NOTES

There is no need to place a sample array in Chip memory, but it

MUST NOT be swapped out! Allocate your sample memory with the

MEMF_PUBLIC flag set.

BUGS

AHIST_L7_1 can only be played using 7.1 audio modes -- it will NOT

be downmixed! It can't be played backwards either.

SEE ALSO

AHI_UnloadSound(), AHI_SetEffect(), AHI_SetFreq(), AHI_SetSound(),

AHI_SetVol(), <devices/ahi.h>

ahi.device/AHI_NextAudioID

NAME

AHI_NextAudioID -- iterate current audio mode identifiers

SYNOPSIS

next_ID = AHI_NextAudioID( last_ID );

D0                         D0

ULONG AHI_NextAudioID( ULONG );

FUNCTION

This function is used to iterate through all current AudioIDs in

the audio database.

INPUTS

last_ID - previous AudioID or AHI_INVALID_ID if beginning iteration.

RESULT

next_ID - subsequent AudioID or AHI_INVALID_ID if no more IDs.

EXAMPLE

NOTES

BUGS

SEE ALSO

AHI_GetAudioAttrsA(), AHI_BestAudioIDA()

ahi.device/AHI_PlayA

NAME

AHI_PlayA -- Start multiple sounds in one call (V4)

AHI_Play -- varargs stub for AHI_PlayA()

SYNOPSIS

AHI_PlayA( audioctrl, tags );

A2         A1

void AHI_PlayA( struct AHIAudioCtrl *, struct TagItem * );

AHI_Play( AudioCtrl, tag1, ...);

void AHI_Play( struct AHIAudioCtrl *, Tag, ... );

FUNCTION

This function performs the same actions as multiple calls to

AHI_SetFreq(), AHI_SetSound() and AHI_SetVol(). The advantages

of using only one call is that simple loops can be set without

using a SoundFunc (see AHI_AllocAudioA(), tag AHIA_SoundFunc) and

that sounds on different channels can be synchronized even when the

sounds are not started from a PlayerFunc (see AHI_AllocAudioA(), tag

AHIA_PlayerFunc). The disadvantage is that this call has more

overhead than AHI_SetFreq(), AHI_SetSound() and AHI_SetVol(). It is

therefore recommended that you only use this call if you are not

calling from a SoundFunc or PlayerFunc.

The supplied tag list works like a 'program'. This means that

the order of tags matter.

INPUTS

audioctrl - A pointer to an AHIAudioCtrl structure.

tags - A pointer to a tag list.

TAGS

AHIP_BeginChannel (UWORD) - Before you start setting attributes

for a sound to play, you have to use this tag to chose a

channel to operate on. If AHIP_BeginChannel is omitted, the

result is undefined.

AHIP_EndChannel (ULONG) - Signals the end of attributes for

the current channel. If AHIP_EndChannel is omitted, the result

is undefined. ti_Data MUST BE NULL!

AHIP_Freq (ULONG) - The playback frequency in Hertz or AHI_MIXFREQ.

AHIP_Vol (Fixed) - The desired volume. If omitted, but AHIP_Pan is

present, AHIP_Vol defaults to 0.

AHIP_Pan (sposition) - The desired panning. If omitted, but AHIP_Vol

is present, AHIP_Pan defaults to 0 (extreme left).

AHIP_Sound (UWORD) - Sound to be played, or AHI_NOSOUND.

AHIP_Offset (ULONG) - Specifies an offset (in samples) into the

sound. If this tag is present, AHIP_Length MUST be present too!

AHIP_Length (LONG) - Specifies how many samples that should be

played.

AHIP_LoopFreq (ULONG)

AHIP_LoopVol (Fixed)

AHIP_LoopPan (sposition)

AHIP_LoopSound (UWORD)

AHIP_LoopOffset (ULONG)

AHIP_LoopLength (LONG) - These tags can be used to set simple loop

attributes. They default to their sisters. These tags must be

after the other tags.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

AHI_SetFreq(), AHI_SetSound(), AHI_SetVol()

ahi.device/AHI_SampleFrameSize

NAME

AHI_SampleFrameSize -- get the size of a sample frame (V4)

SYNOPSIS

size = AHI_SampleFrameSize( sampletype );

D0                          D0

ULONG AHI_SampleFrameSize( ULONG );

FUNCTION

Returns the size in bytes of a sample frame for a given sample type.

INPUTS

sampletype - The sample type to examine. See <devices/ahi.h> for

possible types.

RESULT

The number of bytes, or 0 for invalid types.

EXAMPLE

NOTES

BUGS

This function returned trash for invalid sample types

before V6.

SEE ALSO

<devices/ahi.h>

ahi.device/AHI_UnloadSound

NAME

AHI_UnloadSound -- discard a sound

SYNOPSIS

AHI_UnloadSound( sound, audioctrl );

D0:16  A2

void AHI_UnloadSound( UWORD, struct AHIAudioCtrl * );

FUNCTION

Tells 'ahi.device' that this sound will not be used anymore.

INPUTS

sound - The ID of the sound to unload.

audioctrl - A pointer to an AHIAudioCtrl structure.

RESULT

EXAMPLE

NOTES

This call will not break a Forbid() state.

BUGS

SEE ALSO

AHI_LoadSound()

ahi.device/CMD_FLUSH

NAME

CMD_FLUSH -- Cancel all I/O requests (V4)

FUNCTION

Aborts ALL current requests, both active and waiting, even

other programs requests!

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_FLUSH

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

io_Actual       If io_Error is 0, number of requests actually

flushed.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

This command should only be used in very rare cases, like AHI

system utilities. Never use this command in an application.

BUGS

SEE ALSO

CMD_RESET, <ahi/devices.h>, <exec/errors.h>

ahi.device/CMD_READ

NAME

CMD_READ -- Read raw samples from audio input (V4)

FUNCTION

Reads samples from the users prefered input to memory. The sample

format and frequency will be converted on the fly.

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_READ

io_Data         Pointer to the buffer where the data should be put.

io_Length       Number of bytes to read, must be a multiple of the

sample frame size (see ahir_Type).

io_Offset       Set to 0 when you use for the first time or after

a delay.

ahir_Type       The desired sample format, see <ahi/devices.h>.

ahir_Frequency  The desired sample frequency in Hertz.

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

io_Actual       If io_Error is 0, number of bytes actually

transferred. Starting with V6, io_Actual is also

valid if io_Error is not 0 (like if the request

was aborted).

io_Offset       Updated to be used as input next time.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

It's only possible to read signed mono or stereo samples.

BUGS

SEE ALSO

<ahi/devices.h>, <exec/errors.h>

ahi.device/CMD_RESET

NAME

CMD_RESET -- Restore device to a known state (V4)

FUNCTION

Aborts all current requests, even other programs requests

(CMD_FLUSH), rereads the configuration file and resets the hardware

to its initial state

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_RESET

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

This command should only be used in very rare cases, like AHI

system utilities. Never use this command in an application.

BUGS

SEE ALSO

CMD_FLUSH, <ahi/devices.h>, <exec/errors.h>

ahi.device/CMD_START

NAME

CMD_START -- start device processing (like ^Q) (V4)

FUNCTION

All CMD_WRITE's that has been sent to the device since CMD_STOP

will be started at once, synchronized.

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_START

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

Unlike most (all?) other devices, CMD_STOP and CMD_START do nest in

ahi.device.

BUGS

SEE ALSO

CMD_STOP, <ahi/devices.h>, <exec/errors.h>

ahi.device/CMD_STOP

NAME

CMD_STOP -- stop device processing (like ^S) (V4)

FUNCTION

Stops all CMD_WRITE processing. All writes will be queued, and

are not processed until CMD_START. This is useful for synchronizing

two or more CMD_WRITE's.

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_STOP

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

This command affects ALL writes, even those sent by other

applications. Make sure the code between CMD_STOP and CMD_START

runs as fast as possible!

Unlike most (all?) other devices, CMD_STOP and CMD_START do nest in

ahi.device.

BUGS

SEE ALSO

CMD_START, <ahi/devices.h>, <exec/errors.h>

ahi.device/CMD_WRITE

NAME

CMD_WRITE -- Write raw samples to audio output (V4)

FUNCTION

Plays the samples to the users prefered audio output.

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      CMD_WRITE

io_Data         Pointer to the buffer of samples to be played.

io_Length       Number of bytes to play, must be a multiple of the

sample frame size (see ahir_Type).

io_Offset       Must be 0.

ahir_Type       The desired sample format, see <ahi/devices.h>.

ahir_Frequency  The desired sample frequency in Hertz.

ahir_Volume     The desired volume. The range is 0 to 0x10000, where

0 means muted and 0x10000 (== 1.0) means full volume.

ahir_Position   Defines the stereo balance. 0 is far left, 0x8000 is

center and 0x10000 is far right.

ahir_Link       If non-zero, pointer to a previously sent AHIRequest

which this AHIRequest will be linked to. This

request will be delayed until the old one is

finished (used for double buffering). Must be set

to NULL if not used.

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

io_Actual       If io_Error is 0, number of bytes actually

played. Starting with V6, io_Actual is also valid

if io_Error is not 0 (like if the request was

aborted).

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

32 bit samples (ahir_Type) is only available in V6 and later.

BUGS

SEE ALSO

<ahi/devices.h>, <exec/errors.h>

ahi.device/CloseDevice

NAME

CloseDevice -- Close the device

SYNOPSIS

CloseDevice(ioRequest)

A1

void CloseDevice(struct IORequest *);

FUNCTION

This is an exec call that closes the device. Every OpenDevice()

must be matched with a call to CloseDevice().

The user must ensure that all outstanding IO Requests have been

returned before closing the device.

INPUTS

ioRequest - a pointer to the same struct AHIRequest that was used

to open the device.

RESULT

EXAMPLE

NOTES

BUGS

SEE ALSO

OpenDevice(), exec.library/CloseDevice()

ahi.device/NSCMD_DEVICEQUERY

NAME

NSCMD_DEVICEQUERY -- Query the device for its capabilities (V4)

FUNCTION

Fills an initialized NSDeviceQueryResult structure with

information about the device.

IO REQUEST INPUT

io_Device       Preset by the call to OpenDevice().

io_Unit         Preset by the call to OpenDevice().

io_Command      NSCMD_DEVICEQUERY

io_Data         Pointer to the NSDeviceQueryResult structure,

initialized as follows:

DevQueryFormat - Set to 0

SizeAvailable  - Must be cleared.

It is probably good manners to clear all other

fields as well.

io_Length       Size of the NSDeviceQueryResult structure.

IO REQUEST RESULT

io_Error        0 for success, or an error code as defined in

<ahi/devices.h> and <exec/errors.h>.

io_Actual       If io_Error is 0, the value in

NSDeviceQueryResult.SizeAvailable.

The NSDeviceQueryResult structure now contains valid information.

The other fields, except io_Device, io_Unit and io_Command, are

trashed.

EXAMPLE

NOTES

BUGS

SEE ALSO

<ahi/devices.h>, <exec/errors.h>

ahi.device/OpenDevice

NAME

OpenDevice -- Open the device

SYNOPSIS

error = OpenDevice(AHINAME, unit, ioRequest, flags)

D0                 A0       D0    A1         D1

BYTE OpenDevice(STRPTR, ULONG, struct AHIRequest *, ULONG);

FUNCTION

This is an exec call.  Exec will search for the ahi.device, and

if found, will pass this call on to the device.

INPUTS

AHINAME - pointer to the string "ahi.device".

unit - Either AHI_DEFAULT_UNIT (0), AHI_NO_UNIT (255) or any other

unit the user has requested, for example with a UNIT tooltype.

AHI_NO_UNIT should be used when you're using the low-level

API.

ioRequest - a pointer to a struct AHIRequest, initialized by

exec.library/CreateIORequest(). ahir_Version *must* be preset

to the version you need!

flags - There is only one flag defined, AHIDF_NOMODESCAN, which

asks ahi.device not to build the audio mode database if not

already initialized. It should not be used by applications

without good reasons (AddAudioModes uses this flag).

RESULT

error - Same as io_Error.

io_Error - If the call succeeded, io_Error will be 0, else

an error code as defined in <exec/errors.h> and

<devices/ahi.h>.

io_Device - A pointer to the device base, which can be used

to call the functions the device provides.

EXAMPLE

NOTES

BUGS

SEE ALSO

CloseDevice(), exec.library/OpenDevice(), <exec/errors.h>,

<devices/ahi.h>.