Elentari Demo Board

Functional Specification

Prepared by

BCT Systems

Release Notes

20-Feb-95, Initial Release

- Open Items:

  - Connector pin assignments for connection to Hiroshige or Harunobu, OR should one of these interface IC’s be included on the board?

  - Need to include ISA pin assignments.

  - Add software requirements.

- Should we consider replacing D connectors with headers that can connect to cable mounted D connectors? This might make installation in a chassis easier.

- The manual power switch for the LCD drive circuitry might cause more problems than it solves. The control circuitry will have to keep all logic and supplies off when the switch is off and then sequence them once the switch is turned on. The mechanical switch can’t take advantage of software to sequence the power to the panel.

7-Mar-95, ver 0.2

- Software Requirements placed in separate document.

- ISA connector pin assignments added.

- ISA bus limitation documented.

- Use jumper for power to LCD circuitry and panel, instead of switch. An LED will be provided to indicate LCD power on.

- Use miniature DIN connector for keyboard interface (PS/2 style keyboard)

11-Mar-95, ver 0.3

- Changed FLASH from 32-pin to surface mount  high density devices.

- Added A/D converter for digitizer input.

- Removed serial port connction to digitizer ASIC.

- Added requirement for 8-bit software configuration jumper.

11-Mar-95, ver 1.0

- Changed Supoer I/O to PC87303.

- Changed FLASH to 48 pin TSOP with 5 volt only programming.

- Changed bank 0 DRAM from SIMM to discrete, SMT devices to meet Elentari capacitive drive.

- Added interrupt request to keypad, documented that it is MICROWIRE™.

- Connected keyboard interface to a secondary interrupt input.

- Changed audio timer to general purpose timer 1.

- ISA: Included support for ISA ENDXFR/ signal, documented non-support of BALE.

- Revamped interrupt assignments.

- Revamped DMA assignments.

- Added PCMCIA interface to system memory map.

- Added Section 3, Scanner and Laser Printer Interfaces.

Table of Contents

1. Introduction

2. Functional Specifications

2.1 Functional Block Diagram

2.2 System Memory Map

2.3 Memory

2.3.1 Microsoft At Work Memory Elements

2.3.1.1 System ROM

2.3.1.2 System RAM

2.3.1.3 System NV-RAM

2.3.2 Other Memory Elements

2.3.2.1 EEPROM

2.4 LCD Display.

2.5 PCMCIA Slot

2.6 ECP Parallel Port

2.7 EIA-232 & Infrared Serial Port

2.7.1 SIR Interface

2.7.2 EIA-232 Interface

2.8 Keypad

2.9 LEDs

2.9.1 Debug UART

2.9.2 Keyboard Interface

2.9.3 Floppy Disk Interface

2.9.4 IDE Hard Disk Interface

2.10 Audio Output

2.11 Digitizer Analog Input

2.12 Software configuration jumpers

2.13 Standard and High Speed ISA Interface

2.13.1 Interrupt Structure

2.13.2 DMA Structure

2.14 Power Requirements

3. Add-in Interface Boards

3.1 Scanner Interface

3.1.1 Overview

3.1.2 Scanner software interface

3.1.3 Interface connector pin assignments

3.2 Laser Printer Interface

3.2.1 Overview

3.2.2 LPI Specifications

3.2.3 Interface connector pin assignments

Elentari Demo Board

Preliminary Functional Specification

# Introduction

This document is a functional specification for the Evaluation/Demo Board featuring the Elentari Chip (see Figure 1) in a real-life application.  The board will be used as an evaluation platform for the early sample silicon, the target hardware for National’s port of the Microsoft At WorkÔ Hardware Adaptation Layer (HAL), a demonstration of the Microsoft At WorkÔ Fax, and a functional Winpad device.

The requirements for the demo board were set forth in the Elentari Demo Board, Requirements v0.7. Other than that requirements document, the major source of definition for the demo board was the Microsoft At Work FAX System Adaptation Kit Beta 3 Release, Volume 3: Technical Reference Documents.  Chapter 2 of the details the Hardware Architecture Specification. This Functional Specification was based on the requirements set forth in these two documents.

Figure 1. Elentari Block Diagram.

# Functional Specifications

## Functional Block Diagram

The major blocks of the Elentari Demo Board are shown in the functional block diagram in Figure 2.

System Memory Map

Figure 2. Elentari Demo Board Functional Block Diagram.

DRAM SIMM Bank 1

0-8 Megabytes

Battery Backed SRAM

LED Indicators (8)

20 Key Keypad

DRAM SIMM Bank 0

0.5-8 Megabytes

EPROM

0-8 Megabytes

FLASH

0-4 Megabytes

POWER

CLOCK

RESET

BATTERY

MICRO-WIRE™

A/D for Digitizer

PCMCIA

Type II Socket

ISA Bus Slots

Standard

&

High Speed

1     2     3     4

Super I/O

PC87303

Keyboard Interface

Floppy Disk Interface

IDE Interface

Debug Serial I/O

IEEE 1284 / ECP

Parallel Port

+32V Supply

LCD Bias Gen.

LCD panel I/F

Serial Infrared

or

EIA-232D I/F

DAC

&

Audio Amplifier

MICRO-WIRE™

EEPROM

Cascaded

Interrupt

Controller

The allocation of the external system memory space is shown in the address map in Table 1.

Table 1. Demo Board System Memory Map.

| Starting Address (hex) | Ending Address (hex) | Size | Description |
| --- | --- | --- | --- |
| 00000000 | 00FFFFFF | 16M | System RAM |
| 01000000 | 03FFFFFF | 48M | Unused |
| 04000000 | 04FFFFFF | 16M | Standard ISA |
| 05000000 | 05FFFFFF | 16M | High speed ISA |
| 06000000 | 0603FFFF | 256K | System NV-RAM |
| 06040000 | 06FFFFFF |  | Unused |
| 07000000 | 07FFFFFF | 16M | PCMCIA Interface |
| 08000000 | FEFFFFFF |  | Unused |
| FF000000 | FF7FFFFF | 8M | System secondary ROM (EPROM or FLASH) |
| FF800000 | FFFFFFFF | 8M | System boot ROM (EPROM or FLASH) |

## Memory

### Microsoft At Work Memory Elements

System ROM

Table 2. System ROM Jumper Configuration.

| Jumper Description | Jumper Selections |
| --- | --- |
|  | 29F100 (64K by 16) |
| FLASH device selection | 29F200 (128K by 16) |
|  | 29F400 (256K by 16) |
|  | 29F800 (512K by 16) |
|  | 27C010 (128K by 8) |
| EPROM device selection | 27C020 (256K by 8) |
|  | 27C040 (512K by 8) |
|  | 27C080 (1024K by 8) |
| Upper Group Type (boot) | FLASH |
|  | EPROM |

The System ROM for Microsoft At Work is implemented as a two arrays of devices, one for FLASH and the other for EPROM. The board contains a total of eight sockets to support a maximum of eight megabytes of EPROM and two locations for word-wide surface mount devices for a maximum of four megabytes of FLASH. The EPROM sockets are organized as four banks of two devices each to provide a 16-bit wide data path for the Elentari system bus.

Each 32-pin DIP socket accepts a standard, JEDEC EPROM device. All EPROM  devices must be the same size. The FLASH devices are 48 pin thin small outline (TSOP) surface mount devices that are soldered on the board. The devices, available from AMD or Intel, are programmed using only the 5V supply. The board can be populated with zero, one or two FLASH devices. If there are two FLASH devices, they must be of the same size. Either the EPROM or FLASH devices can be the boot ROM. Table 2 outlines the jumper configurations for the system ROM.As long as the same size devices are used for all banks, the entire EPROM address space will be continguous, with the highest address of the EPROM space ending at 0ffffffffh. The highest address bank within the EPROM space is called the boot bank. A single set of jumpers select the size of all EPROM sockets, so all sockets must have the same size devices. It is therefore recommended that the same size EPROM devices be used in all sockets. However, it is possible to mix sizes provided the jumpers are set to select the largest size EPROM devices installed. Although this is not generally recommended, since the smaller devices will not provide contiguous EPROM, it sometimes does make sense to mix EPROM sizes. One possible reason to use this approach is to have a small diagnostic boot EPROM bank and larger devices in the other banks.

The boot ROM is the ROM that resides at the highest address in the ELENTARI memory space. The boot ROM occupies contiguous address space at the top of system memory (ending at 0ffffffffh). This boot ROM can be either EPROM or FLASH. The devices that are not boot ROM will occupy contiguous address space beginning at address FF000000. The CS0/ output is used to enable all system ROM (boot and non-boot) and must therefore be programmed to cover the entire system ROM address space, i.e. FF000000-FFFFFFFF. Decode for the individual devices is provided in demo board logic.

#### System RAM

The Demo Board is equipped with discrete SMT, DRAM devices soldered on the board for bank 0 and SIMM sockets for bank 1. The discrete DRAM and the two SIMM sockets can provide the full capability of 16Mbyte of memory. The discrete DRAM can consist of six 1 Meg by 4 devices to provide a base RAM size of 2 megabytes or six 4 Meg by 4 devices for 8 megabytes. The default configuration will be 8 megabytes. Industry standard, low-cost DRAM SIMM modules are used. The SIMM modules must be three-chip devices (to comply with Elentari DRAM control drive capability). Parity is supported so users should normally use nine-bit wide SIMMs, as Microsoft At Work requires single bit error detection in the system RAM.

The SIMM sockets support 256K by 9, 1M by 9, or 4M by 9 SIMMs. The two sockets must have the same sized SIMMs and must be less than or equal in memory size to bank 0. The diagnostic firmware sizes the system RAM and programs the Elentari to map the system RAM into contiguous address space beginning at 0h. Table 3 summarizes all possible system RAM configurations supported by the demo board.

Table 3. System RAM Configuration.

| Size | Bank 0 Devices | Bank 1 SIMMs | Address Range |
| --- | --- | --- | --- |
| 2 Meg | 1M by 4 | None | 0h-001FFFFFh |
| 2.5 Meg | 1M by 4 | 256K by 9 | 0h-0027FFFFh |
| 4 Meg | 1M by 4 | 1M by 9 | 0h-003FFFFFh |
| 8 Meg | 4M by 4 | None | 0h-007FFFFFh |
| 8.5 Meg | 4M by 4 | 256K by 9 | 0h-0087FFFFh |
| 10 Meg | 4M by 4 | 1M by 9 | 0h-009FFFFFh |
| 16 Meg | 4M by 4 | 4M by 9 | 0h-00FFFFFFh |

#### System NV-RAM

The system NV-RAM consists of 256 kilobytes of battery-backed, static RAM. The static RAM is organized as two, 128K by 8 static RAM devices to form a 128K by 16 bit memory interfaced to the Elentari system bus.

A NiCad battery is used to maintain the contents of the system NV-RAM when system power is removed. While system power is applied, the NiCad battery is trickle charged. The battery circuitry is designed so that the contents of the system NV-RAM is maintained for 90 days (assuming the battery is fully charged when the system power is removed).

### Other Memory Elements

EEPROM

Table 4. Serial EEPROM Devices

| EEPROM Device | Size (in bits) |
| --- | --- |
| NM93C06 | 256 |
| NM93C46 | 1024 |
| NM93C56 | 2048 |
| NM93C66 | 4096 |

A MICROWIREÔ bus interface serial EEPROM is included for storage of configuration information on the demo board. The demo board interfaces to the EEPROM via the Elentari MICROWIREÔ port. The board is socketed to use any one of the National Semiconductor NM93C series EEPROM devices. The board can therefore accommodate 256, 1024, 2048 or 4096 bits of EEPROM storage by selection of the appropriate device. Table 4 summarizes the device part numbers and their respective sizes.

LCD Display.

Table 5. LCD Display Panel Pin Assignments.

| Pin | I/O | Signal | Description |
| --- | --- | --- | --- |
| 1 |  | V5 | Bias Voltage 3 (1/15 VEE) |
| 2 |  | V2 | Bias Voltage 3 (14/15 VEE) |
| 3 |  | VEE | LCD Power Supply (21-31V) |
| 4 |  | VDD | Logic Power |
| 5 | O | CLF | Frame start signal (FRAME) |
| 6 |  | VGND | LCD Ground return |
| 7 | O | CL1 | Row Clock (LOAD) |
| 8 |  | VSS | Ground |
| 9 | O | DF | Alternates with frame, for AC bias |
| 10 | O | D_OFF/ | Display off |
| 11 | O | CL2 | Dot Clock (CP) |
| 12 |  | V4 | Bias Voltage 4 (2/15 VEE) |
| 13 |  | V3 | Bias Voltage 3 (13/15 VEE) |
| 14 | O | D3 | Data bit 3 |
| 15 | O | D2 | Data bit 2 |
| 16 | O | D1 | Data bit 1 |
| 17 | O | D0 | Data bit 0 |
| 18 |  | - | No Connect |

The Elentari LCD control circuitry supports either the ALPS 324 by 240 or 480 by 320 LCD display. The board includes a DC-DC converter to generate the high voltage (approx. 21V-31V) for the LCD display. A potentiometer to adjust the precise voltage and thus the optimum viewing cone of the panel is included on the board. Circuitry on the demo board divides and buffers this reference voltage to provide the various bias supplies required by the display. A software controlled switch provides correct power sequencing to the LCD. Additionally, a jumper provides redundant power control as extra protection against damage to the panel. Installation of the jumper and presence of LCD power will be indicated by an LED on the demo board. A 1 mm pitch receptacle for flex PCB edge connection is provided to interface to the LCD panel. The pin assignments for the interface connector to the LCD panel are shown in Table 5.

## PCMCIA Slot

A single PCMCIA type II socket is supported on the demo board. This socket complies to PCMCIA Card Standard, Release 2.1. In addition to the capabilities provided by the Elentari PCMCIA interface circuitry, the demo board socket supports the following features:

- Hot insertion / removal of PCMCIA cards (though data integrity is not guaranteed by PCMCIA if removal occurs during operation of the card).

- Switched VCC for software control of power to the socket.

- Switched VPP for software control of the programming voltage. Supports 0, 5V and 12V for programming PCMCIA FLASH cards.

The PCMCIA connector is mounted on the demo board and provides guides to aid inserting PCMCIA cards. The pin assignments for the PCMCIA connector are shown in Table 6.

Table 6. PCMCIA Connector Pin Assignments

| Pin | Signal | I/O | Function |  | Pin | Signal | I/O | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GND |  | Ground |  | 35 | GND |  | Ground |
| 2 | D3 | I/O | Data bit 3 |  | 36 | CD1 | I | Card Detect |
| 3 | D4 | I/O | Data bit 4 |  | 37 | D11 | I/O | Data Bit 11 |
| 4 | D5 | I/O | Data bit 5 |  | 38 | D12 | I/O | Data Bit 12 |
| 5 | D6 | I/O | Data bit 6 |  | 39 | D13 | I/O | Data Bit 13 |
| 6 | D7 | I/O | Data bit 7 |  | 40 | D14 | I/O | Data Bit 14 |
| 7 | CE1 | O | Card Enable |  | 41 | D15 | I/O | Data Bit 15 |
| 8 | A10 | O | Address bit 10 |  | 42 | CE2 | O | Card Enable |
| 9 | OE | O | Output Enable |  | 43 | RFSH | O | Refresh |
| 10 | A11 | O | Address bit 11 |  | 44 | RFU - IORD |  | Reserved - IO Read |
| 11 | A9 | O | Address bit 9 |  | 45 | RFU - IOWR |  | Reserved - IO Write |
| 12 | A8 | O | Address bit 8 |  | 46 | A17 | O | Address bit 17 |
| 13 | A13 | O | Address bit 13 |  | 47 | A18 | O | Address bit 18 |
| 14 | A14 | O | Address bit 14 |  | 48 | A19 | O | Address bit 19 |
| 15 | WE/PGM | O |  |  | 49 | A20 | O | Address bit 20 |
| 16 | RDY/BSY - IREQ | I | Ready/busy - Interrupt Request |  | 50 | A21 | O | Address bit 21 |
| 17 | VCC |  | Card Supply Voltage |  | 51 | VCC |  | Card Supply Voltage |
| 18 | VPP1 |  | Programming and Peripheral Supply 1 |  | 52 | VPP2 |  | Programming and Peripheral Supply 2 |
| 19 | A16 | O | Address bit 16 |  | 53 | A22 | O | Address bit 22 |
| 20 | A15 | O | Address bit 15 |  | 54 | A23 | O | Address bit 23 |
| 21 | A12 | O | Address bit 12 |  | 55 | A24 | O | Address bit 24 |
| 22 | A6 | O | Address bit 7 |  | 56 | A25 | O | Address bit 25 |
| 23 | A7 | O | Address bit 6 |  | 57 | RFU |  | Reserved |
| 24 | A5 | O | Address bit 5 |  | 58 | RESET | O | Card Reset |
| 25 | A4 | O | Address bit 4 |  | 59 | WAIT | I | Extend bus cycle |
| 26 | A3 | O | Address bit 3 |  | 60 | RFU - INPACK | I | Reserved - Input Port Acknowledge |
| 27 | A2 | O | Address bit 2 |  | 61 | REG | O | Register select & IO Enable |
| 28 | A1 | O | Address bit 1 |  | 62 | BVD2 - SPKR | I | Battery voltage detect 2 - Audio Digital Waveform |
| 29 | A0 | O | Address bit 0 |  | 63 | BVD1 - STSCHG | I | Battery voltage detect 1 - Card Status’s Changed |
| 30 | D0 | I/O | Data bit 0 |  | 64 | D8 | I/O | Data bit 8 |
| 31 | D1 | I/O | Data bit 1 |  | 65 | D9 | I/O | Data bit 9 |
| 32 | D2 | I/O | Data bit 2 |  | 66 | D10 | I/O | Data bit 10 |
| 33 | WP - IOIS16 | I | Write Protect - IO Port is 16-bit |  | 67 | CD2 | I | Card detect |
| 34 | GND |  | Ground |  | 68 | GND |  | Ground |

## ECP Parallel Port

The demo board has an IEEE 1284 compliant extended capabilities parallel port (ECP). This uses the Elentari ECP circuitry connected to a standard 25-pin D-type female connector. Appropriate buffering, filter, and signal shaping components are included in the interface. The pin out of the ECP port is shown in Table 7.

Table 7. ECP connector pin out.

| Pin | Signal | I/O |  | Pin | Signal | I/O |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | STB/ | I/O |  | 25 | GROUND |  |
| 2 | PD0 | I/O |  | 24 | GROUND |  |
| 3 | PD1 | I/O |  | 23 | GROUND |  |
| 4 | PD2 | I/O |  | 22 | GROUND |  |
| 5 | PD3 | I/O |  | 21 | GROUND |  |
| 6 | PD4 | I/O |  | 20 | GROUND |  |
| 7 | PD5 | I/O |  | 19 | GROUND |  |
| 8 | PD6 | I/O |  | 18 | GROUND |  |
| 9 | PD7 | I/O |  | 17 | SLCTIN/ | I/O |
| 10 | ACK/ | I |  | 16 | INIT/ | I/O |
| 11 | BUSY | I |  | 15 | FAULT/ | I |
| 12 | PE | I |  | 14 | ATFD/ | I/O |
| 13 | SLCT | I |  | - |  |  |

## EIA-232 & Infrared Serial Port

The Elentari on-chip NS16550 UART supports either EIA-232 interface levels or Serial Infrared (SIR) mode. The demo board has both types of interface circuitry. Configuration jumpers select which interface is used.

### SIR Interface

An infrared LED and receiver are included on the board for infrared remote control application.

### EIA-232 Interface

Because the Elentari MICROWIREÔ is used on the demo board, the CTS/, DCD/ and RI signals are not available for the EIA-232 interface. Only the RTS/, DTR/ and DSR/ control signals plus the data are available for the interface. Interface circuitry on the demo board translates the signal levels from the Elentari voltage levels to the EIA-232 interface levels. The interface is a data terminal equipment (DTE) interface using a 9 pin, male, D type connector. The pin assignments are shown in Table 8

Table 8. Elentari UART EIA-232 Connector Pin Assignments.

| Pin | I/O | Signal | Description |
| --- | --- | --- | --- |
| 1 | I | DCD | Data Carrier Detect (not implemented) |
| 2 | I | RXD | Received Data |
| 3 | O | TXD | Transmitted Data |
| 4 | O | DTR | Data Terminal Ready |
| 5 |  | GND | Signal Ground |
| 6 | I | DSR | Data Set Ready |
| 7 | O | RTS | Ready to Send |
| 8 | I | CTS | Clear to Send (not implemented) |
| 9 | I | RI | Ring Indicator (not implemented) |

## Keypad

A 20 key keypad is mounted on the demo board. The keyboard is a Grayhill series 88, 5 by 4 key, snap dome contact, keypad. The keypad has no legends. An assortment of printed legend labels are supplied for customization by the user. The keypad is mounted directly to the demo board with four studs. The keyboard is approximately 3.2” wide by 3.0” high.

The keypad interface circuitry on the demo board scans the keypad for switch closures, filters key bounce and presents the results to the Elentari. A valid key press detection will generate an interrupt request to the Elentari. The interrupt request is connected through a jumper to ISA bus IRQ12. The data interface for the keypad is a read-only MICROWIRE™ interface.

## LEDs

Eight discrete LED indicators are present on the demo board. These LEDs are directly controllable via software from the Elentari. The LEDs are placed near the keypad so they can be used as indicators for operation in conjunction with the keypad.

## Super I/OÔ Peripherals

A PC87303VUL Super I/OÔ device is included to interface to several additional peripherals. This includes a debug UART, a standard PS/2 keyboard, a floppy disk and an IDE hard drive. The peripherals can use up to five interrupts on the ISA bus. Jumpers are provided for isolating interrupts if the peripheral is not used as well as configuring which interrupt to use for the UART channels. See the following sections for details.

Debug UART

Table 9. Debug UART Pin Assignments.

| Pin | I/O | Signal | Description |
| --- | --- | --- | --- |
| 1 | I | DCD | Data Carrier Detect |
| 2 | I | RXD | Received Data |
| 3 | O | TXD | Transmitted Data |
| 4 | O | DTR | Data Terminal Ready |
| 5 |  | GND | Signal Ground |
| 6 | I | DSR | Data Set Ready |
| 7 | O | RTS | Ready to Send |
| 8 | I | CTS | Clear to Send |
| 9 | I | RI | Ring Indicator |

The debug UART will be the primary interface for software diagnostic and test firmware. This will use standard EIA-232 signal levels. The full EIA-232D signal complement for a 9 pin connector is provided in the male, D-type 9 pin connector. The pin assignments are shown in Table 9. This UART can be jumper configured for use as COM1, COM2, or COM3. Additionally, the interrupt request can be selected to use ISA bus IRQ4 or IRQ3.

Keyboard Interface

Table 10. Keyboard Connector Pin Assignments.

| Pin | I/O | Signal | Description |
| --- | --- | --- | --- |
| 1 | I/O | +DATA | Keyboard Data |
| 2 |  |  |  |
| 3 |  | GND | Ground |
| 4 |  | +5 | +5 Volt Supply |
| 5 | I/O | +CLK | Keyboard Clock |
| 6 |  |  |  |

A standard PS/2 style keyboard interface is provided on the demo board. The connector is a 6-pin, miniature female DIN connector located in the standard AT keyboard location for connection when the demo board is in an AT compatible chassis. The pin assignments for the keyboard are shown in Table 10. The keyboard interface interrupt request is hard wired to the secondary interrupt controller channel 7.

### Floppy Disk Interface

The demo board includes a floppy disk controller (FDC) with a standard 34-pin DIP header for interface to one or more floppy disk drives. The pin assignments for the FDC interface are shown in Table 11. The FDC can optionally use ISA IRQ6 and can also optionally connect to the ISA bus DMA channel 2.

Table 11. Floppy Disk Controller Connector Pin Assignments.

| Pin | Signal |  | Pin | I/O | Signal | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Ground |  | 2 | O | DENSEL | Density Select |
| 3 | Ground |  | 4 | - | - | No Connect |
| 5 | Ground |  | 6 | O | DRATE0 | Data Rate 0 |
| 7 | Ground |  | 8 | I | INDEX/ | Index |
| 9 | Ground |  | 10 | O | MTR0/ | Motor Select 0 |
| 11 | Ground |  | 12 | O | DR1/ | Drive Select 1 |
| 13 | Ground |  | 14 | O | DR0/ | Drive Select 0 |
| 15 | Ground |  | 16 | O | MTR1/ | Motor Select 1 |
| 17 | Ground |  | 18 | O | DIR/ | Direction |
| 19 | Ground |  | 20 | O | STEP/ | Step |
| 21 | Ground |  | 22 | O | WDATA/ | Write Data |
| 23 | Ground |  | 24 | O | WGATE/ | Write Gate |
| 25 | Ground |  | 26 | I | TRK0/ | Track 0 |
| 27 | Ground |  | 28 | I | WP/ | Write Protect |
| 29 | Ground |  | 30 | I | RDATA/ | Read Data |
| 31 | Ground |  | 32 | O | HDSEL/ | Head Select |
| 33 | Ground |  | 34 | I | DSKCHG/ | Disk Change |

### IDE Hard Disk Interface

Use of  one or two hard drives is supported by the integrated drive electronics (IDE) interface. The demo board has a forty-pin, DIP header for interface to a standard IDE ribbon cable connection to the hard drive. The pin assignments for the IDE connector are shown in Table 12. A two pin jumper for connection to an LED indicates IDE disk activity. The IDE can optionally use ISA bus IRQ14.

Table 12. IDE Connector Pin Assignments.

| Pin | I/O | Signal | Description |  | Pin | I/O | Signal | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | O | RESET/ | Reset Drive |  | 2 |  | GND |  |
| 3 | I/O | IDED7/ | IDE Data bit 7 |  | 4 | I/O | D8 | Data bit 8 |
| 5 | I/O | D6 | Data bit 6 |  | 6 | I/O | D9 | Data bit 9 |
| 7 | I/O | D5 | Data bit 5 |  | 8 | I/O | D10 | Data bit 10 |
| 9 | I/O | D4 | Data bit 4 |  | 10 | I/O | D11 | Data bit 11 |
| 11 | I/O | D3 | Data bit 3 |  | 12 | I/O | D12 | Data bit 12 |
| 13 | I/O | D2 | Data bit 2 |  | 14 | I/O | D13 | Data bit 13 |
| 15 | I/O | D1 | Data bit 1 |  | 16 | I/O | D14 | Data bit 14 |
| 17 | I/O | D0 | Data bit 0 |  | 18 | I/O | D15 | Data bit 15 |
| 19 |  | GND | Ground |  | 20 | - | - | No pin key |
| 21 |  | - | No connect |  | 22 |  | GND | Ground |
| 23 | O | IOW/ | I/O Write Strobe |  | 24 |  | GND | Ground |
| 25 | O | IOR/ | I/O Read Strobe |  | 26 |  | GND | Ground |
| 27 |  | - | No connect |  | 28 | O | BALE | Buffered Address Latch Enable |
| 29 |  | - | No connect |  | 30 |  | GND |  |
| 31 | I | IRQ14 | Interrupt Request 14 |  | 32 | I | IOCS16/ | I/O Chip Select 16 |
| 33 | O | A1 | Address bit 1 |  | 34 |  | - | No connect |
| 35 | O | A0 | Address bit 0 |  | 36 | O | A2 | Address bit 2 |
| 37 | O | HCS0/ | Hard Drive Chip Select 0 |  | 38 | O | HCS1/ | Hard Drive Chip Select 1 |
| 39 | I | ACT/ | Activity (LED) |  | 40 |  | GND | Ground |

## Audio Output

A simple audio interface is implemented on the demo board. An eight-bit digital to analog converter (DAC) drives a low power audio amplifier connected to an on board speaker. Elentari timer 1 along with general purpose timer pin T1 can be used with an available DMA channel to provide DMA driven audio capability.

## Digitizer Analog Input

Interface circuitry for a resistive digitizer pad is included on the demo board. The circuitry provides drive and measurement circuitry to a digitizer tablet with a resistance value between 200 and 600 ohms. The circuitry supports determination by the ELENTARI of absence or presence of a pen depression on the tablet. If a depression is present, the X and Y location is resolved to 1 part in 1024. The analog to digital convertor used for the measurement interfaces to the ELENTARI using the MICROWIRE™ interface. Interface to the tablet is through a four pin flex edge connector. The pin assignments for this connector are shown in Table 13.

Table 13. Digitizer tablet connector pin assignments

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | X1 | Horizontal resistive film terminal 1 |
| 2 | Y1 | Vertical resistive film terminal 1 |
| 3 | X2 | Horizontal resistive film terminal 2 |
| 4 | Y2 | Vertical resistive film terminal 2 |

## Software configuration jumpers

Eight bits of software definable configuration jumpers are provided. These jumpers will be used for miscellaneous software configuration options, such as memory speed, wait states, etc.

## Standard and High Speed ISA Interface

Four 16-bit Industry Standard Architecture (ISA) bus slots are included on the demo board. Although only four DMA channels are available, the slots support most standard 8-bit and 16-bit memory and I/O cards. By doubling the address space allocated to the ISA bus interface, each slot is able to support either standard speed ISA operation or high-speed Elentari native mode operation. The slots thus support a 16-bit local bus that provides higher than the normal ISA bus throughput but uses the same form factor, connectors and signals so that prototyping hardware is readily available. Additional circuitry on the demo board controls the Elentari RDY signal to better support the ISA ENDXFR/ and extend the timing to safely support 8-bit cards.

The high-speed ISA is used to implement the high performance DMA required for laser printer engine raster scan drive as well as for high speed scanner input. The standard speed ISA can be used for FAX/modem cards, display cards, LAN cards or many other standard ISA cards that might be useful for Microsoft At Work product development.

The demo board ISA slots do not support the following standard ISA capabilities:

- MEMORY/ signal is not supported. The signal is driven unasserted (high).

- Temporary masters are not supported. The MASTER/ signal is ignored.

- The BALE (address latch enabled) is not supported. The signal is always asserted.

The pin assignments for the ISA bus slots are shown in Table 14 and Table 15.

Table 14. ISA Bus P1 Pin Assignments

| Pin | Signal | I/O | Function |  | Pin | Signal | I/O | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B01 | 0V |  | Ground |  | A01 | IOCHK/ | I | Parity Error |
| B02 | RESETDRV | O | Reset |  | A02 | SD7 | I/O | Data bit 7 |
| B03 | +5V |  | +5 Volt Power |  | A03 | SD6 | I/O | Data bit 6 |
| B04 | IRQ9 | I | Interrupt Request 9 |  | A04 | SD5 | I/O | Data bit 5 |
| B05 | -5V |  |  |  | A05 | SD4 | I/O | Data bit 4 |
| B06 | DRQ2 | I | DMA Request 2 |  | A06 | SD3 | I/O | Data bit 3 |
| B07 | -12V |  | -12 Volt Power |  | A07 | SD2 | I/O | Data bit 2 |
| B08 | ENDXFR/ | I | End Transfer |  | A08 | SD1 | I/O | Data bit 1 |
| B09 | +12V |  | +12 Volt Power |  | A09 | SD0 | I/O | Data bit 0 |
| B10 | 0V |  | Ground |  | A10 | IOCHRDY | I | I/O or memory ready |
| B11 | SMEMW/ | O | Memory Write Strobe |  | A11 | AEN | O | DMA Active |
| B12 | SMEMR/ | O | Memory Read Strobe |  | A12 | SA19 | O | Address 19 |
| B13 | IOW/ | O | I/O Write Strobe |  | A13 | SA18 | O | Address 18 |
| B14 | IOR/ | O | I/O Read Strobe |  | A14 | SA17 | O | Address 17 |
| B15 | DACK3/ | O | DMA Acknowledge 3 |  | A15 | SA16 | O | Address 16 |
| B16 | DRQ3 | I | DMA Request 3 |  | A16 | SA15 | O | Address 15 |
| B17 | DACK1/ | O | DMA Acknowledge 1 |  | A17 | SA14 | O | Address 14 |
| B18 | DRQ1 | I | DMA Request 1 |  | A18 | SA13 | O | Address 13 |
| B19 | REFRESH/ | O | unasserted, logic high |  | A19 | SA12 | O | Address 12 |
| B20 | CLK | O | SYSCLK divided by 3 |  | A20 | SA11 | O | Address 11 |
| B21 | IRQ7 | I | Interrupt Request 7 |  | A21 | SA10 | O | Address 10 |
| B22 | IRQ6 | I | Interrupt Request 6 |  | A22 | SA9 | O | Address 9 |
| B23 | IRQ5 | I | Interrupt Request 5 |  | A23 | SA8 | O | Address 8 |
| B24 | IRQ4 | I | Interrupt Request 4 |  | A24 | SA7 | O | Address 7 |
| B25 | IRQ3 | I | Interrupt Request 3 |  | A25 | SA6 | O | Address 6 |
| B26 | DACK2/ | O | DMA Acknowledge 2 |  | A26 | SA5 | O | Address 5 |
| B27 | TC | O | DMA Terminal Count |  | A27 | SA4 | O | Address 4 |
| B28 | BALE | O | Address Latch Enable (+5V) |  | A28 | SA3 | O | Address 3 |
| B29 | +5V |  | +5 Volt Power |  | A29 | SA2 | O | Address 2 |
| B30 | OSC | O | 14.31818 MHz. |  | A30 | SA1 | O | Address 1 |
| B31 | 0V |  | Ground |  | A31 | SA0 | O | Address 0 |

Table 15. ISA Bus P2 Pin Assignments

| Pin | Signal | I/O | Function |  | Pin | Signal | I/O | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D01 | MEMCS16/ | I | Memory 16-bit chip select |  | C01 | SBHE/ | O | Bus High Enable |
| D02 | IOCS16/ | I | I/O 16-bit chip select |  | C02 | LA23 | O | Address 23 |
| D03 | IRQ10 | I | Interrupt Request 10 |  | C03 | LA22 | O | Address 22 |
| D04 | IRQ11 | I | Interrupt Request 11 |  | C04 | LA21 | O | Address 21 |
| D05 | IRQ12 | I | Interrupt Request 12 |  | C05 | LA20 | O | Address 20 |
| D06 | IRQ15 | I | Interrupt Request 13 |  | C06 | LA19 | O | Address 19 |
| D07 | IRQ14 | I | Interrupt Request 14 |  | C07 | LA18 | O | Address 18 |
| D08 | DACK0/ | O | DMA Acknowledge 0 |  | C08 | LA17 | O | Address 17 |
| D09 | DRQ0 | I | DMA Request 0 |  | C09 | MEMR/ | O | Memory Read Strobe |
| D10 | DACK5/ | O | DMA Acknowledge 5 |  | C10 | MEMW/ | O | Memory Write Strobe |
| D11 | DRQ5 | I | DMA Request 5 |  | C11 | SD8 | I/O | Data bit 8 |
| D12 | DACK6/ | O | DMA Acknowledge 6 |  | C12 | SD9 | I/O | Data bit 9 |
| D13 | DRQ6 | I | DMA Request 6 |  | C13 | SD10 | I/O | Data bit 10 |
| D14 | DACK7/ | O | DMA Acknowledge 7 |  | C14 | SD11 | I/O | Data bit 11 |
| D15 | DRQ7 | I | DMA Request 7 |  | C15 | SD12 | I/O | Data bit 12 |
| D16 | +5V |  | +5 Volt Power |  | C16 | SD13 | I/O | Data bit 13 |
| D17 | MASTER/ |  | Unused |  | C17 | SD14 | I/O | Data bit 14 |
| D18 | 0V |  | Ground |  | C18 | SD15 | I/O | Data bit 15 |

Interrupt Structure

Table 16. ISA to Elentari Interrupt Mappings.

| ISA bus Interrupt | Elentari Interrupt | Cascade PIC Channel |
| --- | --- | --- |
| IRQ3 | E_IRQ1 | N/A |
| IRQ4 | E_IRQ2 | N/A |
| IRQ5 | E_IRQ3 | N/A |
| IRQ6 | E_IRQ4 | N/A |
| IRQ7 | E_IRQ0 | 0 |
| IRQ9 | E_IRQ0 | 1 |
| IRQ10 | E_IRQ0 | 2 |
| IRQ11 | E_IRQ0 | 3 |
| IRQ12 | E_IRQ0 | 4 |
| IRQ14 | E_IRQ0 | 5 |
| IRQ15 | E_IRQ0 | 6 |

An AT compatible ISA bus supports thirteen interrupts. The Elentari only supports 6 external interrupts. Additionally, the Super I/O™ peripherals can use up to five of the ISA bus interrupts (see section 2.10). In order to support the widest possible combination of on board and add-in card usage, the demo boards supports full interrupt capability on the ISA bus. This is accomplished by using a programmable interrupt controller (PIC) external to the Elentari. The PIC uses Elentari E_IRQ0 to add eight additional interrupt request lines for the ISA bus. Table 16 shows the interrupt mapping from ISA bus to the Elentari interrupt inputs.

### DMA Structure

The ISA bus supports a total of 8 DMA channels. ISA channel 4, which is the DMA cascade channel is not supported. Therefore, only seven DMA channels connections are supported by the demo board. Since there are only four Elentari DMA channels available external to the chip, the DMA channels are shared on the ISA bus as shown in Table 19. Additionally, Elentari DMA channel 4 (ISA channel 2) is used by the Floppy Disk controller, so an isolation jumper is provided to permit use of this channel on the ISA bus when the floppy controller is not used

## Power Requirements

Power is supplied to the demo board through two, six pin Molex connectors similar to the AT compatible standard power connectors. This permits use of standard AT compatible power supplies for operation of the board. The board only requires +5V, +12V and -12V for operation, but -5V is supplied by a standard power supply. The -5V is routed to the ISA bus for use by plug-in cards. Pin assignments for the two connectors, PS-8 and PS-9, are shown in Table 17 and Table 18.

Table 17. PS-8 Power Supply Connection

| Pin | Signal |
| --- | --- |
| 1 | Pwr Good |
| 2 | +5V |
| 3 | +12V |
| 4 | -12V |
| 5 | Ground |
| 6 | Ground |

Table 18. PS-9 Power Supply Connection

| Pin | Signal |
| --- | --- |
| 1 | Ground |
| 2 | Ground |
| 3 | -5V |
| 4 | +5V |
| 5 | +5V |
| 6 | +5V |

Table 19. DMA Channel Assignments.

| ISA DMA Channel | Elentari DMA channel | Isolation Jumpers |
| --- | --- | --- |
| 0 | 0 | No |
| 1 | 2 | No |
| 2 | 4 | Yes |
| 3 | 6 | No |
| 5 | 0 | No |
| 6 | 2 | No |
| 7 | 6 | No |

# Add-in Interface Boards

## Scanner Interface

### Overview

The scanner interface is a custom ISA adapter board that interfaces the demo board to a ScanFX scanner. The ScanFX is a simple multi-page, color scanner with keyboard and LCD display supplied by Plustek, Inc.. Plustek also supplies an interface board with the scanner. The custom interface board is a superset of the Plustek supplied interface board. Both the custom interface board and standard ScanFX interface board provide programmed I/O eight bit command and data interface to the scanner. The custom interface board additionally supports eight-bit DMA of scan data from the scanner to the demo board DRAM. This section provides the details of the custom interface board.

### Scanner software interface

The scanner software interface consists of two basic interfaces: the programmed I/O and the DMA. The programmed I/O takes eight contiguous I/O port addresses. The base address of the I/O port address is set by three jumpers to provide eight possible locations. The possible base address settings are selected by jumpers to any one of the following addresses: 220H, 230H, 260H, 270H, 320H, 330H, 360H, or 370H. The interface ports accessed by the software are shown in  Table 20.

Table 20. Scanner Interface Port Definitions

| Port Address | Description |
| --- | --- |
| base + 0 | Data port |
| base + 1 | Command port for write Status port for read |
| base + 3 | Clear Interrupt Request |

An interrupt request signal is jumper selectable from the scanner interface. The following interrupt request signals can be used: IRQ3, IRQ4, IRQ5, IRQ10, IRQ11 or IRQ12.

Jumpers are provided to select which (if any) DMA channel to use. Consistent with standard ISA conventions, only the lower four DMA channels are used for the eight-bit DMA operations. The possible DMA channels that can be used are thus channel 0, 1, 2 or 3. When enabled, the scanner SCANACK  initiates DMA reads from the scanner interface Data port.

### Interface connector pin assignments

Interface to the scanner is through a high density DB-26 connector. The pin configuration of this connector is shown in Table 21.

Table 21. Scanner Interface Connector Pin Assignments.

| Pin | I/O | Signal |  | Pin | I/O | Signal |  | Pin | I/O | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I/O | DATA6 |  | 10 |  | +12 V |  | 19 |  | Ground |
| 2 |  | - |  | 11 | I | SCANACK |  | 20 | I/O | DATA3 |
| 3 | I/O | Data 7 |  | 12 |  | Ground |  | 21 |  | Ground |
| 4 |  | +12 V |  | 13 |  | +5 V |  | 22 | I/O | DATA4 |
| 5 | O | REQUEST |  | 14 | I/O | DATA0 |  | 23 |  | Ground |
| 6 |  | +12 V |  | 15 |  | +5 V |  | 24 | I/O | DATA5 |
| 7 | O | RD_/WR |  | 16 | I/O | DATA1 |  | 25 |  | Ground |
| 8 |  | +12 V |  | 17 |  | +5 V |  | 26 |  | Ground |
| 9 | O | C_/D |  | 18 | I/O | DATA2 |  | - |  |  |

## Laser Printer Interface

### Overview

The laser printer interface (LPI) is a command/control and programmable video interface to a laser printer engine. It is an ISA form-factor compatible board that uses the demo board high-speed ISA interface, though the board is capable of operating in a standard speed ISA slot.

The command/control interface is a low-speed, synchronous serial interface to the print engine. The LPI has parallel to serial and serial to parallel conversion registers for the command / control interface. The Elentari, therefore, writes commands to an eight-bit port, waits for the command to be sent to the engine and then reads the resulting status response returned from the engine by reading an eight-bit status port. An interrupt request can be asserted by the LPI when the status response is available so that the Elentari need not do a polled or programmatic wait for the status response.

The video interface is a 16-bit DMA FIFO interface. Although the average video data transfer rate should not be a problem for the Elentari DMA bus bandwidth, the peak rates on high-speed, high-density laser printer engines will put too stringent a requirement on DMA latency to be practical. (The speed of the Elentari DMA has sufficient speed margin to support fast video rates, but other devices in the system could cause the latency to be the speed limiting factor). Fortunately, a substantial portion of the horizontal trace time is required for non-video transfer (beam re-trace, overhead, etc.) Therefore, a single scan line buffer reduces the peak DMA data rate requirement to the average video data rate. A 512 by 16-bit FIFO on the LPI provides adequate buffering for up to eleven inch wide paper at 600 dots per inch (dpi). The LPI is intended to use Elentari single or demand transfer with buffer chaining. The LPI, along with the FIFO status signals, manages the DMA requests so that the Elentari Byte Count Register can be programmed for up to 64 Kilobytes of DMA transfers between CPU interventions. DMA terminal count (TC) present on a LPI DMA cycle will generate an LPI interrupt request.

The position and width of the laser printer raster scan is controlled by three counters on the LPI. A programmable register is provided for each counter so that the registers only need to be set once and the counters will be automatically reloaded as required. The register/counters perform the following functions:

- Top Margin - One counter controls the number of Beam Detect (BD/) occurrences after the top of the page (VSREQ/ or TOP/) until video data will begin being shifted out. This is an eight bit counter that can provide a top margin of from 1 to 255 beam detect signals.

- Left Margin - One counter controls the time from the left margin (occurrence of Beam Detect)  until the start of video data output for each scan line. This is an eight bit counter that sets the left edge delay in increments of two pixels. Thus the left margin delay can be from 2 to 510 pixels in width.

- Scan width - One counter controls the width of the video data output for each scan line in increments of 16 pixels, i.e. in the number of 16-bit words to be shifted from the FIFO to the video data stream. This is an eight bit counter, so that the number of words per line can be set from 1 to 510.

Control of the printer control signals, printer status signals and interrupt configuration are managed by the printer command and status registers (note these are printer command and status registers in contrast to the engine command and status registers described above). The command register in this interface controls the printer control signals PRNT/ (PRiNT) and CPRDY/ (Controller Power ReaDY). The status register reports the printer status signals PPRDY/ (Printer Power ReaDY), RDY/ (ReaDY) and either VSREQ/ (Video Sync REQuest) or TOP/ (Top Of Page), depending on which print engine is used. The printer command / status interface also controls the enabling of interrupt requests, interrupt acknowledgment and interrupt identification. In addition to the interrupt request from the engine status response and the DMA terminal count, assertion of VSREQ/ (or TOP/) can cause an interrupt request to be asserted.

The programmed I/O for the LPI takes eight contiguous I/O port addresses. The base address of the I/O port address is set by three jumpers to provide eight possible locations. The possible base address settings are selected by jumpers to any one of the following addresses: 220H, 230H, 260H, 270H, 320H, 330H, 360H, or 370H. The interface ports accessed by the software are shown in Table 22.

An interrupt request signal is jumper selectable from the LPI. The following interrupt request signals can be used: IRQ10, IRQ11 or IRQ12.

Table 22. LPI Port Definitions

| Port Address | Size | Operation | Description |
| --- | --- | --- | --- |
| base + 0 | 8-bits | Write | Printer Command Register |
|  |  | Read | Printer Status Register |
| base + 1 | 8-bits | Write | Engine Control Command |
|  |  | Read | Engine Control Status |
| base + 2 | 8-bits | Write | Video Top Margin Register |
| base + 3 | 8-bits | Write | Video Left Margin Register |
| base + 4 | 16-bits | Write | Video Data FIFO input (also target for 16-bit DMA) |
| base + 6 | 16-bits | Write | Scan Width Register |

Jumpers are provided to select which DMA channel to use. Consistent with standard ISA conventions, only the upper DMA channels are used for the sixteen-bit DMA operations. The possible DMA channels that can be used are thus channel 5, 6 or 7.

### LPI Specifications

A summary of the important specifications for the Laser Printer Interface is contained in Table 23.

Table 23. Laser Printer Interface Specifications.

| Description | Specification |
| --- | --- |
| Maximum paper width | 11 inches |
| Maximum engine speed | 8 ppm |
| Maximum resolution | 600 dpi |
| Maximum top margin | 255 lines |
| Maximum left margin | 510 pixels |
| Maximum line scan width | 8,160 pixels |
| Maximum video data rate | 8 Megahertz |
| Maximum left margin variation | 1/8 pixel |
| Maximum demand DMA data rate to FIFO | 25 Megabytes per second |

### Interface connector pin assignments

The LPI connector is a 37-pin, female, DB type connector. The pin assignments for this connector are shown in Table 24.

Table 24. Laser Printer Interface  Pin Assignments.

| Pin | I/O | Signal | Description |  | Pin | I/O | Signal | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | - | Reserved |  | 20 |  | - | Reserved |
| 2 | I | BD/ | Beam Detect/ |  | 21 | I | BD | Beam Detect |
| 3 | I | PPRDY | Printer Power ReaDY/ |  | 22 |  | GND | Return |
| 4 | I | RDY/ | printer ReaDY/ |  | 23 |  | GND | Return |
| 5 | I | VSREQ/ TOP/ | Vertical Sync REQuest/ or Top Of Page/ |  | 24 |  | GND | Return |
| 6 | I | SBSY/ | Status BuSY/ |  | 25 |  | GND | Return |
| 7 | I/O | SC/ | Status Command/ |  | 26 |  | GND | Return |
| 8 | I/O | SCLK/ | Serial Clock/ |  | 27 |  | GND | Return |
| 9 | O | CBSY/ | Command BuSY/ |  | 28 |  | GND | Return |
| 10 | O | PRNT/ | PRiNT/ |  | 29 |  | GND | Return |
| 11 |  | - | Reserved |  | 30 |  | GND | Return |
| 12 | O | CPRDY | Controller Power Ready |  | 31 |  | GND | Return |
| 13 | O | VDO/ | Video Data Out/ |  | 32 | O | VDO | Video Data Out |
| 14 |  | - | Reserved |  | 33 | - | Reserved |  |
| 15 |  | - | Reserved |  | 34 | - | Reserved |  |
| 16 |  | - | Reserved |  | 35 | - | Reserved |  |
| 17 |  | - | Reserved |  | 36 | - | Reserved |  |
| 18 |  | - | Reserved |  | 37 | - | Reserved |  |
| 19 |  | - | Reserved |  |  |  |  |  |