OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	October 11, 1996

TO:	OMCO Business Unit

CC:	iPG / OPSD BIOS Engineering

FROM:	iPG / OPSD BIOS Engineering

SUBJECT:	AP440FX Standard BIOS Beta 4.01

About This Release

o	BIOS 0.01.04.CT1 Beta 4.01 - AP440FX  Standard

Features/Errata Fixed In This Release

Beta 4.01

- Updated AP440FX DMI Information.

- Fix LDCM problems in Memory Upgrade Option, Cache Status, # of PCI/ISA Slots

- PCI Latency Timer Fix

- Include a Date/Time Stamp in Flash

- Include Intel Copyright Information

- Fix VPD Display problem when invalid strings in Flash

Release 3.0

- Banner change from Beta 3.01

Beta 3.01

- Added support for Pentium Pro BIOS Update INT 15 Interface.  This supports the Intel utility that will detect the stepping of the Pentium Pro processor installed and will update the BIOS with a BIOS Update Patch to match.

- Enhanced the DMI Event Logging to include the logging of POST Device Initializer Manager (DIM) errors.

- Added support for setting the ISA VGA frame buffer region to a new Pentium Pro memory type known as “Write Combine” (WC or USWC).

- Added support for Misty BIOS Recovery.

- Enhanced the PCI Latency Timer programming to allow the intelligent programming of PCI devices.

- Updated the basic Chipset register settings for the 440FX and the PIIX3 to match the latest recommended setting from PCD.  Enabled refresh queue, set multi-transaction timer to 20h, set deturbo mode register to 0D0h.

- Added support for displaying the motherboard id and serial number at boot time.

Release 2.0

- Banner change from Beta 2.02

Beta 2.02

- Removed initialization of Heceta Head Hardware monitor and SMM logging support. The part is still programmed to use I/O address 290h but any initialization and setup will need to be done through an external driver or LDCM software.

- Removed support for Trend Anti Virus support including SMM logging support. This may be put back in at a later date once it is decided to be stable.

- Fixed mouse devnode problem where if no mouse was installed devnode 0 which is the PIC is not assigned any resources. No real world problem sighted but simulated test indicated error.

- Fixed problem where ECC error detection would not function if APM was not enabled in setup. Moved enabling of EXTSMI signal to later in post.

- Fixed setup question for ISA LFB showing possible hole sizes of 2Mb and 4Mb. 1Mb hole should be only option.

- Updated DMI info strings for accuracy on this board.

- Changed Slot 3 on the riser card IRQ routing table information to fix bug found with SCSI cards using this slot.

- Fixed problem with Multi bit errors not being logged or reporting type in NMI

- Fixed INT15 cache services bug.

Beta 2.01

- Updated S3 video bios to latest version

- Changes for Win95 suppliment logo acceptance test

- Made all ports indicate 16 bit decode

- Added non-standard I/O address selections 228 for LPT in ECP, EPP modes.

- Fixed a floppy problem where when disabled returned consuming IRQ0

- Fixed issue with the video having the IRQ routed to INTD of the PIIX3 rather than

INTC in which it was connected.

- Ported over to new BIOS code base including the following:

- Graphical setup interface with extended graphical logo support

- More modular and easier to customize

- Added setup questions for USB, Video IRQ, and Heceta hardware monitor. The USB and Heceta questions should only show up if the part is stuffed.

- Added device node for Heceta motherboard device consuming I/O address space 290-297.

- Added Trend Anti virus boot support in BIOS with setup question.

- Added new more extensive event logging support code including logging for Post errors, ECC, Virus detection, and Heceta hardware monitor system events.

- DMI updates for clearing event log added

- Fix for Crystal 4232 coming up disabled in Win95

Release 1.0

- Disabled USB through PIIX3 register

- Added mouse disabling through Keyboard controller enhancement for Microsoft certification.

Beta 1.09

- Fixed an issue where the SMM data area was not cleared keeping recovery from working. Also the PCI access routines were in the wrong place when linked.

Beta 1.08

- Fixed issue with new Manufacturing defaults being overridden with old values.

Beta 1.07

- Fixed problem with 3Com 3C590 card losing its Device ID register contents.  The fix was to perform a double reset upon power-up.

- Updated manufacturing defaults to the latest specification.

- Fixed problem with Audio field in setup disappearing.

- Fixed problem with PnP cards not being recognized/configured when on-board audio was disabled.

- Changed the default value of DENSEL to be set to fix problems with OSes that do not use BIOS Runtime INT 13 interface.

- Fixed problem of disabled devices indicating they are still using resources when in fact, they are not. (Ex. IRQ0 instead of No IRQ).

- Added new algorithm to detect processor speed.

- Added APM 1.2 support.

- Added Secure Flash implementation.

- Added DMI Event Logging capability.

- Added a page to the Setup for Event Log.

- Enabled USB support.

- Masked problem with XcelleNet RemoteWare V1.42.  A key pressed on the host system causes a software generated INT 9 on the target system.   The software correctly places the key pressed by the host system in the keyboard buffer by an INT 16 function 5.  The program should not fake a keyboard hardware interrupt on a system that didn’t have the key pressed.  Depending upon value in port 60h a bogus key could be seen.

- Added support to allow for option ROMs greater than 64M.

- Fixed problem where a PCI card without an ESCD entry was disabled if the previous card/function was disabled.

- Updated DMI BIOS information (TYPE 0) to indicated the BIOS is not socketed, and PCMCIA boot is not supported.

- Added the latest 4236 Crystal Micro-code, and resources.  Resource Version CSC0B36, and Micro-code version BET035B.

- Updated Static DMI information to identify the L2 cache correctly.

- Changed the priority of the resource allocation so mouse IRQ is not allocated unless we are out of resources.

Beta 1.06

- Added cpyright.bin strings making backward compatibility for secure flash

Beta 1.05

- Replaced Audio resource file for Crystal 4232 (wrong file put into 1.04)

Beta 1.04

- Time/date setup fix

- New Crystal 4232 resource code

- Fix for ATAPI devices on secondary channel causing detection problems

- DMI fix for SET DMI STRUCTURE command

- Adaptec ultra wide fix with 512k base memory  in setup fix

- APM manufacturing defaults

- PnPBIOS test with mouse problem fix

- CPU 166Mhz display in setup fix

- 308 resources showing up in ICU as motherboard resources fix

Beta 1.03 Fab C

- Fixed Crystal  4232 Win95 Logo and Win96 compliance issue.

- Added code to support both S3 Virge/V+ 50Ns EDO Video DRAM in the same BIOS.

- Fixed problem with Int 15 AX=DA01h to return proper status when not supported.

- Added capability to get information about BIOS update code specific to the processor.

- Added mechanism to prevent Win95 from enabling a disabled node.

- Fixed a problem with ECP was not being enabled when in Manual mode.

- Fixed a problem where some PCI video cards would not work behind a bridge.  This fix also fixed problems with video cards with built-in bridge.

- Fixed problem where mouse node set to non-configurable when mouse present.

- Removed IR field from Setup since the support is not finalized.

- Fixed problem with Setup hanging when parallel port was set to Port 278h, and ECP mode.

- Rewrote the memory sizing algorithm to support maximum size of memory that Chipset support.

- Fixed problem with ECC code, which was creating an NMI when a single bit error was produced.

- Fixed problem with static version of memory node.

- Added code to support both Crystal 4232/4236 in the same BIOS.

- Moved ISA Shared Memory field from Advanced Peripherals page to PnP page.

- Added  support to release IRQ14/15 to the PnP Screen when the respective IDE controller is disabled.

- Enabled APM on default.

- Added a temporary fix for Adaptec 2940UW card.

- Added Win NT DMI interface.

- Added support in DMI for 256 Byte CMOS GPNV area.

- Added support in DMI for revised security scheme for CMOS GPNV Access.

- Added support in DMI for Generic SIMM reference designators.

- Added support in DMI for Generic CPU reference designators.

- Added support for Auto Start on AC Loss field in the APM Setup page.

- Added support for Power-On COM1 Ring field in the APM Setup page.

- Added Speaker control field in the Boot Options page in setup.

- Fixed problem with Parallel port not being programmed properly for ECP mode.

- Added a work-around for Pentium-Pro CPU to work properly with SMM.

- Fixed problem with ECC single bit errors hanging the system.

- Added all the latest and greatest fixes and updates to the APM code.

- Added support for Soft Off so the system shuts the power supply down after Win95 shuts down.

- Changed the Intel Corporation’s Copy Right to 1996 (X03)

- Fixed programming of PIIX IDE channels with Mode 4 drives. (X03)

- Changed the size of BI? Files to include the Parameter Block 2.  (X03)

- Provided a mechanism to prevent Win95 from enabling a disabled node. (X03)

- Fixed problem in which ESCD Getting trashed if a multifunction PCI card could not be configured.  Failed PCIPost  Test 0Dh.  (X03)

- Added code to clear system RAM just prior to boot.  Non-Clear RAM caused NOVELL UNIX NETWARE to not boot properly.  (X03)

- Fixed problem with PIIX3 IDE resources were not being released when disabled.

- Changed peripheral Page in setup to correctly display the current status of peripherals and also changed the fields to infirmative.

- Added Audio field in the Advanced Setup page in Setup to be able to Enable/Disable the CS4232 Chip.

- Fixed problem with APM not being disabled when set to disabled in Setup.

- Added Natl 308 support specific to the Performance AP platform

- Added DRAM type info bytes in setup

- Fix for TestView and mouse installed

- Preliminary Flash Virus Protection

- BEDO support with delay to support 66Mhz

- Initial PCI IDE Enumeration code

Beta 1.02

- Fixed S-step Pentium Pro patch support

- Added BEDO support in memory detection and recovery

- MTRR programming corrected for non power of 2 memory combinations .

- PIIX3 Slave IDE timing reversal fixed

- New C0 P6 patch included

- 256k vs 512k cache problem corrected

- Added S-step Pentium Pro patch support.

- Changed the attribute for the F000 block to Read only when in normal mode.

- Fixed problem with DRAM Row Type register being programmed inappropriately.

- Changed the DRAM timing to EDO=2-2-2 and FPM=3-3-3.

- Fixed a problem with IDE Slave timing register in PIIX3 was not being enabled.

- Added a work-around to fix the way TEAC 6X CD-56E buffer is displayed.  This is a problem with the drive firmware.

- Added Fault Tolerance code to protect us from losing non-volatile information during a power surge or accidental power down.

- Fixed bug where memory allocated to the Device Initialization Manager was not being de-allocated under all conditions.

- Increased loop count in the Int 13h service to fix the problem with Int 13h function 14h.

- Fixed code overrun so Int 14h function 5h (Extended communication port Control) wouldn’t get over-written.

- Fixed buf in node attribute check in runtime PnP Setnode function.

- Obtained more space in the recovery code by removing unnecessary code.

- Added User Flash Scan Capability for user code to be scanned before video initialization.

- Changed CMOS Error Reporting sequence when in manufacturing mode.

- Paragraph Aligned the protected mode PnP Service Entry Point.

- Enabled APM support in the Setup

- Added BIOS Version display in setup utility.

- Added the new Microcode that Crystal send us to swap the DMA channels.

Beta 1.01

- Removed  unnecessary setup options for ISA LFB because Mars only supports 15M-16M memory hole.

- Added support for programming P6 fixed MTRR register for shadowed option ROMS in core.

- Added bug fix for DRAM type programming to support single density SIMMs  correctly.

- Added code fix to correctly load the P6 B0 and C0 support code

- Enhanced the MTRR programming for 15m-16m memory hole

- Changed defaults for PMC registers according to PCD recommendations

- Included  latest DIM code for enabling ICU functionality

- Changed the C800-DFFF PAM register defaults so that ISA shared memory will now work correctly.

Alpha 1.01.09

- Fixed a problem where the pre-delay questions were added but the core file to call the proper routines was not linked into the BIOS for Alpha 1.01.08.

- The MTRR  P6 cache register programming was updated to fix a bug  for  Variable MTRR register 0 and also reflect the memory hole programming for a 15M-16M memory hole correctly.

Alpha 1.01.08

- ISA Shared Memory questions added to setup

- APM bug fixed and is should now be working

- Changed the Chipset default values for RCD bit to enabled and cache line size to 2 lines

- Added the Pre-Delay question back into setup

- Added PIIX3 support

Alpha 1.01.07

- Corrected problem with IRQ routing where IRQ15 was the only IRQ available

- Made sure the CMOS jumper was correctly clearing ESCD NVRAM area

Alpha 1.01.06

- Added  new and improved IDE initialization code.

- Added Predelay field to the setup utility.

- Made sure all the PCD BIOS Errata list is included.

- Included all Rudy Nelson’s register recommendations.

- Added Audio informative field in the Setup Utility to signify Audio is Enabled or Disabled.

- Fixed problem with Processor Speed not being displayed correctly in the Setup Utility.

Alpha 1.01.05

- Fixed the problem with the Recovery BIOS.  (NOTE: All ROM devices will have to be reprogrammed for this feature to be included; FMUP will not include this fix.)

- Cleaned up the memory sizing code to make it work with All the memory configurations.

- Fixed the problem with the reset after FMUP was performed.

- Added code to make sure that the A20 mask was being set properly before when enabled and disabled.

- Changed the default value for the CPU Latency register to comply with the Sighting report 1014.

Alpha 1.01.04

- Removed In-Order Queue option from the setup utility.

- Removed Deferred Queue option from the setup utility.

- Added capability to dynamically detect  EDO SIMMs .

- Removed the patches for Axx versions of the Pentium® Pro processor.

- Enabled all postings in the Mars Chipset.  (Register 53h).

- Removed APM setup options from setup since it has not been debugged yet.

Any releases previous to this were done for initial bootup

Setup Changes

Beta 1.01

- Removed unnecessary options for ISA LFB questions to eliminate 2m and 4m from the list.  Mars only supports a hole at 15m-16m

Password Changes

Beta 1.01

- None.

Language Support

Beta 1.01

- US language support only.

Hard Disk Changes

Beta 1.01

- None

Plug and Play (Windows 95, etc.)

Beta 1.01

- None

APM 1.1

Beta 1.01

- No changes

New Int 15h Extensions

Beta 1.01

- None.

Other Changes

Beta 1.01

- None

Known Errata With This Release

Beta 1.01

- Audio problems using DMA channel 0/1 with Crystal 4232

Features Still To Be Added / Comments About This Release

Beta 1.01

- None