Z8TBASIC (Z80 TINYBASIC) Version 2.0 by Don McKENZIE  UPDATED 29-August-93(c)

***************************************************************************

Z8TBASIC get up and go instructions, for impatient types like me...

To get Z8TBASIC running from a standard ZLOAD Version 2.0, and burn results

to an EPROM only, Basic Interpreter operating system, do the following:

(1)  Make 3 wire DB-25M dummy plug as described in text, insert into DB-25F.

(2)  Set Load Mode, press Reset. (Led flashes 5 times.)

(3)  COPY ZBAS2RAM.HEX LPT1: (This copies Basic Interpreter to ZLOAD).

After Load, Led continues permanent flash at a fast rate.

(4)  Press and hold Reset, switch to Run mode, release Reset. If Front Panel

I/O is fitted, Version Number 2.0 will be displayed.

(5)  Run FEND.COM. Video should print PBUFF Ready>_

(6)  LOAD GIG (Will Auto-Run. This is a Gigabyte counter.)

(7)  Press F1, Toggle Move on, Esc. (This patches Add 4 in CURRENT.ROM)

(8)  Break program with ^C, (Edit if required,) SAVE MYPROG and Esc to DOS.

This creates and updates Rom image file.

(9)  Burn 2764 from file CURRENT.ROM

(10) Remove ZLOAD EPROM, insert your new MYPROG Basic EPROM.

(11) Set to LOAD position, and the Basic Program will run from the EPROM.

Read on....

***************************************************************************

Z8TBASIC (Z80 TINYBASIC) Version 2.0 by Don McKENZIE  UPDATED 29-August-93(c)

Re-assembled for operation with ZLOAD Version 2.0

Read Zload text files for full information on these changes.

Once again, I have changed Zload to PC write routine. Video still gave

occasional errors.

-------------------------------------------------------------------------------

Z8TBASIC (Z80 TINYBASIC) Version 1.1 by Don McKENZIE  UPDATED 30-July-92(c)

Changed Zload to PC write routine. Video gave occasional errors. I recommend

that Z8TBASIC be run in communication mode from a PC at a ZLOAD speed no

greater than 6.144MHz. That is, ZLOAD running at 6.144Mhz. Stand-alone

Z8TBASIC will run at much faster speeds. To date, Z8TBASIC has been tested

on PC's from 6Mhz 286's to 33Mhz 386's.

-------------------------------------------------------------------------------

Z8TBASIC (Z80 TINYBASIC) Version 1.0 by Don McKENZIE  UPDATED 13-May-92(c)

Program a stand alone Micro-controller with BASIC. A standard IBM PC Parallel

Printer port is used during development.

SOFTWARE OVERVIEW....

Z8TBASIC is an Integer BASIC Language interpreter that runs on my ZLOAD Z80

development kit boards. This can be in an EPROM, or downloaded to a Static

Ram. FEND.COM runs on an IBM PC and is the Front-End program for Z8TBASIC.

This handles Keyboard, Video, and Disk I/O. These two files allow you to use

a PC as a terminal for a BASIC Language micro-controller development system.

Upon completion of program development, you will be able to burn an EPROM to

run a PBUFF board as a Micro-controller in BASIC.

HARDWARE OVERVIEW....

A standard ZLOAD Version 2.0 board is connected via a standard Parallel

printer cable to LPT1 of an IBM PC. This is the communication path for

Keyboard, Video, and Disk I/O. Instead of connecting a Printer to the DB25

female output of PBUFF, a dummy DB25 male plug is connected.

This dummy plug must have the following three jumpers installed:

Pin 2 to pin 14 (Data), pin 3 to pin 16 (toggle Strobe), and pin 10 to pin 17

(Ack).

These three jumpers connect the signals that are the data path from PBUFF to

the PC. IE: Video out and Disk writes. The standard data path from the PC to

PBUFF has been retained so that file dumps (COPY /B FILENAME.EXT LPT1:) can be

done. This is also the data path for Keyboard data, and Disk reads. Upon

completion of program development, you can software disable Keyboard, Video,

and Disk I/O, and remove the printer cable and dummy plug.

***************************************************************************

GENERAL OVERVIEW....

Z8TBASIC can only be used on Revision H (Feb-89) or later PBUFF boards.

PBUFF was developed by me as an 8K to 64K printer buffer kit in Dec-84, and

has appeared in several Electronic magazines as a project. It now supports 4Mb

as a printer buffer. It is the heart of my message board and other projects.

ZLOAD Version 1.0 was originally a PBUFF board and a small satellite board

that had provision for a battery backed Static Ram that could be switched to

address zero in place of the EPROM. It also had a 74LS74 and a 74LS32 that

were used to flip these 28 pin devices and write protect the static ram. It

also had two status LEDs. It was designed as a Z80 Machine Language

Development System.

I have now done away with this satellite board and replaced it with

software. So now, PBUFF and ZLOAD are one and the same, the only difference

being the EPROM firmware, and I have overcome this restriction by using a

27128 EPROM to do both jobs. See my ZLOAD instructions for more details.

***************************************************************************

Disclaimer: To date, my Z80 development system has been very straight forward

in as much as you load your machine language software into my ZLOAD system,

and it performs as per YOUR predictions. Now I have an Integer Basic

Interpreter that may or may not be suitable for your tasks. Needless to say, I

still accept no responsibility as to the operation of this and any other

software that I have written.

***************************************************************************

Future Direction & Development:

I have spent many months writing this software. The only way to really test it

is to get the user to test it fully. It is still in the early stages of

testing and any constructive criticism would be appreciated. A PBUFF Front

Panel Input/Output board, although not necessary, is certainly a very useful

tool to have attached during your own BASIC development. Many Front Panel I/O

demo programs are included.

As my PBUFF board has a 40 pin CPU bus header, it should be very easy for

experienced Technicians to wire up additional hardware on a Prototype board

that can be jumpered with flat ribbon cable and 40 pin IDC crimp header to

this 40 pin CPU header. Please give me any feedback on circuits and driver

software that you develop and are prepared to pass onto other users. The

circuit of my Front Panel hardware is included with Z8TBASIC. This shows the

circuits of the Port decoding methods I have employed. Multiple 40 pin

header drops can be crimped to a 40 wire flat ribbon cable to suit your

needs.

***************************************************************************

Z8TBASIC is my re-hash of an old Public Domain Tiny Basic that was written

originally in 1975 by LI-CHEN WANG for the 8080 Micro.

The TINY BASIC language originated in the pages of Dr. DOBB'S JOURNAL and

PEOPLE'S COMPUTER COMPANY in late 1975 and early 1976. Fed by the enthusiasm

of early computer hobbyists and by the challenges and opportunities created by

the early microcomputer chips, the idea of a tiny basic interpreter quickly

gained popularity and acceptance. The language was a stripped down version of

the ever-popular Dartmouth BASIC with the proviso that it be "useful" with a

minimum of then very expensive memory.

TB8080.DOC and TB8088.DOC should be read as a supplement to this file.

All of these old Tiny Basic files can be found in _TINYBAS.ZIP.

I needed a small Integer Basic Interpreter with I/O and Logical operations

that would suit my "ZLOAD" Z80 Development system. I finally got sick of

people telling me how great the 8052 was to program with Basic. When will you

have Basic on your unit Don?

My early Basic experience was with the old TRS-80 Model 1 Level II. (and 4K

Level I). I would have liked to have used this Z80 Microsoft Basic (circa

1977), but I doubt that Mr Gates would approve without the appropriate

payment. At around 12K of code, It's not exactly suited to simple Controller

applications, and in fact, a bit of an overkill.

At least with Tiny Basic, I have the full source code and hope to adapt it to

suit my needs. Experienced Z80 programmers should also be able to change this

Basic to suit themselves. I have attempted to change and create commands to

match Microsoft Basic syntax.

ZLOAD...

ZBAS2RAM.HEX is the file that can be downline loaded to my ZLOAD Development

kit via a standard PC printer port. This is the method I have used for many

years now to dump machine language code to my ZLOAD board.

Z8TBASIC.ROM...

This was developed using my ZLOAD Z80 Development board. I jumped around the

hardware interrupt and software restart vectors, which will cater for possible

future use. Some of this low area of memory is used for ROM identification,

flag bytes, and a service routine to shut up my standard 555 hardware timer

interrupt and test switch routine. This ROM file is assembled at address

Zero.

ZBAS2RAM.HEX is the Z8TBASIC hex file that is assembled for operation at

address 8000H, and is used with ZLOAD.

The source code for Z8TBASIC.ROM is Z8TBAS_2.Z8T and is found in the

_SOURCE.ZIP file.

FEND.COM...

An 8088 machine language program "Front End" is run on the PC to enable it to

be used as a terminal for ZLOAD. This is FEND.COM. FEND.ASM is the 8088

machine language source code that is also found in _SOURCE.ZIP.

The 8088 front end program and the Z80 PBUFF drivers in Z8TBASIC are my own

work and as such are subject to copyright. The actual Basic interpreter

although now heavily modified by me, is public domain.

_______________________________________________________________

To help explain the files and clean up any possible confusion, here is a map

of the system file and memory usage:

(All addresses are given in Hexadecimal.)

RAM VERSION

FFFFĿ

� DRAM MEMORY. NOT USED FOR ZLOAD

C000��

BFFFĿ

� MIRROR IMAGE OF 8000-9FFF

A000��

����������������������������������������������������������������ͻ

�9FFF���TOP OF MEMORY�Ŀ(MEMORY TEST BYTE) �                     �

�9FF0�����������������Ĵ128 BYTES          �                     �

�     � ZLOAD STACK ETC�RESERVED FOR ZLOAD.�                     �

�9F80�������������������������������������Ķ                     �

�9F7FĿ                                    �                     �

�     �Z8TBASIC HOUSEKEEPING RAM 384 BYTES �                     �

�9E01��                                    � 6264 8K STATIC RAM  �

�9E00ĿBASIC PROGRAM ENDS                  �                     �

�     �3584 BYTES (3.5K)                   �                     �

�9000��BASIC PROGRAM BEGINS                �                     �

������������������������������������������͹                     �

�8FFFĿZ8TBASIC INTEGER INTERPRETER.       �                     �

�     �4096 BYTES (4K)                     �                     �

�8000��ZBAS2RAM.HEX IS LOADED HERE.        �                     �

����������������������������������������������������������������ͼ

7FFFĿ

�EPROM MIRROR IMAGE

6000��

5FFFĿ

�EPROM MIRROR IMAGE

4000��

3FFFĿ

�EPROM MIRROR IMAGE

2000��

����������������������������������������������������������������ͻ

�1FFFĿLAST BYTE IN ROM                    �     ZLOAD.ROM       �

�     �                                    �                     �

�1000��ZLOAD JUMP ROM (ADD 0000)  BANK 1   �  2764 ZLOAD EPROM   �

������������������������������������������͹                     �

�0FFFĿ                                    �OR ONE HALF OF 27128)�

�     �                                    �                     �

�0000��ZLOAD LOADER ROUTINE.      BANK 0   �                     �

����������������������������������������������������������������ͼ

___________________________________________________________________________

ROM VERSION

FFFFĿ

� DRAM MEMORY. NOT USED FOR ZLOAD

C000��

BFFFĿ

� MIRROR IMAGE OF 8000-9FFF

A000��

����������������������������������������������������������������ͻ

�9FFF���TOP OF MEMORY�Ŀ(MEMORY TEST BYTE) �                     �

�9FF0�����������������Ĵ128 BYTES          �                     �

�     � ZLOAD STACK ETC�RESERVED FOR ZLOAD.�                     �

�9F80�������������������������������������Ķ                     �

�9F7FĿ                                    �                     �

�     �Z8TBASIC HOUSEKEEPING RAM 384 BYTES �                     �

�9E01��                                    � 6264 8K STATIC RAM  �

�9E00ĿBASIC PROGRAM ENDS                  �                     �

�     �3584 BYTES (3.5K)                   �                     �

�9000��BASIC PROGRAM BEGINS (AFTER MOVE)   �                     �

������������������������������������������͹                     �

�8FFFĿ                                    �                     �

�     �4096 BYTES (4K) NOT USED.           �                     �

�8000��                                    �                     �

����������������������������������������������������������������ͼ

7FFFĿ

�EPROM MIRROR IMAGE

6000��

5FFFĿ

�EPROM MIRROR IMAGE

4000��

3FFFĿ

�EPROM MIRROR IMAGE

2000��

����������������������������������������������������������������ͻ

�1FFFĿ                                    �                     �

�     �RESERVED (MIRROR IMAGE OF RAM)      �                     �

�1E01��CONFLICTS WITH ZLOAD AND Z8TBASIC   �                     �

������������������������������������������͹     CURRENT.ROM     �

�1E00ĿLAST BYTE IN BASIC PROGRAM          �                     �

�     �                                    �                     �

�1000��BASIC PROGRAM (MOVES TO 9000)       �      2764 EPROM     �

������������������������������������������͹                     �

�0FFFĿ                                    �                     �

�     �Z8TBASIC.ROM (0004=FF)              �                     �

�0000��                                    �                     �

����������������������������������������������������������������ͼ

HARDWARE INTERFACE..

A standard LPT1: type printer port is used for communications between your PC

and ZLOAD.

Standard PC printer OUT hardware is used for keyboard IN to ZLOAD. This is the

data path for file loads to ZLOAD.

ZLOAD video OUT hardware connects to the PC printer IN status bits. Three

signals need to be jumpered on a dummy DB25 male plug. These details are given

near the start of this article. This is 1 Data bit, a toggle Strobe, and an

Ack. This means that you need 8 passes on this channel before the full byte

can be transferred from ZLOAD to a PC. This is the data path for file saves

from ZLOAD.

***************************************************************************

Major changes from 8080 TBASIC to Z8TBASIC.........

I have removed all the old (CPM) Disk, Keyboard, and Video drivers and

replaced them with my own Machine Language drivers. This required 8088 code

(FEND.COM) for the LPT1: printer port, and Z80 code for the PBUFF 8255.

FILE STORAGE......

I have used the front end program to load and save Basic programs in ASCII so

that they can be produced and edited with a standard Word Processor if you

wish. This also gives you the advantage of renumbering them with GWBASIC. The

filename extension must be ".BAZ" for Z8TBASIC and ".BAS" for GWBASIC.

When you first run FEND.COM, a copy of Z8TBASIC.ROM is loaded into the PC

memory if it is resident on the current drive. An error is reported if it's

not. When saving any file, three output files are produced.

(1) FILENAME.BAZ   The ASCII Basic program. (New)

(2) FILENAME.TBI   The Run-time version of the Basic program. (Old)

(3) CURRENT.ROM    A ROM ready file for your 2764 EPROM.

(1) Be gentle with BAZ files. I have not done a lot of error trapping with

this code. Please, no leading blanks on a line, no blank lines, and put a

space between your line number and the start of your statements. Perhaps by

the time you read this, I will have put a lot more time into this problem.

(2) TBI is the original file format for 8080 and 8088 TBASIC. It is also the

file type that is used with the PBUFF board. A better description is given in

full below. It is always 4K (4096 bytes) long, an output file only, never an

input file, and a ROM version of your Basic program.

(3) CURRENT.ROM is 8K (8192 bytes) long. The first 4K is the Z8TBASIC.ROM and

the last 4K is FILENAME.TBI. Z8TBASIC.ROM doesn't have to be exactly a 4K

file. The TBI file will always overwrite it at the halfway point if it is

longer, so be careful if altering your Basic Interpreter. If Z8TBASIC.ROM is

not resident when you run FEND.COM then CURRENT.ROM will not be correct.

***************************************************************************

File formats and Old Tiny Basics...

TBI FILES.........

My Z8TBASIC "TBI" file storage format is now compatible with Michael

Sullivan's 8086 Tiny Basic.

Eg:

LSB Line number, MSB line number.  0AH,00H represents line 10. This is

followed by basic statements in standard ASCII. Each line is terminated with a

single carriage return (0DH) character. The total basic statement table is

terminated with a zero WORD (0H,0H) after the last line carriage return.

LL LH  X  =  1  ,  Y  =  2  ,  Z  =  3 CR LL LH

0000  0A 00 58 3D 31 2C 59 3D 32 2C 5A 3D 33 0D 14 00  ��X=1,Y=2,Z=3���

P  R  I  N  T     X  ,  Y  ,  Z CR TL TH

0010  50 52 49 4E 54 20 58 2C 59 2C 5A 0D 00 00 00 00  PRINT X,Y,Z.....

LL=Line Number LSB. LH=Line Number MSB. CR is Carriage Return.

TL and TH is the WORD which is the double zero listing terminator.

In the Z8TBASIC Z80 Source listing, the Label "TXTUNF" points to "TL".

This is the "BASIC" Statement Table End Of File pointer.

This translates as:

10 X=1,Y=2,Z=3

20 PRINT X,Y,Z

Because I have changed the Command Syntax for Z8TBASIC to be more Microsoft

look-alike, Wang, Sherry Brothers, and Sullivan Basic statements are no longer

useful tools for Z8TBASIC. A Bug in 8080 TBasic TBI file handling means that

it wouldn't load under 8086 TBasic anyway.

Sherry Brother's 8080 Tiny Basic saved an end of text memory pointer (TXTUNF)

along with the file. This was stored at the very start of the file. It meant

that every time you re-assembled the basic interpreter, your old basic

programs would no longer load. This version won't load 8086 TBASIC Files

either.

Note ** Sherry Brothers wrote the CPM Disk I/O routines for Tbasic.

To convert 8080 ".TBI" files to Z8TBASIC and 8086 format, Run Z80MU then: R

"FILENAME.TBI". After the load, this line will be displayed: Low = 100H Next =

200H (Next is the last byte of the disk file.) You can now "W 102 202

FILENAME.TBI". (Add two bytes to each number.) This will write the file

without the first two bytes. These two bytes are a memory pointer reference.

(TXTUNF) You must now find the last line CR (0DH) in the Basic statement

table. The first two bytes (WORD) after this 0DH must be changed to 00H,00H.

This can be done with the "P" command in Z80MU.

***************************************************************************

NOTE **** TBASIC and Z8TBASIC Program statements must be in UPPER CASE ONLY.

Text inside single or double quotes may be lower case.

Spaces can be used freely, just don't use them in the middle of statements.

This is the way all basics should be......

***************************************************************************

General Problems I have fixed....

REM bug. See Below.

Bug in "delete several characters" in a Basic line fixed. Line pointers were

incorrect for end of Basic statement table resulting in trailing rubbish line

numbers and statements.

Delete single character in new line input mode. Cursor update was not

operating correctly on a PC. Now uses the back-arrow key to delete the last

character.

Bug in Save File. A test wasn't done for an empty program buffer on initial

program entry or after a NEW command. It would save the file anyway. In the

case of initial program entry, a copy of random Static Ram data was saved. I

have also fixed this one.

I considered "WAIT" a nightmare. It took me 15 minutes of think music every

time I needed to use it, (just like changing my digital watch to daylight

saving time, or pre-recording a program on my VCR), and one version I found

had bugs in it which didn't help. See Below for a full explanation.

***************************************************************************

My meager attempts to make Z8TBASIC look a little more like Microsoft's

command syntax are as follows:

"OK" Prompt changed to "Ready". Added PBUFF before Ready so that connection to

my board is easily seen. PBUFF Ready>_

PRINT can be entered also as a '?' Eg: 10 ? "HELLO" or 10PRINT'hello' will

both work correctly.

REM can be entered also as a ' Eg: 10 'THIS IS A REMARK ONLY. I found a "REM"

bug in 8080 Tbasic. REM didn't work at all.

Multiple statements on a single line separated by a ':' instead of a ';'

SIZE changed to MEM  Eg: ?MEM  Shows free bytes for Basic program.

Print handling of semicolon and comma.

Because I was familiar with the old Z80 Basic syntax, and now use Borland

Turbo Basic which is very much the same, I found I kept writing statements in

Tiny Basic that always gave an error. I now have these print formatters almost

compatible with the Microsoft syntax. At least they don't error. I didn't see

the point in spending too much time on this section. You only need screen

output for debugging Z8TBASIC Basic code.

The "," separator will now TAB across 14 columns, same as Turbo Basic. The old

TBasic value was 6.

The ";" separator will allow items to be printed shoulder to shoulder. A

trailing ";" will disable Carriage Return generation after the item is

printed.

TBasic has an interesting in-built print formatter that can be used when all

else fails. The "#" symbol can be used and seems to work as per the 8080

"Tiny" Manual. Try it.

As part of this above print formatting re-hash, I also made the "List" command

print the listing from the first column, not at the first "TAB" as the

original was. This looked very odd indeed.

Changed the "#" not equal to symbol to the more familiar "<>".

***************************************************************************

****  AND this is what I needed for my Z80 development kit. ****

Hardware Test Switch acts the same as a ^C, which halts the running Basic

program and returns you to the PBUFF Ready>_ prompt. It also sets all I/O

features to ON, and sets the output from Printer to Video if applicable.

(See Below.) This gives you the ability to debug an established controller

program with a standard PC. The 8255 is also re-programmed for PC

connection. This may result in odd video characters when comms are first

established with a PC. Pressing Enter on the keyboard should overcome this

problem.

If a Front Panel I/O board is connected, the current Version number will be

displayed. (Same feature as PBUFF.)

Upon entry to Z8TBASIC via FEND.COM, CAPS are set to ON, and the printer port

is selected from BIOS Data area 0040:0008 which is the LPT1: port.

The assembled memory map has been altered to suit the ROM and RAM environment

offered by my ZLOAD Z80 Micro Controller and PBUFF. The Memory size shown by

"PRINT MEM" is a reflection of the available memory of Z8TBASIC running on a

PBUFF board.

I have moved IN, OUT, WAIT, PEEK, POKE and other relevant look-up tables to

the top of the compare logic so as to decrease the run time of the interpreter

for control applications.

START, LOAD, and Line 1.  (AUTO-RUN FEATURES)

The START command will restart the system from scratch and if there is a valid

resident Basic program with Line number 1 in the listing, the program will

Auto-Run. This was useful for debugging my line 1 Comms Switches translation

routines. (See below.)

START will also over-ride the NEW command after it has been issued. Try

running this program: 1 START (or 1STA.) This will repeat the sign on message

until ^C is pressed on the keyboard. Great for testing your ZLOAD hardware

after assembly.

If line number 1 is used in the Basic program, this will Auto-Run any valid

resident Basic program during power up, reset, or after a LOAD command. The

resident Basic program can be in ROM or RAM.

Line 1 interpretation. "COMMS SWITCHES"

Simple control is needed to switch off PC communications so that ZLOAD will

run as a stand alone controller running a BASIC Interpreter. There are three

switches. (1) Disk I/O (2) Keyboard, and (3) Video. You may find the need to

set or clear any item independently.

If Line 1 has an ' (apostrophe) remark command, the next three bytes are

translated as follows:  1 'DKV   (Alphabetical order.) where D=Disk,

K=Keyboard, and V=Video

These items can be set or cleared as follows:

(DKV)

1 '010          Disk Off, Keyboard On, Video Off

1 '101          Disk On, Keyboard Off, Video On

1 '111          Disk On, Keyboard On, Video On     (Default)

1 '000          Disk Off, Keyboard Off, Video Off  (EPROM final Version)

When your Z8TBASIC Basic program is fully de-bugged, you may wish to set all

options Off.

If Line 1 and the apostrophe remark command are not found, the default setting

is "ALL ON".

Invalid characters on Line 1 after the ' will result in "ALL ON".

If you use the "REM" remark command in Line 1, the Line will be ignored, as

the Comms Switch translation requires the apostrophe. "ALL ON".

So, it MUST be Line 1, it MUST be an apostrophe, and the next 3 characters

MUST be either a 0 or a 1. OK? ELSE "ALL ON"

Line 1 starts at 9000H, the start of Basic program RAM in ZLOAD.

OLD Command..

Can be issued after a NEW to re-instate the original program.

MOVE Command..

Will move the ROM Basic program at 1000H to RAM at 9000H which is the correct

run address. It moves a total of 3.5K. (3584 bytes.) The missing 512 bytes

is reserved for housekeeping ram for Basic and Zload. See the memory map

supplied.

ROM ADDRESS 4

If address four in ROM contains FF hex, then a MOVE and an OLD will occur on

power up or reset. If line 1 in this program contains the ' rem statement,

this program will Auto-run. This is the normal default setting for the ROM

only version. Use the F1 Menu in FEND.COM to set or clear the move flag.

ROM ADDRESS 5, 6, and 7 are reserved for possible future expansion or your own

program use. These bytes can be "rom-able" set or cleared with the F1 menu in

FEND.COM.

***************************************************************************

USER CALLS:  (Values are passed via the HL register pair.)

These are great fun and very easy to write. I have started a User Jump Table

just after the Test switch (Pin 17) entry point of 102 decimal (66Hex.) Three

bytes are needed for a jump around the table, so the table starts at 105

decimal with 3 byte increments for each jump. These can be easily added to.

X=USR(105,A)  :'USR105 Display L

This will display the LSB (8 bits) of "A" to your Front Panel board if fitted.

This will be in Hex format. EG: If A=1234 (04D2H), the display will show D2.

"X" is a dummy value.

X=USR(108,A)  :'USR108 Display H

This will display the MSB (8 bits) of "A" to your Front Panel board if fitted.

This will be in Hex format. EG: If A=1234 (04D2H), the display will show 04.

"X" is a dummy value.

X=USR(111,A)  :'USR111 Get Input

Get Handshake input from Port 0 of the 8255. This will read a byte that is

input to the 8255. Same as printer byte or keyboard byte in. Byte read will be

the value of "X". "A" is a dummy value.

X=USR(114,A)  :'USR114 LPrint Value.

Use Handshake output to print "A" to Port 1 of the 8255. This is the standard

printer output of PBUFF. "X" is a dummy value.

X=USR(117,A)  :'USR117 Delay on Value.

Value of "A" determines delay. "X" is a dummy value. Crystal Frequency

dependent.

X=USR(120,A)  :'USR120 Printer In, Printer Out.

More of a demo program than a useful feature. OK for testing hardware. Move a

byte to be printed through the system. Calls USR126. "A" and "X" are dummy

values.

X=USR(123,A)  :'USR123 Printer/Video

X=USR(123,0)  :'If A=0 Sets output to Video

X=USR(123,1)  :'If A<>0 Sets output to Printer

Calls USR126. Line 1 Video switch must be on for either of these.

Default power up or reset mode is Video. Test switch will also default to

Video. "X" is a dummy value.

X=USR(126,A)  :'USR126 INIT 8255

Initialize 8255 to A=input, B=output, and C=handshake for A and B ports. This

is standard ZLOAD and PBUFF configuration. You can re-program the 8255 with

standard OUT statements if you wish. "A" and "X" are dummy values.

X=USR(129,A)  :'USR129 DISPLAY HL

The value of A is displayed directly to the Front Panel LEDs with out any

seven segment conversions. This allows custom displays such "CO" to be

displayed. Program DISPHL.BAZ will show the use of this call. If A=62, display

will be "U". If A=57, display will be "C", etc.

And built in ^C and Resets are done with:

X=USR(0,0)      :'Will restart the Micro from address Zero.

X=USR(56,0)     :'555 Timer entry point. Currently ignored.

X=USR(102,0)    :'Is the same as pressing the Test switch.

Anytime the 8255 is re-programmed for PC connection, this may result in odd

video characters when comms are first established with a PC. Pressing Enter on

the keyboard should overcome this.

***************************************************************************

Logical Operators:

The bit manipulation required for a Micro-controller were not supported by the

original TinyBasic, so I had to add them. They are all 8 bit values ONLY,

which is all that is needed. EG: 0 to 255.

In order to cut down on memory and interpretation overhead, I have limited

these Operators to lowercase letters as follows:

"a"

The Logical AND function. EG: X=16a14

"o"

The Logical OR function. EG: X=Zo14

"x"

The Logical XOR function. EG: X=Yx14  To invert or complement a value, XOR

that value with 255.

"l"

The Left shift function. EG: X=Zl2  This will shift the value of Z to the left

by 2 places.  No bit is lost, it is simply rotated left by the value of the

second parameter.

"r"

The Right shift function. EG: X=Zr2  Same as above except the rotation is to

the right. A shift of 8 will return the original value.

Examples of these and other statements are given in the ".BAZ" files.

***************************************************************************

THE OLD WAIT COMMAND..... (Has been changed. See Below.)

WAIT I,J,K

This meant that the data from port I was Anded with data J, and Xored with K,

and wait until the result is not equal to zero. If K was not included in the

statement, then K was equal to zero. This is the same as most modern Basics,

but I don't like it. This WAIT doesn't wait for an entire pattern of bits to

occur, but only for one of them to occur.

Why did I spend so much time with this? There had to be a better way!

***************************************************************************

My New WAIT COMMAND:

This will WAIT for an entire pattern of bits to occur.

WAITPort,Value,Mask

-----------------------------------------------

EXAMPLE ONE:

WAIT2,215       'Will wait for 215 to be present at port 2.

'This is the Copy switch OFF on a PBUFF board.

WAIT2,87        'Will wait for 87 to be present at port 2.

'This is the Copy switch ON on a PBUFF board.

The Mask is default set to 255, or all bits D0 to D7 to be tested.

-----------------------------------------------

For example 1, only data bit 7 is tested. The Mask could have been set to 128.

(D7 only).

EXAMPLE TWO:

WAIT2,215,128   'Will wait for D7 to be high at port 2.

'This is the Copy switch OFF on a PBUFF board.

WAIT2,87,128    'Will wait for D7 to be low at port 2.

'This is the Copy switch ON on a PBUFF board.

-----------------------------------------------

EXAMPLE THREE:

WAIT2,128,128   'Will wait for D7 to be high at port 2.

'This is the Copy switch OFF on a PBUFF board.

WAIT2,0,128     'Will wait for D7 to be low at port 2.

'This is the Copy switch ON on a PBUFF board.

-----------------------------------------------

EXAMPLE FOUR:

WAIT2,128,192   'Will wait for D7 high and D6 low at port 2.

'This is Copy switch OFF and O/P Busy low.

WAIT2,64,192    'Will wait for D7 low and D6 high at port 2.

'This is Copy switch ON and O/P Busy high.

-----------------------------------------------

With this new WAIT mechanism, any bit can be fairly simply tested for any

known state. Any bit can be ignored (Masked out) by this command.

***************************************************************************

Sample program for above.

10 A=INP(2):?A:GOTO10

Run this program and toggle your copy switch. You will see data bit 7 change

state. Busy (PC6) of the output port can be grounded. This will toggle data

bit 6 of port 2. The pause pads must be shorted for this.

This is only a quick sample. See BAZ files for a few more.

AND HERE ARE A FEW MORE:

AROUND.BAZ      Init 8255, read byte in, write out. Direct buffer.

BENCH.BAZ       Beep, Loop 10K times, Beep.

CONVERT.BAZ     Will reverse upper and lower case during print.

COPY2.BAZ       Check copy switch and report OFF/ON.

DELAYS.BAZ      Show Delay routine.

GIG.BAZ         Count 1 Gigabyte on screen. Will take forever.

FLASHER.BAZ     Flash data LED.

FPSW.BAZ        Read Front Panel switches.

FRED.BAZ        A bit of print format types and maths.

INCDIS.BAZ      Increment count to Front Panel LEDs.

KEYS.BAZ        Another version of FPSW.

LOGIC.BAZ       Demo AND, OR, XOR, LEFT, RIGHT

NUMBER.BAZ      Simple count to 32767

ORS.BAZ         Play around with "ORs"

PEEKS.BAZ       Peek at address 0 to 20

PORT96.BAZ      Load FP LEDs one segment at a time, then remove.

PRTOUT.BAZ      All output to printer instead of Video. (PBUFF Printer)

RELOP.BAZ       Test Relational Operators.

SCREEN.BAZ      Write to Video, Auto-run

WAIT.BAZ        Demo WAIT command with Copy and Test switches.

***************************************************************************

BENCHMARKS....

You will find BENCH.BAZ and BENCH.BAS on this disk. Apart from the .EXT name

the only difference is that the BAS version has a few more spaces in lines 30

and 40 so that GWBASIC is happy with it. Z8TBASIC doesn't care about spaces,

as long as they are not in the middle of commands.

The program is a simple 1 to 10,000 loop with no screen output.

I have timed this program as follows:

Z8TBASIC on PBUFF with Z80B installed:

3.5 Seconds 8Mhz Crystal

4.7 Seconds 6Mhz Crystal

7.3 Seconds 3.58Mhz Crystal

GWBASIC on a PC/AT:

1.8 Seconds 17Mhz Landmark speed.

3.5 Seconds 8.6Mhz Landmark speed.

This indicates that the Z8TBasic interpreter performs about the same as a

similar speed PC. This may be true for a simple loop, but it doesn't

necessarily reflect the true performance of this Basic interpreter.

You will find the worst case scenario in the program CONVERT.BAZ. This simply

gets a byte in from the printer port of the PC, converts upper case to lower

and lower case to upper, then outputs it to the printer. It doesn't buffer

bytes, only converts. You will find that this task is performed fairly slowly.

Even at this slow rate (depending on the printer type) it will convert and

output approx. 62 bytes of information every second.

EG: Using a 6Mhz Crystal and doing a full Screen Print with CONVERT.BAZ takes

me 31 seconds until the print is finished. A screen is 80 by 24 or 1920 bytes

of data. This is about 62 bytes a second. Slow? Well it's about 16ms per byte,

but remember the Z80 must read each byte in via the 8255 port handshake, test

it several times, convert it if required, then finally output it via the 8255

output handshake to the printer.

Yes it is slow. You wouldn't program a printer buffer with interpreted Basic.

It would be far too slow. You wouldn't use it as a timer for 8 lanes of

Olympic standard swim or track timing. You wouldn't use it to Auto-Pilot a

NASA space shuttle. (Perhaps a Tiger Moth.)

BUT, where 10, or 50, or perhaps 100ms doesn't really matter, this could be

your quick cost effective answer. I didn't say it wasn't accurate. It is. It

will add, subtract, shift, etc., etc., (Integer only of course) all day long,

it is just a little slower than an equivalent Machine Language program.

BUT, who doesn't know Basic. You could almost count these people along with

the people who have never seen a computer. Most of us know it or will need to

learn it in the very near future.

***************************************************************************

BRIEF HISTORY OF BASIC....

Basic was born from an idea at Dartmouth College in the US in 1962. Kemeny and

Kurtz made it available to the Dartmouth students on May 1st 1964. Gates and

Allen (Microsoft) did major changes to Basic in the mid seventies. (No, not

Burns and Allen.)

Other notable Basic producers around this time were Wang (this Tiny Basic),

Leininger, (Tandy Level I), Wozniak (Apple), and Eubanks (Basice, and CBasic).

Today, Microsoft Basic is the defacto standard on over 50 types of computers.

Bill Gates is now one of the richest men in the US. Gates predicts that Basic

will be very active well into the next century, and how could I but not agree.

Have you had a real look at Basic lately? Quick Basic and Turbo Power Basic

are two very good examples of how far Basic has come since 1964.

***************************************************************************

Files Resident on this Disk:

In the Z8TBASIC directory:

FILENAME(s).BAZ         The ASCII Z8TBasic programs.

FILENAME(s).TBI         The Run-time versions of the Basic programs.

(Do we now, really need these?)

CURRENT.ROM             A ROM ready file for your 2764 or 27(c)64 EPROM.

(Created by saving a Basic program.)

Z8TBASIC.ROM            4K of the ZLOAD version of Z8TBASIC.

Z8TBAS_2.Z8T            The Z80 source code.

ZBAS2RAM.HEX            The Sram Version Intel Hex output of Z8T assembler.

FEND.COM                IBM PC FrontEnd program for Z8TBASIC.

FEND.ASM                8088 Source code for above.

Z8T.COM (or EXE)        The only real Z80 Cross-Assembler for the PC...

Z8T.DOC                 Simple DOC for above. IT'S ALL YOU NEED. All of

my Z80 software is now assembled with Z8T.

***************************************************************************

Z8TBAS_2.Z8T is the Z80 Z8TBASIC Version 2.0 source code. At the start of

this file, you are given the option of assembling at Address Zero for a

ROMable version, or at 8000H for a ZLOAD Ram version.

If you select the Assemble at Zero Version, the output Intel Hex file can be

converted with HBIN.EXE as follows:

Rename Z8TBAS_2.HEX to TEST.HEX. RUN HBIN.EXE. Output file is TEST.BIN.

Rename this to Z8TBASIC.ROM and delete the obsolete TEST files.

If you select the Assemble at 8000 Version, the output Intel Hex file can be

renamed to ZBAS2RAM.HEX. There is a bug in HBIN that doesn't allow a file

assembled at other than Zero to convert correctly. As ZLOAD accepts both

binary and Intel Hex downloads, this bug isn't a problem. You simply COPY

ZBAS2RAM.HEX LPT1: to load it to ZLOAD.

***************************************************************************

In the _TINYBAS.ZIP FILE

TB8080.COM              The 8080 version of Tiny Basic.

TB8080.DOC              Doc file for above.

TB8080.Z8T              The source code. This is converted to Z80

assembly code, and will assemble with Z8T.COM

TB8088.COM              The 8088 version of Tiny Basic.

TB8088.DOC              Doc file for above.

TB8088.ASM              The Machine Language source code.

***************************************************************************

BATTERY BACKUP..............

The 6264 Static Ram in PBUFF can be battery backed up. This may be required

during program development.

The simplest method of providing this is to install a DALLAS SEMICONDUCTOR

DS-1213B SMART-SOCKET between the existing 28 pin socket and the 6264 static

RAM itself. This nifty little device contains all the circuitry required to

maintain the integrity of the 6264 when the power is removed.

These devices are available at many places, but may take a bit of chasing up.

They are available at VELTEK PTY/LTD in Melbourne and Five Dock Sydney and

should cost around $20.

ZLOAD assembly instructions already has the full details for battery backup.

It is the normal mode of operation for ZLOAD.

***************************************************************************

Once you have purchased Z8TBASIC from me in any form, you are entitled to use

it freely for your own Private or Business purposes providing any additional

PBUFF CPU boards are purchased from me. You pay for Z8TBASIC and FEND only

once.

If you have a suitable PBUFF (Rev. H or later), and wish to update to ZLOAD

including Z8TBASIC, $15 for the disk and documentation, and $30 for the disk

documentation, and EPROM.

***************************************************************************