FILENAME: 8255_Z80.DOC   UPDATED 8-August-92 by Don McKenzie.

Original article written by my good friend Rob McAllister (1984).

Interfacing the Z80 to the 8255...

When designing a chip select circuit for an 8255, care must be taken with

the chip select timing otherwise it may not function correctly.

______________________________________________________________________________

A simple circuit diagram is shown below with a Z80 interfaced to an 8255.

��������Ŀ IORQ*          CS*  ����������

���������������������Ĵ

Z80     � RD*            RD*  �  8255

���������������������Ĵ

� WR*            WR*  �

���������������������Ĵ

����������                     ����������

______________________________________________________________________________

Timing sequence for a Z80 I/O write..

IORQ*   ��������Ŀ          � � ���������

�              �

�          �   �

�              �

�          �   �

�              �

����������������

NOTE: IORQ* rises 5ns after WR* (Z80A)

#

WR*   ��������Ŀ          �������������

�          �

�          �   �

�          �

�          �   �

�          �

������������ ���

______________________________________________________________________________

Timing requirement for an 8255 I/O write..

CS*   ��������Ŀ          � � � � � � � � ���������

�                          �

�          �               �

�                          �

�          �               �

�                          �

����������������������������

#   NOTE: The 8255 specifies CS* must be active

for greater than 20 ns after WR* goes high.

WR*   ��������Ŀ          �������������������������

�          �

�          �               �

�          �

�          �               �

�          �

������������ � � � � � � ���

______________________________________________________________________________

The above circuit will NOT satisfy the enable requirements for the 8255,

unless the chip select line is delayed by a time greater than 15ns.   RM84.

______________________________________________________________________________

PBUFF...

This timing delay can be achieved in a number of ways. On the PBUFF board I

have installed a 150pf capacitor between the CS*  of the 8255 and ground.

This trims the timing enough for the circuit to operate reliably.

The propagation delay of one gate of a 74LS04 inverter is 10ns. If two gates

were placed in line to the CS* of the 8255, this would delay this line by

the required 20ns.

If a normal port decoder chip such as a 74LS138 is used, timing will be

correct without adding the 150pf capacitor, in fact, this capacitor must be

removed as the CS* will be delayed by far too much for the circuit to

operate.

This is why the capacitor is installed without a Front Panel, and removed

when a Front Panel is fitted.

In most some cases, late Revision PBUFF boards will operate without the

capacitor installed. My old single sided PBUFF boards would never work

without this capacitor.

______________________________________________________________________________