Home Automation System/2

Version 1.0

COPYRIGHT (c) 1997, Dennis Caffi

The Home Automation System/2 allows you to set up the CP290 X-10 Home

Control Interface. This program is a PM based application.

Installation:

The following files make up the Home Automation System/2:

home2.exe

home2.ico

home2.hlp

First copy these files to the directory of your choice. Change to that directory

and then run the install.cmd script. This will create a folder on the desktop and

an object for the Home/2 program and an object for the readme.doc file.

Using Home Automation System/2:

When you first start the program, you should set the base house code and

communications port.The base house code will allow you to control the modules

with the same house code from the rocker switches on the CP290 control unit.

The program will save these settings in the "home.ini" file.

A log file by the name "home.log" is created in the current directory each

time you start the program.  This file contains the entries that appear in the

program window.

Please be patient when querying settings or setting timer events.  The CP290

communicates at only 600 baud.  Download time for the events increases with

the number of events that are set.

Menu items:

File

Open - Open file containing events.

Save - Save current events.

SaveAs - Save current to another file.

Exit - exit program.

Settings

Set Communications port - Set the port the CP290 is connected to.

This value is stored in the home.ini file

so you only need to set this once unless

you change comm ports.

Set Base House Code - Set the base house code This value is stored

in the home.ini file so you only need to set this once

unless you need to change the base house code. The

new value will replace what is in the home.ini file.

*WARNING* changing the base house code will clear

all events stored in the CP290 control unit.

Set Clock - Sets the clock in the CP290 control interface using the PC time.

Query

Time - gets the time from the CP290 control interface.

Events - Lists all 128 events stored in the CP290 control interface.

Base House & comm port

CP290 - Returns the Base House Code stored in the CP290.

Profile - Returns the values stored in the home.ini file.

Actions

Direct - Used to turn on, off, or dim a single module.

Events - Used to set timer events.  The dialog sets one event at a time with

input obtained from spin buttons in the event dialog.

Timers

Upload from CP290 - Uploads all events from the CP290

Download to CP290 - Downloads the current events to the CP290 Control

Interface.  This may take some time depending on how

may events you have set.

Edit - Allows you to change all 128 timer events.

Help

Help index - Home/2 help for all menu items.

Using help - OS/2 help on using help.

Product information - Info on Home/2.

Fixes / new features:

This version should work with either the SIO drivers installed or with the standard

OS/2 drivers.

You can save the events to a file of your choice.  You can also load different events

stored in another file and download these events to the CP290.

Questions / Comments:

If you have any questions, comments, or suggestions about this program,

feel free to drop me a note. My e-mail address is caffi@ibm.net.