Panasonic ATAPI CD-ROM Device Driver for OS/2 Documentation

-- MKECR57X.ADD Ver.1.11 --

Command Line Parameter

Parameter definitions:

[/V] = Verbose Mode

Displays controller status and drive type during OS/2 initialization.

[/A:n] = Adapter Number

Adapter 0 has default parameters for the primary adapter.

Adapter 1 has default parameters for the secondary adapter.

[/P:nnn] = Base I/O Port Address (hex)

This parameter overrides the default base I/O port address for the adapter

indecated. The default I/O address for primary is (1F0) and for seconday

is (170).

If you will set this parameter, you should set /I: parameter.

[/I:xx]  = IRQ level (dec)

This parameter overrides the default IRQ Number for the adapter indecated.

The default IRQ level for primray is (14) and for seconday is (15).

If you will set this parameter, you should set /P: parameter.

[/NI] = No Interrupt Mode( Polling Mode )

When you use IDE HDD and ATAPI CD-ROM drive at the same adapter(primary or

secondary), the MKECR57X.ADD driver is automatically used this mode.

[/IOR] = I/O Channel Ready Supported Mode

For the adapter with I/O channel ready, the MKECR57X.ADD driver can

designate the fastest read mode.

Examples:

; scan to primary and secondary adapter.

BASEDEV=MKECR57X.ADD /V

; scan to secondary adapter with I/O channel ready.

BASEDEV=MKECR57X.ADD /V /A:1 /IOR

; scan to another adapter(I/O address 1E8  IRQ 11) with I/O channel ready.

BASEDEV=MKECR57X.ADD /V /P:1E8 /I:11 /IOR

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

CAUTION:

OS/2 Warp has ATAPI CD-ROM filter driver(IBMIDECD.FLT). If problems occur

using the IBMIDECD filter, you fix lines in your OS/2 CONFIG.SYS file has

to be edited as follows:

rem BASEDEV=IBMIDECD.FLT

At the end of the file, add the following statement

BASEDEV=MKECR57X.ADD /V

and copy to root directory on the OS/2 system drive.

In this time, if the CR-ROM drive is attached as master to primary or

secondary, you should fix a line in your OS/2 CONFIG.SYS file has to be

edited as follows:

BASEDEV=IBM1S506.ADD /A:x /I

The /A parameter refers to the adapter number that the CD-ROM is attached

to.The /I parameter indicates that the IBM1S506 driver should not attempt

to initialize the adapter indicated.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Trademarks and Copyrights

OS/2 is a trademarks of International Business Machines Corporation.