README.DOC for XFANF.EXE and XFANNT.EXE

=======================================

Software Contents:

------------------

README.DOC - This file.

XFANF.EXE  - Dos serial DNC program.

BAUD.EXE   - Dos utility to set  baud rates greater than  9600 bps.

FIFO.EXE   - Dos utility for 16550 compatible UARTS.

XFANNT.EXE - Windows NT serial DNC program.

PLEASE READ

-----------

Before attempting to use this software, please take the time

to read  the entire text.  I can not  help those who  do not help

themselves. If you  have any comments or questions,  feel free to

write me at  heath@ismi.net. I will do what I  can to assist you.

The license agreement  is contained at the end  of this text.

DO  NOT  SKIP  THE  LICENSE  AGREEMENT  SECTION  IT CONTAINS VERY

IMPORTANT LIABILITY INFORMATION.

What is this software?

----------------------

This  software  is  intended  to  allow  DNC (Direct Numeric

Control)  from IBM  AT class  machines via  standard serial  port

interface using  both the DOS  and Windows NT  operating systems.

Note

----

The DOS version has  been compiled using 80286 instructions,

and will NOT execute from an older 8088/8086 machine.

Software Purpose

----------------

This  software  simply  downloads  CNC  programs  using  the

XON/OFF protocol.  If there is any  demand out there, I  can also

support hardware  handshaking. For now  it is simply  an XON/XOFF

text file transfer utility, nothing more.

This  approach  does  have   it's  advantages.  Firstly,  the

performance  is  high  since  there  are  no  GUIs  demanding CPU

resources, and secondly  it allows end users to  create there own

user interfaces  using Visual tools (Visual  Basic and the like),

and spawn processes to do the file transfers.

Is this software free?

----------------------

READ CAREFULLY.

This software is considered Shareware. Shareware software is

not  free.  The  shareware   software  concept  allows  you,  the

potential customer,  to review the software  for a limited period

of  time for  evaluation purposes.  Once you  have exceeded  this

limited  evaluation  period,  you  are  obligated  to destroy all

copies of this software. The  evaluation period for this software

is 30 days.

If you choose to keep  the software without registering, you

are considered a software  pirate. Software pirating has negative

impact on software innovations, and society in general.

If you  know your company has  not registered this software,

and has  no intention of  registering this software,  you have my

permission to  delete it from the  system. It belongs exclusively

to myself.  Use of this product  without intent to register  is a

breach of the license agreement.  See license agreement at end of

text.

Please keep  in mind that I  am aware that this  software is

intended for a nitch market. In  order to see any kind of return,

I need ALL of this software's  users to purchase the software. Do

not leave  it up to the  other guy to register,  the other guy is

leaving it up to you.

Don't be a priate

-----------------

So please do the right thing. If you are using this tool for

business purposes,  register it today.

Restrictions

------------

All employees of the U.S.  government, both home and abroad

are strickly prohibited from using  this software for any purpose

without prior written approval.

Any version  that does not  display the goverment  contract

agreement  number  is  unauthorized,   and  should  be  destroyed

immediately.  Goverment users  see site  licensing information at

end of text.

Whats included?

---------------

There are two  versions of the software enclosed  in the zip

file. One is for DOS, the other for Windows NT.

The DOS  version is called XFANF.EXE.  There are two support

utilities for  the DOS version,  BAUD.EXE and FIFO.EXE.  BAUD.EXE

and FIFO.EXE are explained in detail later in this text.

The  NT version  is called  XFANNT.EXE. There  are no  other

support files for this version.

NOTE!

-----

Do not attempt to run the DOS version under a NT virtual DOS

machine.  This  will  not  work.  As  you  may  know, the virtual

machines use  virtual device drivers to  isolate the DOS programs

from  the  real  hardware.  Using  this  methodology  is not only

painfully  slow, but  renders the  XON/XOFF handshaking  useless.

Several hundred characters will be buffered by NT by the time the

XOFF is received  by the program. You may have  ran into this had

you attempted  to use standard terminal  software such as PROCOMM

(Datastorm technologies) for DOS.

WIN95 Users

-----------

Although  the  NT  version  of  the  software executes under

Win95,   initial   testing   has    indicated   there   is   some

incompatibility between 95 and NT.  I do not recommend attempting

to use the  NT version under 95. You may  be able to successfully

run the DOS version under 95, but  this may or not be the case. I

am  assuming that  95 is  using virtual  device drivers, in which

case you  will probably experience  overrun errors either  at the

control or at the PC. If you are going to attempt running the DOS

version under 95 disable the  UARTs FIFOs in the device managaer.

I do  not recommend  running either  version under  Win95. Enough

said.

Running the software

--------------------

To view  the help screen  simply run the  utilities from the

command  line without  any arguments.  The DOS  version is called

XFANF.EXE and the  NT version is XFANNT.EXE. Copy  only the files

you need, delete the others  to avoid confusion. The command line

parameters are identical for both versions of the software.

C:\>XFANF

XFANF.EXE Johnson Industrial Controls. V2.0 (c) 1997.

Useage [-x] [-wait] [-Fanuc] <port> <mode> <file>

-------------------------------------------------

-o     = Overwrite file if exist.

-x     = disable XON/XOFF handshaking. enabled by default.

-wait  = wait for XON before transmiting.

-fanuc = if COMx is set to binary mode setting (N,8,1) use the bad

XOFF character sent by Fanuc control. (0x13=0x93)

(See README for more information)

<port> = COM1 thru COM2 only.

<mode> = "tx" or "rx" file.

<file> = filename to transmit or receive.

Note: use dos command MODE to select proper communication

parameters (BAUD,PARITY,WORD_LENGTH,STOP_BITS).

Example: MODE COM1:9600,N,8,1

The -o option

-------------

This option  simply allows you to  confirm a file overwrite

from the command line. If you are attempting to download (receive

a file from  the CNC to the  PC), and the file  already exists on

the hard drive, specifying this option will bypass the prompt for

manual  overwrite confirmation.  Be careful  with this  option if

you  are modifying  important files  on the  CNC and  maintaining

versions on the  hard drive. Once you overwrite,  there is no way

to get it back. Currently it does  not rename the old verion to a

.BAK file. Be careful.

The -x option

-------------

This  option simply  disables XON/XOFF  handshaking. I dont

see a  need for it, but  I thought that someone  out there might.

Beware that if the characters can not be saved to the disk faster

than  they are  being received,   you will  get an  overrun error

message,  and  the  program  will  terminate.  If  you  do get an

overrun, the file is simply closed and is not deleted. Be careful

you  dont get  overruns and  make any  assumptions about the file

content.

The -wait option

----------------

This  option  simply  means  act  as  if  an  XOFF has been

received,  and  do  not  transmit  characters  until  an  XON  is

received.  Some controls  will automatically  output an  XON when

they are ready for character  reception. Using this option allows

you to  prepare for output, walk  over to the control,  and begin

transmision. Otherwise, if  you do not use this  option, you will

have to hit Enter (Return) to begin transmissions.

The -fanuc option

-----------------

For  unknown reasons  outside of  Fanuc GE  Japan, the Fanuc

CNCs (which models?  Don't ask me.), have a  problem dealing with

the  XOFF  character.  The  problem  seems  to  be that they have

hard-coded the XOFF character at some  level such that 8 bits are

transmitted, one of which is stuck high (parity bit position). If

you can use  even parity, 7 data bits, and  2 stop bits, you will

not need this option. However if  you use 8 data bits, no parity,

and 1 stop bit, you *might* need  to use this option. There are a

few screens  on the Fanuc  controls which default  to binary mode

(n,8,1), in which case this option defintely applies.

If you specify this option, and the software determines that

the serial port on the PC is set to binary mode (N,8,1), then the

software will compensate for the XOFF problem.

The port option

---------------

This  simply  specifies  which  communications  port you are

using. The DOS version currently supports only COM1 and COM2. The

NT  version will  try to  open whatever  port you  specify (DIGI1

etc). Thus it may be possible to use any number of ports on an NT

machine, assumming  that the multi-port  board manufacturers have

done  their  homework,  and  are  following  the NT device driver

specification.

To send  files through multiple ports  using the NT version,

open a new window (or process), and specify unique ports for each

download. Two popular multi-port boards are the DigiBoard made by

DigiCom International, and Rocket Port  board by Comtrol. To find

more information, I recommend searching the net.

One thing to  note here is that if you  are using NT, and you

are talking to a Fanuc control,  you probably need to disable and

FIFO/hardware buffering. The reason for  this is simple, when the

Fanuc  tells the  PC to   stop sending  using the  XOFF handshake

character, the PC must stop transmiting within 2 character times.

If you transmit that 3rd  character after the XOFF reception, you

will be likely to lock the control or get a buffer overrun.

Whether or not your multi-port  serial board will stop within

2 character times  is a matter of  investigation. Try downloading

to the CNC,  if you get a buffer overrun,  you probably have some

sort of  extended buffering going  on. In that  case, contact the

board  manufacturer and  ask them  how many  character times will

they continue to  transmit after the reception of  the XOFF. This

should only apply to multi-port intelligent boards.

If you  have 16550 or  compatible UARTS, NT  users can simply

goto  the  control  panel/ports  window,  and  under the advanced

settings, you should find a "Enable  Fifos" check box. If you are

talking  to a  Fanuc, un-check  this box,  and re-boot. DOS users

need  to read  the sections  which describe  the UART utilities I

have enclosed.

The mode option

---------------

Specifies whether  you are sending or  receiving a file. Use

RX to receive files, TX to transmit files.

The file option

---------------

The name of the file you are sending or receiving.

DOS users

---------

Two utility  programs are included to  assist in configuring

your  serial ports.  BAUD.EXE allows  you to  specify higher baud

rates than  the standard MODE.EXE  command. The FIFO.EXE  utility

allows you to configure 16550 uarts.

To see  the command line  options for BAUD.EXE,  simply type

BAUD.EXE without command line parameters.

C:\>BAUD.EXE

BAUD.EXE by Heath Smith, JIC (c) 1997.

Useage: BAUD <port> <baud>

--------------------------

<port> - COM1 through COM4 only.

<baud> - Baud rate to set.

Note: This program does NOT detect UART max baud capability.

The port option

---------------

The port option specifies the communications port. COM1 thru

COM4 may be used. This utility  reads the UARTs base address from

BIOS,  so make  sure that   your BIOS  has not  been reconfigured

incorrectly. Some newer BIOS versions  allow you to specify which

connector is to  be used for which communications  port. This can

be confusing if the BIOS has been changed, and you are unaware of

this. Most  systems use a 9  pin connector for COM1  and a 25 pin

connector for COM2.

The baud option

---------------

The baud  option is simply the  baud rate to use.  DOS (6.22

and prior  versions) only allow  baud rates up  to 9600. You  can

specify  19200, 38400,  57600, and  115200 as  possible baud rate

settings.  Beware, older  UARTs like  the 8250  allow bauds rates

ranging  from 38400  and below.  See UART  documentation for more

information on maximum baud rates.

Even if your  uart can use the higher  baud rates, without a

FIFO mechanism,  you will probably encounter  UART overrun errors

when receiving.  If this is  the case, step  down the baud  rates

until the overruns cease.

Example

-------

First set the communcations parameters other than the baud.

C:\>MODE COM2:9600,N,8,1

Then run the baud command.

C:\>BAUD COM2 115200

Thats all you need to communicate with 115200 baud.

The FIFO.EXE utility

--------------------

The  FIFO  utility  allows   you  to  configure  your  16550

compatible  UARTS. There  are a  couple of  reasons why you might

want to use this utility. One reason is if you are receiving data

faster than 9600 baud, the other is if you are transmitting files

to a Fanuc control.

The  FIFO mechanism  in  16550  compatible UARTs  allows the

hardware  to buffer  characters (up  to 16),  rather than  only 2

characters (like old 8250s). This  approach has both positive and

negative effects.

There are  several variations of 16550  compatible UARTS out

there, and I have done my best to detect them. One version that I

do not currently deal with the the 82510 UART which has FIFOs. If

anyone has the  data sheet on it, and they  need the FIFO utility

to be updated, e-mail me.

If you  are *transmitting* files  to a Fanuc,  then you will

want to  disable your UART's  FIFOs. Setting the  receive trigger

level to 1 will not work, this will not affect the transmit FIFO.

I did not  find a method  to independently enable  or disable the

transmit and receive FIFOs. They are either both on or off.

The  reason for  this was  mentioned earlier,  if the  Fanuc

sends an XOFF  to the PC, the PC must  stop transmission within 2

character times. If the FIFOs are enable, up to 16 characters can

be  buffered  by  the  hardware.  Once  buffered,  they  will  be

transmitted,  thus you  will get  a buffer  overrun on  the Fanuc

control.

On  the other  hand, if  you are  *receiving* files from the

CNC, at  high baud rates (+9600),  then will want to  enable your

FIFOs. This  prevents character overruns,  a unrecoverable error.

The best  receive trigger level  is 1 for  very high baud  rates.

This gives more  time to the interrupt service  routine to handle

the incomming characters.

To see  the command line  options for FIFO.EXE,  simply type

FIFO.EXE without command line parameters.

C:\>FIFO.EXE

FIFO.EXE By Heath Smith, JIC (c) 1997

USEAGE: FIFO.EXE [-e [n]] [-d] <port>

-------------------------------------

-e     - Enable the UART's FIFOs.

n     - Receive trigger level (1,4,8,14), default=14.

-d     - Disable the UART's FIFOs.

port   - COM1 thru COM4 only.

To enable the FIFOs on COM2 with a trigger level of 1, type:

C:\>FIFO.EXE -e 1 COM2

To disable the FIFOs on COM2 type:

C:\>FIFO.EXE -d COM2

If you do not have a  16550 compatible UART the program will

terminate with an  error message. To verify your  UART types, run

MSD.EXE, then select the COM PORTS menu box.

If you  have an older UART  which does not have  FIFOs, then

you will  probably only be able  to *receive* at baud  rates less

than or equal to 9600. You can *transmit* (send to CNC) using the

UARTs highest possible  baud rate. Older UARTs might  be 8250s or

16450s. These UARTs do not have FIFOs.

Future Enhancements

-------------------

There are several enhancements which I may provide depending

on the  demand. The first is  to allow any number  of UARTS under

DOS, allowing  simultaneous downloads. I would  like to provide a

solution  for up  to 4  standard UARTS  with shared IRQs. Another

enhancement would be support for multi-port boards under DOS.

The  NT version  could be  changed to  allow the  program to

monitor  directories for  file uploads.  If a  file is  found, it

would be sent to the device automatically. I believe you could do

this already with batch files, or using a Visual Basic solution.

I  would   take  the  time  to   investigate  problems  with

multi-port serial  boards if problems  arise under NT.  If you do

run  into problems,  or if  you find  a fix,  please pass  on the

information.

Other Products

--------------

Depending on  the success of  this Shareware software,  I am

considering  developing a  high speed  DNC package  for NT.  This

would only be available to Fanuc users with remote buffer boards.

Currently I do  have a solution for OS/2 (Warp),  but not for NT.

This  would require  an NT   device driver  to be  written.

Other  companies already  have such  a product.  There is at

least  one major  problem with  their solutions,  they are  using

non-intelligent boards. Non-intelligent  interface boards place a

high demand on the system,  in which case your throughput usually

suffers.

The  intelligent boards  not only  give you  full throughput

with low CPU  overhead, it allows you to  easily support multiple

simultaneous connections. If you have  a need for such a product,

feel free to e-mail me.

I am considering  creating an NT C++ library  for DNC-2. The

would be available  to system developers as well  as Visual Basic

programmers.  The  DOS  source  is  available  from  GE, but I am

unaware of  an NT version.  I might also  port this same  code to

OS/2 depending on the demand.

Currently I  have a embedded C  development system for Fanuc

15 users. This allows you to write your own applications right on

the control.  This does not use  the same compiler that  GE Fanuc

recommends.  Future development  will provide  linkable libraries

for all system calls.

Another  product which  I am  interested in  developing is a

tape reader simulator  for all types of CNCs.  Currently I have a

DOS embedded solution availble, but I  would like to create an NT

version, allowing any number of  connections to a single machine,

even over the network.

=================================================================

LIMITED USE SOFTWARE LICENSE AGREEMENT

This   Limited  Use   Software  License   Agreement  (the

"Agreement") is a legal agreement  between you, the end-user, and

Heath  Smith. By  continuing the  installation of  these software

programs, by  loading or running  the software, or  by placing or

copying the  software program onto your  computer hard drive, you

are agreeing to be bound by the terms of this Agreement.

SOFTWARE LICENSE

1.  Grant  of  License.  Heath  Smith  grants  to you the

limited right  to use the Software  DNC program (the "Software"),

for the  exclusive, limited purpose of  testing the compatibility

of your computer system with  the Software. You are not receiving

any ownership  or proprietary right,  title or interest  in or to

the Software  or the copyright  or other rights  related thereto.

You are granted  a 30 evaluation and or  use period prior to

purchase.  If  after  the  30  day  period  has  elapsed, you are

required to either purchase the  software, or, destroy all copies

of the software, regaurdless of form.

For  purposes  of  this  section,  "use"  means  loading the

Software  into RAM,  as well  as installation  on a  hard disk or

other  storage device.  You agree  that the  Software will not be

shipped, transferred or exported into any country in violation of

the U.S.  Export Administration Act  (or any other  law governing

such matters) and that you will not utilize, in any other manner,

the Software in violation of any applicable law.

2.  Prohibited  Commercial  Use.  Under  no circumstances

shall you,  the end-user, be permitted,  allowed or authorized to

commercially exploit the Software. Neither you nor anyone at your

direction shall do any of the following acts:

a.      Rent the Software;

b.      Sell the Software;

c.      Lease or lend the Software;

d.      Offer the Software on a pay-per-use basis;

e.   Distribute  the   Software  (except   as  noted   in

4. hereinbelow) by any means,including,  but not limited to direct

mail, retail, mail order or other means; or

f. In any other manner  and through any medium whatsoever

commercially  exploit the  Software or  use the  Software for any

commercial purpose.

3.  Additional Prohibited  Uses: You  shall not  make the

following  uses or  take the  following action  in regard  to the

Software:

a.      Translate the Software;

b.      Reproduce the Software;

c.      Publicly display the Software; or

d.      Prepare derivative works based upon the Software.

4.      Electronic Distribution is Permitted:  So long as

this Agreement accompanies the Software at all times, Heath Smith

grants  to Providers  the limited  right to  distribute, free  of

charge, except normal access fees,  and by electronic means only,

the  Software;  provided,  however,   the  Software  must  be  so

electronically distributed only in  a compressed format. The term

"Providers,"  as  used  in  the  foregoing  sentence,  shall mean

persons whose business it is to provide services on the Internet,

on commercial online networks, or on the BBS. Anyone who receives

the Software  from a Provider shall  be limited to all  the terms

and conditions of this Agreement.

5. Copyright. The Software is owned by Heath Smith and is

protected  by  United  States  copyright  laws  and international

treaty  provisions. You  must treat  the Software  like any other

copyrighted material, as required by  17. U.S.C., 101 et seq. and

other applicable law.  You agree to use your  best efforts to see

that any  user of the  Software licensed hereunder  complies with

this Agreement.

6. NO  WARRANTIES. HEATH SMITH  DISCLAIMS ALL WARRANTIES,

WHETHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO, IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE

WITH  RESPECT  TO  THE  SOFTWARE  AND  THE  ACCOMPANYING  WRITTEN

MATERIALS,  IF  ANY.  HEATH  SMITH  DOES  NOT  WARRANT  THAT  THE

OPERATION OF  THE SOFTWARE WILL  BE UNINTERRUPTED, ERROR  FREE OR

MEET  YOUR SPECIFIC  REQUIREMENTS. ADDITIONAL  STATEMENTS SUCH AS

PRESENTATIONS,  WHETHER  ORAL  OR   WRITTEN,  DO  NOT  CONSTITUTE

WARRANTIES BY HEATH SMITH AND SHOULD NOT BE RELIED UPON.

7. Venue and Exclusive  Remedies. This Agreement shall be

construed  in accordance  with and  governed by  the laws  of the

State of  Michigan. Copyright and other  proprietary matters will

be governed by United States  laws and international treaties. IN

ANY  CASE,  NEITHER  HEATH  SMITH  NOR  HEATH  SMITH'S  OFFICERS,

EMPLOYEES, DIRECTORS, AGENTS, LICENSEES, SUBLICENSEES, SUCCESSORS

OR ASSIGNS  SHALL BE LIABLE  FOR LOSS OF  DATA, LOSS OF  PROFITS,

LOST  SAVINGS,  SPECIAL,  INCIDENTAL,  CONSEQUENTIAL, INDIRECT OR

OTHER SIMILAR DAMAGES ARISING FROM  BREACH OF WARRANTY, BREACH OF

CONTRACT, NEGLIGENCE,  OR OTHER LEGAL THEORY  EVEN IF HEATH SMITH

OR HIS AGENT HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES,

OR FOR  ANY CLAIM BY ANY  OTHER PARTY. Some jurisdictions  do not

allow the exclusion or  limitation of incidental or consequential

damages, so  the above limitation  or exclusion may  not apply to

you.

8.  General Provisions.  Neither this  Agreement nor  any

part or  portion hereof shall be  assigned or sublicensed, except

as described  herein. Should any  provision of this  Agreement be

held to  be void, invalid,  unenforceable or illegal  by a court,

the validity and enforceability of the other provisions shall not

be  affected  thereby.  If  any  provision  is  determined  to be

unenforceable, you  agree to a modification  of such provision to

provide for enforcement of the  provision's intent, to the extent

permitted by  applicable law. Failure  of a party  to enforce any

provision of this Agreement shall  not constitute or be construed

as a  waiver of such  provision or of  the right to  enforce such

provision.  If  you  fail  to  comply  with  any  terms  of  this

Agreement, YOUR LICENSE IS AUTOMATICALLY TERMINATED.

YOU ACKNOWLEDGE  THAT YOU HAVE  READ THIS AGREEMENT,  YOU

UNDERSTAND THIS AGREEMENT, AND  UNDERSTAND THAT BY CONTINUING THE

INSTALLATION OF THE SOFTWARE, BY LOADING OR RUNNING THE SOFTWARE,

OR  BY PLACING  OR COPYING  THE SOFTWARE  ONTO YOUR COMPUTER HARD

DRIVE,  YOU  AGREE  TO  BE  BOUND  BY  THIS AGREEMENT'S TERMS AND

CONDITIONS. YOU  FURTHER AGREE THAT, EXCEPT  FOR WRITTEN SEPARATE

AGREEMENTS  BETWEEN  HEATH  SMITH  AND  YOU,  THIS AGREEMENT IS A

COMPLETE AND EXCLUSIVE STATEMENT OF THE RIGHTS AND LIABILITIES OF

THE PARTIES. THIS AGREEMENT SUPERSEDES ALL PRIOR ORAL AGREEMENTS,

PROPOSALS OR UNDERSTANDINGS, AND ANY OTHER COMMUNICATIONS BETWEEN

HEATH  SMITH  AND  YOU  RELATING  TO  THE  SUBJECT MATTER OF THIS

AGREEMENT.

USE ON MORE THAN ONE COMPUTER

-----------------------------

To  use  JIC  products  in  a  commercial,  educational or

governmental agency on more than one computer, it is necessary to

purchase a SITE LICENSE. The Site License allows the organization

to  use the  software package  on up  to the  number of computers

licensed.  Site License  fees are  based on  the total  number of

computers  that will  use the  software package.  Please use  the

"Multiple Use Order Form"

NOTE  THAT  THE  SITE  LICENSE  INCLUDES  ONE  COPY OF THE

SOFTWARE. The Site  License allows you to copy  it for the number

of machines licensed. Distributing,  repackaging, or reselling of

the software  to third parties  is not allowed.  All licenses are

prepaid.

=================================================================

ORDERING INFORMATION

--------------------

DATE: ___________

SEND TO:

--------

Heath Smith

6919 Somerset Dr.

Brighton, MI 48116

FAX 810-229-4924

heath@ismi.net

FAX NUMBERS

------------

810-229-9072

619-753-3667

FROM: __________________________________

Name: __________________________________

(Company): __________________________________

Address: __________________________________

City: __________________________________

State, ZIP Code: __________________________________

(Country): __________________________________

Daytime

Phone #: _________________________  FAX #: ____________________________

E-mail address: ____________________________________

Note: registered  users will receive a  free upgrade via attached

to an e-mail.

INDIVIDUAL USE

* Diskette with programs and on-line Documentation ....... $125  _____

MULTIPLE USE

* SITE LICENSE for the use of:

XFANF.EXE,XFANNT.EXE,BAUD.EXE,FIFO.EXE

(INCLUDES ONE DISKETTE WITH PROGRAMS AND ON-LINE DOCUMENTATION)

2 to   9 computers ..... at $120 each    # computers ___x 120  ______

10 to  24 computers ..... at $110 each    # computers ___x 110  ______

25 to  49 computers ..... at $100 each    # computers ___x 100  ______

50 to  99 computers ..... at $90  each    # computers ___x 90   ______

100 to 199 computers ..... at $80  each    # computers ___x 80   ______

200 to 500 computers ..... at $70  each    # computers ___x 70   ______

WRITE OR CALL FOR PRICING ON QUANTITIES OVER 500.

Shipping & Handling (Postal) US & Canada $5.00/item

Outside US & Canada $15.00/item                                    ______

MICHIGAN RESIDENTS add applicable State & County Sales Tax         ______

TOTAL ENCLOSED US FUNDS                                      US$________

TERMS:

Check or  Money Order drawn on  a U.S.A. bank in  U.S.A. funds.

Corporate  Purchase orders  (Net 30  days) accepted  for software

from large corporations within the USA & Canada. ALL LICENSES ARE

PREPAID ONLY. ALL ORDERS OUTSIDE  OF THE UNITED STATES AND CANADA

MUST BE  PREPAID. Please make remittance  payable to Heath Smith.

*** Prices and terms subject to change without notice.

=================================================================

DISTRIBUTION LICENSE

--------------------

A Distribution License is required by an organization, company or

government  agency   to  use  JIC  programs   to  facilitate  the

distribution of software or data to outside parties.

To   use  JIC   programs  within   your  organization,   company,

educational  or government  agency, please  see the  Site License

information.

Complete  and return  the following  Application for Distribution

License  to  receive  a  quotation.  There  is  no  obligation to

complete and  return the Application to  receive a quotation. You

may  fax  or  mail  the  completed  Application.  JIC will send a

written quotation.

The Distribution  License is non-exclusive  and non-transferable.

All licenses are prepaid.

APPLICATION  FOR  DISTRIBUTION  LICENSE

(NOTE: there is no obligation to complete the

application for a quotation.  Please print or type.)

NAME:  ____________________________________________________

COMPANY:  ____________________________________________________

TITLE:  _________________________________  DATE:____________

ADDRESS:  ____________________________________________________

CITY:  _______________________________   STATE:  __________

ZIP CODE:  _____________   (COUNTRY:) __________________________

PHONE #:  (_____)______________  FAX #:  (____)________________

PLEASE COMPLETE AS BEST AS POSSIBLE.

1)  I am interested in distributing the program(s):

(CHECK ALL THAT APPLY):

XFANF  ___  for MS-DOS

XFANNT ___  for NT

2)  In what quantity do you estimate distributing the software

indicated above?   ________ copies.

Per year ____  or  one time ____ (CHECK ONLY ONE).

4)  Please complete the following:

(feel free to attach additional sheets)

-   Describe the intended usage of the JIC programs indicated

in question 1.

-   Describe the software packages(s) that you intend to distri-

bute with or using the JIC programs, and their price or

price range.

-   List any other relevant information:

5)  State the names(s) of the software package(s)/program(s)

which will be distributed using the JIC programs.

6)  Please indicate the state or country in which your

organization is incorporated:

=================================================================