MAXMODE by AudioJack/RDG

(c) Copyright Renegade Demo Group, 1996.  All Rights Reserved.

Portions (c) Copyright Advanced Gravis, 1995.  All Rights Reserved.

ULTRASOUND PNP

~~~~~~~~~~~~~~

The Gravis UltraSound PnP is Advanced Gravis' most adventures

project so far.  It uses an InterWave soundchip which features

32 voices, 44.1kHz playback with effects like chorus and reverb

for each channel (and still its UltraSound MAX compatible).

THE PROBLEM

~~~~~~~~~~~

Gravis claimed that the GUS PnP is 100% UltraSound MAX

compatible.  But if you run the SETUP program, you will notice

that SETUP actually configures the CODEC chip's port (usually

port 32Ch) but when you run programs that detects the GUS MAX

specifically, you will find that the program cannot use the

CODEC chip (detects your card as a GUS instead of a GUS MAX).

THE SOLUTION

~~~~~~~~~~~~

From what I can tell, the IWINIT program will only initialize

the GUS PnP card in InterWave mode and GF1 mode.  The program

does not initialize the CODEC chip and as such, the CODEC is

not "visible" to programs searching for the CODEC chip.

What MAXMODE does is to first call the IWINIT program (to

initialize the GUS PnP in both InterWave and GF1 mode), then

set the ULTRA16 enviroment settings in DOS.  To initialize

the CODEC chip, MAXMODE will call the ULTRINIT program found

in the UltraSound Installation Disks v4.11 which is initialize

the GUS card in GF1 mode as well initialize the CODEC chip.

Once this is done, programs that detect the CODEC chip will be

able to locate the chip and thus be able to detect the "GUS MAX"

in your GUS PnP.  After calling the ULTRINIT program, MAXMODE

will call ULTRAMIX to initialize the cards mixer settings.

TO INSTALL MAXMODE

~~~~~~~~~~~~~~~~~~

Simple run MAXINSTL and it will install MAXMODE into your

UltraSound directory.

COPYRIGHT MESSAGE

~~~~~~~~~~~~~~~~~

ULTRAMIX and ULTRINIT are copyrighted programs belonging to

Advanced Gravis and were used/distributed without permission.