Date: Tue, 14 Jan 92 23:01:16 EST

From: jeff@millie.loc.gov (Jeff Mallory)

Subject: General MIDI Level Spec

****    Brief Overview of Proposed General MIDI Level 1 Spec   ****

The heart of General MIDI (GM) is the _Instrument Patch Map_, shown in

Table 1 (see below). This is a list of 128 sounds, with corresponding

MIDI program numbers. Most of these are imitative sounds, though the

list includes synth sounds, ethnic instruments and a handful of sound

effects.

The sounds fall roughtly into sixteen families of eight variations

each. Grouping sounds makes it easy to re-orchestrate a piece using

similar sounds. The Instrument Map isn't the final word on musical

instruments of the world, but it's pretty complete

General MIDI also includes a _Percusssion Key Map_, show in Table 2

(see below). This mapping derives from the Roland/Sequential mapping

used on early drum machines. As with the Instrument Map, it doesn't

cover every percussive instrument in the world, but it's more than

adequate as a basic set.

To avoid concerns with channels, GM restricts percussion to MIDI

Channel 10. Theoretically, the lower nine channels are for the

instruments, but the GM spec states that a sound module must respond

to all sixteen MIDI channels, with dynamic voice allocation and a

minimum of 24 voices.

General MIDI doesn't mention sound quality of synthesis methods.

Discussions are under way on standardizing sound parameters such as

playable range and envelope times. This will ensure that an arrangement

that relies on phrsing and balance can play back on a variety of

modules.

Other requirements for a GM sound module include response to velocity,

mod wheel, aftertouch, sustain and expression pedal, main volume and

pan, and the All Notes Off and Reset All Controllers messages. The

module  also must respond to both Pitch Bend and Pitch Bend Sensitivity

(a MIDI registered parameter). The default pitch bend range is +-2

semitones.

Middle C (C3) corresponds to MIDI key 60, and master tuning must be

adjustable.  Finally, the MIDI Manufacturers Association (MMA) created a

new Universal System Exclusive message to turn General MIDI on and off

(for devices that might have "consumer" and "programmable" settings).

Table 3 (see below) summarizes these requirements.

General MIDI has room for future expansion, including additional drum

and instrument assignments and more required controllers. Also under

discussion is an "authorizing document" that would standardize things

such as channel assignments (e.g., lead on 1, bass on 2, etc.) and setup

information in a MIDI file.

Copies of the Level 1 Specification documents for General MIDI ($5 each

at last notice) are available from the Internation MIDI Association,

5316 West 57th Street Los Angeles, CA 90056, (213) 649-6434. The first

issue of the Journal of the MMA (back issues, $15 each) contains an

article by PassPort Designs and Stanley Junglieb about General MIDI.

Roland's GS Standard

When Warner New Media first proposed a General MIDI standard, most MMA

members gave it little thought. As discussions proceeded, Roland

listened and developed a sound module to meet the proposed

specification. At the same NAMM show where the MMA ratified General MIDI

Level 1, Roland showed their Sound Brush and Sound Canvas, a Standard

MIDI File player and GM-compatible sound module.

Some companies feel that General MIDI doesn't go far enough, so Roland

created a superset of General MIDI Level 1, which they call GS Standard.

It obeys all the protocols and sound maps of General MIDI and adds many

extra controllers and sounds. Some of the controllers use Unregistered

Parameter Numbers to give macro control over synth parameters such as

envelope attack and decay rates.

The new MIDI Bank Select message provides access to extra sounds

(including variations on the stock sounds and a re-creation of the MT-32

factory patches). The programs in each bank align with the original 128

in General MIDI's Instrument Patch Map, with eight banks housing related

families. The GS Standard includes a "fall back" system. If the Sound

Canvas receives a request for a bank/program number combination that

does not exist, it will reassign it to the master instrument in that

family. A set of Roland System Exclusive messages allows reconfiguration

and customization of the sound module.

This means that a Roland GS Standard sound module will correctly play

back any song designed for General MIDI. In addition, if the song's

creator wants to create some extra nuance, they can include the GS

Standard extensions in their sequence. None of these extensions are so

radical as to make the song unplayable on a normal GM sound module.

After all, compatibility is what MIDI - and especially General MIDI - is

all about.

Music authors interested in the GS Standard should contact Tom White

at RolandCorp USA, 7200 Dominion Circle, Los Angeles, CA 90040, (213)

685-5141.

****     TABLE 1  -  General MIDI Instrument Patch Map      ****

(groups sounds into sixteen families, w/8 instruments in each family)

Prog#     Instrument               Prog#     Instrument

(1-8        PIANO)                   (9-16      CHROM PERCUSSION)

1         Acoustic Grand             9        Celesta

2         Bright Acoustic           10        Glockenspiel

3         Electric Grand            11        Music Box

4         Honky-Tonk                12        Vibraphone

5         Electric Piano 1          13        Marimba

6         Electric Piano 2          14        Xylophone

7         Harpsichord               15        Tubular Bells

8         Clav                      16        Dulcimer

(17-24      ORGAN)                      (25-32      GUITAR)

17        Drawbar Organ             25        Acoustic Guitar(nylon)

18        Percussive Organ          26        Acoustic Guitar(steel)

19        Rock Organ                27        Electric Guitar(jazz)

20        Church Organ              28        Electric Guitar(clean)

21        Reed Organ                29        Electric Guitar(muted)

22        Accoridan                 30        Overdriven Guitar

23        Harmonica                 31        Distortion Guitar

24        Tango Accordian           32        Guitar Harmonics

(33-40      BASS)                        (41-48     STRINGS)

33        Acoustic Bass             41        Violin

34        Electric Bass(finger)     42        Viola

35        Electric Bass(pick)       43        Cello

36        Fretless Bass             44        Contrabass

37        Slap Bass 1               45        Tremolo Strings

38        Slap Bass 2               46        Pizzicato Strings

39        Synth Bass 1              47        Orchestral Strings

40        Synth Bass 2              48        Timpani

(49-56     ENSEMBLE)                      (57-64      BRASS)

49        String Ensemble 1         57        Trumpet

50        String Ensemble 2         58        Trombone

51        SynthStrings 1            59        Tuba

52        SynthStrings 2            60        Muted Trumpet

53        Choir Aahs                61        French Horn

54        Voice Oohs                62        Brass Section

55        Synth Voice               63        SynthBrass 1

56        Orchestra Hit             64        SynthBrass 2

(65-72      REED)                         (73-80      PIPE)

65        Soprano Sax               73        Piccolo

66        Alto Sax                  74        Flute

67        Tenor Sax                 75        Recorder

68        Baritone Sax              76        Pan Flute

69        Oboe                      77        Blown Bottle

70        English Horn              78        Skakuhachi

71        Bassoon                   79        Whistle

72        Clarinet                  80        Ocarina

(81-88      SYNTH LEAD)                   (89-96      SYNTH PAD)

81        Lead 1 (square)           89        Pad 1 (new age)

82        Lead 2 (sawtooth)         90        Pad 2 (warm)

83        Lead 3 (calliope)         91        Pad 3 (polysynth)

84        Lead 4 (chiff)            92        Pad 4 (choir)

85        Lead 5 (charang)          93        Pad 5 (bowed)

86        Lead 6 (voice)            94        Pad 6 (metallic)

87        Lead 7 (fifths)           95        Pad 7 (halo)

88        Lead 8 (bass+lead)        96        Pad 8 (sweep)

(97-104     SYNTH EFFECTS)                (105-112     ETHNIC)

97        FX 1 (rain)              105       Sitar

98        FX 2 (soundtrack)        106       Banjo

99        FX 3 (crystal)           107       Shamisen

100        FX 4 (atmosphere)        108       Koto

101        FX 5 (brightness)        109       Kalimba

102        FX 6 (goblins)           110       Bagpipe

103        FX 7 (echoes)            111       Fiddle

104        FX 8 (sci-fi)            112       Shanai

(113-120    PERCUSSIVE)                  (121-128     SOUND EFFECTS)

113        Tinkle Bell              121       Guitar Fret Noise

114        Agogo                    122       Breath Noise

115        Steel Drums              123       Seashore

116        Woodblock                124       Bird Tweet

117        Taiko Drum               125       Telephone Ring

118        Melodic Tom              126       Helicopter

119        Synth Drum               127       Applause

120        Reverse Cymbal           128       Gunshot

****    TABLE 2  -  General MIDI Percussion Key Map    ****

(assigns drum sounds to note numbers. MIDI Channel 10 is for percussion)

MIDI   Drum Sound                MIDI    Drum Sound

Key                              Key

35     Acoustic Bass Drum        59      Ride Cymbal 2

36     Bass Drum 1               60      Hi Bongo

37     Side Stick                61      Low Bongo

38     Acoustic Snare            62      Mute Hi Conga

39     Hand Clap                 63      Open Hi Conga

40     Electric Snare            64      Low Conga

41     Low Floor Tom             65      High Timbale

42     Closed Hi-Hat             66      Low Timbale

43     High Floor Tom            67      High Agogo

44     Pedal Hi-Hat              68      Low Agogo

45     Low Tom                   69      Cabasa

46     Open Hi-Hat               70      Maracas

47     Low-Mid Tom               71      Short Whistle

48     Hi-Mid Tom                72      Long Whistle

49     Crash Cymbal 1            73      Short Guiro

50     High Tom                  74      Long Guiro

51     Ride Cymbal 1             75      Claves

52     Chinese Cymbal            76      Hi Wood Block

53     Ride Bell                 77      Low Wood Block

54     Tambourine                78      Mute Cuica

55     Splash Cymbal             79      Open Cuica

56     Cowbell                   80      Mute Triangle

57     Crash Cymbal 2            81      Open Triangle

58     Vibraslap

****     TABLE 3  -   General MIDI minimum sound module specs    ****

Voices:

A minimum of either 24 fully dynamically allocated voices

available simultaneously for both melodic and percussive sounds or 16

dynamically allocated voices for melody plus eight for percussion.

Channels:

General MIDI mode supports all sixteen MIDI channels. Each channel can

play a variable number of voices (polyphony). Each channel can play a

different instrument (timbre). Keybased Percussion is always on

Channel 10.

Instruments:

A minimum of sixteen different timbres playing various instrument

sounds. A minimum of 128 preset for Intruments (MIDI program numbers).

Note on/Note off:

Octabe Registration: Middle C(C3) = MIDI key 60. All Voices including

percussion respond to velocity.

Controllers:

Controller #   Description

1             Modulation

7             Main Volume

10             Pan

11             Expression

64             Sustain

121             Reset All Controllers

123             All Notes Off

Registered      Description

Parameter #

0               Pitch Bend Sensitivity

1               Fine Tuning

2               Coarse Tuning

Additional Channel Messages:

Channel Pressure (Aftertouch)

Pitch Bend

Power-Up Defaults:

Pitch Bend Amount = 0

Pitch Bend Sensitivity = +-2 semitones

Volume = 90

All Other Controllers = reset

(after Electronic Musician, 8/91 issue)