The Newcastle Handirom - an aid for the handicapped

Introduction:

The Handirom was developed to enable a person with limited motor control

to operate a microcomputer.  The  control  over  the  computer,  although

limited,  will  present  the  user  with full control over wordprocessing

utilities such as View 3.0 and  Wordwise  Plus,  various spreadsheets and

databases  or  even to write and run their own  programs  in  a  language

environment such as Basic, Comal, Pascal or Lisp etc.

In operation, the system claims the lower part of the screen on which it

draws up a grid representing  the  keys  on  the computer keyboard (in an

optimised  order).  The  grid  is scanned vertically whenever the program

requires keyboard input. Operation of  a  button  connected to the "fire"

button inputs on the Analogue socket at the  rear  of  the  computer will

cause  the  grid  to  be scanned horizontally until either the button has

been pressed again to select  a  letter  or command, or the grid has been

scanned horizontally twice. In this case the line is assumed to have been

wrongly selected and the vertical scan resumes.

A useful feature of the  Handirom  is  that  the  keyboard  operation is

mostly unaffected and the two input devices may be used together.

Five grid layouts are available; lower case, upper case, common words, a

control screen with function key  emulator and a "*" command screen. Each

of the grids also contains a  number  of commands which will be described

later.

The microcomputer system must be  fitted  with a bank of sideways ram in

order for the Handirom to work. In this way, the whole of the user memory

is available for documents or programs.

In  addition to the grids  mentioned  above,  custom  wordlists  may  be

created as  described  under  the section "Editing wordlists" on page 9.

These wordlists (of which three are supplied on the disc) are loaded into

sideways ram also, again maximising the amount of user ram.

Note:

As a number of sideways  ram  modules  are commercially available, it is

unlikely that the Handirom has been tested with all of them. In the event

of the Handirom not working please write to  the address given at the end

of this guide giving full system details and the  type  of  sideways  ram

fitted.

Page 1

The Newcastle Handirom - an aid for the handicapped

Handirom initialisation:

Having verified that sideways ram  is  available  and  with  some form of

switch  connected  to  the  analogue  port,  you  are ready to  load  the

Handirom.

Insert the disc into the  default disc drive and, while holding down the

SHIFT key, depress the BREAK key momentarily. If all is well, you will be

greeted  by  the  Handirom  welcome  screen.  Operating   the  switch  or

depressing any key (with the exception of the escape and break keys) will

prompt you for the sideways ram bank  that  you  wish  to  use  (refer to

manufacturers data).

If you do not wish to use socket 7 (Master) or socket 15 (BBC) (assuming

that they are available) then  selecting  the  NO  option  will  list the

sideways  firmware  installed  and  prompt  for bank 4-7 (Master) or 8-15

(BBC).

Upon selection of a suitable  socket,  the  Handirom  is loaded from the

disc and the user is asked to select the  speed  of the scan. Don't worry

if the speed is not quite right, as it can be adjusted later.

The sound on/off option is  next.  This just "blips" on each step of the

horizontal or vertical scan. Again, this option can be adjusted later.

Now select the wordprocessor option.  This  simply means that the system

will automatically insert spaces after a word  has been selected from the

wordlist or common words grid or even after  a  full  stop  (useful  when

writing  a  letter).  In  addition,  the full stop will cause the capital

letters screen to be selected for  one  character  only, after which, the

lower case screen will be selected.

The wordlist required may now  be  loaded.  Three  examples:  W.STDWORD,

W.TEST  and W.NEWWORD are supplied on the disc. The program "EDIT",  also

on the  disc  will allow the editing or creation of custom wordlists with

the Handirom active.  About 9K of a 16K bank of sideways ram is available

for the wordlist selected.

Page 2

The Newcastle Handirom - an aid for the handicapped

Saving the configuration:

If you are unfamiliar with  the  Handirom  please  answer  no  to  this

option.

A  yes  answer to save  the  configuration  will  prompt  for  a  blank

formatted disc to  be  placed  into the current drive.  Pressing a key or

the  switch  will  save the whole  of  the  sideways  ram  (Handirom  and

wordlist) together with details of  socket number and options selected to

the disc.  By copying the program  "SRLOAD"  onto  the disc (BBC B only),

the system may be loaded in with just SHIFT-BREAK,   without  the need to

step through the welcome and option screens again.

This method has the added advantage that it allows easy distribution of

the program throughout a class,   enabling each pupil to have his/her own

copy without the expense of multiple master copies (no,  we don't mind!).

The command below will work with most disc filing systems:

*COPY <source> <destination> <filename>

e.g. Assuming a single disc drive as drive number 0,  type:

*COPY 0 0 SRLOAD

followed by return.

Page 3

The Newcastle Handirom - an aid for the handicapped

Using the Handirom:

By now you should be  aware  of  a  scanning  grid at the bottom of the

screen and have probably tried pressing the button.  The grid  looks like

this:

>> :SCRN :WL :SPC :RET :DEL :CAP :ESC

i  t  r  o  l  p  .  ,  {  }  :REPEAT

e  a  c  m  x  v  :  ;  (  )  :CTRL

n  s  h  y  k  z  !  ?  <  >  :WWESC

d  f  g  q  +  -  *  /  =  &  \  ~

u  b  w  j  "  '  @  `  $  #  ^  |

0  1  2  3  4  5  6  7  8  9  [  ]

Speed 8  Word-processor

Try selecting :SPC from the grid.  To do this,  wait until the cursor

is  on  the  top line  and  press  the  button.   The  cursor  now  scans

horizontally.  Press your button once when :SPC is pointed to and a space

will be printed on the upper screen.

Other ":" directives:

:RET      this is identical to pressing the RETURN key on the keyboard.

:DEL      DELETEs the previous character.

:CAP      selects the capital letters screen.

:ESC      emulates the ESCAPE key.

:REPEAT   repeats the last letter typed until the button is pressed.

:CTRL     followed by a  letter  allows  control  keys to be emulated.

This is most useful for wordprocessor operation.

:WWESC    generates  an ESCAPE event and turns off  the  Handirom  grid

until the  button  is  pressed.   Useful for returning to the

Handirom  after  reading  a  screen that  would  normally  be

corrupted.

:WL       after selecting the initial letter  of  a word,  this command

will bring up a screen consisting of words beginning with the

same letter unless no wordlist has been loaded.

e.g. after selecting "a" and ":WL";

>> :NL  :QL

after       another       august

afternoon   anyone        away

again       anything      awry

almost      anywhere

always      apple

angry       april

Speed 8  Word-processor

Page 4

The Newcastle Handirom - an aid for the handicapped

:NL       on  this screen will bring up the next list if there are more

words beginning with the letter selected.

:QL       quits the wordlist screen.

:SCRN     will  allow  you  select any of the grids below (grid 0 being

the lower case screen).

SCREEN 1 - Upper case screen

>> :SCRN :WL :SPC :RET :DEL :CAP :ESC

I  T  R  O  L  P  .  ,  {  }  :REPEAT

E  A  C  M  X  V  :  ;  (  )  :CTRL

N  S  H  Y  K  Z  !  ?  <  >  :KEY

D  F  G  Q  +  -  *  /  =  &  \  %

U  B  W  J  "  '  @  `  $  #  ^  |

0  1  2  3  4  5  6  7  8  9  [  ]

Speed 8  Word-processor

You may have noticed the :KEY command that has crept in here.

This turns off the  grid,   clears the screen and then does a

RETURN.   Useful  when the whole screen  is  preferable  e.g.

LISTing a basic program  or  *TYPEing  text  from disc.  If a

CTRL-N (list in paged mode),  the button  acts like the SHIFT

key until the listing is finished.

A further press of the button will bring the grid back again.

SCREEN 2 - Commonly used words

>> :SCRN :WL :SPC :RET :DEL :CAP :ESC

and   cant   me    see    this  where

are   for    mine  that   to    will

be    have   my    the    we    with

but   know   not   then   was   would

can   like   of    there  what  you

his   her    by    their  on    off

Speed 8  Word-processor

SCREEN 3 - Control screen (for wordprocessors etc)

>> :SCRN :WL :SPC :RET :DEL :CAP :ESC

:CU     :CD     :CL     :CR   :REPEAT

:SCU    :SCD    :SCL    :SCR  :CTRL

:CCU    :CCD    :CCL    :CCR  :WWESC

:FK     :SFK    :CFK    :SCFK :KEY

:COPY   :SCOPY  :CCOPY  :SCCOPY

0  1  2  3  4  5  6  7  8  9

Speed 8  Word-processor

This  may look a little daunting but is really very simple.  When using

a wordprocessor  such  a  View  3.0,   the function keys are used for all

manner of things which is exactly what screen three allows you to do.

Page 5

The Newcastle Handirom - an aid for the handicapped

Control screen directives:

:CU   Cursor Up           :CD   Cursor Down

:CL   Cursor Left         :CR   Cursor Right

:SCU  Shift Cursor Up     :SCD  Shift Cursor Down

:SCL  Shift Cursor Left   :SCR  Shift Cursor Right

:CCU  Control Cursor Up   :CCD  Control Cursor Down

:CCL  Control Cursor Left :CCR  Control Cursor Right

:FK   Function key        :SFK  Shift Function Key

:CFK  Ctrl Function Key   :SCFK Shift Control Function Key

:COPY   COPY key          :SCOPY  Shift COPY key

:CCOPY  Control COPY      :SCCOPY Guess this one yourself

It  is  left  up to the user to become familiar with the intricacies of

the particular wordprocessor or spreadsheet etc which they prefer to use.

SCREEN 4 - Command screen.

:SCRN  :SPC  :RET  :DEL  :KEY  :ESC

*H.H.   *NOBEEP *WPON  *BEEP  *WLOAD

*BOOT   *SPEED  *WPOFF *OFF   *HELP

*CAT    *ROMS   *DISC  *TAPE  *ADFS

*TYPE   *INFO   *EXEC  *RUN   *LOAD

*BASIC  *WORD   *KEY   *SAVE  *FREE

*DUMP   *MOUNT  *TIME  *MOTOR *COPY

Speed 8  Word-processor

Page 6

The Newcastle Handirom - an aid for the handicapped

Star commands:

Typing *HELP HANDIROM or an  abbreviation  down  to  *H.H.  followed by

return will bring up the help screen shown below:

>*H.H.

Newcastle Handirom 1.20

BEEP

NOBEEP

BOOT    (<drv>)

CATALL

CLOSE

EXPALL

EXPLAIN

FILESYS

ON

OFF

ROMS    (<rom>)

SPEED

WLOAD   <fsp>

WPON

WPOFF

>

These commands will now be described:

Command: *BEEP / *NOBEEP

Description: Turns grid blip on or off.

Command: *BOOT (<drv>)

Description: Execute !BOOT file on disc.

Command: *CATALL

Description: Catalogue all files on ADFS disc.

Command: *CLOSE

Description: Close any open files. Equivalent to CLOSE#0 in BASIC.

Command: *EXPLAIN

Description: Catalogues disc and if any BASIC programs are found that

have a REM statement in the first line, EXPLAIN will echo the associated

text to the screen. Very useful for finding programs, latest versions etc.

Command: *EXPALL

Description: Performs the EXPLAIN command in all directories from the

currently selected one on an ADFS disc.

Command: *FILESYS

Description: Returns currently selected filing system.

Page 7

The Newcastle Handirom - an aid for the handicapped

Command: *ON / *OFF

Description: Turns Handirom  on or off.

Command: *ROMS (<rom>)

Description: Sideways firmware lister.

Command: *SPEED <0 - 9>

Description: Selects scanning speed for Handirom.

Command: *WLOAD <fsp>

Description: Loads a custom wordlist into Handirom.

Command: *WPON / *WPOFF

Description: Turns wordprocessor mode on or off.

Commands may be entered in upper or lower case and may be prefixed by the

letter "H" if necessary (e.g. *HON etc)

Page 8

The Newcastle Handirom - an aid for the handicapped

Editing Wordlists:

The wordlists may be edited,  or new ones created,  with or without the

Handirom active.

To do this,  type CHAIN"EDIT" or CH."EDIT" followed by RETURN (you must

be in BASIC to do this).

The editor is easy to use and prompts the user firstly for the filename

of  the wordlist to edit or RETURN for a new wordlist.  The wordlists are

all kept  in  directory  W but there is no need to specify this directory

with.

Try loading STDWORD then add  and  delete words to familiarise yourself

with the editor.  Using the F)inish option will prompt for a filename.

Do not use the original filename or you run the risk of corrupting your

data (assuming that the file is not locked).

Limitations:

Handirom  will NOT work with the Inter Word wordprocessor,  Acornsoft's

Basic Editor or any basic program using INKEY(n).  It is also unlikely to

work with programs or roms which use text or graphics windows.

A further  limitation  is  that  the  cursor  editing  and copying keys

produce   unpredictable  results  in  the  command  type  of  environment

i.e.  where  a  "*"  command  may  be  issued.  The best way to return to

normal  operation  after an anomaly has occurred is to  press  escape  or

return from the keyboard.

When using a  wordprocessor  such  as  View  3.0,   which  obeys window

definitions  on  startup,   it  is  necessary to have the Handirom active

before entering the command *WORD and  to  issue  the *WORD command after

each change of screen mode (within View,  of course).

Although  the  Handirom  is compatible with the BBC micro or the Master

with or without the  6502  Tube,   the  Tube  itself must be switched off

before attempting to load into sideways ram.  Once loaded,   the Tube may

be switched on and a CTRL-BREAK issued to initialise it.

Page 9

The Newcastle Handirom - an aid for the handicapped

Problems:

If  any  problems  are  experienced  with the Handirom,  please contact

either G J Flanagan or Rob Scott at:

Department of Medical Physics

Medical School

University of Newcastle upon Tyne

Newcastle upon Tyne

NE2 4HH

Page 10