The following notes describe the low level operation of the Amstrad Notepad

computers. They are intended for third party developers who want to program

the Notepad in machine code.

As always, I will try to help out if anyone has questions about this but I

cannot give an absolute guarantee to be able to provide support on the low

level operation of the machine.

It is our intention that these firmware routines and system variables should

be maintained in future issues of the software but we cannot give an absolute

guarantee about this.

Cliff Lawson                            CIS: 75300,1517

Notepad project manager               Phone: (+44) 277 208341

Amstrad Plc                             Fax: (+44) 277 208065

169 Kings Road

Brentwood

Essex

CM14 4EF

ENGLAND

I/O Specification for Amstrad NC100

<<<<<<<<<<<<<<<<<>>>>>>>>>>>>>>>>>>

All numbers are in hexadecimal unless suffixed with a "b" for binary or

"d" for decimal. (Address line numbers A19, A18, etc are in decimal).

SUMMARY

=======

Address                 Comment                 R/W

=======                 =======                 ===

E0-FF                   Not Used                -

D0-DF                   RTC (TC8521)            R/W

C0-C1                   UART (uPD71051)         R/W

B0-B9                   Key data in             R

A0                      Card Status etc.        R

90                      IRQ request status      R/W

80-8F                   Not Used

70                      Power on/off control    W

60                      IRQ Mask                W

50-53                   Speaker frequency       W

40                      Parallel port data      W

30                      Baud rate etc.          W

20                      Card wiat control       W

10-13                   Memory management       R/W

00                      Display memory start    W

In Detail

=========

Address = 00                                    Write only

start address of display memory

----------------------------------------------------------

bit 7           A15

bit 6           A14

bit 5           A13

bit 4           A12

bits 3-0        Not Used

On reset this is set to 0.

The display memory for the 8 line NC computers consists of a block of 4096

bytes where the first byte defines the state of the pixels in the top left

hand corner of the screen. A 1 bit set means the pixel is set to black. The

first byte controls the first 8 dots with bit 7 controlling the bit on the

left. The next 59 bytes complete the first raster line of 480 dots. The bytes

which define the second raster line start at byte 64 to make the hardware

simpler so bytes 60, 61, 62 and 63 are wasted. There are then another 64 bytes

(with the last 4 unused) which defines the second raster line and so on

straight down the screen. That is (all numbers decimal):

byte00   byte01   byte02      byte60   byte61     byte63

Bit Number      76543210 76543210 76543210 .. 76543210 76543210.. 76543210

Pixel Number    01234567 89012345 67890123 .. 23456789  wasted .. wasted

(read bottom    00000000 00111111 11112222    77777777

to top decimal) 00000000 00000000 00000000    44444444

....and so on for subsequent lines. (Second line = bytes 64..127 etc.)

Address = 10..13                              Read/Write

Memory management control

--------------------------------------------------------

10              controls 0000-3FFF

11              controls 4000-7FFF

12              controls 8000-BFFF

13              controls C000-FFFF

On reset all are set to 0.

For each address the byte written has the following meaning:

bit 7           together they select ROM, internal RAM, card RAM

bit 6                   00b = ROM

01b = internal RAM

10b = card RAM

bits 5-0        determine address lines 19 to 14.

Therefore, 00 is the first 16K of ROM, 01 is the second 16K, etc.

40 is the first 16K of internal RAM, 41=second 16K, etc.

80 is the first 16K of card RAM, 81=second 16K, etc.

So, for example, if you want to switch the third 16K of internal RAM so the

processor sees it at 4000-7FFF you would output the value 42 to I/O address

11.  42 has bits 7,6 = 01b and bits 5-0 are 00010b which is the third 16K of

internal RAM.

Address = 20                                    Write only

Memory card wait state control

----------------------------------------------------------

bit 7 = 1 for wait states, 0 for no wait

On reset this is set to 1. The bit should be set if the card RAM/ROM is

200nS or slower.

Address = 30                                    Write only

Baud rate etc.

----------------------------------------------------------

bit 7     select card register 1=common, 0=attribute

bit 6     parallel interface Strobe signal

bit 5     Not Used

bit 4     uPD4711 line driver, 1=off, 0=on

bit 3     UART clock and reset, 1=off, 0=on

bits 2-0  set the baud rate as follows

000 = 150

001 = 300

010 = 600

011 = 1200

100 = 2400

101 = 4800

110 = 9600

111 = 19200

On reset all data is set to 1.

If programming the UART directly ensure that TxD clock is operating x16.

Address = 40                                    Write only

Parallel interface data

----------------------------------------------------------

The byte written here is latched into the parallel port output register. To

print it you must then take the Strobe signal (I/O address 30 bit 6) low and

then high again. If the printer sends ACK this may generate an IRQ if the mask

bit is set in I/O address 60 - IRQ mask.

Address = 50..53                                Write only

Sound channels period control

----------------------------------------------------------

50      channel A period low

51      channel A period high

52      channel B period low

53      channel B period high

On reset all data is set to FF. The top bit in the high byte (51 and 53)

switches the resepective sound generator on or off - 1=off, 0=on.

The frequency generated is determined as:

Frequency = 1,000,000d

----------

data * 2 * 1.6276

So if the data word programmed into 50 and 51 was 7800 (ie 50=0, 51=78) then

the frequency generated would be:

freq = 1,000,000           =  1,000,000          = 1,000,000   = 10Hz

---------              ---------            ---------

7800h * 2 * 1.6276     30720 * 2 * 1.6276      99,999.7

Address = 60                                    Write only

Interrupt request mask

----------------------------------------------------------

bits 7-4        Not Used

bit 3           Key Scan interrupt (every 10mS)

bit 2           ACK from parallel interface

bit 1           Tx Ready from UART

bit 0           Rx Ready from UART

On reset all bits are 0. For each bit, 1=allow that interrupt source to

produce IRQs. 0 = interrupt source is masked.

Address = 70                                    Write only

Power off control

----------------------------------------------------------

bits 7-1        Not Used

bit 0           1 = no effect, 0 = power off

On reset this is set to 1.

Address = 90                                     Read/Write

IRQ status

-----------------------------------------------------------

bits 7-4        Not Used

bit 3           Key scan

bit 2           ACK from parallel interface

bit 1           Tx Ready interrupt

bit 0           Rx Ready interrupt

When an interrupt occurs this port should be read to determine the source of

the interrupt. The bit will be set to 0 to identify the interrupting device.

The interrupt can then be cleared by writing 0 to that bit.

Address = A0                                     Read only

Memory card/battery status

----------------------------------------------------------

bit 7           Memory card present 0 = yes, 1 = no

bit 6           Card write protected 1 = yes, 0 = no

bit 5           Input voltage = 1 if >= to 4 Volts

bit 4           Mem card battery. 0 = battery is low

bit 3           Alkaline batteries. 0 if >= 3.2 Volts

bit 2           Lithium battery. 0 if >= 2.7 Volts

bit 1           Parallel interface BUSY (0 if busy)

bit 0           Parallel interface ACK  (1 if ACK)

Address = B0 - B9                                 Read only

Keyboard data

-----------------------------------------------------------

B0..B9          each key of the 64 on the keyboard

will set a bit in one of these bytes

while pressed.

The gate array scans the keyboard every 10mS and then generates an

interrupt. The program should then read these 10 I/O locations to

determine which key(s) is pushed. When I/O address B9 is read the

key scan interrupt is cleared automatically and the next scan cycle

will start from B0.

Address = C0                                      Read/Write

UART control/data

------------------------------------------------------------

C0              UART data register

C1              UART status/control register

The UART is the NEC uPD71051. Programmers are advised to study the data

sheet for that chip for more information. The Serial interface requires

that the uPD4711 line driver chip be truned on by writing a 0 to bit 4 of

I/O address 30. While turned on power consumption increases so this should

only be done when necessary.

Address = D0                                    Read/Write

Real Time Clock chip (TM8521)

----------------------------------------------------------

D0..DC          Data

DD              Control register

DE              Control register        (Write only)

DF              Control register        (Write only)

See data sheet of chip for more information.

=================================================================

NC100 operating system firmware

<<<<<<<<<<<<<<<<>>>>>>>>>>>>>>>

notes for external program writers

==================================

To get external programs executed on the Notepad you could either POKE

them into memory in BBC BASIC (or even use its built-in Z80 assembler)

and then CALL the entry point. However, this does have the drawback of

needing to transfer the code back to the machine each time it crashes

(as it inevitably will).

The simplest way to develop for the Notepad is to get a PCMCIA drive

for your PC and write a binary image direct to the card using that. If

this isn't possible then small programs (up to 16K) can be developed by

transferring the binary card image into the Notepad using Xmodem from

the PC. The use the "Make program card" feature in the File, transfer

menu to write that file onto a newly formatted PCMCIA RAM card.

In either case, to run the resultant code, you just press Function-X

(eXecute) and the first 16K page of the RAM card will be switched to

the Z80 memory map at C000..FFFF. A Check is made that location C200

holds the ASCII text "NC100PRG" and also that locations C210..C212

contains a long jump to C220. All being well, the Z80 starts executing

code at C210 so that, once you have control, you can take over

completely if you wish (driving all hardware functions directly). Most

people will probably want to cooperate with the in built firmware as it

provides most of the routines that one would require anyway.

The ASCII text "NC100PRG" must appear at C200h

program origin is C210h

program MUST start with jp C220h

the program name is at C213h, max 12 characters, zero terminated

org C200h

db "NC100PRG"

org c210h

jp start

db "PROGRAM NAME",0

org C220h

start

available workspace A000h to A3FFh (shared with other programs)

also A800h to AFFFh (this is overwritten if selectfile is called)

the program MUST handle yellow events :-

either exit when Stop is pressed

or check for yellow event with kmgetyellow and return if carry set

Serious developers may be interested in contacting Ranger Computers Ltd

on (+44) 604 589200 as they can produce a device that looks like RAM to

a PC but ends in a PCMCIA header plug that connects directly to the

Notepad's card slot and the "PC RAM" appears as card RAM to the Notepad.

The following sequence is a working(!) piece of code written for the

AVMACZ80 assembler on a PC, which, when assmembled produces a binary

file that can be programmed onto a PCMCIA card and executed. The

program just reads keys and prints them back until "Q" is pressed.

Notice that exit from the program is just by a RET back to the

operating system that called it:

;VVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVVV

include "nc100jmp.inc"          ;The list of firmware routine

;addresses given later in this

;file

DEFSEG  Fred, CLASS=CODE, START=0

SEG     Fred    ;Seg will be linked to RUNSAT C000h

jp      start   ;put a jump at the start in case this code is

;ever programmed into a ROM page where the entry

;will almost certainly be made at the more

;normal C000.

ds      509     ;waste first 512 bytes of card to start at C200

;

; following 16 bytes are Arnor's header for card at C200

;

db      "NC100PRG",0,0,0,0,0,3,0,1

;

; then card program must start with this long jump at C210

;

jp      start                   ;this is at C210h

db      "CLIFFS PROG",0,0       ;0's pad to C220h

start:

call    kmreadchar

ld      a,c

cp      "q"

jr      z,finish

call    txtoutput

jr      start

finish:

ret

end

;^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

; code is assembled with:

;       AVMACZ80 TEST.ASM

;which produces a .OBJ file which is then linked to produce a .HEX file

;with the command

;       AVLINK @TEST.LNK

;where TEST.LNK contains:

;       TEST.HEX=TEST.OBJ -RUNSAT(Fred, 0C000h)

;finally the Intel .HEX file is converted to .BIN using a HEX2BIN converter

;The .BIN file is either written to the PCMCIA card using a PC based

;card drive or it can be Xmodemed across to the Notepad and written to

;the card using "Make program card". Finally, Function-X executes it.

In other assemblers you may not have "segments" and must use a direct

ORG to locate code at C000 but watch out for the resultant .HEX file

being padded out with 48K of "0"s from 0000 to BFFF!!

Alphabetic list of routine entry points

=======================================

To use any one of these routines just load the registers as described in

the following and then call the relevant address. Although the running of the

routine may involve a different ROM bank being switched in, this mechanism

is invisble to the caller. So, for example, to print a capital A one might use:

txtoutput       EQU     B833

LD      A,"A"

CALL    txtoutput

col1            equ B818h

col1text        equ B81Bh

editbuf         equ B800h

fclose          equ B890h

fdatestamp      equ B8C9h

ferase          equ B893h

fgetattr        equ B8CFh

finblock        equ B896h

finchar         equ B899h

findfirst       equ B89Ch

findnext        equ B89Fh

fnoisy          equ B917h

fopenin         equ B8A2h

fopenout        equ B8A5h

fopenup         equ B8A8h

foutblock       equ B8ABh

foutchar        equ B8AEh

fquiet          equ B91Ah

frename         equ B8B1h

fseek           equ B8B4h

fsetattr        equ B8CCh

fsize           equ B8B7h

fsizehandle     equ B8BAh

ftell           equ B8BDh

ftesteof        equ B8C0h

heapaddress     equ B87Eh

heapalloc       equ B881h

heapfree        equ B884h

heaplock        equ B887h

heapmaxfree     equ B88Ah

heaprealloc     equ B88Dh

kmcharreturn    equ B803h

kmgetyellow     equ B8D2h

kmreadkbd       equ B806h

kmreadchar      equ B9B3h

kmsetexpand     equ B809h

kmsettickcount  equ B80Ch

kmsetyellow     equ B8D5h

kmwaitkbd       equ B80Fh

lapcat_receive  equ B8D8h

lapcat_send     equ B8DBh

mcprintchar     equ B851h

mcreadyprinter  equ B854h

mcsetprinter    equ B857h

padgetticker    equ B872h

padgettime      equ B875h

padgetversion   equ B8DEh

padinitprinter  equ BA4Fh

padinitserial   equ B85Ah

padinserial     equ B85Dh

padoutparallel  equ B860h

padoutserial    equ B863h

padreadyparallel equ B866h

padreadyserial  equ B869h

padresetserial  equ B86Ch

padserialwaiting equ B86Fh

padsetalarm     equ B878h

padsettime      equ B87Bh

pagemodeon      equ BA49h

pagemodeoff     equ BA4Ch

readbuf         equ B812h

selectfile      equ B8C3h

setdta          equ B8C6h

testescape      equ B815h

textout         equ B81Eh

textoutcount    equ B821h

txtboldoff      equ B83Fh

txtboldon       equ B842h

txtclearwindow  equ B824h

txtcuroff       equ B827h

txtcuron        equ B82Ah

txtgetcursor    equ B82Dh

txtgetwindow    equ B830h

txtinverseoff   equ B845h

txtinverseon    equ B848h

txtoutput       equ B833h

txtsetcursor    equ B836h

txtsetwindow    equ B839h

txtunderlineoff equ B84Bh

txtunderlineon  equ B84Eh

txtwrchar       equ B83Ch

Notepad memory map

==================

16K code/data sections always mapped to C000h

���������������Ŀ        �����������������������������������������������Ŀ

�   video RAM   �        �Protext �Dictionary�Con-�Calc�Addr�Diary� BBC  �

�---------------�        �        �  data    �trol�    �book�     �BASIC �

�    RAM        �        � 1 & 2  � 6 blocks �    �    �    �     �      �

���������������Ĵ C000   �������������������������������������������������

�stack/variables�       \

�---------------� B000  | common RAM (accessible by all programs)

�    RAM        �       /

���������������Ĵ 8000                        ���������������Ŀ

�               �                             �               �

�    RAM        �                             �     PLS       �

�               �                             �               �

���������������Ĵ 4000   ���������������Ŀ    �    spell      �

�               �        �OS- remaps high�    �   checking    �

�    RAM        �        �---------------�    �     code      �

�               �        � Startup code  �    �               �

����������������� 0      �����������������    �����������������

general notes:

most routines return carry set if successful

unless otherwise stated assume AF corrupted, other regs preserved

"all registers preserved" includes flags, but NOT alternate registers

the ALTERNATE register contents can NEVER be assumed to be preserved

(they are used as scratch registers in time critical routines)

==============

editbuf = B800

==============

line editor with options

zero-terminated string may be passed in buffer (HL)

this will display the initial contents

ENTRY - HL : pointer to input buffer

B : size of buffer (excluding terminating zero)

A : flags.      b2=1 -> ## terminate entry

b3=1 -> input not echoed

b6=1 -> dotty background (character 176)

b5=1 -> edit unless characters entered

b4=1 -> delete trailing spaces

Other bits must be set to zero.

EXIT  - c=0 z=1 ESC pressed

c=1 z=1 empty string input

c=1 z=0 at least one character entered

HL preserved

BC = last key token (or -1 if ESC used to terminate)

===================

kmcharreturn = B803

===================

returns a token to the keyboard buffer

ENTRY - BC = the token

EXIT - all registers preserved

================

kmreadkbd = B806

================

Gets a key token if there is one, does not wait

(Checks put back character and expands macros)

Returns tick event tokens if enabled

ENTRY - none

EXIT  c=1 : BC = token (B=0 for simple character)

c=0 : no key token available

=================

kmreadchar = B9B3

=================

This routine is the same as kmreadkbd but macros are

exapnded and one or two other "behind the scenes" tasks

are performed. By using this routine you can be sure that

the Ctrl+Shift+S screen dump mechanism works in your code

==================

kmsetexpand = B809

==================

Defines a macro string

ENTRY - BC = macro token (between 256 and 383)

- HL points to new macro string (first byte is the length,

followed by the string - need not be zero terminated)

EXIT  - c=1 if macro defined successfully

c=0 if insufficient room in buffer

(The buffer size is user configurable)

=====================

kmsettickcount = B80C

=====================

Enables the ticker event

There are 100 ticks per second

When a ticker event occurs t.tickevent is returned by kmreadkbd

ENTRY - HL : number of ticks before first event

DE : number of ticks between events

EXIT - all registers preserved

================

kmwaitkbd = B80F

================

Waits for a key token, uses kmreadkbd

(Checks put back character and expands macros)

Returns tick event tokens if enabled

ENTRY - none

EXIT  c=1 : BC = token (B=0 for simple character)

==============

readbuf = B812

==============

line editor. See also editbuf.

ENTRY - HL : pointer to input buffer (empty)

B : size of buffer (excluding terminating zero)

EXIT  - c=0 z=1 ESC pressed

c=1 z=1 empty string input

c=1 z=0 at least one character entered

HL preserved

BC = last key token (or -1 if ESC used to terminate)

=================

testescape = B815

=================

tests whether an ESC key has been pressed (STOP or FUNCTION)

waits for a key if one is found in the keyboard buffer

ENTRY - none

EXIT  - c=1 if no ESC key in buffer

c=1 if ESC key in buffer but STOP not pressed

c=0 if ESC key in buffer and STOP then pressed

A is preserved

===========

col1 = B818

===========

if cursor is at start of a line do nothing

otherwise move cursor to start of next line (within window)

ENTRY - none

EXIT  - none

===============

col1text = B81B

===============

same as textout, but calls col1 first

==============

textout = B81E

==============

displays string

ENTRY - HL : pointer to zero-terminated string

***************************************************************

WARNING - HL must not point into an upper ROM!

***************************************************************

EXIT  - none

===================

textoutcount = B821

===================

as textout, returns character count in B

=====================

txtclearwindow = B824

=====================

clears current window and moves cursor to top left

================

txtcuroff = B827

================

removes the cursor from the screen

ENTRY - none

EXIT  - all registers preserved

===============

txtcuron = B82A

===============

displays the cursor on the screen

ENTRY - none

EXIT  - all registers preserved

===================

txtgetcursor = B82D

===================

returns the cursor position

ENTRY - none

EXIT  - H = column (between 0 and 79)

L = row (between 0 and 7)

===================

txtgetwindow = B830

===================

returns the window coordinates

ENTRY - none

EXIT  - H = left column (between 0 and 79)

L = top row (between 0 and 7)

D = right column (between 0 and 79)

E = bottom row (between 0 and 7)

c=0 if window is whole screen

c=1 if a smaller window has been

================

txtoutput = B833

================

displays a character or acts on control code

ENTRY - A = character

A=7 : beeps   A=10 : LF   A=13 : CR

All other values displayed as character (PC char. set)

EXIT - all registers preserved

===================

txtsetcursor = B836

===================

moves the cursor

ENTRY - H = column (between 0 and 79)

L = row (between 0 and 7)

EXIT - none

===================

txtsetwindow = B839

===================

defines a new window

ENTRY - H = left column (between 0 and 79)

L = top row (between 0 and 7)

D = right column (between 0 and 79)

E = bottom row (between 0 and 7)

EXIT  - none

================

txtwrchar = B83C

================

displays a character

ENTRY - A = character. All values displayed (PC char. set)

EXIT - all registers preserved

======================

txtboldoff      = B83F

txtboldon       = B842

txtinverseoff   = B845

txtinverseon    = B848

txtunderlineoff = B84B

txtunderlineon  = B84E

======================

These six routines enable or disable various display

attributes. They have no entry conditions and preserve all registers.

==================

mcprintchar = B851

==================

sends a character to the printer

ENTRY - A=character

EXIT  - c=1 if successful

c=0 if not sent

A preserved

=====================

mcreadyprinter = B854

=====================

tests whether the printer is ready

ENTRY - none

EXIT  - c=0 if busy

c=1 if ready

A preserved

===================

mcsetprinter = B857

===================

sets the printer type to be used by mcprintchar and mcreadyprinter

ENTRY - A=printer type, 0=parallel, 1=serial

EXIT  - none

====================

padinitserial = B85A

====================

initialises the serial port using the global configured settings

turns on the UART and 4711

do not call this until needed - to prolong battery life

ENTRY - none

EXIT - none

==================

padinserial = B85D

==================

reads a character from the serial port

ENTRY - none

EXIT  - c=1 if successful, A=character

c=0 if no character read

=====================

padoutparallel = B806

=====================

sends a character to the parallel port

ENTRY - A=character

EXIT  - c=1 if successful

c=0 if not sent

A preserved

===================

padoutserial = B863

===================

sends a character to the serial port

ENTRY - A=character

EXIT  - c=1 if successful

c=0 if not sent

A preserved

=======================

padreadyparallel = B866

=======================

tests whether the parallel port is ready

ENTRY - none

EXIT  - c=0 if busy

c=1 if ready

A preserved

=====================

padreadyserial = B869

=====================

tests whether the serial port is ready

ENTRY - none

EXIT  - c=0 if busy

c=1 if ready

A preserved

=====================

padresetserial = B86C

=====================

turns off the UART and 4711

call this when finished using the serial port to prolong battery life

ENTRY - none

EXIT - none

=======================

padserialwaiting = B86F

=======================

tests whether there is a character waiting to be read

from the serial port

ENTRY - none

EXIT  - c=1 if character waiting

c=0 if no character waiting

===================

padgetticker = B872

===================

returns the address of a 4 byte 100Hz ticker

ENTRY - none

EXIT  - HL is the address of the least significant byte

=================

padgettime = B875

=================

reads the time and date from the RTC

ENTRY - HL points to an 7 byte buffer to use

EXIT  - HL preserved

data returned as above (see padsettime)

==================

padsetalarm = B878

==================

sets the ALARM date and time (within next month)

ENTRY - HL points to 3 byte data area

byte 0=date 1=hour  2=minute

EXIT  - none

=================

padsettime = B87B

=================

sets the RTC date and time

ENTRY - HL points to 7 byte data area

bytes 0,1 = year (low,high)

2=month  3=date

4=hour  5=minute  6=second

EXIT  - none

==================

heapaddress = B87E

==================

obtains the address of a memory block for a given memory handle

ENTRY - DE = memory handle

EXIT  - HL = pointer to memory block

================

heapalloc = B881

================

allocates a block of memory from the heap

ENTRY - DE = number of bytes to allocate

EXIT  - HL = memory handle in range [1,63] if successful

HL = 0 if failed

Note:   heapaddress must be used to get a pointer to the memory block

Unless the block is locked with heaplock, heapaddress must be

called each time the memory block is used. IT MAY HAVE MOVED!

===============

heapfree = B884

===============

frees a block of memory

ENTRY - DE = memory handle, returned by heapalloc or heaprealloc

EXIT  - none (preserves HL,BC)

Note: the memory handle passed must be a valid handle returned by

heapalloc or heaprealloc. This is not validated.

===============

heaplock = B887

===============

locks or unlocks a memory block

ENTRY - DE = memory handle

BC = non zero - the block is locked. It will not be moved

until unlocked so fixed addresses can be used as

pointers into the block

BC = 0 - the block is unlocked

==================

heapmaxfree = B88A

==================

returns the largest block size that can be allocated

ENTRY - none

EXIT  - HL = largest free block size in bytes

==================

heaprealloc = B88D

==================

changes the size of an allocated memory block

ENTRY - DE = memory handle

BC = new size for memory block

EXIT    HL = zero if failed to reallocate

The old block will not be freed but could have moved

HL = non-zero if successful

Note: if the block is being expanded, it must be assumed that the

base of the memory block will be moved (even if the block cannot

actually be expanded) so heapaddress must be called afterwards.

If the block is being contracted, the base will not move.

=============

fclose = B890

=============

closes a file

ENTRY - DE = file handle

EXIT  - c=1 if successful, c=0 if failed

=============

ferase = B893

=============

erases a file

ENTRY - HL = zero-terminated filename

EXIT  - c=1 if OK, c=0 if error (file not found)

===============

finblock = B896

===============

reads a block from a file

ENTRY - DE = file handle

- HL = buffer

- BC = number of bytes to read (> 0)

EXIT  - c=1 if end of file not reached

- c=0 if eof (or error?)

- BC = number of bytes read

- HL = address after last byte read

KNOWN BUG (1.00,1.01) - finblock does not set the file position

so repeated calls will always read from the start of the file

Workaround: call fseek after calling finblock to set the pointer

============

finchar B899

============

reads a byte from a file

ENTRY - DE = file handle

EXIT  - c=1 if successful, A=character

c=0, A corrupt if end of file reached

other regs preserved

================

findfirst = B89C

================

finds first file. setdta must have been called first

ENTRY - none

EXIT  - HL=0 if no files

- HL points to file info structure if file found

- 1st item in structure is the filename, zero-terminated

-       (up to 12 characters long)

- offset 13 is attribute byte

- offset 14/15 is the file size in bytes

===============

findnext = B89F

===============

finds next file. findfirst must have been called first

ENTRY - none

EXIT  - HL=0 if no more files

- HL as findfirst if file found

==============

fopenin = B8A2

==============

opens a file for input

ENTRY - HL points to zero-terminated filename

EXIT  - c=1 if successful, DE=file handle

c=0 if failed (file not found)

DE corrupt if error

A corrupt, other regs preserved

===============

fopenout = B8A5

===============

opens a file for output

ENTRY - HL points to zero-terminated filename

EXIT  - c=1 if successful, DE=file handle

c=0 if failed (out of memory/too many files/file exists)

DE corrupt if error

A corrupt, other regs preserved

==============

fopenup = B8A8

==============

opens a file for input and output.

the file must exist already

ENTRY - HL points to zero-terminated filename

EXIT  - c=1 if successful, DE=file handle

c=0 if file not found

DE corrupt if error

A corrupt, other regs preserved

================

foutblock = B8AB

================

writes a block to a file

ENTRY - DE = file handle

- HL = buffer

- BC = number of bytes to write (> 0)

EXIT  - c=1 if OK

- c=0 if error

- BC = number of bytes written

- HL = address after last byte written

===============

foutchar = B8AE

===============

writes a byte to a file

ENTRY - DE = file handle

- A  = character

EXIT  - c=1 if successful

c=0, A corrupt if end of file reached

A corrupt, other regs preserved

==============

frename = B8B1

==============

renames a file

ENTRY - HL = zero-terminated old filename

- DE = zero-terminated new filename

EXIT  - c=1 if OK, c=0 if error (file not found)

============

fseek = B8B4

============

moves the file pointer to a position within a file

ENTRY - DE = file handle

- BC = offset from start of file

EXIT  - c=1 if successful

c=0 if offset past end of file (pointer not changed)

KNOWN BUG (1.00,1.01) - leaves error messages enabled (fnoisy)

Workaround: call fquiet after fopenout if necessary

============

fsize = B8B7

============

finds size of file

ENTRY - HL = zero-terminated filename

EXIT  - c=1  HL=size in bytes, if found

- c=0 if not found

==================

fsizehandle = B8BA

==================

finds size of an open file

ENTRY - DE = file handle

EXIT  - HL=size in bytes

============

ftell = B8BD

============

returns the value of the file pointer

ENTRY - DE = file handle

EXIT  - HL = current file position

===============

ftesteof = B8C0

===============

tests whether end of file has been reached

ENTRY - DE=file handle

EXIT  - c=1 if not eof, c=0 if eof

=================

selectfile = B8C3

=================

displays the file selector (clears the screen first)

shows all files and allows a selection to be made

using the cursor keys and RETURN

ENTRY - none

EXIT  - c=1 if a file selected (RETURN pressed)

HL = filename

- c=0 if STOP pressed

=============

setdta = B8C6

=============

set memory block to be used by findfirst/findnext

ENTRY - DE= address of buffer (at least 35 bytes long)

buffer must be in common RAM (8000h-BFFFh)

EXIT  - none

=================

fdatestamp = B8C9

=================

sets file date/time to current date/time

ENTRY - HL = zero terminated filename

EXIT  - c=1 if successful

- c=0 if not found

===============

fsetattr = B8CC

===============

sets the attribute byte for a file open for output

if the file is open for input only there is no effect

ENTRY - DE = file handle

- C  = attribute byte

bit 0 = system file

bit 1 = hidden file

bit 2 = BASIC program

bit 3 = binary file

EXIT  - c=1 if successful

- c=0 if not found

===============

fgetattr = B8CF

===============

returns attribute byte of file

ENTRY - HL = zero-terminated filename

EXIT  - c=1  A=attribute, if found

- c=0 if not found

preserves HL

==================

kmgetyellow = B8D2

==================

ascertains whether a 'yellow event' is pending

(so called because the FUNCTION key is coloured yellow)

a yellow event occurs (i) when the user has pressed one of the

the FUNCTION+key combinations that cause an immediate context switch

(FN+red, FN+green, FN+blue, FN+menu)

or (ii) when the machine is powered up and (because the option to

preserve context has not been set) needs to return to the main menu

ENTRY -  none

EXIT  -  c=1, BC=token if yellow event pending

An application should exit normally as quickly as possible

Any UNSAVED FILES should be SAVED AUTOMATICALLY!

-  c=0, BC=0 if no yellow event pending

Note: each of the yellow event keys return the ESC token (2FCh)

An application should call kmgetyellow whenever an ESC is read,

this distinguishes between a yellow event and an ordinary ESC.

==================

kmsetyellow = B8D5

==================

sets up a yellow event. Specialised use only.

ENTRY - BC = a yellow event token

EXIT  - none

=====================

lapcat_receive = B8D8

=====================

reads a character from the parallel port using Lapcat protocol

ENTRY - none

EXIT  - c=1 if successful, A=character

c=0 if no character read

==================

lapcat_send = B8DB

==================

sends a character to the parallel port using Lapcat protocol

ENTRY - A=character

EXIT  - c=1 if successful

c=0 if error

====================

padgetversion = B8DE

====================

gets the firmware version number

ENTRY - none

EXIT  - HL = version number (*100)

Thus, 1.03 returns 103

System variables

<<<<<<<<>>>>>>>>

The following are the RAM based variables used by the operating system. It

is hoped that they will always use these locations in subsequent versions

of the software - but this is not guaranteed.

B000   copyofmmu0      ds 1    ; copy of MMU0 since it's a write-only port

B001   copyofmmu1      ds 1    ; copy of MMU1 since it's a write-only port

B002   copyofmmu2      ds 1    ; copy of MMU2 since it's a write-only port

B003   copyofmmu3      ds 1    ; copy of MMU3 since it's a write-only port

B004   gotcontext      ds 1

B005   __savepearlmmu  ds 1    ; extra vars needed in case we mustn't save context

B006   __saveaf        ds 2

B008   __savehl        ds 2

B00A   saveaf          ds 2    ; to save context, we need to save all the registers ...

B00C   savebc          ds 2

B00E   savede          ds 2

B010   savehl          ds 2

B012   saveix          ds 2

B014   saveiy          ds 2

B016   savepc          ds 2

B018   savesp          ds 2

B01A   saveafdash      ds 2

B01C   savebcdash      ds 2

B01E   savededash      ds 2

B020   savehldash      ds 2

B022   savemmu0        ds 1    ; ... and the memory state

B023   savemmu1        ds 1

B024   savemmu2        ds 1

B025   savemmu3        ds 1

B026   savecritpc      ds 2

B028   savecritsp      ds 2

B02A   savingcontext ds 1

B02B   nmimagic        ds 4

B02F   nmichksums      ds 8    ; checksum bytes of first 8 roms

B037   criticalpc      ds 2    ; save pc,sp for recovery from NMI during IRQ

B039   criticalsp      ds 2

B03B                   ds 80   ; A small stack which we only use in initialisation.

; It can't sensibly overlap with anything in case we get an NMI

; requring immediate shut down after saving context.

; Subsequent power on will have to restore the context

B08B   initstack

B08B   diagnostics?    ds 1    ; flag used in start-up, nonzero to do diagnostics

B08C   saveprinstat    ds 1

B08D   kbdstate1       ds 10   ; 1 bit per key, 1=down 0=up  corresponds to matrix

B097   kbdstate2       ds 10

PADKEYBUFLEN           equ 32          ; this MUST be 2^n for positive integer n

B0A1   padkeybuf       ds PADKEYBUFLEN*2

B0E1   padnextin       ds 1            ; offset into padkeybuf

B0E2   padnextout      ds 1

B0E3   padbufempty     ds 1            ; nonzero if empty

B0E4   lastkbdstate    ds 2

B0E6   thiskbdstate    ds 2

B0E8   caps.state      ds 1            ; 0=off FF=on

B0E9   savecaps        ds 1

B0EA   justswitchedon? ds 1

; variables above here are preserved after timeout

PADSERBUFLEN           equ 32          ; this MUST be 2^n for positive integer n

B0EB   padserbuf       ds PADSERBUFLEN

B10B   padsernextin    ds 1

B10C   padsernextout   ds 1

B10D   padserbufempty  ds 1

B10E   padserin_xoff   ds 1    ; non-zero when XOFF has stopped inward transmission

B10F   padserout_xoff  ds 1    ; non-zero when XOFF has stopped outward transmission

B110   disablexonxoff  ds 1    ; nonzero to disable software handshake

B111   ackirq          ds 1    ; set non-zero when ACK interrupt occurs

B112   rptdelay        ds 1    ; centisecs

B113   rptrate         ds 1    ; centisecs

B114   rpttimer        ds 1    ; count down timer for key repeat

B115   keytorepeat     ds 1    ; key number

B116   rptkeystates    ds 1    ; shift states

B117   rtcbuf          ds 13

B124   d.alarmday      ds 6    ; alarm day,hour,min ready for rtc chip

B12A   alarmhappened   ds 1    ; non zero when alarm has gone off, message pending

B12B   alarmhappenedgotmsg     ds 1    ; non zero when alarm has gone off, got message & pending

B12C   soundcounter    ds 1    ; non-zero if we're playing a tune

B12D   soundptr        ds 2    ; pointer to array of frequency,duration

B12F   soundrepcount   ds 1

B130   soundrepptr     ds 2

B132   poweroffminutes ds 1    ; configured time to power off

B133   minutesleft     ds 1

B134   minutecounter   ds 2

B136   eventhappened   ds 1

B137   preservecontext ds 1    ; 0=return to main screen at power on

B138   dontpreservecontext     ds 1    ; 1=dont preserve (diag/batt)

B139   mainprog        ds 1    ; 6=inbasic, 128=inexternal (foreground program id)

B13A   currentprinter  ds 1    ; 0 for parallel, 1 for serial

B13B   currentmenu     ds 2    ; pointer to current menu

B13D   wasmenusel      ds 1    ; after kmwaitchar this is 1 if menu used, 0 if not

; need this in fsel to know whether redraw needed

B13E   lastsecond      ds 1    ; checked to see whether to update the time

B13F   clockon?        ds 1    ; uses in Protext, non-zero when clock is enabled

B140   sdumpname       ds 4    ; s.a, s.b, s.c etc. for screen dump name

; force d.workspace to an 8 byte boundary

B148   d.workspace     ds 8    ; for massaged copy of symbol data (eg inverse/underline)

B150   d.datebuf       ds 9+MAXMONTHLEN        ; 27 January 1992

B162   d.asciitime     ds 12                   ; hh:mm:ss xm\0

B16E   currentcfg      ds cfg.len

B1BA   g.outstream     ds 1    ; bit 0 for screen, 1 for printer, 2 for file

B1BB   g.h.outfile     ds 2    ; file handle for charout if bit 2 set

B1BD   g.pos           ds 1    ; current column number (charout)

B1CE   def.fname       ds      MAXPNLEN+1      ; Name of current file being edited

; first byte not zero if document open

; (yellow/red goes to edit mode, transfer from addrbook works)

B1DD   def.first       ds 1

; DO NOT CHANGE THE LAYOUT OF THE FIRST 21 BYTES

0024            	len.findinfo	equ 36

000D            	o.findinfo.attr	equ 13

000E            	o.findinfo.size	equ 14

0010            	o.findinfo.time	equ 16

0012            	o.findinfo.date	equ 18

0023            	o.findinfo.mhandle equ 35

B1DE   d.findinfobuf   ds len.findinfo

0002             o.file.size     equ 2

0005            	o.file.mhandle	equ 5

000D             o.file.attr     equ 13

000D            	o.direntry.attr	equ 13

000E            	o.direntry.size	equ 14

0010            	o.direntry.time	equ 16

0012            	o.direntry.date	equ 18

;       char name[13];  /* 12 chars plus \0 (the file we found) */

;       char attribute;

;       uint size;              /* filesize can't be bigger than 64k */

;       uint time,date; /* if we allow time & date stamping */

;       char flags;             /* memory block flags */

;       char handle;    /* memory block handle */

;******************************************************************************

; PEARL.TXT DATA

;; The following 8 bytes are saved for each stream

B202   d.thisstream    ds      8-8

B202   d.colrow        ds      2-2     ; keep next 2 together

B202   d.row           ds      1       ; 0-based within window

B203   d.col           ds      1       ;

B204   d.winlefttop    ds      2-2     ; keep next 2 together

B204   d.wintop        ds      1

B205   d.winleft       ds      1

B206   d.winsize       ds      2-2     ; keep next 2 together

B206   d.winheight     ds      1       ; height -1

B207   d.winwidth      ds      1       ; width -1

B208   d.winset?       ds      1       ; NZ if window

B209   d.state         ds      1       ; bit 7 if inverse on

;; The following are recalculated from the above (in txtstrselect)

B20A   d.colrowcount   ds      2-2     ; keep next 2 together

B20A   d.rowcount      ds      1

B20B   d.colcount      ds      1       ; how many more cols to print on this line

B20C   d.stream        ds      1       ; current stream number

B20D   d.fastpos       ds      2       ; needed for quick screen update

B20F   d.streamwsp     ds      8*NSTREAMS      ; 8 streams of 8 bytes each

B24F   d.dateptr       ds 2            ; non null for expanding time/date

B251   d.kmcharret     ds 2            ; Returned character

B253   d.kstate        ds 2            ; Key locks state

B255   d.caslocks      ds 1            ; Shift states set by sticky key press

B256   d.sticky        ds 1            ; non-zero in sticky key mode

B257   d.yellow        ds 1            ; low byte of yellow/other key token

; stored by p.xlattoken which then returns ESC

B258   d.calcmode      ds 1            ; nonzero if keyboard in calculator mode

B259   d.kmexplen      ds 1            ; expansion string length

B25A   d.kmexpptr      ds 2            ; expansion string pointer

B25C   d.expbuffer     ds 2            ; address of expansion key buffer

B25E   d.expbufptr     ds 2            ; pointer to free byte

B260   d.expbufend     ds 2            ; last byte in buffer

B2A1   macro_buf       ds 256

; file selector variables

B3A7   fs_clicat       ds      1       ; non-zero if CAT command, not fsel

B3A8   fs_showsizes    ds      1       ; non-zero if showing file sizes (pad default=off)

B3A9   fs_showsys      ds      1       ; non-zero if showing system files

B3AA   fs_curfile      ds      1       ; current file number offset from top left

B3AB   fs_topleftfile  ds      1       ; file number displayed top left

B3AC   fs_numcols      ds      1

B3AD   fs_colwidth     ds      1

B3AE   fs_numshown     ds      1

B3AF   fs_maxfiles     ds      1       ; max files that can be shown

FS_NUMROWS             .equ    7       ; display rows

FS_NUMCOLS             .equ    5

FS_COLWIDTH            .equ    16

B3B0   fs_handle       ds      2

FS_NUMSHOWN            .equ    FS_NUMCOLS*FS_NUMROWS

; number of files shown

B3B2   fs_numfilerows  ds      1       ; rows of files in CAT command

B3B3   fs_startlist    ds      2       ; start of file list

; zero if doing unsorted list

B3B5   fs_startdir     ds      2       ; start of directory entries

B3B7   fs_endlist      ds      2

B3B9   fs_numfiles     ds      1       ; number of files in directory

B3BA   fs_lastshown    ds      1       ; last filenumber currently shown

; 	fs_topleft + FS_NUMSHOWN

B3BB   tickcount           ds 4    ; 32 bit counter needed for basic

B3BF   ticksleftuntilevent ds 2

B3C1   tickreloadvalue     ds 2

B3C3   tickeventpending    ds 1

B3C4   countdowntimer      ds 2

B3C6   savestream      ds 1

B3C7   password        ds 5    ; encrypted

B3CC   pwbuf           ds 5    ; clear

B3D1   realpwbuf       ds 5    ; the real password saved for encrypting

B3D6   haspassword     ds 1    ; non-zero if has password

;passwdmsg		ds 2

B3D7   passwdlen       ds 1

B3D8   passwordlocked  ds 1    ; non-zero if locked (disallow soft reset)

B3D9   editingsecret   ds 1    ; non-zero when editing secret file (can't delete it)

B3DA   inmenu?         ds 1    ; non-zero when inside menu - macros disabled

B3DB   macro_count     ds      1

B3DC   recording?      ds      1

B3DD   macro_token     ds      2

B3DF   printfailed     ds 1            ; flag set by mccheckprinter

; stops "finished printing" message

B3E0   wasmemoryerr    ds 1

B3E1   inprotext       ds 1            ; used in file selector, 0=was Fn-L, nonzero=Fn-2

**** End ****