{.HE%F}

{.FOSJDimon %F Page # %#}

Program : PRINTCHK.PAS

Purpose : PRINTCHK is a Function  that is  used to determine  if a printer is

On and OnLine. This is VERY dependant on the configuration of  your

system, and must be tested on each PC before it is given the  "OK!"

Use     : Make sure  that the  function   is in an  accesible disk  directory.

The  function returns a Boolean value.  TRUE if (it thinks) that the

printer is On and OnLine. False otherwise. You will have to test the

function to determine the table of values for your configuration  of

system.

Source  : Following is the Source Code for the PRTEST Function.

{.PA}

Function PrTest : Boolean;

{*****************************************************************************

**                                                                          **

**                       Function PRTest                                    **

**                                                                          **

**        Purpose - To determine if the printer is in an active or an       **

**                  inactive state. THIS IS NOT a guarantee of "perfection" **

**                  simply a 'best guess' of if the printer is On and       **

**                  On line. Status will be unknow, just True or False.     **

**                                                                          **

**        Returns - Boolean. TRUE (On+OnLine), FALSE (any other case)       **

**                                                                          **

**                  Written By : SJDimon 1985                               **

*****************************************************************************}

Type

Regtype = Record

AX,BX,CX,DX,BP,SI,DI,DS,ES,Flags: Integer

End;

Var

Reg : Regtype;

I : Integer;

Begin

Reg.Ax:=$02 Shl 8;

Reg.Dx:=0;

Intr($17,Reg);

I:= Hi(Reg.AX);

If I = 144 Then

PrTest := True

Else

PrTest := False;

End;