NU-EPSON

The Pop-Up Printer Controller

Version 2.3

Copyright (C) 1985

By

Charles Rich

NU-EPSON

The Pop-Up Printer Controller

NU-EPSON is a memory resident printer controller that pop's up

for you any time you need to set your printer.  With NU-EPSON

you can change your printer's style and format instantly.  You'll

never need to exit to DOS to change your printer again.  You can

stay inside your word processor, editor or whatever.

To load NU-EPSON, put your program disk in your default drive,

type NU-EPSON and then press carriage return.  When the brief

installation message comes up, NU-EPSON is loaded.  After that

you can call NU-EPSON any time by pressing Alt-Caps Lock.

Just hold the Alt key down and press Caps Lock.

When summoned, NU-EPSON pop's up with a menu of printer styles

and motion controls.  The styles are on the left side of the

menu.  The motion controls are on the right.  The styles are

Compressed, Double-strike, Emphasized, Italic, Tiny, and Wide.

The motion controls are: Form feed, Line feed, Quiet, and

Uni-directional.  NU-EPSON also offers Hex Input mode and a Reset

feature.  On a color monitor the styles are displayed in

cyan, and the motion controls are white. This is a visual

reminder.

NU-EPSON is ready for your selections as soon as it comes up.

Just type the letters corresponding to the combination of

features you want.  Press the backspace key to delete any entries.

Submit your selections when everything is just right, by

pressing the carriage return.  NU-EPSON will disappear, sending

your commands to the printer.

You can enter up to eight selections on the regular entry line.

Although you'll probably never need to send that many.  If you

try to enter more than eight selections NU-EPSON will beep to

alert you.  If you really want to send more than eight commands,

just submit the first eight and call NU-EPSON a second time and

submit the remaining commands.  You can repeat this as often as

you need.

If you call NU-EPSON while your printer is turned off, or is off

line, you will get a flashing "Printer" message.  Just turn your

printer on, or put it back on line and NU-EPSON will continue.

Press the carriage return if you want to exit instead.

The next few pages cover all the features of NU-EPSON in detail.

If you have any questions or comments please let me know about

them.  My address is at the end of this documentation.  If you'd

like a reply, please enclose a self-addressed stamped envelope.

Enjoy!  Charles Rich.

NU-EPSON

Page 1

NU-EPSON'S PRINT STYLES

The following is a detailed description of NU-EPSON's print

styles.  The best way to get familiar with them is to experiment.

They can be combined almost any way you'd like, and many combina-

tions are quite useful.

C  Compressed Mode

This is easily the most popular print width after the

standard pica.  You get 132 characters per line with Com-

pressed Mode instead of 80.  This is great for any reports

and spreadsheets that need extra width yet must retain high

legibility.  Your printer will print 17.16 characters per

inch instead of the usual 10.

D  Double-Strike Mode

This print mode gives you darker type by filling in the

vertical spaces between dots.  Your printer reprints each

line after shifting the paper down 1/216".

E  Emphasized Mode

This mode reprints each line after shifting the print head

slightly, filling in the horizontal space between dots.  This

is darker than Double-Strike, and the two modes used to-

gether make a very dark, solid type style.  Note that the

Epson printer won't print Emphasized and Compressed Modes

together, it will ignore the Compressed Mode setting.

I  Italic Mode

Use Italic Mode when you want your printing to look a little

fancier.  You can mix Italic Mode with any other mode such as

Compressed, Wide, Tiny or any combination.

T  Tiny Type

This is a combination of Compressed and Superscript modes,

with 1/12" line feeds.  This is the tiniest print you can get

from the Epson, and it's still really legible.  This is great

for long program listings, archiving or novelty effects.

You'll find other uses--it grows on you.

W  Wide or Expanded Mode

This doubles the width of your type.  When combined with

Double Strike and Emphasized modes, this is the most atten-

tion getting style.  This is a good combination when printing

bulletin board notices and announcements.  You can also mix

Wide Mode with any other mode for some interesting type

styles.

---- END OF NU-EPSON'S PRINT STYLES ----

NU-EPSON

Page 2

NU-EPSON'S MOTION CONTROLS

F  Form Feed

This is an instant action key for Nu-Epson.  Pressing "F"

sends a form feed to your printer, advancing the paper to the

top of the next page.  This lets you begin print jobs at the

top of a fresh sheet of paper.  It spares you from reaching

over, taking your printer off-line, pressing the form feed

button and then putting it back on line.  This doesn't echo

an "F" on the entry line, and you can issue it as many times

as you like.  It won't fill the entry buffer.

L  Line Feed

This is another instant action key for Nu-Epson.  When

you press "L", Nu-Epson sends a line feed to your printer,

advancing the paper one line.  This lets you skip lines

instantly.  This too, won't echo an "L" on the entry line,

and you can issue it as many times as you like.  It won't

fill the entry buffer.

Q  Quiet or Half-Speed Mode

This runs your Epson printer at half its normal speed for a

slightly quieter print job.

U  Unidirectional Mode

This makes your Epson print in one direction only.  This is

good for graphics output in non-standard print modes since it

reduces misalignment from row to row.

H  Hex Input Mode

This gives you the "Hex:" prompt so you can send customized

control strings with your menu choices.  When you're through

with Hex Input, hit the carriage return to come back to the

regular entry line.  You'll see an "H" in the command

line as a reminder of your Hex string, if you entered one.

Your Hex string will then be sent along with your menu

choices.  Hex Input is discussed more fully below.

R  Reset

This sends the "Master Reset" code to your Epson.  After

that, your printer is in the same state as when it's first

turned on.  This means that all the mode settings are

cleared and the current line position is the new top of form.

When used with the line feed key, this is an easy way for

setting the top of form.  Just advance to where you want

the new top of form to be and press Reset.  NU-EPSON will

chirp and flash "Reset" to alert you, when pressed.  Reset

also clears the command entry line.

---- END OF NU-EPSON'S MOTION CONTROLS ----

NU-EPSON

Page 3

NU-EPSON'S HEX INPUT MODE

In the back of your Epson User's Manual you'll find a complete set

of control codes for your printer.  There are commands for

setting your printer's margins, underlining, changing character

sets, defining form lengths, line feeds and so on.  These are all

the possible ways you can set your printer.  With Hex Input Mode,

NU-EPSON lets you send all of these, easily.

Press "H" to invoke the Hex Input Mode.  The "Hex:" prompt will

appear and you can create your hex string by simply typing it from

the keyboard.

As an example, let's turn on underlining.  You can make your

printer print underlines by sending it this sequence: <ESC>"-1".

So, in Hex Input mode press the escape key, then the negative

sign, then the number 1.  NU-EPSON will convert them to hex for

you.  You'll see these hex values echoed on the Hex Input line

as: 1B 2D 31.  Press the carriage return and you'll go back to the

usual input line.  The "H" in the entry line reminds you that you

have defined a hex string at that point.  Press the carriage

return again and NU-EPSON will disappear, submitting your commands

to the printer.  That's all there is to it.  No more fooling

around with hex conversion tables and firing up the BASIC inter-

preter every time you want to change your printer.

The Hex Input Mode will let you send up to nine hex characters to

your printer.  If you try to send more than nine characters,

NU-EPSON will beep to alert you.  Since most control codes are

about three characters long this lets you send two or three

configurations at once.

For example, you could set the left and right margins, and

redefine the number of lines in a page with just one hex input

string.  We'll do this later.  If you need to send a longer

sequence simply call NU-EPSON again and submit the rest of the

sequence.  You can repeat this as many times as you need to.

If you make mistakes in your hex entry, press the backspace key

to correct them.  This will step the cursor backwards, clearing

entries one by one.

Press the carriage return when you're happy with your hex input

string.  This returns you to the regular entry line where you

can continue setting your printer from the menu if you want.  If

you created a hex string in Hex Input mode you'll see an "H" on

the regular entry line where you called Hex Input.  This is

to remind you of your Hex input string.  If you didn't enter

anything in Hex Input mode, there won't be an "H" in the regular

command line.  Since NU-EPSON holds only one line of hex input,

NU-EPSON

Page 4

you can only create one hex string at a time.  This means you can

only have one "H" on your command entry line.

If you try to get into Hex Input Mode twice from the regular

command line NU-EPSON will beep, alerting you that you have

already created a hex input line.

If you want to get into Hex Input Mode anyway, just delete the "H"

from the previous hex input.  NU-EPSON will then let you enter hex

commands again.

If you need to submit a hex string that's too long for one entry

line, simply send it piece by piece.  Enter the first nine hex

characters and send them.  Then call NU-EPSON again and send

another nine characters.  You can repeat this as many times as you

like.

USING THE ALT-KEYPAD

Some of the Epson printer commands need you to give a number for

a parameter.  For instance, you need to give a number when setting

margins, or the number of lines to print per page, or for the

number of line feeds per form, and for character spacings, etc.

The control codes in the back of the User's Manual ask for these

numbers like this: CHR$(n), with "n" being the number you want.

The Alt-Keypad lets you send these numbers to NU-EPSON.  Whenever

a control code says CHR$(n), you'll use the Alt-Keypad to send

that number.

The Alt-Keypad is a normal feature of the IBM PC's keyboard.  If

you hold the Alt key down and then enter a number using only the

numeric keypad on the right hand side of the keyboard, the

keyboard will send the ASCII character corresponding to that

number.  In Hex Input mode, NU-EPSON converts all your keystrokes

into the hexadecimal value for the character you pressed, in-

cluding characters entered from the Alt-Keypad.

So, to put the number 50 in NU-EPSON's Hex Input line, hold the

Alt key down and press the 5 and 0 on the numeric keypad.  when

you release the Alt key you'll see a 32 in the hex string.  That

is the number 50 in hexadecimal.  As another example, to put the

number 127 into a hex input command, hold the Alt key down and

type 127 on the numeric keypad, release the Alt key.  This

generates a 7F in the entry line.  7F is 127 in hexadecimal

numbers.

Note that when the Caps Lock key is turned on, your computer will

invert the case of the letters of the alphabet, changing "A"'s to

"a"'s and vice versa.  This means then, that if Caps Lock is set

and you enter the number 100 from the Alt-Keypad, which corres-

ponds to the letter "d", with Caps Lock on, your computer will

NU-EPSON

Page 5

intervene and convert that lowercase "d" to a capital "D".  Now,

the "D" character is ASCII character number 68, not 100 as you

intended.  Since NU-EPSON converts keboard input into hexadecimal,

you'll get the hex number 44 instead of hex 64.  The upshot of

all this is that if you start getting numbers that aren't right,

check the Caps Lock key.

The numbers from 65 to 90 correspond to uppercase letters.  They

should all start with 4's or 5's in hexadecimal.

The numbers from 97 to 122 correspond to lowercase letters.

They should all start with 6's or 7's in hexadecimal.

Note that if you forget to hold the Alt key down when you type

on the keypad it will generate 00's.  This is useful for when

you need to send a CHR$(0).  Also, ProKey users should remember

that ProKey wants you to press Shift-Alt while using the numeric

keypad, otherwise you'll only get 00's, too.

AN EXAMPLE OF HEX INPUT

Hex Input Mode is a very simple way to send your custom codes.

You'll get the hang of it quickly.  for practice, try sending "Hi

there" to your printer by just using the Hex Input Mode.  Call up

the Hex Input Mode and type "Hi there" right from the keyboard.

You should see: 48 69 20 74 68 65 72 65.  Those are the hexa-

decimal values for those letters.  Before sending it do one more

thing.  Enter the number 10 from the Alt-Keypad.  Hold the Alt key

down and press 1 and 0.  This generates a 0A, the line feed

character. We need the line feed to flush the printer buffer and

print our message right away.  Your Hex Input Line should look

like this now: 48 69 20 74 68 65 72 65 0A.

Now, let's submit it.  Press the carriage return and get back to

the regular entry line.  Then press carriage return again to exit

NU-EPSON and submit the hex string.  Your printer will print

"Hi there".  Sending hex command strings to your printer is the

very same thing.  But instead of being sequences of characters

that spell something meaningful to you, they say something

meaningful to your printer.

AN EXAMPLE USING MANY FEATURES

When you use Tiny Type to print programs source code, or long

lists, you'll find that margins of 26 and 106, with skip over

perforation set for 12 lines make nicely formatted listings.

You can set it as follows.

NU-EPSON

Page 6

Start by re-establishing the top of form three lines down from

the perforation, about 1/2-inch down.  Simply tap "L" three

times, until the paper is positioned correctly, then press "R"

to send a Master Reset to the printer.  That is now the new top

of form.  You'll see why we did this when we set our skip over

perforation "clear zone," below.

Next, press "T" to invoke Tiny Type.

Press "H" for Hex Input Mode.  In Hex Input Mode press Escape, l

(lower case "L"), and use the Alt-Keypad to send the hexadecimal

number for 26.  This will set the left margin at 26 spaces.

Next press Escape, Q (capital "Q"), and use the Alt-Keypad to send

106.  This establishes the left margin at 106 spaces.

Finally, press Escape, N (capital "N"), and use the Alt-Keypad to

send the Number 12.  This sets the skip over perforation zone to

twelve lines.  We want to reserve 1/2-inch above and below the

perforations.  One-half inch is equal to six tiny lines, since

we're setting the printer to Tiny Type.  That's why we set the

top of form 1/2-inch below the perforation.  Now, subsequent

perforations will fall right in the middle of our 12 line perfor-

ation zone, giving us 1/2-inch top and bottom margins.

Your Hex Input entry line should say: 1B 6C 1A 1B 51 6A 1B 4E 0C.

Press carriage return and you'll come back to the regular entry

line.  It should look like this: "TH".  "T" for Tiny Type, and "H"

for your Hex Input string.  Press the carriage return to send your

new configuration to the printer, and that's it.  That's all there

is to it.  In practice it's very simple.

Now print a listing from your word processor, or from DOS.  It

will be centered in the pages with a half inch margin at the top

and bottom of each page.

---- END OF NU-EPSON'S HEX INPUT MODE ----

NU-EPSON

Page 7

SOME USEFUL COMMAND STRINGS

These are some of the control codes you'll probably use the

most.

Set Elite Mode On

<Esc>M    That's Escape, then uppercase "M".

Set Elite Mode Off

<Esc>P    That's Escape, then uppercase "P".

Set left margin at #

<Esc>l#   That's Escape, then lower case "l", then

use the Alt-Keypad to enter the number for

your left margin.

Set right margin at #

<Esc>Q#   Press Escape, then capital "Q", then the

number for your right margins.

Skip over perforation, variable

<Esc>N#   Press Escape, then capital "N", then the

number of lines for your "perforation zone".

Set the form length in lines

<Esc>C#   Press Escape, then capital "C", then use the

Alt-Keypad to enter the number of lines for

your new page definition.

Set the form length in inches

<Esc>C(00)#   Press Escape, capital "C", hex 00 (get

this by pressing any of the numeric keypad

keys without pressing the Alt key), then use

the Alt-Keypad and send the length of your new

page in inches.

Turn underline on

<Esc>-1   Press Escape, the minus sign, then press the

number 1.  Use the numbers at the top of the

keyboard for this.

Turn underline off

<Esc>-0   Press Escape, then minus sign, then type the

number 0.  Again, use the numbers at the top

of the keyboard for this.

Sound the printer bell

7         Use the Alt-Keypad and press 7.  The printer

bell will ring once.

NU-EPSON

Page 8

A Word About ShareWare

Shareware is based on the concept that good

software shouldn't cost an arm and a leg.  Share-

Ware gives you a chance to use your software before

paying for it.  Also, you are actually encouraged

to copy ShareWare programs and share them with your

friends.  Once you know you like a program, then

send a check or money order for the price of the

utility.  Prices are usually very modest.

NU-EPSON is a ShareWare program.  Please make as

many copies as you like and share it with others.

I only ask that you don't alter NU-EPSON or this

documentation, and that you include this documen-

tation when passing it around.  You can support

ShareWare development, and share back by sending

your $25.00 donation to:

Charles Rich

P.O. Box 95650

Seattle, WA  98145-2650

Thanks! and enjoy NU-EPSON

NU-EPSON

Page 9

#

NU-EPSON