PDATA DOCUMENTATION

-------------------

Release version : 3.52 final

Date            : Saturday, 3rd February 1995

(This textfile was last modified on 25th August 1995)

Copyright  1993-1995 Deadbeat/The Sharks

(C64 coding, optimized cable schematics, documentation)

Copyright  1993 Hude

(Amiga coding)

Please notice that the Amiga executable file is now called "XData"!

(It's 100% compatible with PData; read FUTURE for more information)

1. OVERVIEW

PData is an utility for transfering both binary and text files between

the C64/C128 and Amiga. It's about 25 times faster than a null modem

cable and it even leaves your Amiga's serial port free for other

applications (The maximum BPS rate in a _standard_ C64 is around 2.4K).

2. DISTRIBUTION AND DISCLAIMER

PData may be freely distributed as long as all files are included

intact and as long as no more than a nominal fee is charged to cover

time and copying costs. PData may NOT be used for commercial purposes

without a written permission from the author(s). The author(s) can't be

held liable for any damage directly or indirectly caused by the use or

misuse of this manual and/or the program it describes. PData is provided

"as-is" and it is a subject to change without notice.

Internet users will most likely find the latest version of PData from

nic.funet.fi. You could also try to /msg SHARKBITE for some HELP on IRC.

Those unprivileged ones (Persons without 'net access) should call any

of THE SHARKS boards, and they'll get the latest version.

If you like this program, feel free to send me (Deadbeat) a postcard.

On the other hand, you could always drop me with some e-mail...

3. REQUIREMENTS

The C64 end of PData will work in every C64/C128. The Amiga end of PData

will - hopefully - work in every model, but Kickstart v37.175 (v2.04) or

above is required to utilize all features. A special cable between the

C64 and Amiga is required. The following things are needed for the cable:

- One card edge connector (C64 user port)

- One male D-25 connector (Amiga parallel port)

- Suitable length of flat cable (Width 12 or more)

Additionally, you'll need the following files for your Amiga...

- Devs/Parallel.device

- L/Port-handler

- Libs/Asl.library

- Libs/Locale.library

4. PDATA CABLE SCHEMATICS

C64 user port              Amiga parallel port

Pin      Description       Description      Pin

B        FLAG*             STROBE           1

C        PB0               DATA 0           2

D        PB1               DATA 1           3

E        PB2               DATA 2           4

F        PB3               DATA 3           5

H        PB4               DATA 4           6

J        PB5               DATA 5           7

K        PB6               DATA 6           8

L        PB7               DATA 7           9

8        PC*               ACK*             10

M        PA2               BUSY             11

N        GND               GND              17

5. USING PDATA C64

Using PData C64 is fairly easy. You'll first be prompted for your work

drive. Any device between 8 and 11 will do. Then you can select whether

to handle your files as binary or text. If you select binary, the files

will be transfered without any alterations. Selecting text mode will

cause everything to be translated from PETSCII to ASCII and vice versa.

The next step is to decide whether to send or receive files. At this

point, you can press "," to access a SEQ file handler, using which you

can print out or view sequential files. Assuming that you didn't enter

the SEQ file handler, you now can press F1 to toggle between normal/zip

mode. If zip mode is on, PData will add those 1! 2! 3! and 4! prefixes

in front of each zip disk-packed file automatically and you only need

to worry about fourteen last characters. If you start receiving a file

in single mode, you'll be prompted whether to process files as PRG or

SEQ. If a file exists, you'll be given a chance to regret. You can use

"$" (CR = $ as Default) to see directory, "@" for DOS commands and "?"

to see number of free blocks available on disk. Pressing "=" at all

times (Except in multi-file transfer menu) will take you one step

"backwards" in the menus. In zip mode, you can individually decide

which of the files should be overwritten. Pressing F3 will take you

to multi-file transfer menu. In multi-file transfer menu, press F1 for

sending files, F3 for receiving files, F5 for directory and F7 for main

menu. After pressing F1, you can use Y/N to select files. Pressing CR

(Return) will quit or start transfer, depending if you've got any files

selected. After pressing F3, you can enter target filenames. Leave one

blank line and hit CR to start transfer. Multi-file transfer can handle

255 files both directions. It handles only binary PRG files and never

overwrites it's target files. At ALL times, pressing restore will clear

the channel, initialize vectors and restart program. If you're in doubt

of something, just try it. There's no way the program could crash.

6. USING XDATA AMIGA

xdata (s/r/?) (path/filename)

(s = send / r = receive / ? = help)

Example : xdata r dh1:dl/c64_rules.txt

Pressing CTRL+C will - hopefully - abort transfer. Use it in combination

with RESTORE in the C64 end to gain maximum effect. If you have a

multifacecard (MFC) in your Amiga, remember you must use the XData.CFG

in S: directory to re-route PAR: into PIT:. Edit the file so that it

reads PIT:<CR>. If you're not a hard drive user, I suggest you redirect

the files into RAM: when receiving something from the C64, if you don't

have lots of spare time to waste (See missing features).

7. MISCELLANEOUS

Configuring your regular directory tool for PData makes your life much

easier. Once you've made the first transfer after turning both of your

computers on, you can ignore the C64's on-screen prompts about setting

the other end to send/receive mode. This is because once the Amiga's

port-handler has been "activated", PData can't "lose" anything. When

transfering PData mainfile (PDATA.C64) to the C64, you MUST use the

filename "PDATA 3.5-SHARKS". To transfer the file, try a null modem

cable. Schematics are supplied below. You'll need a RS-232 adapter for

the C64 and a suitable terminal program for both ends. If, however, you

should experience problems in doing so, e-mail the official PData

helpline at this address: mysti@stekt.oulu.fi and beg for help! :)

(Mysdee/The Sharks just volunteered to help all the people in need!)

8. NULL MODEM CABLE SCHEMATICS

Pin     Description       Description       Pin

2       TXD               RXD               3

3       RXD               TXD               2

4       RTS               DCD               8

6       DSR               DTR               20

7       GND               GND               7

8       DCD               RTS               4

20      DTR               DSR               6

9. BUGS AND MISSING FEATURES

C64 end:   - None known, although there has been some reports about

problems with multi-file transfers.

Amiga end: - Sometimes refuses to work if a filename ends with "+".

- Refuses to work if a filename contains any spaces.

- Receive buffer missing.

...

PData doesn't send a header containing a filename. This is NOT a bug.

By the way, did you know that PData can be used to transfer data between

two C64's or two Amiga's with minor cable modifications? Hardware wizards

may figure this one out on their own, full cable schematics will be

released later on for the public.

10. FUTURE

This version of PData is the very final ever. I've finally decided to

share my views about the future with the public. PDATA IS DEAD. LONG LIVE

XDATA. After careful consideration and lots of time, I've finally decided

to make some major changes. The changes are so radical, that I've decided

to change the name as well. Expect the initial release of XData soon...

In fact, I've had the C64 end finished for months now, but finding a

qualified Amiga coder has been the most difficult thing. Hude lost his

interest towards this project, and he even managed to lose the source

codes somewhere. Anyways, there'll be lots of new features. The entire

C64 end has been reprogrammed from scratch. Finally there should be a PC

version as well. There has been pretty radical ideas for future versions,

like a TwinExpress with GUI, but much better and more advanced. Data

transfer rates will be *MUCH* higher, no more typing in hundreds of

filenames thanks to neat headers - and hopefully we'll finally get rid

of the ugly ASL file requester!... Reqtools rules... Just wait and see.

The Amiga executable is indeed already named "XData", but it's exactly

the same as "PData". I just patched the program a bit to make it look

better. Besides, I finally wanted to get rid of the "PData" name!

11. GETTING IN TOUCH WITH ME...

Alias:           Deadbeat/The Sharks

Snailmail:       Timo R

Linjakuja 3

FIN-74120 Iisalmi

Finland

E-mail:          deadbeat@freenet.hut.fi

IRCnick:         Deadbeat

(I'm usually on channels #C-64 and #Amigascne)

Feel free to /msg me on IRC, if you see me around. Please don't

snailmail me letters begging for cables, I have _NO_ time to make any.

Consider the making of a PData cable as a lamer-protection! ;)

If you really have to ask me something, try e-mail first (Prefered).