This file contains information on building the 64NET/2 cable.

*64NET/2 uses a _DIFFERENT_ cable to 64net v1*

(This is for improved reliability, as well as speed).

The 64NET/2 cable uses 12 pin-to-pin connections as follows:

Printer Port	User Port

1		B

2		C

3		D

4		E

5		F

6		H

7		J

8		K

9		L

10		8

11		M

18		N

HINTS:

A 64NET v1 cable can usually be easily modified to this configuration

provided it uses 12 core (or greater) cable.

If you are constructing a new 64NET/2 cable, buy a printer cable

designed for an IBM compatible, remove the centronics connector, and

solder the C64 user port connector in place.  This saves the hassle of

soldering the DB25 (which is by far harder than soldering a low-guage

edge connector, ie user port).

TROUBLE SHOOTING:

If you have difficulty building the cable, contact your local 64NET

dealer, where you will be able to purchase one at a reasonable price.

It is unlikely that this cable will operate with runs of >5m (due to

the general cruftiness of most PC Parallel ports).  You may be

fortunate enough to have longer runs function, however it is

recommended that you do not.  If you *really* *really* have to, buy

high quality cable (eg type 5 cable desgined for 100mbit networking,

or for "think net").  Oxygen free copper is also a help.  It is

usually a combination of interference and drop of signal (PC printer

ports are usually pushed through a set of capacitors, which

drastically limit current) which kills the connections.  Be aware that

the 64NET/2 cable can be strobing signals at upto 33Mhz, so reasonable

soldering is necessary.