1/1/97

-- INSTALL.DOC

DigiBoard,- 6400 Flying Cloud Drive Eden Prairie, MN 55344 (612) 912-4800

DigiBoard, GmbH Schmittgasse 64 5000 Cologne 90 Germany 49-2203-81083

Installation Guide Reference Manual for Universal OS/2 Device Driver for

DigiBoard Intelligent Communications Boards

92000100C

DigiBoard, DigiBoard, PC/Xi, PC/16i, PC/8i, PC/Xe, PC/16e, PC/8e,

PC/4e, PC/2e, Xem, PC/Xem, MC/Xem, EISA/Xem, COM/Xi, COM/8i, COM/4i,

MC/Xi, MC/16i, MC/8i, MC/4i, EPC/X, C/X and AccelePort are trademarks of

DigiBoard. All other brand and product names are the trademarks of their

respective holders.

Digi International, Inc.  All Rights Reserved

Information in this document is subject to change without notice and

does not represent a commitment on the part of DigiBoard.  DigiBoard

provides this document as is, without warranty of any kind, either

expressed or implied, including, but not limited to, the particular

purpose.  DigiBoard may make improvements and/or changes in this manual

or in the product(s) and/or the program(s) described in this manual at

any time.  This product could include technical inaccuracies or

typographical errors.  Changes are periodically made to the information

herein; these changes may be incorporated in new editions of the

publication.

RESTRICTED RIGHTS:

For non-U.S. Government use: These programs are supplied under a

license.  They may be used, disclosed, and/or copied only as permitted

under such license agreement.  Any copy must contain the above copyright

notice and this restricted rights notice.  Use, copying, and/or

disclosure of the programs is strictly prohibited unless otherwise

provided in the license agreement.

For U.S.  Government use: Use, duplication, or disclosure by the

Government is subject to restrictions as set forth in sub-paragraph

(c)(1)(ii) of the Rights in Technical Data and Computer Software clause

of DFARS 52.227-7013.

Table of Contents                               1

Introduction                                    2

The Distribution Diskette                       2

Installing the Device Driver                    3

Global Parameters                               3

Per-Card Parameters                             4

Examples                                        6

Testing the Ports                               8

Troubleshooting                                 8

Error Messages                                  18

DMODE I/O Port Configuration Utility            10

DMODE Examples                                  11

Appendixes                                      12

Introduction

This release of the DigiWARE device driver for OS/2 supports the

entire line of DigiBoard intelligent asynchronous serial communications

controllers:

Micro Channel (MCA) bus:

DigiBoard MC/Xi 2, 4, 8 and 16 port boards

DigiBoard MC C/X cluster controller system

DigiBoard MC/Xem PC/AT

DigiBoard MC EPC/X

ISA bus:

DigiBoard PC/Xe 2, 4, 8 and 16 port boards

DigiBoard PC/Xi 8 and 16 port boards

DigiBoard COM/Xi 4 and 8 port boards

DigiBoard ISA C/X cluster controller system

DigiBoard PC/Xem

DigiBoard AccelePort

DigiBoard ISA EPC/X

EISA bus:

DigiBoard EISA C/X cluster controller system

DigiBoard EISA/Xem

DigiBoard EISA EPC/X

The device driver software supports multiple boards and multiple board

types in a single machine, and can handle up to 256 ports from any

combination of the boards listed above.  As new products are introduced,

the driver will be upgraded to include those devices as well.  The

driver takes all its configuration information from the device= command

line in CONFIG.SYS, and all DigiBoard intelligent boards in a system

are configured from a single command line.

In addition to the device driver, the configuration utility DMODE.EXE is

provided to change communication parameters for individual ports without

rebooting.  DMODE.EXE is similar to the OS/2 MODE command, but has some

added features, and supports communications devices beyond OS/2's range

of COM1 to COM8.

Before installing this device driver software, complete the installation

of the DigiBoard intelligent communications board(s) in your system,

according to the hardware installation manual provided with the

board(s), and write down your configuration!  The driver supports all

legal I/O and memory address settings, but must be informed of these

settings in the device driver command line.

The Distribution Diskette accompanying this manual contains all of the files

and documentation needed to install the DigiWARE OS/2 Universal Intelligent

Device Driver in your system.  A list of these files follows:

XALL.SYS        The installable device driver.

DMODE.EXE       A MODE work-alike program for configuring I/O ports beyond COM8

INSTALL.DOC     Installation instructions for the current release of the driver.

RELNOTES.DOC    Release notes.

DMODE.DOC       Instructions for the DMODE program.

XALL.DOC        Programmers information about the driver and applicable

system calls.

*.bin           Miscellaneous binary files used by the driver.

Installing the Device Driver.  Copy all of the files from the distribution

diskette to a single directory on your hard drive.  By default, driver

and data files are expected to be in \DIGI.  The device driver is

invoked by placing its name and configuration arguments into the

systems CONFIG.SYS file.  All DigiBoard intelligent asynchronous

serial communications boards must be installed from a single command

line, which contains complete set-up information for each board.  The

format of the command line is:

device=\digi\xall.sys [/n:names] [/b:baud] [/d:dtype] [/f:digipath]

[/v:messagepath] [/l:lptnames] [/s:sigs_off] /t:card /p:port /m:mem

[, /t:card /p:port /m:mem] [, ...]

Note that the path for XALL.SYS is \DIGI.  This is the default path.  If an

alternative path is specified with the /f:digipath option, substitute that

path for \DIGI.

The above command line must appear on a single command line in the

CONFIG.SYS file.  This line may be up to 255 characters long.

Global Parameters. The first six parameters in the command line above are

global parameters, and affect all boards installed.  These parameters

are optional, and may be omitted.  If omitted, default values will be

used.

/n:names        Defines the naming convention to be use for the

DigiBoard ports.  Names consists of an optional

stem followed by the starting device number.  The new

stem must be enclosed in double-quotes ("").  If

the stem is omitted, COM will be used.  If the

number is omitted, 3 will be assumed.  If the /n

parameter is not specified, the default name COM3 will

be assigned to the first port.

/n:"digi"0      DIGI0, DIGI1, DIGI2, ...  DIGIn

/n:4            COM4, COM5, COM6, ...  COMn

/n:"qdev"       QDEV3, QDEV4, QDEV5, ...  QDEVn

/b:baud         Sets the default baud rate to be used for all ports.

If this parameter is not specified, all ports will be

set to the OS/2 default of 1200 baud.  Baud rates of

up to 230,000 are supported.  Use the DMODE.EXE

command to set baud rates of individual ports.

/b:9600         Initializes all ports to 9600 baud.

/d:dtype        Sets the initial data type (parity, character

length and number of stop bits).  If this parameter

is omitted, the OS/2 default data type of even

parity, 7 data bits and 1 stop bit is used.  The

syntax is [pds], where p is E, O or N; d is 8, 7, 6

or 5; and s is 1 or 2.  All three values must be

supplied, in the correct order.

/d:N81          No parity, 8 data bits, 1 stop bit.

/f:digipath     Defines an alternative directory path for the driver

and data files.  If this option is present, all files

from the distribution diskette must be present in this

directory.

/f:d:\drvr\digi         All data files are in D:\DRVR\DIGI.

/v:messagefile  Forces the driver to direct all verbose messages

to the file named.  If /v: is given without a

filename, messages will be placed in \XALL.MSG.

This flag prevents any information from being

displayed on the console; useful in some situations

as when the machine is a headless server, where

calls to Standard Output will cause problems.  Without

this flag, all messages will appear on the

system console.  The message file is overwritten

each time the system is started.

/v:errmsg       Writes banner and other info to file \ERRMSG.

/v:             Writes banner and other info to file \XALL.MSG.

/a:a            Enable the alternate pin assignment of Data Carrier

Detect (DCD) for all ports (see discussion of Alt-Pin

processing.

/a:q            Disables the performance enhancing quick-write mode.

The driver defaults to quick-write mode enabled. When

enabled, DosWrite calls return as soon as the transmit

data is copied to the DigiBoard hardware transmit

buffer. When this mode is disabled, DosWrite calls

will return only after the transmit data has actually

left the DigiBoard hardware, mimicing the OS/2 COM.SYS

driver. A side effect of using quick-writes is that

when DosClose is called, the driver must wait for any

transmit data on the DigiBoard hardware to drain

before finishing the close processing.

/s:sigs_off     Forces modem control output signals off on port open.

/s:d            DTR remains low or off on port open.

/s:r            RTS remains low or off on port open.

/s:dr           DTR & RTS remain low or off on port open.

/l:lptnames   This permits the user to define names for the 8emp

printer ports so that they are accessable to the OS/2

print manager. The convention is similar to the

/n:names option mentioned above.

Names consists of an MANDITORY stem followed by the

starting device number.  The new stem must be

enclosed in double-quotes ("").  If the stem is

omitted, the parameter is invalid.  If the number is

omitted, 3 will be assumed.  If the /l: parameter is

omitted, the 8emp printer port will be named sequent-

ially with the other 8emp ports. If more than one

8emp module is used, the printer ports will be num-

bered sequentially starting with the stem and number

supplied with this option. If any DigiBoard supported

port has the same name as the selected 8emp printer

name, the 8emp printer port will be assigned the

specified name and the other port will be given the

name the printer port would have had, had this option

not been used.

The algorithm used to assign the names is as follows:

1) Assign names to all ports (which will includ the

printer ports) using the driver's normal naming

convention, "/n:".

2) Assign new names to the printer ports using the

names supplied by the "/l:" parameter.

3) If step 2 would result in a printer port's name

being changed to a name assigned to another port,

the printer port gets the "/l:" name and the comm

port gets the printer ports original name.

/l:"com"2 /n:digi0

8emp Ports

1     2     3     4     5     6     7     8    9(prtr)

|     |     |     |     |     |     |     |    |

digi0 digi1 digi2 digi3 digi4 digi5 digi6 digi7 com2

/l:"com"2

8emp Ports

1    2    3    4    5    6    7    8     9(prtr)

|    |    |    |    |    |    |    |     |

com3 com4 com5 com6 com7 com8 com9 com10 com2

The 8emp ports start with "com3" since the "/n:"

parameter is ommited and com2 is assigned to the

printer port.

/l:"com"

8emp Ports

1    2     3    4    5    6    7    8     9(prtr)

|    |     |    |    |    |    |    |     |

com11 com20 com5 com6 com7 com8 com9 com10 com3

10    11    12    13    14    15    16    17   18(prtr)

|     |     |     |     |     |     |     |    |

com12 com13 com14 com15 com16 com17 com18 com19 com4

The printer ports are named sequentially starting with

the name "com3", so ports 9 and 18 are named "com3" and

"com4" respectively. Since the "/n:" option was not

used, the com ports would have started their names with

"com3". Since port 9 and 18 are already assigned "com3"

and "com4", port 1 (normally com3) is given the name

"com11" and port 2 (normally com4),is given the name

"com20", since these are the names that would have been

assigned to ports 9 and 18 had the "/l:" option not

been used.

/l:"com"2 /n:"com"4

8emp Ports

1    2    3    4    5    6    7     8     9(prtr)

|    |    |    |    |    |    |     |     |

com4 com5 com6 com7 com8 com9 com10 com11 com2

10    11    12    13    14    15    16    17   18(prtr)

|     |     |     |     |     |     |     |    |

com13 com14 com15 com16 com17 com18 com19 com20 com3

Here, the standard com port names are not modified

because they have been specified to start with "com4"

using the "/n:" option, which don't conflict with the

specified printer port names.

Per-Card Parameters. The remaining three parameters are per-card

parameters, and are used to inform the driver of the type of board and

where it is to be addressed in the hosts I/O and memory buses.  These

parameters are mandatory, and must be given for each board!  The

per-card parameters must be given in complete sets, and the sets must be

separated by commas.

/t:card         Defines the card type to the driver.  Legal values

for card are:   a for AccelePort

i for the ISA C/X,

e for the EISA C/X,

m for the MC C/X,

p for Xem (PC/Xem, MC/Xem or EISA/Xem)

x for COM/Xi boards.

c for all EPC/X cards (ISA, EISA, MCA)

This parameter must be omitted for PC/Xe, PC/Xi and

MC/Xi boards.  The designators i, e, and m must be

followed by an additional two digits defining the

number and arrangement of DigiBoard C/CON-16 or

C/CON-16e concentrators on each host adapter channel.

If the concentrator(s) are connected to the host

adapter via a dial-up or leased line (remote

concentrators), each of the two digits must be

followed by a parameter, enclosed in parentheses,

specifying the synchronous communications mode to

be used for that line.  See the Appendix A for a

list of available synchronous communications modes.

/t:x            COM/Xi (4 or 8 channels)

/t:a            AccelePort

/t:p            PC/Xem, MC/Xem or EISA Xem

/t:i10          ISA C/X, one local concentrator on line 1

only.

/t:e11          EISA C/X, one local concentrator on each of

lines 1 and 2.

/t:m21          MC C/X, two local concentrators on line 1 and

1 on line 2.

/t:e12          EISA C/X, one local concentrator on line 1,

and two on line 2.

/t:i22          ISA C/X, two local concentrators on each of

lines 1 and 2.

/t:i2(21)3(27)  ISA C/X, two remote concentrators on line 1,

and two remote concentrators on line 2.

The concentrators on line 1 will use mode

#21 (76,800 baud, 8-wire, external clock) to

communicate with the host adapter, and the

concentrators on line 2 will use mode #27

(614,400 baud, 8-wire, internal clock).  See

the Appendix for a list of synchronous

communications modes.

/t:c22          An ISA, EISA, or MCA EPC/X with 2 concentrators

on each line. Please see Appendix B for a more

detailed explanation of EPC/X configuration

specifications.

/p:port         Defines the I/O port address for the card.

/p:300          Board is addressed at I/O port 300h.

/p:2005         EISA board (EISA C/X or EISA/Xem host adapter)

is in slot 2 (EISA I/O addresses are 4

digits : the slot number, followed by 005).

When plugging an ISA board (PC/Xe, ISA

C/X host adapter, etc.) into an EISA

machine, the /p parameter must contain the

3-digit I/O address as set by the DIP switches

on the board, and not the 4-digit address

based on the EISA slot number.

/m:mem          Dual-ported memory starting address.  This is the full

32-bit hexadecimal address of the beginning of the

boards dual-ported memory.

/m:d0000        Sets dual-ported memory start address to

0D0000h (in the BIOS expansion area between

640K and 1 megabyte).

/m:e80000       Sets start address of E80000h (in the

fifteenth megabyte).  EISA and ISA boards

(except the COM/Xi), may all share the same

memory starting address.  COM/Xi and all

Micro Channel boards must each have

unique memory addresses.

Examples The following examples start out with just the minimum to

install a DigiBoard board.  After locating an example for your type of

board, read the rest of the examples for optional situations that may be

appropriate to your application.  Default options will be altered by

your application program(s) when you configure the application.

--------

Example 1:      COM/Xi One COM/Xi at I/O port 300h, memory start address

D8000h; devices to be named COM5 COM6, etc.  All data files are in the

directory \DIGI:

device=c:\digi\xall.sys /n:5 /p:300 /m:d8000 /t:x

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 2:      PC/Xe, PC/Xi One PC/Xe (or PC/Xi) at I/O port 220h,

memory address D0000h   Ports will be named COM3, COM4, etc.

(default names).  All data files are in the directory \MISC\DIGI:

device=c:\misc\digi\xall.sys /f:\misc\digi /p:220 /m:d0000

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 3:      MC/Xi One MC/Xi at I/O port F1F0h, memory address

FC0000h.  Devices will be named DIGI1, DIGI2, etc.  and will have an

initial baud rate of 38,400.  All data files are in the default

directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /p:F1F0 /m:FC0000

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 4:      Two board installation with optional global parameters.

Two PC/Xi (or PC/Xe) boards at I/O ports 320h and 300h, with dual-ported

memory of both starting at D0000h.  Ports will be named COM0, COM1,

etc., and will be initialized to 38,400 baud, no parity, 8 data bits and

1 stop bit; messages will be contained in the file MESSAGES.MSG located

in the \DIGI directory.  All data files are in the default directory

\DIGI:

device=c:\digi\xall.sys /n:0 /b:38400 /v:\digi\messages.msg /d:N81

/p:320 /m:D0000, /p:300 /m:D0000

(Note that the above must all appear on one line in CONFIG.SYS)

---------

Example 5:      PC/Xem One PC/Xem at I/O port 224h, memory start address

C8000h; devices to be named COM3 COM4, etc.  All data files are in the

default directory \DIGI:

device=c:\digi\xall.sys /p:224 /m:C8000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

---------

Example 6:      ISA C/X One ISA C/X host adapter at I/O port 228h,

memory address D0000h, with one concentrator on line 1 and two on line

2.  Devices to be named DCX1, DCX2, etc.  and will have an initial baud

rate of 38,400.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"dcx"1 /b:38400 /p:228 /m:d0000 /t:i12

(Note that the above must all appear on one line in CONFIG.SYS)

-----------

Example 7:      EISA C/X with a remote concentrator via an RS-232

synchronous modem.  One EISA C/X host adapter in slot 3, memory address

D8000h, with two local concentrators on line 1 and one remote

concentrator at 9600 baud and external clocking on line 2.  Devices to

be named COM5, COM6, etc.  and will have an initial baud rate of 19,200.

All data files are in the directory \OS2\DIGI:

device=c:\os2\digi\xall.sys /n:5 /b:19200 /f:c:\os2\digi /p:3005 /m:d8000

/t:e21(17)

(Note that the above must all appear on one line in CONFIG.SYS)

----------

Example 8:      MC C/X and MC/Xem One MC C/X host adapter at I/O port

228h, memory address D8000h, with one concentrator on line 1 and one

concentrator  on line 2; one MC/Xem at I/O Port 304h, memory address

D0000h.  Devices will be named DIGI1, DIGI2, etc.  and will have an

initial baud rate of 38,400, Alt-Pin enabled, no parity, 8 data bits,

and 1 stop bit.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /a:a /d:N81 /p:228

/m:d8000 /t:m11, /p:304 /m:d0000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

(Note that for MC C/X and MC/Xem, the addresses 104 and 108 should not be

used to avoid possible conflicts)

----------

----------

Testing the Ports The simplest (and probably the best) way to check the

driver and I/O connections is to connect an ASCII terminal to one of the

ports and redirect some output to that port.  If the terminal is

connected to COM3, try something like the following:

dmode com3

(this will verify that the driver can find the portthe ports parameters will

be displayed.)

dmode com3 octs=off odsr=off

(this insures that the hardware handshaking signals CTS and DSR will

not prevent outputwe are not concerned with flow control at this time.)

dir > com3

(this redirects the current directory to COM3.) The console should simply

respond by repeating the C:\> prompt, and the current directory

(in this case, C:\) will appear on the terminal.

Troubleshooting If the C:\> prompt doesn't reappear, and the system

seems to be hung, you probably used a memory start address that is

being used by some other device, or the board is not set for the address

specified in CONFIG.SYS.  Try a different address.  If it still hangs,

try a different I/O port.  If your computer has a memory cache and the

boards memory start address is above 1 megabyte, try either disabling

the cache or moving the board to a low-memory address (D0000 is usually

a good choice).  It is also possible that a conflict exists with another

device driver which has been loaded via CONFIG.SYS.  The XALL.SYS line

should be placed near the end of the CONFIG.SYS, and must not be placed

before the DEVICE=TESTCFG.SYS line.  Due to potential device naming

conflicts, the pair of lines

DEVICE=\OS2\COM.SYS and

DEVICE=\OS2\VCOM.SYS

may cause problems.  Be sure that the XALL.SYS line is placed before

these lines in CONFIG.SYS.  If the XALL.SYS driver generates an error

message, try temporarily putting REM in front of the above two lines.

If this works, there is probably a naming conflict.  You may need to

add a /n: command line flag to the XALL.SYS line to force the driver to use

a different base name or start numbering the COM devices at 5 or above.

If the C:\> prompt does reappear, but there is no output on the terminal

screen, there is probably a cabling problem.  Make sure that all cable plugs

are securely seated in receptacles, and that they are configured correctly

(see the appropriate section of the hardware installation guide for your

board).  If the terminal displays output, but the data on the screen is

garbled, the terminal is probably not set with the same baud rate and data

type as specified in CONFIG.SYS (or the defaults, if not specified).

Verify these settings.  If everything checks out and you still have

problems, there could be a problem with your board.  Contact your distributor

or DigiBoard Technical Support for further assistance.

Error Messages If, when the driver is loaded, it cannot find the

board(s), or if they fail to respond properly to commands, certain error

messages may be sent to the console.  These error messages are defined

in the following table:

Could not find BIOS file

Could not find FEP file

Could not find data file        One or more files were missing from the

digipath directory.  The default digipath

directory is /digi.  If you have loaded the

driver and data files into a different

directory, that directory must be

specified in the xall command line with

the /f:digipath parameter.

IO Port Not Responding          I/O port address set incorrectly, or

conflicting with some other device.  Verify

setting, or try a different address.

DigiBoard BIOS Will Not Boot

DigiBoard BIOS Will Not Respond

DigiBoard FEPOS Will Not        DigiBoard board will not respond to

commands from the driver.  Typically due to a

memory conflict, or a memory cache conflict.

Driver Failed to Hook Timer

System Cannot Allocate Memory    A system call to OS/2 has failed.

Invalid Command Line Arguments  Syntax error in the command line in

CONFIG.SYS.  Double-check your entries.

========

DMODE I/O Port Configuration Utility DMODE is DigiBoards I/O channel

configuration utility.  DMODE functions like the OS/2 MODE command, but

is more flexible, and provides some added features.

-Supports communications device names other than COM1-COM8.

-Any single parameter may be changed without causing all others

to be reset to default values.

-Can be used to enable or disable the Alt-Pin feature on

individual ports.

-Maintains the familiar MODE syntax.

DMODE arguments may be entered in any order, and in many cases may be

abbreviated.  For example, a baud rate of 19,200 may be entered simply

as 19, since within the context of the DMODE command, 19 cannot be

interpreted as anything else.  Arguments may be separated by spaces or

commas, and unspecified parameters are left unchanged.  DMODE returns

the settings of all parameters after any requested changes have been

made.  If no parameters are given, DMODE simply returns the current

settings.  The syntax of DMODE is: dmode port_name [parameters]

Port names and parameters are not case sensitive, so upper and lower

case letters may be freely mixed without effect on the operation of the

DMODE command.  The following parameters are accepted:

Baud Rate       Baud rates can be specified with 2 to 5 digit designations,

according to the following table:

Accepted Values         Baud Rate

11 110                     110

15 150                     150

30 300                     300

60 600                     600

12 120 1200                1200

24 240 2400                2400

48 480 4800                4800

96 960 9600                9600

19 192 1920 19200          19200

38 384 3840 38400          38400

56 567 5670 56700          56700

115 1152 11520 115200   115200

230 2300 23000 230000   230000

Character Length Characters may be 5, 6, 7 or 8 bits long.  Enter

the appropriate number in the DMODE arguments.

Parity:  Enter the desired parity checking method with a single

letter E, O, N, M or S for Even, Odd, None, Mark or Space, respectively.

Stop Bits: Enter the desired number of stop bits.  Recognized

values are 1, 2 and 1.5.

Flags : DMODE supports the following flags in the same way as

the OS/2 MODE command.  Refer to your OS/2 documentation for

explanations of the flags.  To set a flag, enter the mnemonic for the

flag, followed by an equals sign (=), and the desired value (ON, OFF,

etc.).

TO=ON           Write timeout in effect.

TO=OFF          No timeout (infinite write timeout)

XON=ON          XON/XOFF Transmit handshaking enabled.

XON=OFF         XON/XOFF Transmit handshaking disabled.

XON=BOTH        XON/XOFF Transmit and receive handshaking enabled.

IDSR=ON         Enables Input DSR Sensitivity.

IDSR=OFF        Disables Input DSR Sensitivity.

ODSR=ON         Enables Output DSR Sensitivity.

ODSR=OFF        Disables Output DSR Sensitivity.

OCTS=ON         Enables CTS output flow control.

OCTS=OFF        Disables CTS output flow control.

DTR=ON          Sets DTR to ON or HIGH.

DTR=OFF         Sets DTR to OFF or LOW.

DTR=HS          DTR used for input flow control.  RTS Signal Control

RTS=ON          Sets RTS to ON or HIGH.

RTS=OFF         Sets RTS to OFF or LOW.

RTS=HS          RTS used for input flow control.

RTS=TOG         Toggles RTS on every transmit request. (Not supported)

ALT=ON          Internally swaps DSR and DCD signals (see explanation below).

ALT=OFF         DSR and DCD are not swapped.

QW=ON           Enables quick-write mode on the specified port.

QW=OFF          Disables quick-write mode on the specified port.

DigiCHANNEL products use 10-pin RJ-45 connectors in order to support all

of the signals normally available on a 9-wire RS-232 connection.

However, many commercially available RJ-45 connectors have only 8 pins.

8-pin RJ-45 plugs will fit in a 10-pin jack, but the first and last pins

(numbers 1 and 10) are not connected.  DigiBoard uses these pins for the

modem control signals RI (Ring Indicator, pin 1) and DCD (Data Carrier

Detect, pin 10).  While Ring Indicator is rarely a concern (since most

modems have auto-answer capability), Data Carrier Detect is often

monitored to insure that the modem has established, and is maintaining,

a valid connection with a remote modem.  To facilitate the use of 8-pin

RJ-45 connectors in modem applications, a special DMODE flag, ALT, has

been provided.  ALT instructs the device driver to monitor pin 1 of an

8-pin RJ-45 for DCD.  See the Hardware Installation guide for your board

for cabling diagrams and pin assignment charts for DB-25 and RJ-45

connectors.

Examples

dmode digi12

(Returns the current settings of device digi12.  No changes are made.)

dmode com5 2

(Configures COM5 for 2 stop bits.  All other parameters remain unaffected.)

dmode com5:38 rts=on

(Configures COM5 for a baud rate of 38,400 and sets RTS to ON or HIGH.)

dmode com5 8

(Configures COM5 for 8 data bits.  No other parameters are affected. )

dmode comdv83 idsr=off, to=off e 7 12

(Sets COMDV83 to disable Input DSR Sensitivity, disable output timeout,

select even parity, 7 data bits and a baud rate of 1200. )

dmode com5 38 alt=on

(Sets up COM5 for a baud rate of 38,400, and enables Alt-Pin Processing.)

dmode com5 e alt=off

(Sets COM5 for even parity checking and disables Alt-Pin processing.)

dmode com3 odsr=off octs=off

(Disables sensing of output flow control signals Clear To Send (CTS)

and Data Set Ready (DSR). )

===========

Appendix A

Synchronous Communications Modes

The DigiCHANNEL C/X host adapter communicates with attached C/CON-16

concentrators via two RS-422 synchronous channels (newer concentrators and

host adapters also support RS-232 synchronous communication.  See your

hardware Installation Guide).  The default operating mode for these

channels is 1.2 megabaud, 8-wire, internally clocked.  This provides the

fastest possible communication between the host adapter and locally

connected concentrators (in this context, local indicates that concentrators

are connected directly, via a 4 or 8 wire cable, to the host adapter;

remote refers to concentrators that are connected to the host adapter via

data communications devices such as modems or DSUs).  Local concentrators

always communicate with the host adapter in either an eight wire internally

clocked mode or a four wire self-clocked mode (in the four wire modes the

clock is encoded with the data instead of being carried on separate lines as

in the eight wire modes).  Remote concentrators always use an eight wire

externally clocked mode (the clock signal is provided by the synchronous

modem). The table on the following page shows the modes that are supported by

the XALL.SYS device driver.

=====================

The currently available communications modes are as follows:

Mode    Bit Rate        Clocking Mode

_____________________________________________________________________________

00      115K            8-wire internal clock

01      230K            4-wire self-clocked

02      460K            4-wire self-clocked

03      2400            8-wire internal clock

04      4800            8-wire internal clock

05      9600            8-wire internal clock

06      19.2K           8-wire internal clock

07      38.4K           8-wire internal clock

08      57.6K           8-wire internal clock

09      76.8K           8-wire internal clock

10      115K            8-wire internal clock

11      230K            8-wire internal clock

12      460K            8-wire internal clock

13      920K            8-wire internal clock

14      1.2M            8-wire internal clock

15      2400            8-wire external clock

16      4800            8-wire external clock

17      9600            8-wire external clock

18      19.2K           8-wire external clock

19      38.4K           8-wire external clock

20      57.6K           8-wire external clock

21      76.8K           8-wire external clock

22      115K            8-wire external clock

23      230K            8-wire external clock

24      460K            8-wire external clock

25      920K            8-wire external clock

26      1.2M            8-wire external clock

33      14000           8-wire external clock

35      2400            8-wire external clock RS-232

36      4800            8-wire external clock RS-232

37      9600            8-wire external clock RS-232

38      14000           8-wire external clock RS-232

39      19.2K           8-wire external clock RS-232

40      38.4K           8-wire external clock RS-232

41      57.6K           8-wire external clock RS-232

42      64000           8-wire external clock RS-232

43      76.8K           8-wire external clock RS-232

44-59   Reserved in future EPC Concentrator modes

60      115K            4-wire self-clocked

61      115K            4-wire self-clocked

62      230K            4-wire self-clocked

63      230K            4-wire self-clocked

64      460K            4-wire self-clocked

65      460K            4-wire self-clocked

66      921K            4-wire self-clocked

67      921K            4-wire self-clocked

68      1.843M          4-wire self-clocked

69      1.843M          4-wire self-clocked

70      1.843M          8-wire internal clock

71      2.458M          8-wire internal clock

72      3.686M          8-wire internal clock

73      7.373M          8-wire internal clock

74      10M             8-wire internal clock

In 8-wire internal clock mode, each device provides it's own transmit clock

from an internal baud-rate generator at the rated speed.  In 8-wire external

clock mode, the transmit clock must be supplied from a modem or similar

source.  External clock rates are approximates used to set up timeouts

tuning parameters only.  Values need not be exact; the user should select

the speed closest to the actual speed of the modem.

On one line, if any device uses 4-wire mode, all devices on that line must

use the same 4-wire mode.  8-wire modes have no such restrictions, and may

be mixed in any desired combination.

Modes 60-74 are only supported by the EPC/X product.

Appendix B - EPC/X command line configuration.

Specifying an EPC/X command line is similar to specifying a C/X command

line but with more options. These options are a result of the EPC/X's

ability to have up to 3 EBI modules attached to each EPC concentrator. The

driver supports up to a limit of 256 ports total across all EPC/X cards.

The default line speed is 1.2M since both C/CON-16 and EPC/CON-16 concentra-

tors may be attached to an EPC/X host adapter card. Only one line speed may

be specified for each EPC/X line and all concentrators on the line will

communicate at that speed. For example, If you are using remote concentrators

via a synchronous modem at 9600 baud, then all concentrators will communicate

at this speed.

A series of progressively more difficult configuration examples are presented

below to describe the EPC/X configuration command line.

The card type specifier (/t:_) 'C' indicates an EPC on any bus (ISA, EISA,

MCA). The type specification is followed by information about the number of

concentrators on each line. These examples are the same as examples for C/X

configurations. Note too, that these can be C/X or EPC/X concentrators.

Example : device=xall.sys /p:328 /m:d0000 /t:c11

One concentrator on each line at the default speed of 1.2M.

Example : device=xall.sys /p:328 /m:d0000 /t:c32

3 concentrators on line 1 and 2 concentrators on line 2 at the

default speed of 1.2M.

Example : device=xall.sys /p:328 /m:d0000 /t:c02

No concentrators on line 1 and 2 concentrators on line 2 at the

default speed of 1.2M.

The next level of complexity is to specifiy a non-default communications

line speed. This is specified by a speed number in parenthesis immediately

following the number of concentrators as specified for each line. Parenthesis,

in general, mark the beginning of a more complex configuration for a given

line. These examples also apply to C/X configurations with the exception of

line speed 74, which is exclusive to the EPC/X.

Example : device=xall.sys /p:328 /m:d0000 /t:c1(74)0

1 concentrator on line 1 communicating at 10M (mode 74, Appendix A)

and no concentrator on line 2.

Example : device=xall.sys /p:328 /m:d0000 /t:c2(14)3(19)

2 concentrators on line 1 communicating at 1.2M (mode 14) and 3

concentrators on line 2 communicating at 38.4k (mode 19).

In this case it's redundant to specify mode 14 as this is the

default mode.

Example : device=xall.sys /p:328 /m:d0000 /t:c23(9)

2 concentrators communicating at 1.2M (default) on line 1 and 3

concentrators on line 2 communicating at 76.8k (mode 9).

The last level of complexity is to specify EBI modules connected to EPC/X

concentrators. To do this, each concentrator (EPC/CON or C/CON) must be

specified with the square bracket characters,'[' and ']' and each EBI module

attached to the concentrator is specified within the bracket pair using their

number of ports. Multiple EBI modules are seperated by a semi-colon. Both

concentrator and EBI modules must be specified in the order they are attached

to the EPC/CON. Place holders ( empty brackets, "[]") must be used to specify

concentrators with no EBI modules if concentrators further downline have EBI

modules attached to them.

Example : device=xall.sys /p:328 /m:d0000 /t:c1([16])0

/t:c1([16])0

||||   \- No concentrators on line 2.

|||\----- Denotes one 16 port EBI on the concetrator.

||\------ Denotes 1st and only concentrator.

|\------- Denotes start of expanded configuration on line 1.

\-------- Denotes one concentrator on line 1.

Example : device=xall.sys /p:328 /m:d0000 /t:c2([16])0

/t:c2([16])0

||||   \- No concentrators on line 2.

|||\----- Denotes one 16 port EBI on concetrator #1.

||\------ Denotes 1st concentrator.

|\------- Denotes start of expanded configuration on line 1.

\-------- Denotes two concentrators on line 1.

Note that the 2nd concentrator was not specified since no EBI

modules are attached to it and it's the last concentrator on the

line.

Example : device=xall.sys /p:328 /m:d0000 /t:c2([][16])0

/t:c2([][16])0

||| ||   \- No concentrators on line 2.

||| |\----- Denotes one 16 port EBI on the 2nd concetrator.

||| \------ Denotes 2nd concentrator.

||\-------- Denotes 1st concentrator with no EBI modules.

|\--------- Denotes start of expanded configuration on line 1.

\---------- Denotes two concentrators on line 1.

Note that the first concentrator was specified with a place holder,

"[]", since a concentrator followed it with an attached EBI module.

Example : device=xall.sys /p:328 /m:d0000 /t:c2([8][9;16])0

/t:c2([8][9;16])0

|||| ||||   \- No concentrators on line 2.

|||| |||\----- Denotes 2nd EBI module, a 16em.

|||| ||\------ Semi-colon seperates EBI modules.

|||| |\------- Denotes 1st EBI module, an 8emp (8 + parallel)

|||| \-------- Denotes 2nd concetrator.

|||\---------- Denotes 8em on 1st concentrator.

||\----------- Denotes 1st concentrator.

|\------------ Denotes start of expanded configuration on line 1.

\------------- Denotes two concentrators on line 1.

Example : device=xall.sys /p:328 /m:d0000 /t:c2(74[8][9;16])3(12([][][16;16;8])

/t:c2(74[8][9;16])3(12[][][16;16;8])

||| || ||||   ||| | | ||  |  \- 3rd 8em EBI module on conc #3.

||| || ||||   ||| | | ||  \---- 2nd 16em EBI module on conc #3.

||| || ||||   ||| | | |\------- 1st 16em EBI module on conc #3.

||| || ||||   ||| | | \-------- 3rd concentrator.

||| || ||||   ||| | \---------- 2nd concentrator, No EBI's.

||| || ||||   ||| \------------ 1st concentrator, No EBI's.

||| || ||||   ||\-------------- 460K speed on line #2.

||| || ||||   |\--------------- Line #2 expanded configuration.

||| || ||||   \---------------- 3 concentrators on line 2.

||| || |||\-------------------- 2nd 16em EBI module on conc #2.

||| || ||\--------------------- Semi-colon seperates EBI modules.

||| || |\---------------------- 1st 8emp EBI module on conc #2.

||| || \----------------------- 2nd concentrator.

||| |\------------------------- 8em EBI module on conc #1.

||| \-------------------------- 1st concentrator.

||\---------------------------- 10M speed on line #1

|\----------------------------- Line #1 expanded configuration.

\------------------------------ 2 concentrators on line #1.