Installation Guide for

DigiBoard Intelligent Communications Boards

UNIX System V/386 Release 4

DBI 92000154A

Introduction

The DigiBoard device driver software for UNIX is installed by the DigiBoard

MenuPort Interface package; the software development system is not required to

create a new kernel.  The link kit, however, must be installed.

Software Installation

Installation of the DigiBoard device drivers is a two-part procedure:

First, the DigiBoard MenuPort Interface (MPI) package is installed onto your

hard disk. Second, the MPI package is run and the appropriate device drivers

for your system are installed and configured.

Software changes more rapidly than printed documentation can keep up. For this

reason, some of the screens or prompts may not appear exactly as shown.

Installing the MenuPort Interface (MPI) Package

System V, Release 4

1.  Log onto the console as super-user (root).

2.  To begin the installation, insert the DigiBoard MenuPort Interface device

driver diskette into drive A and type:

pkgadd -d diskette1

(If you wish to load the driver package from drive B, substitute

"diskette2" for "diskette1" in the above command.)

3.  Now run the MPI program.

Running the MenuPort Interface (MPI) Program

To run the MenuPort Interface program, log in as root and enter: mpi

After the restricted rights legend, you will see the Main menu. Items in this

menu are:

*  Config, which lets you install, change and remove device drivers for specific

boards and operating systems.

*  System, which lets you build a new kernel, update installed device drivers

and create an image of the kernel.

*  Monitor, which runs DigiBoard Port Authority (DPA), a software diagnostic

tool that allows users to easily monitor the status of the Front End

Processor/Operating System (FEP/OS), which runs on all DigiBoard intelligent

serial port boards. The DPA software also shows the status of the individual

ports. With DPA, system managers can tell instantly if a board is no longer

working, if its performance level decreases, of if a port is nonfunctional,

enabling them to work more productively and solve system errors more quickly.

DPA also provides a loop back test to confirm that the board can transmit and

receive data.

*  Notes, which displays the release notes for the supported device drivers.

MPI features context sensitive help screens; press <F1> at any time for an

explanation of the currently displayed information and choices.

System V/386, Release 4

PC/Xe, PC/Xi, MC/Xe and MC/Xi Boards

Format used for port names: /dev/term/ibp, /dev/pribp

i	Base name.

pri	Base name for DigiPrint transparent printing.

b	Board number (1-4).

p	Port identifier:

a-p     (lower case) are used for non modem control ports.  These

ports do not require DCD (Data Carrier Detect) to be asserted

before they can transmit data, and RTS and CTS pacing (see

ditty rtspace and ditty ctspace, on page 33) are not

active by default.  Primarily used with directly connected terminals

and printers.

A-P     (upper case) are used for modem control ports.  These ports

require DCD (Data Carrier Detect) to be asserted before they can

transmit data, and RTS and CTS pacing (see ditty rtspace

and ditty ctspace, on page 33) are active by default.

Primarily used with modems.

For example, Port 4 of Board 2 will be /dev/term/i2d.  The same port

with modem control enabled would be named /dev/term/i2D.

C/X, EPC/X and Xem Systems

Format used for port names: /dev/term/cpt, /dev/prcpt

/dev/term/

Directory path for tty devices.

pr	Base name for DigiPrint transparent printing. Transparent printer

devices are placed in /dev.

c     	Concentrator or EBI module letter I.D.  Legal letters are a-z.

p	Port number on the individual concentrator or EBI module (01-64).

t	Type of port (s or m).  Indicates whether port is a "standard" (s) or

"modem control" (m) port.

Standard (non modem control) ports do not require DCD (Data

Carrier Detect) to be asserted before they can transmit data, and

RTS and CTS pacing are not active by default.  Primarily used

with directly connected terminals and printers.

Modem ports require DCD (Data Carrier Detect) to be asserted

before they can transmit data, and RTS and CTS pacing are active

by default.  Primarily used with modems.

For example, standard tty port 4 on concentrator or EBI module "b"  will be

/dev/term/b04s. The same port with modem control enabled would be named

/dev/term/b04m.

Enabling Ports - System V, Release 4

Please refer to your UNIX System Administrator's Guide, under the Peripherals

Setup section, for the details on how to enable serial ports. The following

example shows how to set up serial ports for use with terminals:

Connect terminals to the ports (using a null modem, if necessary) and test the

connections to each terminal by entering the following command for each port

added: date > /dev/term/i1a

(Assuming the terminal is connected to i1a.)

Please note that in the above, the "date" command is used as a simple test, to

provide text output that can be redirected; there is no other significance to

"date" in this test. Also note that the default settings for terminals is 9600

baud, 7 data bits, even parity and 1 stop bit.

*  If the date appears on the terminal's screen, the device is properly

connected.

*  If the date does not appear on the terminal's screen, then that terminal

is not receiving data; check the power, cables, connections, etc.

*  If nonsense characters are printed on the terminal's screen, check the baud

rates, data bits, stop bits, and parity setting on your terminal. (UNIX default

parameters are 9600 baud, 7 data bits, 1 stop bit, and even parity.)

Another test you can perform will verify that a port can both transmit and

receive data:

Connect the port's transmit and receive lines together (pins 2 and 3 of a DB-25

connector, or the middle two pins of an RJ-45 connector), and enter the

following commands:

cat </dev/term/i1a &

cat /etc/termcap > /dev/term/i1a

The first command runs in the background, and directs all input from

/dev/termi1a to stdout (your screen). The second command transmits the

termcap file to /dev/termi1a. If the port is working, the data is sent out the

port, received back by the same port and displayed on your screen. If the port

fails, check another port. If more than one port fails, the problem is probably

due to memory contention, rather than defective hardware. Try reinstalling the

device driver with a different memory start address. Once you can redirect

output to your screen with the test above, perform the following steps to

enable the ports for monitoring by ttymon.

1.  Log onto the console as super-user (root).

2.  Enter: sysadm

This will bring up the System Administration menu. From this menu, select

"Ports".  The Ports menu will now be displayed.  From the Ports menu,

select "Quick Terminal".  Now select "Add".

You will now be shown a list of port numbers that are currently being

monitored for activity (the list may be empty at this time).

3.  Press <F2> ("Choices").  You will be shown a list of available ports.  Use

the arrow keys to move through the list of ports.  Press <F2> to mark ports to

be enabled (an asterisk will appear by the marked choices).

IMPORTANT!  Do not select both modem and standard devices for the same

port number (e.g. /dev/term/i1a and /dev/term/i1A).

4.  When you have completed your selections, press <F3> ("Enter").  The list

you saw in Step 3 will now include the ports you just selected.  To accept the

list, press <F3> ("Save").

5.  Now press <F3> again ("Continue"), press <F7> ("Cancel") to exit the

Ports Setup menu.  The ports are now enabled.

The default UNIX SYSTEM V REL 4 settings for these ports are: 9600, ixon,

cs7, e, 1 (these defaults may have been changed by your system administrator).

These settings can be modified by going again to the Ports menu, selecting

Ports Services, then selecting Modify.

Uninstalling the MPI Software Package

Uninstalling the MPI software package is a two-part procedure.  First, you must

remove any device drivers that were installed via the MPI software, then you

remove the MPI software package itself.

Removing Device Drivers

1.  To run the MenuPort Interface program, log in as root and enter: mpi

2.  From the Main menu, select Config.

3.  Now select Remove, from the Config menu.

4.  From the Pick List, select the device driver you wish to remove. Repeat

this step for all installed device drivers.

System V, Release 4

1.  Log onto the console as super-user (root).

2.  Enter the following command: pkgrm -n mpi

It will take some time for the new kernel to be built. When it is finished,

perform a system shutdown and reboot to activate the new kernel.

Error Messages

The following error messages are generated by the driver:

***WARNING*** No board found at port 0x320, check switch settings.

or

***WARNING*** PC/Xi at port 0x320 not resetting, check switch settings

What it means:	The board doesn't respond to reset.

Action to take:

Make sure the board is fully seated in the computer's slot.

Make sure the board's jumper settings and the device driver's settings match.

Potential hardware problem.

***WARNING*** Driver memory allocation failed for PC/Xi at port 0x320.

What it means: The operating system would not allocate memory to the driver.

Action to take: Adjust kernel resources.

***WARNING*** No memory at 0xd8000 for PC/Xe at port 0x320, check switch

settings

What it means: The driver cannot read the board's dual ported memory.

Action to take:

Make sure the board is fully seated in the computer's slot.

Make sure the board's settings and the device driver's settings match.

Make sure the jumpers or DIP switches are in their proper positions.

Make sure that there is no memory contention with other devices.

Make sure you haven't selected an 8K address for a 64K board.

Potential hardware problem.

***WARNING*** PC/Xi at port 0x320 failed diagnostics. Error #(N1,N2).

What it means:

The driver encountered an error executing on-board BIOS.  N1 and N2 are

status bits at the time of the failure.

Action to take:

Make sure the jumpers or DIP switches are in their proper positions.

Make sure there is no memory contention with other devices.

Potential software problem.

Potential hardware problem.

***WARNING*** PC/Xi at port 0x320 failed BIOS function #X. Error #(N1,N2).

or

***WARNING*** FEPOS for PC/Xe at port 0x320 not functioning. Error #(N1,N2).

What it means:

The driver encountered an error executing on-board FEPOS.  X is the BIOS

function that failed, N1 and N2 are status bits at the time of the failure.

Action to take:

Make sure the jumpers or DIP switches are in their proper positions.

Make sure there is no memory contention with other devices.

Potential software problem.

Potential hardware problem.