README.DOC File

5250 Local Gateway        1994/01/12

------------------------------------------------------------

Contents

*  About New 5250 twinax (8841-A3) Card

*  Terminating the 5250 Twinax Card

*  Notes on Using EMM386.EXE

*  Troubleshooting Tips

If the Emulator Can't Find the 5250 Twinax Card ...

If You Don't Get a Sign On Screen ...

************************************************************

------------------------------------------------------------

About New 5250 twinax (8841-A3) Card

1. Change the P2 RJ-11 (6 pin) to RJ-45 (8 pin) socket.

2. Change SW1 Switch to S2 DIP Switch (Pin 3) .

Change J5 to S2 DIP Switch (Pin 4) .

->  DIP Switch Pin 4  -- ON  (default) .(PC GND. connect to HOST GND.)

->  DIP Switch Pin 3  -- OFF  (default) .

->  DIP Switch Pin 1,2 no use .

The 8841-A2 card had support terminator switch function,

but our T-connector had support self-terminating cable .

Now the 8841-A3 card are not support terminator switch .

If you use Twisted pair wiring connector to the RJ-45

socket and need terminator ,

you can SET S2 Pin 3 ON to terminating.

------------------------------------------------------------

Terminating the 5250 Twinax (8841-A2) Card

The installation instructions for your 5250 twinax card tell

you to set the card's terminator switch to TERM if your PC

is the last physical 5250 device on a twinaxial line. Some

newer cards, however, ship with self-terminating twinaxial

cables that require the card to be set to THRU even if the

PC is the last device on the line.

If a plastic T-connector is molded to the end of the cable

supplied with your card, you have a self-terminating cable.

Set the terminator switch on the card to THRU, regardless of

where your PC is on the twinaxial line. Setting the card to

TERM while using a self-terminating cable results in double

termination, which can interfere with communications.

------------------------------------------------------------

Notes on Using EMM386.EXE

If you have a memory manager, remove it or use an

EXCLUDE statement to reserve the memory address the

5250 twinax card uses.

For example, if you use address D000 for twinax card

when you configure Memory Address option from Card Address dialog,

you should EXCLUDE the memory address range of D000 to D7FF (32K).

device=c:\windows\emm386.exe x=D000-D7FF

------------------------------------------------------------

Troubleshooting Tips

If the Emulator Can't Find the 5250 Twinax Card ...

1. Have you used the Emulator before? If not, go to Step 2.

If so, determine what has changed on your system that

could affect Emulator operation.

2. Is the correct 5250 twinax card properly installed

in your PC? If so, go to Step 3. If not, install

the card as directed in the installation instructions

accompanying it.

3. Do the card's settings match the software configuration?

If so, go to Step 4. If not, set the proper address(es)

on the card and/or reconfigure the software to match.

4. Vary memory address and I/O port address settings,

making sure the card's settings and the software

configuration match.

You can change the memory address only through

software configuration, and not with hardware

switches. Valid memory addresses are C000, C800,

D000, D800, E000 and E800. Each uses a 32K segment.

On a PC bus card, you must use hardware switches to

change the I/O port address.

If you have a micro channel card, you must run the

reference diskette to alter card settings.

Go to Step 5 if you cannot find addresses that work.

5. Identify and resolve possible conflicts in your personal

computer environment. Below are some suggestions:

Find out what settings other cards in your PC use.

Remove or reconfigure another card if necessary.

Check the commands in your CONFIG.SYS and AUTOEXEC.BAT

files. Edit unknowns from those files or boot from a

DOS diskette to bypass those files.

If you have an RCC-TWINAX PC bus card, jumper together

pins 1 and 2 on jumper J1 to enable memory at startup.

If you have a memory manager, remove it or use an

EXCLUDE statement to reserve the memory address the

5250 twinax card uses.

If you have a memory cache, disable it.

If you have shadow RAM, disable it.

If you have a mouse, disable it.

If you have a 16-bit video card, set it to 8-bit.

Run a diagnostic utility to help identify conflicts in

your system configuration.

Call Engineering Support if you cannot find or resolve a

conflict in your system.

If You Don't Get a Sign On Screen ...

The cursor position and the SA (System Available) status

line indicator can help determine where communications fail

between the PC and the host or controller.

If the cursor is in the upper right and SA isn't on the

status line, no communications exist. This symptom can

appear if the host or controller is not polling your device

(go to Step 1). A break in the physical connection also can

cause this symptom (go to Step 6).

A cursor in the upper left and SA flashing on the status

line points to interference in communications. This symptom

can result from a configuration error (go to Step 1) or a

physical problem (go to Step 6).

Communications do exist if the cursor is in the upper left

and SA is on the status line. Go to Step 1 to check the

logical connection for a configuration error.

1. Check the logical connection between your personal

computer and the host or controller. Make sure the host

is configured for the attached device. The Emulator's

device type and station address must match the host's

configuration. Make sure the device is varied on. If

the host configuration is correct and you are using a

controller, go to Step 2. If you aren't using a

controller, go to Step 3.

2. Make sure the controller is configured for the attached

device. If so, go to Step 3.

3. If you are using twisted pair wiring with the 5250

twinax card, go to Step 4. If you are using twinaxial

cable, make sure the card at the end of the run is

terminated and all other devices are set to pass-

through. If so, go to Step 5.

4. If you are using twisted pair wiring, make sure all the

5250 twinax cards on the line are terminated. If so, go

to Step 5.

5. Check the status and addresses of other devices on the

same line as the Emulator (seven devices is the maximum

total allowed). Check to see if you get a sign on screen

when the Emulator is the only active device on the line.

Or try running the Emulator using a station address that

works for another device on the line. If you find no

problems in the logical connection, go to Step 6.

6. Check the hardware connection. If you are using twisted

pair wiring, go to Step 13. If you are using twinaxial

cable, make sure the cable is attached to the 5250

twinax card. If so, go to Step 7.

7. Make sure the twinaxial cable is attached to the

preceding device on the line. If so, go to Step 8.

8. Make sure the twinaxial cable is intact. Polarity must

be correct (won't work if wires are flipped). Use a

different cable or run a continuity test. If the cable

is intact, go to Step 9.

9. Unscrew the twinaxial cable from the T-connector and

examine both cable pins. Make sure they are not broken.

Make sure they protrude far enough to make a solid

connection. If so, go to Step 10.

10. Try using a different T-connector. If the problem is not

in the T-connector, go to Step 11.

11. Examine the pigtail for damage or use a different

pigtail. If the problem is not in the pigtail, go to

Step 12.

12. Ground noise on the line may cause interference. Remove

the D-shell from the pigtail. If doing so resolves the

problem, wrap the silver braid with electrical tape and

put the D-shell back on the cable. If the problem is not

in your twinaxial cable, go to Step 15.

13. There are several standards for twisted pair wiring and

baluns. Polarity must be correct. Check the number of

flips - even works, odd doesn't. If you are using two

baluns, make sure both do the same flips. If so, go to

Step 14.

14. Disconnect unused pins. Only the two pins carrying the

twinaxial signal should go into the RJ-11 socket on the

5250 twinax card. Pin 1, 2 or 3 connected to pin 4, 5 or

6 is a valid combination of pins for the RJ-11 socket.

If the problem is not in your twisted pair wiring or

baluns, go to Step 15.

15. Power surges or lightning can ruin a card. Try using a

different 5250 twinax card. If you don't have another

card or if another card doesn't resolve the problem,

call Engineering Support.

------------------------------------------------------------

Keystroke Compiler Discontinued

Please disregard references to the Keystroke Compiler

in your documentation. Support for that program has been

discontinued, and it is no longer included on your master

diskettes.