KVGSETUP program

----------------

Since the 100VG LAN Adapter is designed with no switches or jumpers, the

configuration settings are stored in the EEPROM on the adapter and any

configuration change can be made via a software setup program.  The KVGSETUP

program supplied in the driver diskette is used for this purpose and it works

with KTI 100VG ISA, EISA, and PCI LAN adapters.

Before running KVGSETUP program, make sure:

1. it is running in DOS environment with version 3.0 or higher.

2. your network software and drivers are not running.

3. the base I/O address (factory default 300-33Fh) of an ISA adapter

does not conflict with any other system adapters, if you are using

ISA adapter. Note that all ISA adapters shipped from factory

have the same default base I/O address setting.

Just type the following command to execute the program:

A:\SETUP\KVGSETUP <enter>

Once KVGSETUP program is loaded, it detects how many 100VG adapters are

installed in the computer and shows the configuration of each adapter. The

adapter configuration is shown in a lower window on the screen. You may use

"PgUp" or "PgDn" hot keys to browse the configuration of each adapter and

designate which adapter's configuration you want to change.

The program supports either mouse or keyboard user interface.

Hot Key Functions:

PgUp  - switch to another detected adapter (if multiple cards are found)

PgDn  - switch to another detected adapter (if multiple cards are found)

F10   - main menu for "Setup" or "Diagnostics" selections

F2    - Save configuration changes into the target adapter

Alt-S - "Setup" pop-up menu

Alt-D - "Diagnostics" pop-up menu

Alt-X - Exit to DOS

Tab   - move cursor between "Ok", "Cancel", and selection block

Arrow keys - move menu bar or selection bar

For the target adapter, KVGSETUP program performs two functions in the menu:

Setup Pop-Up Menu

-----------------

Allows you to change the adapter configuration including:

Base I/O address (for ISA adapter only)

Boot ROM function & ROM address range

The configurable settings are summarized in the table below:

Settings                  ISA Adapter

-----------------     -------------------------------

Interrupt             3,4,5,6,7,9,10,11,12,15

Base I/O address      200,240,280,2C0,300,340,380,3C0

Operating mode*       I/O-mapped, memory-mapped

Boot ROM function     Disable/Enable

Boot ROM address      C0000,D0000,E0000

Settings                  EISA Adapter

-----------------     ------------------------------------------

Interrupt             3,4,5,6,7,9,10,11,12,15

Base I/O address	allocated by the system for each EISA slot

Operating mode*       I/O-mapped, memory-mapped, Bus-master mode

Boot ROM function     Disable/Enable

Boot ROM address      C0000,D0000,E0000

Settings                  PCI Adapter

-----------------     ------------------------------------------

Interrupt             allocated by the system PCI BIOS

Base I/O address      allocated by the system PCI BIOS

Operating mode        I/O-mapped

Boot ROM function     not available

* The interrupt and base I/O address for the PCI adapters are allocated by

the system PCI BIOS automatically for the PCI slot where the adapter is

installed.

* The available Operating mode selections are:

Memory-mapped at C0000H

Memory-mapped at C8000H

Memory-mapped at D0000H

Memory-mapped at D8000H

Bus-Master (for EISA adapter only)

* If memory-mapped mode is selected, a memory map address is required.

You also may enter the memory-mapped address manually if it is not in

the list. The valid address can be in the range of c0000 - ee000

or above 100000 and must be at 2000 boundry such as c2000, c4000, c6000..,etc.

<Important> If your adapter is going to operate under DOS environment, do

not specify an address above 100000. If the adapter is

going to be used in Netware server, the address segment can be

above 100000.

Do not specify a location that:

. is in the area of your system memory

. has been allocated for other system devices

. is controlled by a memory manager software

. is reserved by system BIOS for shadow RAM or memory caching.

Any configuration change which results in a conflict between the installed

adapters will prompt a warning message. All changes will not be stored into

the adapter EEPROM until a "save .." command is invoked.

Setup menu also provides an option to restore the factory default configuration

to the adapter EEPROM.

Diagnostics Pop-Up Menu

-----------------------

The program performs a Diagnostics procedure to the designated adapter. The

tests performed include :

. Registers  --- verify the functionality of the VG controller

chip and the compatibility of the selected base I/O address

. Card memory -- test the buffer memory on the adapter using the selected

operating mode

. Interrupt ---- test the functionality of the selected interrupt

After Setpup

------------

After you finished the setup procedure for the adapter on you computer, it is

recommended to reboot your system.

If your computer is an EISA computer, you must run your EISA

Configuration utility program for the adapter to store the

new settings (such as IRQ, I/O port, Slot, Memory-mapped address,

and boot ROM address) into the system EISA BIOS.

The system BIOS maintains the information of the resources (inlcuding IRQ, I/O

port, and memory address) allocation for the adapter and make sure the

resources will not be allocated to other devices when a new device is added

into your system.