Point Enabler Notes:                                   951026kvc

--------------------

The Point Enabler may be loaded either in CONFIG.SYS or

AUTOEXEC.BAT.

If loading in CONFIG.SYS, see the enclosed APEXDFM.TXT file.

If loading in AUTOEXEC.BAT, the following instructions apply.

The Point Enabler allows the Toshiba computer to use the

Noteworthy PC Card (PCMCIA) modem without the need to load

Toshiba Card Manager software.  This provides more available

conventional memory when other memory resident programs must be

loaded, for example network drivers, etc.

To use the Noteworthy Point Enabler with the default COM port

configuration of COM2: and IRQ3, type APEXDFM at the DOS prompt.

C:\>apexdfm

The enabler loads and prints the following message:

PCMCIA Super Client Driver, Version 1.00

Copyright (C) Apex Data Inc. 1994, All Rights Reserved.

Checking system configuration......

Card services not found

Socket     = A

Comport    = 2

IO address = 2f8

IRQ        = 3

Modem is ready for use

To see a list of commands, type apexdfm -h at the DOS command

line.

A:\>apexdfm -h

If you wish to change the COM: port or the IRQ you may do so

using the following parameters after the APEXDFM command:

-K[A,B,C,D] select socket (default A)

-P[1,2,3,4] select comm port (default COM2)

-I[2,3,4,5] select interrupt number (default IRQ 3)

-M[C8-DF] select memory (default D000)

-T TSR when no card services detected

-B beep (default no beep)

-N no check on function ID (default check ID)

-D display debug information (default no debug)

-H or ? display help messages

Example:

--------

To set up the Noteworthy modem as COM1:, use the following

parameters:

apexdfm -P1 -I4

Baud rate and other parameters are set in the communications

program.

***End of APEXDFM.DOC***