| Field Note: 139 |  | Distribute freely |
| --- | --- | --- |

Subject:  INI file setting to improve Captivator Pro/TV Tuner scanning

Date Created: July 28, 2026

Product: Captivator Pro/TV

Some users may experience problems with channel scanning on Captivator Pro/TV.  This may be due to the picture lock delay time being too short, the default setting is 100 milliseconds.  Extending this to 200 milliseconds can improve the reliability scanning.  The picture lock delay can be set using an .INI setting in TUNER.INI, e.g.:

[System]

PicLockReadDelay=200

Nick Jurascheck

July, 26