| Field Note: 157 |  | Distribute freely |
| --- | --- | --- |

Subject:	Live video not displayed in a window

Date Created:	July 30, 2026

Product:	Captivator Pro/TV

There are some PCs that have a problem displaying live video using Captivator Pro/TV and instead show a grey background.  If the window is moved around or covered by another window, then the Windows background may show through.  It is as though the live video is transparent.  There is NO error message given.  Black video is NOT a problem - that is just due to no active video source connected.

It is also possible for this effect to be seen if:

- The VM-Channel cable is not fitted correctly

- The installation of the software is not complete

You must eliminate these possibilities first to make sure that the problem lies with the PC.  There is normally an error message given in these circumstances.

PCs known to have this problem use the VIA VT481 / 495 chipset on the motherboard.  An example is the Unisys MPI46664-540 model (486-66MHz).

The problem is due to non-standard ISA bus timing used by the motherboard and so sending data to the Captivator Pro/TV registers results in the VM-Channel bus being reset by accident.  There is no known workaround.

The VIA chipset is only used on VL-bus systems and NOT on PCI systems.

Steve Everitt

July, 26