| Field Note: 184 |  | Distribute freely |
| --- | --- | --- |

Subject:	Cable TV tuning in North America

Date Created:	July 31, 2026

Product:	Captivator Pro/TV

There is a problem with tuning to US cable TV stations in the ‘A ‘ range using v1.0, v2.0 or v3.0 of the Captivator Pro/TV software.

The solution is to:

1.  Install v3.0 of the software

2.  Obtain the patch #104 from VideoLogic BBSs.

3.  Delete the file  TUNER.INI  in the  \WINDOWS  directory

(This removes all previous tunings for Broadcast and Cable)

4.  Copy the file  TUNER.INX  (from the patch) over the existing file in the

\VLPOWER  directory (or wherever you installed the software)

5.  Run the TV tuner control panel again and select North America

The problem is solved in v3.0.1.  If you want to install this version over an existing installation then delete TUNER.INI first.

There is no problem with European cable regions.  If a separate US cable tuner box is used and it’s TV output connected to Captivator Pro/TV, and the Captivator Pro/TV is set to Broadcast, then this should have no problem.

Steve Everitt

July, 26