| Field Note: 66 |  | Distribute freely |
| --- | --- | --- |

Subject:  Compatibility with Orchid Fahrenheit VA/VLB (local bus VGA)

Date Created: July 31, 2026

Product: DVA-4000/ISA

Problem with Orchid Fahrenheit VA/VLB & DVA-4000 (BIOS Version 1.2)

When used with the Orchid Fahrenheit VA/VLB the DVA-4000 produces no display.  This problem is caused by the Orchid Fahrenheit incorrectly setting the register 3C6h on power-up, or following a mode change.

Register 3C6h is the PAL Mask register of the VGA palette. According to the IBM Technical Reference Manual, 3C6h should be set to 0FFh by the BIOS at power-on or on completion of a mode change.  Version 1.2 of the Orchid BIOS does not appear to set this register correctly resulting in a blank output from the DVA-4000.

A partial fix is available in the form of a program called RESET3C6.EXE. RESET3C6.EXE is a TSR which sets 3C6h to 0FFh following a mode change. This can be run from AUTOEXEC.BAT in order to enable the DVA-4000 display. Unfortunately, the normal power-on display will not be visible on the DVA-4000 display. This patch is available from Regional Support Groups and CompuServe.

The most satisfactory solution would be for the Orchid BIOS to be modified to write the correct value into 3C6h, and VideoLogic are communicating with Orchid to this end.

Colin Crawford

July, 26