| Field Note: 141 |  | Distribute freely |
| --- | --- | --- |

Subject:  Profiling with AVI's

Date Created: July 31, 2026

Product: GrafixStar

Within Windows whenever an AVI clip is to be played for the first time in its current graphics mode, Windows will do a Graphics Profile of the display, providing it has not already done so for that particular mode in the past.

However, a problem has been found when viewing Indeo 3.2 and Cinepak AVI’s of a non regular size through the DCI (the device control interface that GrafixStar drivers use to handle video clips playback). Each time an AVI clip that has a pixel size which is a non-multiple of 16 is viewed, Windows fails to remember the information it derived from its Graphics Profiling. Additionally SmartScale fails to function with these odd size clips.

This means that whenever a non regular sized AVI clip is viewed using a GrafixStar driver, the Graphics Profiling occurs each time the clip is to be played.

This is not a serious problem in that AVI clips that are non-multiple’s of 16 are rare. However, one example of an application that contain a few clips of this nature is Encarta 94.

Dave Marsland

July, 26