| Field Note: 202 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Using SmartTools with Non-English versions of Windows 95

Date Created: 	July 31, 2026

Product: 	GrafixStar 300/400/500/700

SmartTools version 1.0.1 is a patch (Patch 110) release that is fundamentally SmartTools version 1.0 with  the capability to run on the German, Italian and Swedish versions of Windows 95. In order for SmartTools to run on other Non-English versions of Windows 95, the registry requires additional information. To update the registry an executable text file has to be created that informs SmartTools of the specific language reference used for various strings.

An example of the registry update file (Smartnls.reg) is provided as part of Patch 110 and can be seen in Figure 1 below -

REGEDIT4

[HKEY_CURRENT_USER\Software\VideoLogic\SmartNLS]

[HKEY_CURRENT_USER\Software\VideoLogic\SmartNLS\Strings]

"UseCustom"="Yes"

"Apply"="&Apply"

"Display"="Display Properties"

"Settings"="Settings"

Figure 1 - 	The file ‘SmartNLS.reg’

By modifying the ‘SmartNLS.reg’ file, you can update the registry for any language version of Windows 95. As an example, this field note documents the steps necessary to update the registry for the Dutch version of Windows 95.

Firstly, when running Windows, run up the Display Properties Control Panel (right click on the desktop, and select the last entry of the pop-up menu), and then replace the values in the file ‘SmartNLS.reg’ with the Apply button text, Display Properties control panel title and the Settings page title. A pictorial example of the text required is given below.

Figure 2 -	The English Display Properties Control Panel

By looking at the English version of the Display Properties Control Panel and comparing the Non-English version being used, it is possible to ascertain the text required for the text file ‘SmartNLS.reg’.

Figure 3 below, depicts the Dutch Display Properties page with Settings being the active Tab.

Figure 3 -	The Dutch Display Properties Control Panel

IMPORTANT NOTE: the Apply button text includes a keyboard accelerator. The character which appears underlined (in the button text) must be preceded with an ampersand, hence in the example the string is "&Apply" NOT "Apply". This is usually the first character, but may not always be.

Only the right hand side of each line should be changed. The UseCustom key can actually have any value - so long as it is not empty these strings will be picked up instead of those compiled in (which default to English, if the language is not yet supported.

REGEDIT4

[HKEY_CURRENT_USER\Software\VideoLogic\SmartNLS]

[HKEY_CURRENT_USER\Software\VideoLogic\SmartNLS\Strings]

"UseCustom"="Yes"

"Apply"="&Toepassen"

"Display"="Eigenschappen voor Beeldscherm"

"Settings"="Instellingen"

Figure 4 - 	The executable registry update for the Dutch

version of Windows 95.

The file ‘SmartNLS.reg’ is an executable and running it will instantly update the registry, with a Windows reboot not being required. It should be noted however that any future updates of the same area of the registry will result in the previous information being overwritten.

Dave Marsland

July, 26