| Field Note: 236 |  | Distribute freely |
| --- | --- | --- |

# Subject: Installing 'DISK 2' for GrafixStar 400 or 600 through the Interactive Installation Guide of the Software Library CD-ROM version 3.

# Date Created: 3 March 1997

# Product: GrafixStar 400, GrafixStar 600

When installing DirectVideo as part of driver installation for GrafixStar 400 or 600 through the Interactive Installation Guide on the Software Library CD-ROM version 3, you are asked to insert the GrafixStar DISK 2, as shown below:

This is caused by the Interactive Installation Guide calling the mechanism to provide options after a manual display driver installation.

To continue, browse to the file \GS600\WIN95\DISK2\REDIST.IHV on your CD-ROM, and click OK.

This is unique to installing through the Interactive Installation Guide; manually installing the display drivers using the ‘Software installation for Windows 95’ documented in the readme files VLGX400A.DOC and VLGX600A.DOC is not a problem.

VideoLogic Support Team
March, 97