| Support Note: 240 |  | Distribute freely |
| --- | --- | --- |

# Subject: Installing Windows NT 3.51 drivers from the VideoLogic Software Library

# Date Created: March 11, 1997

# Product: GrafixStar Range

When installing graphics drivers under Windows NT 3.51, the system expects the drivers to be located on a floppy disk.

The Windows NT 3.51 drivers supplied on the VideoLogic Software Library CD-ROM therefore have to be copied to a floppy disk before being installed.

Follow the steps below to install NT 3.51 drivers for any GrafixStar card:

1.	Copy the driver files from the others\NT351 subdirectory of the relevant GrafixStar folder of the VideoLogic Software Library CD-ROM to a floppy disk.

2.	In the Main Program group, double click the Control Panel icon.

3.	In the Control Panel double-click the Display icon.

4.	Click Change Display Type.

5.	Next to Adapter Type, click Change.

6.	Click Other.

7.	Insert the floppy disk with the driver files into your floppy drive and click OK.

At the Driver Select window, click Install.

After the files have been copied, Windows NT prompts you to restart your system.

After your system has restarted, you can change resolution by double-clicking the Display icon on your Control Panel.

VideoLogic Support Team
March, 97