| Field Note: 0220 |  | Distribute freely |
| --- | --- | --- |

Subject:  GrafixStar 600 Maintenance Release for Windows 95

Date Created: July 31, 2026

Product: GrafixStar 600

There are two known problems with the GrafixStar 600 GrafixMAX v4.1.1 display driver.  It can be found that under certain conditions, the ‘START’ button and it’s banner declaring ‘Windows 95’ may be displayed incorrectly.  The other problem is Xing MPEG player performance in 1024x768, 256 colors when in fullscreen playback mode.  Both problems have been fixed and is available as a Windows95 display driver maintenance release.

The maintenance release is provided in the form of a driver installation set (This will not include SmartTools).  To receive the maintenance release, you need to obtain PATCH220.ZIP from VideoLogic’s Online Services.  See the support information found in your GrafixStar 600 manual for details on VideoLogic’s Online Services.

Once downloaded, prepare a blank formatted 1.44MB 3.5” diskette and label accordingly.  The PATCH220.ZIP file needs to be extracted with an extraction program which is available on the ‘VideoLogic Software Library: Drivers and documentation CD’ and can be found in the UTILS folder from CD release v1.1 and later.  When using the extraction program, please allow the extraction utility to create stored directories.  Examples of this can be found later on in this document.

WINZIP

Check the folder names option found under the Actions command on the menu bar and then choose Extract (CTRL-E).  You will see the following dialog box.

PKUNZIP

[-d] option to restore/create Directory structure stored in .ZIP file.

Eg. PKUNZIP.EXE -D PATCH220.ZIP *.* A:\

Installation

- Switch your computer on and start Windows 95.

- Insert the PATCH220 diskette into your system.

- Right click the desktop and choose the ‘Settings’ page.

- In the ‘Adapter Type’ section, click on the ‘Change’ button.

- From the ‘Select Device’ dialog box, choose ‘Have Disk’.

- Choose Browse and locate the floppy drive.  Choose OK twice, select GrafixStar 600 GrafixMAX from the list and click OK.

- When the GrafixStar files have been copied to your PC, a message will appear asking you to restart Windows. Choose OK.

- Windows 95 will restart and a 'VideoLogic GrafixStar Installation' dialog box will appear, allowing you to install SmartTools, DirectVideo which includes DirectDraw, or to Cancel the options.  Choose the desired option:-

SmartTools:

VideoLogic SmartTools installation will be executed on selection of this option.  Please follow the instructions that appear on the screen.

Please Note: You will need to supply the SmartTools installation diskette if you want to install SmartTools.

DirectVideo:

Microsoft DirectDraw v1.0 installation will be executed on selecting this option which will install DirectVideo.  In order to utilise this software, the user needs to restart Windows 95.

VideoLogic Support Team

July, 26