VideoLogic GrafixStar 750 for Windows 95

26th February 1997

Contents

1. 	Introduction

2. 	Installing GrafixStar 750

3.	Removing GrafixStar 750 from Windows 95

4.	Removing SmartTools

5.	Accelerated Video Modes and DirectDraw

6.	Feature Connector

7. 	Troubleshooting

1. Introduction

This file contains installation instructions, late-breaking information, troubleshooting information, and notes on the GrafixStar 750.

If you are reading this file outside of the VideoLogic Installer and want to install GrafixStar 750 manually, refer to section 3 below or the User Guide for complete instructions on how to do this.

2. Installing GrafixStar 750

Introduction

The easiest way to install your GrafixStar 750 is to install the hardware first, referring to the User Guide, and then install the software using the Interactive Installation Guide which runs automatically when you insert the CD into your PC. However, if you don’t have a User Guide, or you want to install the software manually, you should refer to the following instructions.

Handling GrafixStar 750

Follow these guidelines to minimize the risk of electrostatic charges damaging the card:

Keep the card in its protective packaging until you are ready to install it.

Just before handling the card, touch the metal frame of your computer with the back of your hand to discharge any static electricity which may have built up on you or your clothes.

Hold the card by the adapter bracket or by its edges; do not touch the printed circuit board.

Do not place the card on top of your computer or on any other metal surface.

Make sure that the card is not accidentally touched by anyone else.

Handle the card with care to avoid damage to the fragile electronic components mounted on the circuit board.

Installing the hardware

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

1.	Switch off your computer, monitor, and any hardware devices such as printers connected to your computer. Leave the power cable connected to the power outlet so that your computer is grounded.

2.	Disconnect your monitor cable from your computer.

3.	Remove the computer’s cover. You may need to refer to the documentation supplied with your computer for instructions on how to do this.

4.	Remove your existing display card or disable your built-in graphics controller. Refer to the documentation supplied with your computer for instructions on how to disable a built-in graphics controller.

5.	Locate an empty PCI bus expansion slot and remove the slot cover. Keep the screw, you will need it later to secure the card.

6.	Touch the bare metal chassis of your computer with the back of your hand to discharge any static that may have built up on you or your clothes.

7.	Align GrafixStar 750 with the slot you have chosen. Firmly press the card into the slot. You will need to use some pressure to push the card into position, but do not use excessive force.

8.	Secure GrafixStar 750 using the screw that you removed previously.

9.	Replace the cover of your computer.

10.	Connect the monitor cable to GrafixStar 750. All cables connected to GrafixStar 750 should be properly shielded and comply with radio frequency interference standards.

11.	Switch on your computer and your computer monitor. You are now ready to install the software.

Installing the software

The easiest way to install the software is to use the Display Properties window. Installation varies slightly depending upon which build of Windows 95 you are using. Use the following steps to find out which build of Windows 95 you are using, then follow the instructions for that version.

1. 	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. 	Double-click the System icon to open the System Properties Window.

3. 	Ensure that the General tab is selected. This page has details about the System, Registration information, and Computer details. The build number is located in the System section, under the 'Microsoft Windows 95' line.

Software installation for Windows 95 build 4.00.950 or 4.00.950a

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Change Display Type button.

5.	Make sure that the Monitor Type line contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

6.	Click the Change button for the Adapter Type.

7.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs750\win95\disk1 where [disk] is your CD drive.

8.	Click GrafixStar 750 GrafixMAX from the list. Click OK.

9.	When the GrafixStar files have been copied to your PC, the Change Display Type window is displayed again. Click Close.

10.	The Display Properties windows is displayed again. Click Apply.

11.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

12.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

13.	Follow the instructions that appear on the screen.

Software installation for Windows 95 build 4.00.950 B (otherwise known as OSR2)

1.	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double-click the Display icon to open the Display Properties Window.

3.	Click the Settings tab. The Settings page is displayed.

4.	Click the Advanced Properties button.

5.	Click the Monitor tab. The Monitor page is displayed.

6. 	Make sure that the Monitor page contains the name of your monitor. If not, click the Change button and click the name of your monitor on the list.

7. 	Click the Adapter tab. The Adapter page is displayed.

8.	Click the Change button to change display driver.

9.	Insert the GrafixStar CD into your computer (close the Interactive Installation Guide if it autoruns at this point) and click Have Disk from the Select Device menu. Browse to the folder [disk]:\gs750\win95\disk1 where [disk] is your CD drive.

10.	Click GrafixStar 750 GrafixMAX from the list. Click OK.

11.	When the GrafixStar files have been copied to your PC, the Advanced Display Properties window is displayed again. Click Close.

12.	The Display Properties windows is displayed again. Click Apply.

13.	A message appears asking whether you want to restart your computer. Click Yes. Windows 95 restarts with the new GrafixStar Windows 95 drivers.

14.	When your computer has restarted a dialog box is displayed asking whether you want to install SmartTools with DirectVideo, or just SmartTools, or neither.

SmartTools is desktop control software that enables you to configure and customize your display, set a virtual desktop, and build your own toolbars. Click SmartTools to install SmartTools and DirectVideo.

DirectVideo is a set of drivers that optimize your computer’s performance when running DirectDraw video applications. If you only want to install DirectVideo, click DirectVideo.

If you don’t want to install either SmartTools or DirectVideo, click Cancel. Note that video playback performance will not be optimized without DirectVideo.

15.	Follow the instructions that appear on the screen.

3. Removing GrafixStar 750 from Windows 95

You remove the card from Windows 95 in two stages: first the hardware, then the software.

To remove GrafixStar 750 from your Windows 95 system:

1. 	Open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2. 	Double-click the System icon to open the System Properties Window.

3.	Click the Device Manager tab.

4.	Double-click Display Adapters.

5.	Click GrafixStar 750 GrafixMAX, and click Remove.

6.	A dialog box is displayed asking you if you want to restart Windows. Click No.

7.	Shut down Windows 95.

8.	Switch off your computer, monitor, and any hardware devices such as printers 
connected to your computer. Leave the power cable connected to the power outlet 
so that your computer is grounded.

9.	Disconnect the monitor cable from your computer.

Warning: Always switch off your computer before removing the cover and observe the warnings specified in the manufacturer’s documentation.

10.	Remove the computer’s cover. You may need to refer to the documentation supplied 
with your computer for instructions on how to do this.

11.	Remove the GrafixStar card from its PCI slot and replace it with a different graphics card.

12.	Replace the cover of your computer.

13.	Switch on your computer and your computer monitor. Windows 95 detects the new graphics card and installs drivers for it.

14.	When Windows 95 is running with the new graphics card’s drivers you can delete the following files from the disk on which you are running Windows 95:

\windows\vl750dd.ini

\windows\system\s3dtkw.dll

\windows\system\s3kernel.dll

\windows\system\vlenable.sys

\windows\system\vlgx750a.drv

\windows\system\vlgx750a.vxd

\windows\system\vlgxcpl.dll

\windows\system\vlregio.dll

\windows\system\vlspdd32.dll

4. Removing SmartTools

Note: Save any open documents before removing SmartTools, as you will need to restart Windows.

To remove SmartTools:

1.	Click the Start button, and point to Settings.

2.	Click Control Panel.

3.	Click Add/Remove Programs.

4.	From the list of programs displayed, click VideoLogic SmartTools, and click Add/Remove.

5.	Follow the instructions that appear on the screen.

5. Accelerated video modes and DirectDraw

Introduction

This section describes the display modes available with overlay video acceleration, and how GrafixStar 750 interacts with DirectDraw.

Display modes with video acceleration

The GrafixStar 750 uses an intelligent graphics engine which provides true-color video playback even when the display is set to 256 colors. This means that you don’t have to sacrifice speed and functionality by being in a true-color display mode; instead, you can play video clips in true-color while using a 256 or 65k color mode. However this is NOT available in all display modes. The modes in which this is possible are listed below and are only achievable depending upon the video clip size and player component (DirectVideo, ActiveMovie, or Xing MPEG player) used.

Display size	Colors	Refresh rate (Hz)

640 x 480	256	60, 72, 75, 85, 90, 100, 120, 150

800 x 600	256	56, 60, 72, 75, 85, 90, 100, 120, 150

1024 x 768	256	43i, 60, 70, 75, 85, 90, 100

1152 x 864	256	60, 70, 75, 85, 90

1280 x 960 	256	60, 70, 75

1280 x 1024	256	43i, 60, 70, 75

640 x 480	65,536	60, 72, 75, 85, 90, 100, 120, 150

800 x 600	65,536	56, 60, 72, 75, 85, 90, 100, 120, 150

1024 x 768	65,536	43i, 60, 70, 75, 85, 90, 100, 120

1152 x 864	65,536	60, 70, 75

1280 x 960   	65,536	60, 70, 75

1280 x 1024 	65,536	43i, 60, 70

640 x 480	16.7m	60, 72, 75, 85, 90, 100, 120, 150

800 x 600	16.7m	56, 60, 72, 75, 85, 90, 100, 120, 150

1024 x 768	16.7m	43i, 60, 70, 75, 85, 90, 100, 120

1280 x 1024 	16.7m	43i, 60, 72

All other modes display AVI files using software only, which gives poorer quality playback. If you want to playback MPEG files without special hardware, you can obtain a FREE copy of Xing MPEG Player by returning your GrafixStar 750 registration card.

Using games with DirectDraw

GrafixStar 750 display drivers for Windows 95 are compatible with DirectDraw 2.0.

The GrafixStar 750 display drivers do not automatically install DirectVideo with DirectDraw 2.0 components. To install, run GS750\WIN95\DISK2\REDIST.IHV\SETUP.EXE from the VideoLogic Software Library CD, after which Windows 95 needs to be restarted. To remove DirectDraw from your PC, delete the following files, where [drive] is the hard drive on which you are running Windows 95.

File	Location

Ddhelp.exe	[drive]:\windows\system

Dsound.vxd	[drive]:\windows\system

Dsound.dll	[drive]:\windows\system

Ddraw.dll	[drive]:\windows\system

Ddraw16.dll	[drive]:\windows\system

D3drm.dll	[drive]:\windows\system

D3dxof.dll	[drive]:\windows\system

D3dim.dll	[drive]:\windows\system

D3drampf.dll	[drive]:\windows\system

D3drgbf.dll	[drive]:\windows\system

D3drm8f.dll	[drive]:\windows\system

D3drm16f.dll	[drive]:\windows\system

D3drm24f.dll	[drive]:\windows\system

D3drm32f.dll	[drive]:\windows\system

D3drg8f.dll	[drive]:\windows\system

D3drg16f.dll	[drive]:\windows\system

D3drg24f.dll	[drive]:\windows\system

D3drg32f.dll	[drive]:\windows\system

D3dhalf.dll	[drive]:\windows\system

Dplay.dll	[drive]:\windows\system

Dpserial.dll	[drive]:\windows\system

Dpwsock.dll	[drive]:\windows\system

Dvideo.dll	[drive]:\windows\system

6. Feature Connector

The GrafixStar 750 feature connector is disabled by default.

Before enabling the feature connector, you must set a display mode of 640x460 pixels with 16 colors, or 800x600 pixels with 16 colors. If you have SmartTools installed, you must deinstall SmartTools or install a generic SVGA display driver to be able to change to 16 colors.

Follow the steps below to enable the feature connector:

1.	Click the Start button, point to Settings, and click Control Panel

2.	Click Display, then the Settings tab.

3.	Change the display mode to either 640x460 pixels with 16 colors, or 800x600 pixels with 16 colors.

4.	Add the following line to the start of the CONFIG.SYS file on the drive on which you are running Windows 95:

\windows\system\vlenable.sys

5. 	Reboot your system.

7. Troubleshooting

Introduction

This section describes common problems you may encounter when running GrafixStar 750.

Display Properties

Errors when running virtual desktops and/or changing modes

Some applications, especially those that display video, report errors when running a virtual desktop or changing modes. Every effort is made to prevent such problems but if they do occur we recommend that you change to a non-virtual desktop mode, or close the problematic application before changing modes.

Modes

16-color modes

SmartTools is not designed to work with a 16 color palette (4 bits per pixel). You must change to a higher palette via the Display Settings dialog box of Control Panel to enable SmartTools to work.

Monitors

Display sizes

Windows 95 does not restrict the generic SVGA monitor settings to a given set of display sizes, so SmartTools allows any display size to be selected. If you have a standard monitor selected, there is therefore a possibility that you could zoom out to a display size that your monitor does not support.

Choose the correct monitor type, or contact your monitor supplier for more information.

Corrupt or blank display

Windows 95 does not restrict the generic SVGA monitor settings to a given set of refresh rates, so SmartTools allows any refresh rate to be selected. If you have a standard monitor selected, there is therefore a possibility that you could restart your system with a refresh rate that your monitor does not support. This usually results in a corrupted or black display in Windows 95. To rectify this problem, restart Windows 95, holding down the CTRL key on your keyboard. This starts Windows 95 in the lowest possible refresh rate for the resolution selected.

Toolbars

Disappearing toolbars

When using SmartDesktop to zoom into the desktop, you may lose sight of the SmartTools toolbars. You can relocate a toolbar within your current display using the Find Now feature. This is located by clicking the Properties button of the toolbar on the Toolbar Properties page of SmartTools. Click the Find Now button to move the active SmartTools toolbar to a visible location within the display.

Toolbar Properties page of SmartTools

Maximize in Display

The SmartDesktop Maximize on Display utility maximizes your applications to the full size of your display, so part of your application window may disappear behind the Windows 95 taskbar. If you use this feature, we recommend that you set the taskbar to auto-hide.

Moving and deleting buttons from toolbars

You can move buttons around on the toolbars by holding down the SHIFT key and dragging them to the desired location. To remove a button, drag it off the toolbar while holding down the SHIFT key.

HotKeys

DOSBox

The SmartDesktop hotkeys will not function when a DOS box is active.