************************************

*                                  *

*     MIC System - Release Note    *

*                                  *

*          Release 3.0.10          *

*                                  *

************************************

*************************  I M P O R T A N T  *************************

1.  MIC Release 3.0.10 must be used with MS-DOS version 3.1 or above.

2.  MIC Release 3.0.10 cannot easily be installed on a PC which is

not equipped with a hard disk.  If your machine does not have

a hard disk you must contact VideoLogic Customer Support for

assistance.

3.  MIC Release 3.0.10 is distributed on either 2 x 5.25" diskettes

or 1 x 3.5" diskette.  Where the following text refers to the

"first" diskette this implies the single diskette when using

the 3.5" medium.

4.  Installation procedure.

The installation procedure must be used as the majority of the

files on the distribution diskette are held in a compressed

form.  Merely copying the files will NOT result in a successful

installation.

To install from A: to C: insert the first distribution

diskette in drive A: and at the C: prompt type:

A:INSTALL A: C:

The installation procedure performs the following steps:

a. Installs the VideoLogic Applications Management System

(AMS) program and data files into the directory \AMS of

the designated destination drive (C: in the example given

above).

If the directory does not already exist it will be created.

If AMS is already installed on your machine your

Applications Library File (AMS.ALF) will be updated to

include several new MIC-related applications; the

original contents will be preserved.  If you have

modified the AMS help file (AMS.HLP) you should take a

copy of this file as it will be overwritten - any changes

you have made will have to be re-applied.

The following files will be installed, overwriting any

existing copies:

AMS.EXE

AMSMENU.EXE

AMS.HLP

b. Installs the MIC System Software Release 3.0.10 into the

directory \MICFILES of the designated destination drive.

It is recommended that MIC be installed onto the C: drive.

If the directory does not already exist it will be created.

If the directory already exists its files will be DELETED.

If there are any files in \MICFILES which you wish to keep

you should take a copy of them (in another directory) before

starting the installation procedure.

If you have a previous version of MIC software in any

directory other than \MICFILES we recommend that you

remove the program and data files to avoid any possibility

of using incompatible versions.

c. Invokes the MIC Set-up Program (MICSETUP.EXE) to allow

you to define your hardware configuration and start-up

software state.  Your CONFIG.SYS and AUTOEXEC.BAT files

will be modified appropriately - non-MIC related

statements in these files will be preserved.

Detailed information on the MIC Set-up Program can be

found in the "MIC User Guide".

d. Invokes the MIC Set-up Program (MICSETUP.EXE) to allow

you to disable a limited sub-set of MIC Commands which

may not be supported by your hardware (ie. INTERLACE).

Detailed information on the MIC Set-up Program can be

found in the "MIC User Guide".

5.  Use of interrupt 67h

By default MIC v3.0.10 System Software still uses software interrupt

67 (hex).  However, if it is important that MIC does not use

this interrupt (when you are using EMS, for example) then the

parameter /NOINT67 can be used when loading MIC-2000.EXE.

The MIC Loading System will automatically apply this parameter

to MIC-2000.EXE if the NOINT67 keyword appears as the first

statement of the Dynamic Configuration File being used to load

MIC.

Dynamic Configuration Files are described in the "MIC User

Guide".

*************************  N E W   F E A T U R E S  ************************

- Support for the Pioneer LD-V2200 & LDP-4300 videodisk players

- MS DOS 5.0 support

************************  S P E C I A L   F I L E S  ***********************

When MIC has been installed, the following undocumented files are placed

in the MICFILES directory.

4.1  TEN-MIC.EXE  - Allows you to use MIC commands from the

TenCORE authoring system (versions 2.0-2.03).

TEN-MIC is used in conjunction with the TenCORE

source unit MICD.SRC.  This routine executes

an MIC command given within a TenCORE 'PACK'

command.

ex:  PACK        MICCON;MICLEN;STILL 1025

DO MICD

PACK        MICCON;MICLEN;AUDIO ON/A1

DO MICD

PACK        MICCOM;MICLEN;TRANSC ALL

DO MICD

etc.

Variables within the TenCORE Defines Unit

must contain the following:

MICCOM,80

MICRET,80

.    keytype,5

.    keycode,5

.    filler,70

MICLEN,2

Include TEN-MIC within your AUTOEXEC.BAT and

reset the TenCORE default of 240 lines to 200

lines.

eg: AUTHOR/D=EGA 200

Please Note:  These files have been made redundant

by subsequent releases of TenCORE, and have been

provided to maintain compatibility with past

applications.

4.2  MICD.SRC     - A TenCORE source module providing the

functionality embodied within the "DO MICD"

statements in the TenCORE/MIC interface example

described in the section above (on TEN-MIC.EXE).

4.3  PCSB-MIC.EXE - Allows you to use MIC from IBM's PC Storyboard.

** Note ** This interface is valid only for PC

Storyboard, not the newly released PC Storyboard Live.

PCSB-MIC is a memory resident program. It must

be loaded before you enter Storyboard.

The program works by copying a file (MIC-SB.nnn)

containing MIC commands to the MIC Device Driver.

You edit sequences of MIC commands into

individual files named MIC-SB.nnn where the

extension (ie, the nnn part) is a three digit

number between 001 and 999. These sequences are

then incorporated into your storyboard program

and are invoked by using the /INT 66h command.

Example:

To issue the commands held in MIC-SB.014

insert /INT 66h,14 into your story. The

"14" indicates file extension .014.

Although the interface is essentially output only

(it is not possible to get response back from the

Virtual Device Interface), you can still use READKEY

to wait for keyboard input or return at the end of

the current video segment.

4.4  MIC-MARK.EXE - MIC-MARK.EXE and MIC-REL.COM are used to remove

MIC-REL.EXE    memory resident programs from memory.

The programs are used in the following way:

1).   Run the program MIC-MARK before installing

the memory resident programs you wish to

deinstall later.  This will mark the current

memory position, save the processor interrupt

vectors and the state of the MIC System.

2).   Install whatever memory resident programs you

wish to use in the normal way.

3).   When you what to deinstall the memory resident

programs above the last 'mark' simply run the

program MIC-REL. This will release the memory

resident programs above (and including) the

last MIC-MARK, and restore the interrupt vectors

and the state of the MIC System.

MIC-MARK and MIC-REL may be 'stacked' as many times

as desired. MIC-REL releases all the memory above

and including the last MIC-MARK.

Due to the way DOS handles batch files it is not

possible to perform the following:

First, from the command line:

MIC-MARK

MIC-LM

Then, from within a batch file:

MIC-REL

LOTUS

MIC-MARK

MIC-LM

This will result in a large 'hole' in memory.  It

is better to invoke MIC-REL from the command line

before executing the batch file to run LOTUS,

MIC-MARK and MIC-LM.

However, it is possible to run MIC-MARK at the

begining of a batch file and MIC-REL at the end.

This would allow you to remove any memory resident

programs loaded by the batch file.

*****************************    F I L E S  *****************************

Your distribution disk contains the following files:

README.DOC      - This file.

README.EXE      - The README file display program.

INSTALL.BAT     - First stage batch file for installing MIC

Release 3.0.10 program and data files.

INSTALL2.BAT    - Second stage batch file for installing MIC

Release 3.0.10 program and data files.

ENDINSTA.BAT    - Terminating batch file for installing MIC

Release 3.0.10 program and data files.

PRODDATE.DAT    - Production and modification information file.

INS*.*          - Installation procedure program and data files.

RW.EXE          - Program to change the attributes of a file.

DIRCHK.EXE      - Program to check for the existance of a directory.

ARCE.COM        - File decompressor.

AMS.ARC         - Compressed library containing the VideoLogic

Application Management System program and data

files.

DISK1.ARC,

DISK2.ARC       - Compressed libraries containing the MIC

Release 3.0.10 program and data files.

MIC-2000.EXE    - The MIC System Kernel.

SCREEN.SYS      - The MIC Virtual Screen Input System.

#