Field Note: 119

Subject:	PCI Movie & IRQs

Product:	PCI Movie

It is possible under certain conditions Windows will not function correctly with PCI

Movie.

These conditions arise when another add-in card uses either IRQ 2 or 9.  Under these

conditions an IRQ conflict will occur, causing problems in Windows.  IRQ 2 is a

cascade of IRQ 9, and therefore they are effectively the same IRQ.

To rectify this problem, ensure that all other add-in cards use alternative IRQs.

To change the IRQ on a PCI add-in card...

Run the system BIOS and ensure that IRQ 9 is available to the PCI Bus (see the

PC's user manual).

If another PCI add-in card is using IRQ 2 or 9 swap slots with PCI Movie.

To change the IRQ on a ISA , EISA, VLB add-in card...

Refer to the add-in card's user manual.

To change the IRQ on a "Plug and Play" system...

Refer to the "Plug and Play" user documentation.

Martin J Parkes

Date: 09 September, 94