| Field Note: 170 |  | Distribute freely |
| --- | --- | --- |

Subject:	PCIMovie Windows 95 (Non VMC) display drivers

Date Created:	July 28, 2026

Product:	PCIMovie

This release of PCIMovie display driver does not support VMC and video acceleration.  If you wish to use VMC devices please read Fieldnote FN168.DOC.

Supported Modes

The supported modes for this driver are detailed in the table below:

| Display Mode | Colour Depth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 16 colours | 256 colours | 65k colours | 16.7m colours |
| 640x480 | 4 | 4 | 4 | 4 |
| 800x600 | 4 | 4 | 4 | 4 |
| 1024x768 |  | 4 | 4 |  |
| 1152x900 |  | 4 |  |  |
| 1280x1024 |  | 4 |  |  |

These drivers are supplied by Weitek, and have changes to the INF file to ensure that only supported modes are offered with the use with the PCIMovie.

Installation

To install the driver:-

1. From the desktop, right click the mouse to open the 'Display Properties' window.

2. Press the 'Change Display Type' button on the 'Settings' page.

3. Under the 'Adapter Type' section press the 'Change' button.

4. Select the 'Have Disk' button and use the 'Browse' button to point to the directory or disk that contains the driver information.

5. Select the 'PCIMovie (Non VMC)' driver from the available list.

6. When Windows has installed the PCIMovie driver restart your computer.

The Windows 95 PCIMovies are available through our On-line services on in the form of PATCH100.ZIP:-

VideoLogic BBS,

Compuserve (GO VIDEOLOGIC),

MSN (VideoLogic Forum),

Internet (http://www.videologic.com).

Alan Lee

July, 26