| Field Note: 186 |  | Distribute freely |
| --- | --- | --- |

Subject:	PCI Movie BIOS Upgrade

Date Created:	July 31, 2026

Product:	PCI Movie

Due to a problem in the Weitek Power 9100's PCI bus interface implementation, PCI Movie causes the system to hang when trying to use Windows on systems fitted with Intel Aries or SIS chipsets.  This problem is fixed by upgrading PCI Movie's BIOS to v1.46 included with the attached patch release. It can be installed as a TSR from the AUTOEXEC.BAT.

Martin J Parkes

July, 26