These are the instructions for setting up a TL810 or 4/52 to run the pp.tst

macro.

Load a cartridge in the top and bottom bin of each bin pack, i.e.: bins 0, 7,

8, 15, 16, 23, 24, 31, 32, 39, 40, and 47.

Perform a Home All command to set the actuators at a know location.

Verify the Drives are unloaded and ready to accept a tape.

Perform an Inventory to load the unit with the knowledge of where each

cartridge is located.

Select loop mode and PP under User Commands.

The unit will start picking and placing to drives and bins and will do so

until commanded to stop.  The best way to stop the test is to use the "end"

key on the PC. This will stop the test at the end of the loop. The test may

then be restarted without moving cartridges via commands from the PC or

manually. If cartridges are moved manually, it will be necessary to

reinventory the unit prior to starting the loop again.

The loop takes approx. 15 minutes 30 seconds and consists of 131 commands.

Version 3.00I or later is required to run this (date stamp 2/24/95).