These are the instructions for setting up a ESL9000 or P2000/3000 to run the PP6.TST macro.

Load a cartridge into bins 0-5 and bin 36.

Perform a Home All command to set the actuators at a known location.

Verify the Drives are unloaded and ready to accept a tape.

Perform an Inventory to load the unit with the knowledge of where each

cartridge is located.

Select loop mode (and loop count) and PP6.TST under User Commands.

The unit will start picking and placing to drives and bins and will do so

until commanded to stop.  The best way to stop the test is to use the "end"

key on the PC. This will stop the test at the end of the loop. The test may

then be restarted without moving cartridges via commands from the PC or

manually. If cartridges are moved manually, it will be necessary to

reinventory the unit prior to starting the loop again.

The loop takes approx. 7 minutes and consists of loading D0 - D5, spinning

down the tapes one at a time, then unloading them to their origonal bin

positions. While the tapes are spinning up, the tape in slot 36 is moved

to various bin locations as a test and to use the time while the tapes are

spinning up.

Version C1.02 or later is required to run this test.