MSL 5000 Utility

# Requirements

- MSL5000 Utility, provided on two diskettes with your Library, or a zipped file

from the service web site.

- Firmware image obtained from service web site.

- PC with available serial port.

- CA Assy, Serial RJ11> DB9F (Compaq PN 252850-001)

# Firmware Flashing Procedure

- Copy the MSL5000 Utility into the desired directory of your Windows PC and install using the setup program.

- Obtain the Firmware image and copy into the same directory.

- Connect the Library Diagnostic port to the PC serial port using the RJ11 > DB9F Cable Assy (Compaq PN 252850-001) supplied with the Library.

- Double click on the Q icon to start the utility.

- Click on the Connect tab, select the proper serial port and click OK.

- Click the download tab and then click browse to select the directory with the firmware image and highlight the image file.

- Select OK and the firmware will automatically download.

Important: If the flash process does not complete, select a lower baud rate option

in the “Connect” window and try again.

- Cycle power on the MSL5000 and verify the firmware is now at the new version.