# ProLiant GbE2

## Introduction

ProLiant GbE2 Interconnect Switch configuration files are specified in text format. Trivial File Transfer Protocol (TFTP) or Secure Copy (SCP) services can to be used to upload and download these text-based configuration files. When these configuration files are downloaded and applied to the GbE2 Interconnect Switch, the GbE2 Interconnect Switch interprets the contents and applies it to the system as the active configuration.

The configuration settings can be saved to NVRAM, and they become the backup configuration for the GbE2 Interconnect Switch. These settings are then used every time the GbE2 Interconnect Switch is rebooted.

IMPORTANT: The TFTP server must be running TFTP server software to perform a file transfer. TFTP server software is included as part of the Windows switch utilities package.

When inserting a new or replacement GbE2 Interconnect Switch into a production environment, you must be certain that the GbE2 Interconnect Switch configuration is compatible with the production network.  Since the factory default switch configuration may not be same as the production configuration, it may compromise the security aspects and VLAN isolation used by the production network.

A new or replacement GbE2 Interconnect Switch should be preconfigured in a "safe mode" such that it addresses these security concerns and still allows the administrator to configure the switch and finalize switch deployment on the production network. The administrator can simply download a final configuration template with the safe mode configuration to deploy the GbE2 Interconnect Switch in the production environment.

## User Account Information

User password information is not saved in the configuration file for security reasons and must be managed by the GbE2 Interconnect Switch administrator. User account information can be manually added to the configuration file after it is uploaded, or it can be re-entered using  the CLI.

## Safe Mode

When inserting a new or replacement GbE2 Interconnect Switch into a production environment, you must be certain that the GbE2 Interconnect Switch configuration is compatible with the production network. Compatibility with a network can include items such as:

- Specific configuration of the network in general—problems, such as loops, can occur within the network without specifically enabling Spanning Tree Protocol (STP)

- Security of connected network entities—security issues can arise as a result of configuration items, including such attributes as subnet-to-subnet communication and blade-to-blade communication where communication between these entities is not acceptable

- Security of the GbE2 Interconnect Switch—these attributes include controlled access to the GbE2 Interconnect Switch such as user-password and SNMP read-write community strings

- Other configuration attributes, such as VLAN settings

The GbE2 Interconnect Switch factory default, as well as previously configured GbE2 Interconnect Switches, may not have configurations that are compatible with the production network.  If the configuration of the GbE2 Interconnect Switch is not compatible with the production network, the GbE2 Interconnect Switch may be pre-configured in a pre-production network environment before inserting in to production environment. Also, the GbE2 Interconnect Switch can booted up using the Diagnostic Station for preconfiguration, to make it ready for production network.

There are several preconfiguration options:

- Configure the GbE2 Interconnect Switch for its exact position in the production network.

- Partially configure the GbE2 Interconnect Switch so that it is compatible with multiple positions in the production network. The configuration can then be completed for the GbE2 Interconnect Switch through scripting, manual operation, or a configuration download. This more globally appropriate configuration is called a safe mode configuration. Note that a single safe mode configuration may or may not be appropriate for all production environments. A spare GbE2 Interconnect Switch, however, configured to a safe mode, may be appropriate for multiple production environments.

## GbE2 Interconnect Switch Replacement Scenario using a Safe Mode Configuration

When an in-production GbE2 Interconnect Switch fails:

- Remove the failed GbE2 Interconnect Switch.

- Insert the spare GbE2 Interconnect Switch that was previously configured for safe operation.

- Upgrade to new firmware if appropriate.

- Download the correct configuration for that particular GbE2 Interconnect Switch.

## Safe Mode Configuration File Templates

An example template configuration file, named Safe_Mode_Script_GbE2.txt, has been provided as a basis to create an appropriate safe mode configuration. This file, delivered with the GbE2 Interconnect Switch Management Utilities package, is located on the utilities and user documentation CD included in the interconnect kit and at the following website:

www.hp.com/support

NOTE: The template provided is only an example, and should not be used in a production environment without modifications required for your specific production network. This template has commented sections such as for IP configuration, password protection, and port selection.

## Safe Mode Configuration File Template Modification

To modify the safe mode configuration file template:

- Review the readme file located on the utilities and user documentation CD.

- Copy the example safe mode template from the utillities and user documentation CD to a new file with custom name.

- Perform modifications appropriate for your production environment. Your environment may require additional settings such as VLAN configuration, IP settings, password settings, TFTP/SCP settings, and so on.

## How to use the Safe_Mode template configuration file to set the GbE2 Interconnect Switch in Safe Mode

### Method 1: Applying the Safe_Mode configuration template via the switch serial port using command line interface (CLI)

Step 1. To apply safe mode configuration template, power up the switch using Diagnostic Station or in a non-production blade enclosure. Login to the switch CLI via the serial console as Admin.

Step 2. Open the Safe_Mode_Script_GbE2.txt template configuration file using any text editor, such as MS WordPad, and make the changes as suggested by the comments in that template configuration file.

Step 3. Copy all the content from the template configuration file with your edits and paste it as a command to the CLI.

Step 4. Issue the /apply command on the CLI.

Step 5. Issue the /save command and type y when prompted.

Step 6. Now, power up the switch in a production blade enclosure to boot up in Safe Mode with all ports except an uplink port (19) disabled.

Step 7. Access any of the switch management interfaces through that uplink port (19) and complete the switch configuration appropriate for your production network. Another way is to use serial port to complete the switch configuration appropriate for production network.

### Method 2: Applying the Safe_Mode configuration template via any the switch management interface using TFTP functionality

Step 1. To apply safe mode configuration template, power up the switch using Diagnostic Station or in a non-production blade enclosure. After resetting the switch to factory settings (if required), login to the switch CLI via the serial console as Admin.

Step 2. If switch cannot get BootP assigned IP settings, configure the switch IP address settings so that it can access a TFTP server on the attached network.

Step 3. Open the Safe_Mode_Script_GbE2.txt template configuration file using any text editor, such as MS WordPad, and make the changes as suggested by the comments in that template configuration file.

Step 4. From any switch management interface, enable TFTP download of the template configuration file with your edits.

Step 5. Apply the changes and save to flash.

Step 6. Now, power up the switch in a production blade enclosure to boot up in Safe Mode with all ports except an uplink port (19) disabled.

Step 7. Access any of the switch management interfaces through that uplink port (19) and complete the switch configuration appropriate for your production network. Another way is to use serial port to complete the switch configuration appropriate for production network.

----------------------------------------------------------------------------

Hewlett-Packard Company shall not be liable for technical or editorial errors or omissions contained herein. The information in this document is provided "as is" without warranty of any kind and is subject to change without notice.  The warranties for HP products are set forth in the express limited warranty statements accompanying such products. Nothing herein should be construed as constituting an additional warranty.

(c) 2002-2004 Hewlett-Packard Development Company, L.P.