DigiWARE DMODE for OS/2 (DMODE V1.4.6)

Introduction

DMODE is a 'work-alike' of the MODE command under OS/2 but with a

number of added features and a few different rules of behavior.

It came to the attention of the DigiBoard Technical Support Staff

that the standard MODE command -- though useful for many things --

was limited in the following ways:

-- No communications device names besides COM1 - COM8 could be used.

-- Timeout, Handshake signals, etc. could not be set without having

to re-specify the Baud rate, Parity and Data type parameters.

-- If any of the set of Baud_rate:Parity:Databits:Stopbits was left

off the command line, MODE would assign them to ITS defaults of

1200,e,7,1, instead of the defaults of whatever those values are

CURRENTLY set to.  Thus a user wanting only to change Baud rate

would unknowingly also re-set the Data type and Parity of the

device.

DMODE addresses these issues while attempting to remain compliant

with the familiar MODE command syntax.

In general, DMODE attempts to forgive as much as possible in terms

of both the order of the arguments and of each argument's

requirements for other unrelated arguments.  Given the limited range

of values that most of the parameters can assume, DMODE will assign

a parameter based on its value instead of its position in the command

line.  The syntax of the DMODE command is then

DMODE device-name[:] [arguments]

where the device name must be the name of a character-device that

supports Generic IOCtl system calls.  The Colon is optional and is

there only to be backward compatible with MODE.  The remainder of

the arguments are parsed according the following rules.  The

arguments may appear in any order and must be separated from

each other with a space and/or comma.  Any parameters left unspecified

on the command line will be left in their current settings.  In any

case, DMODE will report the current settings of all parameters AFTER

such changes as requested on the command line have been assigned.

Thus with no arguments (i.e. no changes to current settings), DMODE

simply reports the current settings.

Baud Rate Rules:

Baud Rates parameters are recognized to be any 2 to 5 decimal

digit argument to be mapped as follows.

Accepted values....                     Assigned Baud rate

-----------------------------------------------------------

11  110                                           110

15  150                                           150

30  300                                           300

60  600                                           600

12  120  1200                                    1200

24  240  2400                                    2400

48  480  4800                                    4800

96  960  9600                                    9600

19  192  1920 19200                             19200

38  384  3840 38400                             38400

57  576  5760 57600                             57600

115  1152 11520 115200                     115200

230  2300 23000 230000                     230000

Data Bit Rules:

Data Bit parameters are recognized to be any 1 digit

consisting of 5,6,7, or 8.

Stop Bit Rules:

Stop Bit parameters are recognized to be any argument with

the value of 1, 2, or 1.5.

Parity Rules:

The Parity parameter is recognized to be any single-letter

argument with the value of   E, O, N, M, S  -- For (E)ven,

(O)dd, (N)one, (M)ark, and (S)pace.   Upper or lower case letters

are acceptable.

Handshake Signals and Timeout parameter Rules:

These arguments follow the documentation for standard MODE command.

The accepted signal names are listed below.  In all cases the

name and assignment may be in any mix of upper an lower

case characters.

Full name                  Argument     State Assignments

-----------------------------------------------------

Read-timeout                    TO      ON  OFF

Tx Xon/Xoff handshake           XON     ON  OFF  BOTH (Tx/Rx Xon/Xoff)

Input DSR sensitivity           IDSR    ON  OFF

Output DSR sensitivity          ODSR    ON  OFF

Output handshake on CTS         OCTS    ON  OFF

DTR signal control              DTR     ON  OFF  HS

RTS signal control              RTS     ON  OFF  HS  TOG

ALT_PIN processing: [--intelligent DigiBoards ONLY--]

In some cases it is useful to have the driver report changes in

DSR as if they were changes in CD (the modem Carrier Detect Signal).

This is sometimes done to get around a limitation of the

8-wire RJ45 connector -- a connector which leaves pin1 and pin10

of the standard 10-wire RJ45 unterminated.  Since pin10 is defined to

be Carrier Detect and so many communications programs depend on

accurate reporting of the state of this CD signal use of 8-wire

RJ45 connectors can prevent these programs from working.  A work-around

is to slightly change the wiring scheme of the 8-wire RJ45 so that

the CD wire is brought into the middle 8 wires which DO make contact.

Of course this means that the signal that USED to be defined for that

location must be moved out to pin10, where CD used to be.  In

general the LEAST payed-attention-to signal is DSR (on Pin2) so if

CD is wired to Pin2 and DSR is wired to Pin10 close to full modem

control can be acheived on an 8-wire RJ45.  The final step in this

trick is then to instruct the Device driver that for THIS channel

pin2 is to be interpretted as CD instead of DSR.  DMODE permits

this ALTERNATE PIN ASSIGNMENT PROCESSING mode to be established on

channels by issueing the requisite custom command to the XALL driver.

dmode {devname} ALT=ON          ;; report PIN2 as CD

or

dmode {devname} ALT=OFF         ;; report PIN2 as DSR (default)

Those interested in implementing this custom call in software should

look at the file XALL.DOC on the distribution diskette for information.

Examples:

dmode digi12            [ interrogative; no settings changed ]

dmode com5 2            [ the '2' assumed to be stop bits..  ]

dmode com5:38 rts=ON    [ the '38' is parsed to the baud rate]

[ of 38.4K baud, and RTS is turned ON]

dmode com5 8            [ the '8' parsed as  data bits       ]

dmode comdev83 IDSR=OFF,TO=off e 7 12

[ IDSR is turned OFF, timeout on reads]

[ is set to off, Even-parity is set,  ]

[ 7 data bits is set, 1200 baud is set]

dmode com5 38 ALT=ON    [ set 38.4Kbaud, report PIN2 as CD      ]

dmode com5 e alt=off

[ set EVEN parity, report PIN2 as DSR , ]