INDEX

CONTENTS

Introduction	1

Using the Disc Drive	3

Using the Utility Disc	5

Disc File Commands	12

Care of Discs	22

Transferring Tapes to Disc	23

Operating System Disc Routines 23

Guarantee	27

Page		Page

*ACCESS	16	fsp		12

afsp	13	*HELP		15

*BACKUP	16	*INFO		18

!BOOT	22	*KEY		23

*BUILD	20	*LIB		19

*CAT	14	*LIST		21

CHAIN	11	LOAD		13

*COMPACT	16	MENU		5

*COPY	17	*RENAME		19

*DIR	18	*OPT		21

*DISC,DISK	20	SAVE		23

*DELETE	17	Shif t		5

*DESTROY	17	*TAPE		23

*DRIVE	18	*TITLE		19

drv	13	Track/Sector		6

*DUMP	20	*TYPE		21

*ENABLE	18	Utility Cmds		20

Error Codes	25	Verif y		11

Format	7	*WIPE		19

INTRODUCTION

The purpose of this booklet is to enable you to use the powerful disc

operating system with your BBC Machine. This will enable programs to be

loaded and saved much faster than cassette operation. Included with this is

a special 'Utility' Disc which contains programs for setting up and checking

the discs prior to storing data and programs.

The booklet will give a description of the various disc commands and

their uses.

The modifications to your machine consist of adding IC's and

changing links and may also require soldering work in some cases. We

would recommend that only competent persons who are used to working

with electronic equipment carry this out although the actual work involved is

fairly straightforward.

Your computer should have been supplied with a Disc Operating

System fitted. In many cases an increase in the speed of operation can be

gained by fitting links 3 and 4 on the keyboard. The Opus 3" Microdrive has

a track to track search time of 3 milliseconds (three thousandth of a

second).

To carry out these modifications a Model 'B' Machine fitted with the

1.2 Operating System must be used. Model 'A' Machines may be used but

these must be upgraded to the 'B' version first.

To establish which operating system is fitted inside your machine,

enter *FXO. To work a disc system you also require a Utilities Disc and at

feast one blank disc on which to store your programs.

Switch on and listen for the double bleep. If all is well the screen

should display a legend similar to:

BBC Computer 32k

Acorn DFS BASIC

1

are:

Having obtained the correct opening message you are now

ready to connect up your disc drive unit. The descriptions which follow

cover normal 5 1/4" floppies as well

4

as the 3" Microdrive. This is done in order that the user becomes familiar

with both systems.

There are various types of drive configuration. These

single sided,

double sided,

single sided,

double sided,

40 track drive 40

track 80 track

80 track

These can come either singly or as dual units. The BBC can cater for up

to 4 drives (allocated as DRIVE 0,1,2 or 3) with. the 4 drive option

normally consisting of two doubled-sided drives mounted in a single

housing.

You will find that there are two cables attached to the drive. One

of these cables will be a flat ribbon cable which is the data cable and the

other is a power cable with a small 6-way connector attached. Both

these leads are connected into the BBC Machine. Your OPUS 3"

Microdrives are doubled-sided 40 track drives.

Connections

Firstly, switch OFF the computer. The ribbon cable must be plugged into

the 34-way connector underneath the BBC Machine marked "disc drive".

The correct orientation of the connector is with the ribbon cable

downwards away from the machine. Some connectors will have a raised

'key' on the top which engages with a slot in the BBC connector. Again

the ribbon cable should come downwards away from the machine, (see

diagram on Page 3). When correctly fitted the clips at either end of the

plug will spring upwards to hold the socket in place. To release the

socket press both clips outwards.

View from underside of BBC Machine

The power cable supplied with the single drive units plugs direct

into a connector mounted underneath on the power supply on the left of

the computer. This connector is 'polarised' and will therefore only fit one

way round.

Re-check that all connectors are correctly fitted.

Using the Disc Drives

Switch ON the computer and the disc drive unit (dual drives with

separate mains supply) and then if all is well you should again hear a

double bleep and see the message:

BBC Computer 32k

Acorn DFS

BASIC

You are now ready to try out the disc drive.

Carefully remove the protective packing card (if fitted) from the

drive by opening the door flap and pulling the card out. The idea of the

card is to prevent the two read/write heads from banging together

during transit.

Check that a Utility Disc is supplied with your drive unit.

Without this it may not be possible for you to use the system. Two

types of 5 1/4" floppy discs are available, 40 and 80 track and must

be used with the correct drive. Some drives however, typically the 80

track drives, are sometimes fitted with a Switch Option which allows

80 and 40 track discs to be used.

3" Microdrive discs are inserted in the drive with the label

marked 'A' uppermost. Should your drive be mounted vertically the 'A'

side would face to your lefthand side. The Utilities cartridge has a

protection notch which is covered by pushing a small plastic lug across.

Use the tip of a ball point pen or similar object.

5 1/4" floppies are inserted with the label uppermost for drives mounted

with the slot horizontally or with the label to the right in drives mounted

with the slot vertically. There is also a small notch cut in the side of the

disc (on the Utility Disc this notch has been taped over to prevent any

accidental overwriting of information similar to the way in which

cassettes can be prevented from being recorded over). This notch

should be towards the left hand side of the slot for single drives, or

towards the top of the slot for vertically mounted drives. For dual drive

units the disc should be inserted into the LEFT hand drive. The disc

MUST be inserted the right way round otherwise the computer will be

unable to access information on it. Then close the door flap. Incorrect

insertion however should not damage the disc or the drive.

Single drive, single sided	DRIVE 0

Single drive, double sided	DRIVE 0 & 2

Dual drive, single sided	DRIVE 0	(1.h.drive)

DRIVE 2	(r.h.drive)

Dual drive, double sided	DRIVE 0 & 2 (1.h.drive)

DRIVE 1 & 3 (r.h.drive)

Each particular drive can be accessed by specifying its drive number, and

this will be discussed later on. If no drive number has been specified, then

DRIVE 0 is assumed to have selected and the programs on the Utility Disc

have been formed to operate when Drive 0 is selected.

Using the Utility Disc

An automatic start-up option has been included on the disc and this

means that you can gain access to the programs via a "MENU" program

which will display a list of the programs available on the disc. To use this,

hold down the SHIFT key and press the BREAK key. This is called

'Autobooting'. Try it now.

You should find that the light on the drive comes on and you should

hear the motor on the drive running. A few seconds later the motor should

stop and the light go out and an introductory message display come up on

the screen. If this fails to happen and the message "Disk Fault" comes up,

then you need to check that:

i)

The drive connectors are correctly inserted and in the case of the

dual drives, that the mains switch on the drive unit is ON.

ii)	The disc has been inserted into the correct drive and that

it is the RIGHT WAY ROUND. Check earlier for

mention of the way to insert the disc.

After about 15 seconds the display should be replaced

with a "menu" listing all the programs available. The first thing you need

to do is to make a back-up copy of the Utility Disc. This is advisable in

case anything goes wrong when you are using the drive and data on the

disc gets corrupted, and also for safeguarding any important programs

you may have.

Assuming that you have bought a batch of blank discs, you will

need to 'format' them ready to accept data in a form understandable to

the computer. In general, like cassette tapes, a disc produced on one

computer may not necessarily work on another manufacturer's one.

Data is recorded as a series of 'tracks' and these can be either

40 or 80. Each track is sub-divided into ten 256 byte blocks called

'sectors'. 40 tracks will hold up to 100k bytes of data and 80 tracks will

hold up to 200k bytes. These tracks and sectors are absent on blank

discs and therefore need to be recorded before you can save any

programs.

40 track double sided drives such as the Opus 311 Microdrive

will hold 100k bytes per side, giving a total of 200k bytes. 'k' in computer

terms means kilo and represents 1024 bytes. Thus the drive has a

memory capacity of 204,800 bytes. It is the intention of OPUS

SUPPLIES LTD. to bring out what is known as 'double density' early in

1984. This will provide a method of increasing the capacity to 400k bytes.

Club Members will be kept informed as usual.

There is a program on the Utility Disc called "FORMAT" and by

loading and running this program you can pre-record the required tracks

and sectors. The program is also used to set-up the Catalogue required

for holding the information about files saved on the disc.

To run the program is simply a matter of typing the appropriate

letter next to the program name as it appears in the menu listing. The

Menu program will automatically load and run the program. Try it now.

The computer allocates drive numbers, as mentioned

earlier, in the following way:

You should find that the disc drive will start-up again and the

formatting program will be loaded and run. You could also access the

formatting program direct by using the CHAIN command as used with

cassette files, i.e:-

CHAIN"A.FORMAT"

(The significance of the W in front of the filename - the filename 'directory'

letter - will be described in the next section).

The program will then ask you a series of questions about the disc

you wish to format:

Drive No.?

Type 0 to indicate that you want to use drive .?

The message "REMOVE FORMAT DISW' will appear to remind

you to remove the Utility Disc other wise all the information on it could be

erased were it not protected by the notch cover.

The message:

Insert blank disk into drive 0

will appear to prompt you to put your blank disc into the drive.

You will then be asked:

Ready to format drive (Y/N)?

Type Y and the program will start with the acknowledgement:

Formatting Drive 0 - Please wait

As the program proceeds, a series of numbers (hex) will appear.

These represent the track numbers as they are being formatted.

On completion you should see:

Drive 0 formatting complete

0 Format errors

0 Verify errors

Format another disk (Y / N)?

The 'format errors' count gives the number of errors

encountered when attempting to write to the tracks. The 'verify errors'

count indicates any errors encountered when reading back the

information recorded when the tracks were formatted. These errors can

be seen as '?' appearing next to the track numbers:

Type the number 80,40 or 35 depending on what type of drive you

are using. 80 track drives may however be used for formatting 40 and 80

track discs if you have a "Switch Option" fitted. Make sure this switch has

been set for the correct number of tracks. This is important particularly

when formatting 40 track discs as it is possible to format for 40 tracks with

the drive switched for 80 tracks without any error being detected. You

would then find that the disc would produce error messages when used on a

40 track drive. However, trying to format for 80 tracks using a 40 track

drive would produce error messages by the formatter program

Select 40 when using Y' Microdrives.

7

If errors have occurred, you will be asked:

Another try (Y / N)?

Type Y and the formatting process will be repeated. The

program ' will assume the same Drive number and the same number of

tracks. If after three attempts you are still getting errors then the disc is

probably faulty. If you are getting '??' after each track number, check

that the drive unit is correctly connected. Check also that the disc is not

write protected.

The next question asked will be. No. of

tracks required (80,40,35)?

e.g.	00? 01. -format error on track 0

00 ?01. -verify error on track 0

00??01. . -format and verify errors

If no errors have occurred you can format another disc by replying

'Y' to the message:

Format another disc (Y/N)?

The program will assume you want the same drive number and

number of tracks.

To change to a different drive number, e.g. to format the other side

of a disc in a double sided drive, or to change the number of tracks required,

simply press 'ESCAPE' and type 'RUN' and the Formatter will re-start.

This concludes the explanation on the formatter and it is a good idea

to format all your blank discs at the same time. Included on your Utilities

Disc are programs which will dump High Resolution Pictures from the

monitor screen on to various types of dot matrix printers. Also to move long

programs (originally on tape) down memory, after a disc load, before a

RUN commences. Full instructions explaining the use of these programs are

included on the disc.

Now you have formatted some discs you can now make a back-up

copy of the Utility Disc. To do this, there are two commands:

*ENABLE

*BACKUP <source drive> <destination drive>

The command *ENABLE must be used in this and for certain other

disc commands which have drastic effects on disc files.

In the *BACKUP command the <source drive> is the drive which

contains the disc you wish to copy from and the

<destination drive> , the drive containing a blank (but formatted) disc ready

to be copied to.

9

1

if you only have a single drive unit you need to specify source

and destination drives as being number 0 (or 2 if you have a double sided

drive). When you enter the command, the computer will prompt you to

insert the source disc and then ask you to remove this disc and insert the

blank (destination) disc. This will happen several times as the computer

only transfers one file at a time. Be careful that you do not get the discs

mixed during these transfers. To help you, place a small adhesive label

(normally supplied with blank discs) over the "Write Protection" notch

(mentioned earlier on) cut in the side of the Utility Disc if this is not

already fitted. This will prevent any information being overwritten on the

Utility Disc.

An error message will be produced if an attempt is made to

write to protected disc:

Disk write protected

For single drive users enter the commands:

*ENABLE

*BACK 0 0

and then follow the instructions given by the computer.

For those with dual drives, you can insert the Utility Disc in

Drive 0 (left-hand drive) and the blank disc in drive 1 (right-hand). In this

way, unlike the single drive unit, the computer will transfer all the files

from one drive to the other without any intervention.

Use the commands:

*ENABLE

*BACK 0 1

You will be able to see the two drives being activated alternately

(watch the drive LED lamps where fitted).

You now have a copy of the Utility Disc which you can use

when needed whilst keeping the original as a master.

10

Another program on the Utility Disc is VERIFY which is used for

checking the entire disc for any errors (or 'corruptions') which may have

occurred. This works in a similar manner to the Formatter by asking you for

the Drive number containing the disc to be checked.

You can access the program from your Utility Disc by calling (or

'booting) the Menu program. This is done by holding down the SHIFT key

and pressing the BREAK key the same way as before. Pressing the

appropriate letter when the Menu is listed will load and run the program.

Alternatively you can enter:

CHAIN"A. VERIFY"

The program should load and you should get the message;

Drive No.?

Leave the Utility Disc in the drive and for this exercise verify the

Utility Disc you have just made. Type 0 if the disc is in Drive 0.

The following message should appear:

Ready to verify drive (Y/N)?

Type 'Y' and the program will start. There is no need to specify the

number of tracks as the program reads the disc size from the Catalogue

sectors recorded on track 0.

As with the Formatter a series of numbers corresponding to each

track as it is checked will appear. When complete, the results of the verify

will be displayed:

40 Track Disk

Drive 0 verify complete

0 Verify errors

This is what you should get if successful (this is assuming Drive 0

selected and a 40 track disc used).

11

Any errors found, again, will be shown by a '?' following the

track number. If a disc does contain errors then all the files on it should

be transferred to another disc if possible and the problem disc re-

formatted.

The message '40 Track Disk' tells you the number of tracks the

disc has been formatted for. If the program has not read a valid disc size

i.e. 35,40 or 80 tracks then you would get the message:

Disk not formatted

This verify program can therefore prove useful for checking

discs that apparently fall to work by detecting corruption of the data on

them or if you have forgotten to format them before use.

Disc File Commands

The filename is at its best when composed of six letters and a

number. The name cannot be any length as with the cassette filenames.

Symbols can also be used but not *:!.#or spaces, since these all have a

special meaning.

The names as applied to discs usually involve more than the

mere name, the Drive number <drv> and the directory letter <dir> are

also used. This combination is called the File Specification <fsp>. The

format would therefore be:

--.:zdrv-

:<drv>.<dir>.<filename>

Note the use of ":" and "." as part of the fsp. Thus for a full load

command you might type:

LOAD ":0.$.FRED"

In practice, however, the fsp can be shortened.

12

This can be accomplished by considering the following two commands:

i) *DRIVE <drv>

This command sets the drive number for any future disc operations

and this saves you having to specify the drive number in the File

Specification. On power-up the computer will assume Drive 0 until

you change it using the above command.

The example load command can be reduced to:

LOAD "$.FRED"

Within the Catalogue files are placed in different 'directories'. This

allows you to use files with the same name as long as they are in

different directories. In the above command, the file FRED is stored

in directory '$'. .This particular directory is the one that is assumed by

the computer unless changed using the following command:

ii) *DIR <dir>

The directory letter may be any alpha-numeric character, but again,

*:!. should be avoided.

Using this command you can reduce the example command to:

LOAD "FIZED"

With this you can see that the command is now the same as the

cassette command and called the Alternative File Specification

<afsp>.

The selected Drive number and Directory letter may be changed at

any time and if you need to use an alternative drive or directory letter

temporarily you can specify the full command.

WILDCARD FACILITY

This is a method in which you can process a group of files with

similar directory letters or filenames. The following symbols are used: '*'

and '#'. The '#' symbol is used to replace a single character and the '*'

symbol, a group of characters.

For example, if you had the following group of files:

$.HELLO

A.HELP

A.HELP2

then by using an afsp of:

*.HE*

you would be able to use the *COPY, *INFO, *DESTROY, *WIPE,

*ACCESS commands (described later) to operate on all files starting

with the letters 'HE' and in any directory.

Another example might be:

*$.T##T

fsp = $.T##T

Operation on directory $ files:

$.TEST

$.TENT

$.THAT

would be allowed, i.e. all files in directory $ with filenames of four letters

and the same first and fourth letter 'T'.

command:

To see that files you have in the catalogue, use the

*CAT (which may be abbreviated to *.)

14

A listing of all the files in directory order will be produced. Files in

the currently selected directory are listed first (without a preceeding letter).

The catalogue heading will tell you the current directory and also the drive

being used. All other files will be shown in groups with the directory letter

shown.

Try putting the Utility disc in and asking for the Catalogue by typing

*.

You will see files without a directory letter, e.g. !BOOT and MENU

and other files such as FORMAT, VERIFY all listed under directory 'A'.

To obtain listings of other drive catalogues simply type:

*. <drv>

where <drv> can be a number 1 to 3 inclusive.

KEYWORDS

Keywords such as used in the commands just described are

reserved for the Disc Operating System and the following is a description

of the others available.

*HELP

The Operating System and Filing System version numbers will be

displayed.

*HELP DFS

Used to list out all the available commands in the Disc Filing (or Disc

Operating) system.

15

*HELP UTILS

A list of general purpose commands for special file operations.

These will be described later on.

*ACCESS afsp L

Access locks or unlocks a file. Locking a file prevents it from

being accidentally overwritten. Omitting the 'L' will unlock the file. For

locking all your files at once type using the 'wildcard' facility:

*ACCESS *.* L

*BACKUP src drv dest drv

A means of copying all files from one disc to another, overwriting

any files already on the destination disc. Any command that can cause this

type of damage has to be preceeded by *ENABLE. ANY PROGRAM

IN THE COMPUTER WILL BE AFFECTED therefore make sure you

save any program you want beforehand.

e.g. *ENABLE

*BACK 0 1

*COMPACT drv

This has the effect of moving all the files to the start of the disc to

take up space left by files that have been deleted. This leaves all free

space after the end of the last stored file. As the files are moved,

information about load, execution addresses, length and sector start

address will be displayed.

e.g. $.TONY FF1900 FF801F OOOOFF 002

A.MIKE FF1900 FF801F 0001FF 003

A.STEVEFF1900 FF801F 0001FF 005

B.LEE 002000 0020000 OOOOFF 007

16

This command need only be used if you do delete any files. Again

any programs in the computer will be affected.

*COPY <src drv> <dest drv><afsp>

Copies are named program from one drive to another. Quote marks

are not required on the fsp . A series of files all starting with the same

letters can be copied using:

*COPY <src drv><dest drv,- xx*

where 'xx' are the start letters. Wildcard facility is allowed e.g.

*COPY 0 1 *.HE*

Note that computer memory is affected when using the COPY command.

*DELETE <fsp>

This will delete a single file. Once deleted, a file cannot be restored.

*DESTROY <afsp>

A whole series of files with the same starting letters can be deleted

in one single action using the wildcard facility. This command has to be

enabled:

*ENABLE

*DESTROY *.xx*

(xx-starting letters)

The files to be destroyed will be listed and you will be asked to

confirm the command e.g.

*ENABLE

*DESTROY *.T*

$.TEXT1

$.TEXT2

Delete WNP

17

message:

Typing 'Y' will delete all the named files. The

Deleted

is displaced when complete. cancel

the command.

*DIR <dir>

Typing anything else will

dir can be any letter. After a BREAK keypress the computer

will assume '$' as the current directory. After this command, saved files

will be placed in the specified directory.

*DRIVE <drv>

.<drv> can be a number 0 to 3 representing up to four drive units.

After a BREAK keypress, Drive 0 is automatically selected.

*ENABLE

A command which is necessary before any destructive

commands such as BACKUP or DESTROY are used.

*INFO

This provides additional information about a file in the following

order:

.<dir> <fsp,> (L) <start>< exec><length> < sb>

where 'start' is the start address, 'exec' the execution address, 'length' the

length of the file in bytes and finally 'sb' the start block. or sector address

where the file is located on the disc.

All files on the disc may be accessed using:

*INFO *.*

18

*LIB :<drO. <dir>

Sets the library option on the currently selected drive to transfer

action to the selected drive and directory, e.g. if you ran a program on drive

0 which required to access a program from drive 1, directory A then you

would enter the command:

*LIB : 1.A

When Chaining in the main program, it would be loaded from Drive 0

and run and then Drive 1 would be activated to load in the other program.

*RENAME <old fsp> <new fsp>

In addition to just changing the basic name, it is also a facility for

moving a file from one directory to another e.g.

*RENAME G.Prog E.Prog

If a file already exists with the new name then an error message will

be produced.

*TITLE <disc name>

A method of giving a useful name to your disc when the programs on

the disc are all related to a particular topic, e.g. UTILITY, DATA etc. The

title can be 12 letters long, but if the title contains spaces it must be placed

inside quotation marks, e.g.

*TITLE "OPUS UTILITIES"

*WIPE <afsp>

Removes all files starting with the same specified letters. This is

similar to the DESTROY command but will list out each file in turn and ask

you to confirm deletion of that file.

e.g. *WIPE *.HE*

19

UTILITY COMMANDS

The list given in response to the keyword *HELP UTILS are

explained below, in the order in which they occur.

*BUILD <fsp>

Used to construct text files consisting of ASCII text as opposed

to say machine code or BASIC programs. The file is closed by pressing

ESC. Line numbers are presented automatically. Colour may be added to

the text (Mode 7 only) by using the SHIFT key together with one of the

user function keys (see User Guide pages 153,154).

If the fsp = !BOOT and drive 0 is selected with option *OPT 4,3

set (explained later), then the lines will be treated as program lines and

executed:

1. CHAIN MENU

2. ESCAPE

This will execute if BREAK is pressed whilst the SHIFT key is

held down. This is how the Utility disc menu is brought onto the screen.

It is important to note that !BOOT is a reserved filename and should not

be used as a general file.

*DISC

*DISK

Changes the current filing system to disc operation. Note that on

power up, the computer will assume disc operation arid NOT tape. To

regain tape operation simply type *TAPE.

*DUMP <fsp>

20

Produces a hexadecimal listing of a file with ASCII equivalent

characters where possible. Mainly used for examining machine code

programs.

*LIST < fsp>

Lists a text file such as produced by *BUILD or *SPOOL (used for

producing ASCII text of a BASIC program) with each line numbered. Not

to be confused with the BASIC List command.

*TYPE <fsp>

This again gives a listing of text files but no line numbers are given. If

you try this with a 'tokenised' BASIC program i.e. as Saved under BASIC

you will find that rubbish is produced as the 'tokenising' process replaces the

BASIC keywords with special internal codes.

*OPT N n

These options enable you to set certain actions on the computer. This

command should not be confused with the BASIC OPT N command.

1 . *OPT 0 0 The default (or start-up) condition.

2. *OPT 1 0 Disable the *INFO command

3. *OPT 1 n n 0 = enable *INFO

4. *OPT 2 n Tape error detection

5. *OPT 3 n Interblock gap command

6. *OPT 4 0 Auto start Option off

7. *OPT 4 1 Load !BOOT file

8. *OPT 4 2 Run machine code file !BOOT

9. *OPT 4 3 Execute !BOOT file

Options 2,3 are used to prevent information about the files being

disclosed to another user if you wish to protect your files. This like other

commands may be included in your BASIC programs.

Options 4,5 are only of importance for users trying to produce tapes

that can be read by other computers and therefore not of use for the

ordinary user.

21

Options 6,7 are used to enable or disable the file !BOOT which

you have seen can be used, for example, to load in the Menu program on

the Utility Disc.

Option 8 is used to enable loading of a machine code program

placed in the !BOOT file.

Option 9 is used to Execute commands contained in the !BOOT

file as you have seen in the setting up of the Utility Disc.

TBOOT

See under *BUILD

Care of Discs

1)	Always replace discs not in use into their

protective wallets.

2) Magnetic fields can damage your disc, so do not place them

on top of equipment or near magnets.

3) Never touch the disc surfaces where they are exposed

through the black case or attempt to remove the disc from its

case.

4) Never write on the case or onto the wallet whilst it contains

the disc with a ball-point pen or other sharp object. Use only a

felt-tip pen and employ only light pressure.

5)	When the drives are not in use, keep the access

doors closed.

6)	Keep the discs away from sources of heat such

as radiators or the sunlight through windows.

22

7)	Do not fold or roll up the disc.

8)	Try to keep the disc free from dust and other

contaminants.

9)	3" discs are encased in a rigid plastic sleeve from

which they are never removed.

Transferring tapes to disc

A BBC Computer fitted with a disc filing system automatically

assumes disc operation. To load a Tape program, the user must first change

the filing system by entering *TAPE. To restore the disc filing system, enter

*DISC or *DISK. If you wish to transfer a lot of programs, it helps to

define the red user keys as follows:

*KEY 0 "*TAPE M"

*KEY 1 "*DISC M:*DRIVE<drv>,M"

<drv> = Drive No. (0-3)

You can now press f0 for tape and load in the program, then press f1

1 for disc and SAVE <fsp>. Remember, the name can only be seven

characters with no spaces allowed in the middle of the filename.

Operating System calls

FILE HANDLING

For file handling in BASIC, command *CAT(or *.), OPENIN,

OPENOUT, OPENUP (later versions of BASIC), INPUTf, PRINT*,

B13UTf, PTIZ*, EXT*, EOF*and CLOSE are all available.

23

Operating System calls such as OSFIND, OSARGS, OSGBPB

etc. may be used for discs files as standard (see User Guide pages 451-

456 for full details). Note that the CiSFIND call will return the opened

file channel number IN THE W REGISTER. Error codes are listed in

the next section.

DISC TRACK AND SECTOR ACCESS

In addition to the above OS calls, there is an OSWORD call

which enables you to read from or write to single or multiple sectors.

OSWORD call with A=&7F

On entry to this routine the X register (low byte) and Y register

(high byte) must point to an instruction block in memory. The contents of

the block given as an 'offset' is as follows:

Example:

Command = &53 (read), &4B (write)

No. of parameters = 3

Parameter 1 = Track address

Parameter 2 = Sector address (&00-&09)

Parameter 3 = Sector length/number of sectors to access

24

Offset

0	Drive number (0-3)

1-4	Start address in memory of Source or Destination of

the data

5	The number of parameters required for the complete

command

6	Command (&53 = read, &4B = write)

7+n Parameters

8+n Result of operation

The 3rd parameter bits are made up:

Bits 0-4 = No. of sectors to be accessed

Bits 5-7 = n, where length of sector = 128 x 2n

(e.g. n=l for 256 bytes (standard)

This would give &21 for a single sector of 256 bytes.

On exit from the routine, the Result parameter will contain &00 if the

operation was successful, otherwise one of the following disc error codes will be

present:

DISC ERROR CODES

&BD	Not enabled

&BE	Catalogue full

&BF	Can't extend file

&CO	More than 5 files (max) open

&Cl	File read only

&C2	File already open

&C3	File locked

&C4	File already exists

&C5	Drive fault

&C6	Disc is full

&C7	Faulty disc

&C8	Wrong disc inserted

&C9	Disc read only

&CA	Bad Checksum

&CB	Bad Option

&CC	Bad File Specification

&CD	Bad Drive number

&CE	Bad directory

&CF	Bad attribute

&D6	File not found

&FE	Bad Command

25

FOR FURTHER READING AND A

GREATER UNDERSTANDING TRY:

PUBLISHED BY PRENTICE-HALL INTERNATIONAL

Available from THE COMPUTER USERS CLUB,

69 Hadlow Road, Welling, Kent.

Please add £1 for postage and packing when ordering from the above

address.

26

or local book shops -	Price

GUARANTEE

This equipment is guaranteed only against defects in design, materials and

workmanship, from the date of purchase for the relevant period and subject

to the conditions detailed below.

Registration: This guarantee shall only be effective if the whole

Guarantee Card is completed and signed by the customer and mailed within

7 days of delivery to Opus Supplies Ltd., 158 Camberwell Road, London

SE5.

Period of Guarantee: The is Guarantee is effective for six months

calculated from the date of purchase.

Conditions

1.	This Guarantee is personal to the original user, is not

transferable and shall not be valid unless the equipment

was purchased or take on deferred or hire purchase terms

from an appointed OPUS Dealer, to whom the equipment

was sold by the Company or director from the Company.

2.	This Guarantee will be invalidated if the equipment is

misused or modified in any way without the written

consent of the Company or if any original component or

accessory has been replaced by any component or

accessory of a type not recommended or approved by the

Company, or if operated other than in accordance with

the Instruction Manual.

3.	Any claims made under this Guarantee must whenever

possible be made through the Dealer from whom the

equipment was originally purchased, or taken on deferred

or hire purchase terms. If this is not possible a claim

may be made to any other Opus Dealer (a list of whom

can be supplied on application). The customer's portion of

the certified guarantee must be produced when any claim

is made. The cost of carriage to and from the Dealer

must be paid by the customer.

27

4.	The Company's own transport will collect and return

at no charge from an OPUS dealer only any

equipment which after agreement by the Company is

found necessary to be forwarded to them for repair.

5.	In the event of equipment being returned which on

test is found to comply with published specification,

the Company reserves the right to charge a

reasonable fee for testing the equipment and for

return carriage.

6.	The liability of the Company under this Guarantee

shall be limited to the cost of repair or complete

replacement (at the discretion of the Company) of

the defective equipment.

7.	The Company does not accept responsibility for any

loss or damage during transit to or from the Dealer or

the Company, but every effort will be made to

investigate complaints of loss or damage if these

arise.

This guarantee does not affect the purchaser's statuatory

rights.

28

Guarantee Registration Customer's copy

Model / Invoice / Ref.No.	Serial No. Date of Purchase

Dealer

Address

This guarantee is only valid if it is completed and signed below by the

customer and mailed to OPUS SUPPLIES LTD., 158 Camberwell Road,

London SE5 within ten days of delivery to the original user.

This guarantee is valid in U.K. only

Guarantee Registration OPUS SUPPLIES LTD. copy

Model / Invoice / Ref.No.	Serial No. Date of purchase

Customer

Address

I accept the terms of this guarantee.

Customer's signature