COIL.EXE version 2.00

Authors: Steve MOSHIER and Emil LAURENTIU

Last rev: Sunday, 29 September 1996

This program computes the electrical self inductance of various shapes and

sizes of air core coils and the LC circuit parameters at resonance:

USAGE:

Select the desired coil geometry from the menu. Enter mean diameter, length,

and thickness of winding, and total number of turns, as prompted. The

program will display the inductance then loop back for more input. "Mean

diameter" is the arithmetic average of the inner and outer diameters of the

winding; for a single layer coil it is measured to the center of the wire.

"Thickness," for a single layer coil, is the wire diameter (see below for

the correction for empty space in the winding).

For each item the previously entered value is displayed in parentheses and

will be kept if you just hit the carriage return key.

Wire gauge calculation, prompts for the wire gauge and the dimensions of a

rectangular area to be filled. It displays the wire diameter and the number

of turns that will fit into the indicated space.

The formulas for a multilayer circular solenoid were obtained from National

Bureau of Standards publications and are very accurate. They work for

arbitrary winding thickness and length. A flat spiral disc coil is obtained

by setting the length = 0.

Also very precise is the NBS formula for a helical solenoid of round wire.

It models the size of the wire and the nonuniform current density inside the

wire. The only low precision formula included is the one for a multilayer

square solenoid; it is an approximation from the CRC handbook.

In the case of a single-turn loop or a straight piece of wire, a skin effect

correction is computed. This requires that you enter the frequency of

operation. The wire is then assumed to have the conductivity of copper.

Unless the wire diameter or winding thickness is explicitly requested, the

coil is modeled as a zero thickness current sheet. Except for the circular

solenoid of round wire, the formulas assume uniform current density

throughout the winding, modified only when skin effect is included.

In the cases that assume uniform current density, there is no correction for

empty space in the winding. An approximate correction for close-wound coils

is (Rosa, 1906)

dL = 0.00097 d N

where d is the mean diameter of the winding, in centimeters, and N is the

total number of turns. This correction, in microhenrys, is added to the

inductance.

All calculations assume both the core material and the wire are nonmagnetic.

For magnetic cores a correction factor (mu) can be used.

FILES:

coil.c	       main program for coil calculator

coil.doc       this file

makefile.wat   Watcom makefile

makefile.bor   Borland makefile

formulae.c     inductance formulas from NBS publications

lyle.h	       approximations to Lyle's tables

mathl.c	       elliptic integrals and other math functions

tables.c       computes tables found in engineering handbooks

textrout.asm   routines for directly writing text on the screen

OPTIONS:

Use /b on the command line for black and white colors.

The program may be configured to accept dimensions in either millimeters,

centimeters or inches.

REFERENCES:

Grover 1918:

Frederick W. Grover, "Additions to Inductance Formulas,"

Sci. Pap. #320,  Bulletin of the Bureau of Standards 14, 555-570

Grover 1922a:

Frederick W. Grover, "Tables for the calculation of the

inductance of circular coils of rectangular cross section," Sci.

Pap. #455, Scientific Papers of the Bureau of Standards 18,

451-487 (1922)

Grover 1922b:

Frederick W. Grover, "Formulas and tables for the calculation

of the inductance of coils of polygonal form," Sci. Pap. #468,

Scientific Papers of the Bureau of Standards 18, 737-762 (1922)

Grover 1946:

Frederick W. Grover, _Inductance Calculations, Working Formulas

and Tables_, Van Nostrand, 1946; Dover, 1962

Rosa 1906:

Sci. Pap. #31,  Bulletin of the Bureau of Standards 2, 161

Skilling 1948:

H. H. Skilling, _Fundamentals of Electric Waves_,

Wiley, 1948, pp 99-101.

Snow 1952:

Chester Snow, "Formulas for Computing Capacitance and Inductance,"

National Bureau of Standards circular #544

See the section "Radio Formulae" in _Handbook of

Chemistry and Physics_, Chemical Rubber Publishing Co.,

for toroidal and rectangular coils, loops, straight wire, etc.

AUTHORS:

Program by	Steve MOSHIER

email:	moshier@world.std.com

and		Emil LAURENTIU

email:	emil@interlog.com

This program is "cardware" that means if you like this program and you use

it you should send me a post card. (This is a _must_, otherwise I will never

upload my programs again :-). Please send me a postcard with a nature

landscape from your country on the following address:

Emil LAURENTIU

Str. Drumea Radulescu nr.42 sect.4

Bucharest cod 75126

ROMANIA

I am in Canada now, but please send the post-cards to the address in

Romania because that one is a stable one.

My latest email address (together with my PGP public key) can be found by

querying the PGP key-servers (send email with subject line "MGET laurentiu"

to one of these servers (for example to pgp-public-keys@kub.nl)).