This file contains information about the files on your JAWS disk.

Read it to learn how to run JAWS or JAWSDEMO, select

synthesizers, change communication parameters, and other

pertinent information.  For tips on using JAWS with specific

application programs and specific configuration files refer to

"WORDPROC.DOC", "SPREADSH.DOC", and "EMULATOR.DOC".  These files

are not meant to replace the JAWS manual which is also available

on disk, the files have the extension ".MAN".  You can also refer

to the JAWS Installation cassette tape and JAWS Training tapes

for more detailed information.

USING YOUR JAWS SCREEN READER

Your distribution disk will contain some of the following files:

"JAWS.EXE"  is the front end, it is installed with your name on

it.  It also should know what synthesizer you will be using as a

default or first choice.  When you execute JAWS  by entering

"JAWS" and pressing enter this program displays the JAWS logo,

your name and address, the copyright information, and then

schedules a TSR (Terminate and Stay Resident) program based on

your choice of synthesizer.

If you have a demo disk you will find "JAWSDEMO.EXE" instead of

"JAWS.EXE".  In the following instructions you should substitute

the file name "JAWSDEMO" wherever we use the file name "JAWS".

"JAWSDEMO.EXE" is the demonstration front end, it does some

initializing procedures then schedules the main part of the JAWS

system, the TSR program.

Files starting with "J_" and ending with the extension "TSR" are

the various TSR (Terminate and Stay Resident) programs for the

various synthesizers.  If you have a demo disk the TSR files

begin with "J_" and have the extension of ".EXE".

"J_BNS.TSR" is the JAWS TSR program for the Braille 'n Speak from

Blazie Engineering.

Remember that the JAWS system actually consists of 2 files: the

JAWS.EXE or JAWSDEMO.EXE front end and the TSR program.  When you

run JAWS you should set the default disk drive to the one where

the JAWS files are.  If they are in the "A" drive be sure you

have a "A>" prompt before entering "JAWS".  If you have copied

the JAWS files over to a subdirectory on a hard disk then default

your system to that disk drive and subdirectory.

To default to a particular disk drive enter the letter of the

disk followed by a ":", e.g. "C:" and press enter.  To default to

a given subdirectory first default to the proper disk, then enter

"CD\" followed by the name of the subdirectory, e.g. "cd\JAWS"

and press enter.  Then you can execute JAWS and the default TSR

program by entering "JAWS" and pressing enter.

To execute a JAWS TSR That is not the default one simply follow

the "JAWS" with the name of the synthesizer, e.g. "JAWS BNS" for

the Braille 'N Speak, or "JAWS ACCENT" for the Accent PC

synthesizer.  Note that in this case the name of the synthesizer

does not include the "J_" or the ".TSR".

To execute the JAWS demo type in the name "jawsdemo" followed by

the name of the synthesizer you want to demo.  For example:

JAWSDEMO S200

JAWSDEMO ACCENT

JAWSDEMO BNS

JAWSDEMO PROSE

One or both of these files, the front end and/or the TSR, can be

preceded by a disk drive letter designator and a path name to

tell DOS where to find it.

Now we will explain more about using the particular versions of

JAWS.

Braille 'n Speak:  To use the Braille 'n Speak as a synthesizer

for JAWS follow these steps:

1. connect the serial interface cable (supplied with the optional

interface kit), the small end into the left side of the BnS, the

large end into COM1 (first serial port) of your computer.  You

will probably need the gender bender provided with the kit. 2.

Perform a Warn Reset on the BNS by pressing all 7 keys at the

same time, then press them again.

3.  Turn on the BNS speech box mode, dots 345-cord, press all 6

dot keys, e-cord.

4.  Turn on xon/Xoff handshake: P-cord, H-cord, S, E-cord.

5.  Enter "JAWSDEMO BNS", or "A:JAWSDEMO A:BNS" if your demo disk

is in the A drive.  It should start talking within ten or twenty

seconds.

If you want to use a different Com port or different Baud rate,

Parity, etc. use the following examples:

"JAWS BNS COM2:1200,E,7,1"

"JAWSDEMO BNS COM3:4800,N,8,1"

Note that the syntax is similiar to the Mode command, except that

no spaces are allowed following the COM       port number.  You

must also change the parameters on the Braille 'n Speak to match

the parameters defined for JAWS.

Some older versions of the BNS will lose characters at high Baud

rates, if your's sounds funny or inconsistent reduce the Baud

rate to about 1200, or ask Blazie for an upgrade.

Also, the new JAWS is designed to control the volume, rate, and

pitch of the BNS.  This works fine with the newer BNS, but not

with the old ones.  The best thing to do is to upgrade your BNS.

Otherwise ask us for an old version of JAWS.

Refer to the JAWS manual and training tapes for details on JAWS

features.

Files with the extension "MAN" are part of the JAWS manual, the

extension "DOC" means documentation file.  Either may have

helpful information for you.