Copyright

Copyright 1993 - 1996 by SpartaCom S.A. All rights reserved. No Part of this publication or software may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language in any form by any means, without the written permission of SpartaCom S.A.

Disclaimer

Information in this document is subject to change without notice.

Trademarks

SAPS and SpartaCom are trademarks of SpartaCom S.A.

All other company names and product names are registered trademarks or trademarks of their respective companies.

SpartaCom USA

3425-D Corporate Way

Duluth, GA. 30136

USA

Phone: 770 622 2820

Fax: 770 232 9536 Technical Support: 770 946 9149

E-Mail: sales@spartacom.com

WWW: http://www.spartacom.com

Document ref. Sapsman.doc 97-01-08

1. SAPS INTRODUCTION

2. MINIMUM REQUIREMENTS

3. SAPS SERVER FOR WINDOWS NT

3.1 FEATURES

3.2 INSTALLATION

3.3 CONFIGURING SAPS SERVER FOR NT

3.4 SETTING SAPS OPTIONS

3.5 STARTING SAPS SERVER FOR NT

3.6 PAUSING SAPS SERVER FOR NT

3.7 VIEWING CURRENT SAPS USERS

3.8 SAPS EVENT VIEWING

3.9 USING SECURITY FOR SAPS SERVER FOR NT

3.10 MANAGING REMOTE SAPS SERVERS FOR NT

3.11 UNINSTALLING SAPS SERVER FOR NT

3.12 SAPS SERVER FOR NT AND RAS SERVER

3.13 SAPS AND ZETAFAX SERVER

3.14 ADVANCED CONFIGURATION

4. SAPS SERVER FOR WINDOWS

4.1 FEATURES

4.2 INSTALLATION

4.3 CONFIGURING SAPS SERVER FOR WINDOWS:

4.4 STARTING SAPS SERVER FOR WINDOWS

4.5 SAPS SERVER FOR WINDOWS ADMINISTRATION

4.6 SAPS SERVER FOR WINDOWS PERFORMANCE CONSIDERATIONS

4.7 SAPS SERVER FOR WINDOWS PARAMETERS

4.8 UNINSTALLING SAPS SERVER FOR WINDOWS

5. SAPS CLIENT FOR WINDOWS NT

5.1 FEATURES

5.2 INSTALLATION

5.3 CONFIGURING SAPS CLIENT FOR WINDOWS NT

5.4 DISCONNECT ON INACTIVITY

5.5 OPTIONS

5.6 ADMINISTRATION

5.7 UNINSTALLING SAPS CLIENT FOR WINDOWS NT

6. SAPS CLIENT FOR WINDOWS 95

6.1 FEATURES

6.2 INSTALLATION

6.3 MODEM DETECTION

6.4 CONFIGURING SAPS CLIENT FOR WIN 95

6.5 ERRORS

6.6 SAPSCTRL

6.7 ADDING A PORT

6.8 UNINSTALLING SAPS CLIENT FOR WIN95

7. SAPS CLIENT FOR WINDOWS

7.1 FEATURES

7.2 INSTALLATION

7.3 CONFIGURING SAPS CLIENT FOR WINDOWS

7.3.1 TOOLBAR DESCRIPTION

7.3.2 SHARED COM PORT POOLS

7.3.3 AUTOMATIC CONNECT/DISCONNECT

7.3.4 SAPS SPECIAL KEYBOARD COMMANDS

7.4 CONNECT TO A SHARED COM PORT

7.5 DISCONNECT FROM A SHARED COM PORT

7.6 ADDITIONAL INSTALLATION NOTES

7.7 SAPS ERROR MESSAGES

7.8 SAPS CLIENT FOR WINDOWS PARAMETERS

7.8.1 SAPS.INI FILE

7.8.2 SAPSUSER.INI FILE

7.9 UNINSTALLING SAPS CLIENT FOR WINDOWS

8. SAPS CLIENT FOR DOS

8.1 FEATURES / REQUIREMENTS

8.2 INSTALLATION

8.3 CONFIGURING SAPS CLIENT FOR DOS

8.4 USING SAPS CLIENT FOR DOS

8.5 UNINSTALLING SAPS CLIENT FOR DOS

9. AUTOMATED CONFIGURATION

9.1 SCRIPT FILE

9.2 LOG FILE

9.3 AUTOMATED CONFIGURATION FOR SAPS CLIENT FOR WIN 95

9.4 AUTOMATED CONFIGURATION FOR SAPS CLIENT FOR NT

9.5 SAMPLE SCRIPT FILES

10. CONFIGURING OS/2 SERVERS

11. USING SAPS WITH TCP/IP OVER A ROUTER

12. HOW TO USE SAPS WITH NOVELL NETWARE

13. QUESTIONS AND ANSWERS

# SAPS INTRODUCTION

SAPS (SpartaCom Asynchronous Port Sharing) is a group of software products which allow peer to peer sharing of COM ports between LAN workstations.

Sharing COM ports means sharing resources like modems, ISDN adapters, X.25

PADs or in fact any device that connects to a COM port.

SAPS gives you completely transparent access to shared resources for outbound

communications and operates in most networking environments:

Windows NT, Windows 95, Windows 3.X and DOS.

SAPS supports all NetBIOS compatible protocols: TCP/IP, NetBEUI, IPX, etc.

SAPS also supports Microsoft LAN Manager and IBM LAN Server COM port sharing on OS/2 Servers.

The SAPS software consists of two parts: SAPS Server and SAPS Client.

The network station with one or more COM ports to be shared must have the SAPS Server software installed.

Each workstation that is to use shared COM ports must have the SAPS Client software installed.

Both the SAPS server software and the SAPS client software can reside on the same computer. SAPS does not require a dedicated server.

Once SAPS is installed and configured, the workstation users can run any Windows communications program to access the shared COM port resources, just as if the devices were attached to their own computer.

This User Guide applies to SAPS version 3 and its components :

SAPS Server for Windows NT

SAPS Server for Windows (95 and 3.X)

SAPS Client for Windows NT

SAPS Client for Windows 95

SAPS Client for Windows 3.X

SAPS Client for DOS

The SAPS server for Windows supports sharing up to 9 COM ports (COM1-COM9). The SAPS server for Windows NT supports an unlimited number of shared COM ports.

Access security is provided through the use of passwords for each shared COM port.  Share level security is supported on the Windows SAPS server, and both User level and Share level security are provided on the Windows NT SAPS server.

Pools of COM ports (modems or other Asynchronous devices) can be defined and shared.  When a Workstation COM port is redirected to a shared COM port pool, SAPS will locate the first available COM port in the pool and assign that port to the requesting application.  Pools can consist of shared COM ports residing on several servers.

SAPS for Windows (95 and 3.X) is a Windows only product, and therefore does not support the sharing of COM ports by applications running under DOS, nor applications running in a Windows DOS shell.  However, the SAPS Client for Windows NT does support DOS applications using shared COM ports.

DOS applications using BIOS (Int 14h) are supported by the SAPS Client for DOS.

# MINIMUM REQUIREMENTS

SAPS Server for Windows NT

- Any PC equipped to run Windows NT version 3.51 or higher.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

- One or more COM ports available for sharing.

SAPS Server for Windows

- Any PC equipped to run Windows 3.1, Windows for Workgroups 3.11 or  Windows 95.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

- One or more COM ports available for sharing.

OS/2 Server

- Any PC equipped to run OS/2.

- Microsoft LAN Manager version 2 or IBM LAN Server version 1.2.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...)

- One or more COM ports available for sharing.

SAPS Client for Windows NT

- Any PC equipped to run Windows NT version 3.51 or higher.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

SAPS Client for Windows 95

- Any PC equipped to run Windows 95.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

SAPS Client for Windows

- Any PC equipped to run Windows 3.1 or Windows for Workgroups 3.11.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

SAPS Client for DOS

- Any PC running Microsoft MS-DOS or IBM PC-DOS.

- Network adapter and NetBIOS compatible network (TCP/IP, IPX, NetBEUI, ...).

- Communications program using BIOS (Int 14h) to communicate with COM port.

# SAPS SERVER for WINDOWS NT

## FEATURES

- Simultaneous sharing of local communication ports (COMn).

- Implemented as a Windows NT Service with automatic startup when the system is switched on.

- Password protection (optional) incorporated for accessing each shared resource.

- Verification (optional) as to whether the user is logged on the workstation and member of an authorized SAPS group.

- Microsoft RAS Server compatible.  Pools of modems previously dedicated to RAS communications can also be used for outbound calls.

- Significant events which occur on any of the shared communications ports (user connected, calls, etc...) are recorded in the Windows NT event log.

- Hardware independent (multiport cards, internal/external modems, external ISDN adapters, etc.).

- Local or remote administration of the SAPS NT Server from any Windows NT or Windows 95 computer in the network.

- Pools of shared resources can be defined on the server, transparent to the users.

## INSTALLATION

SAPS Server for Windows NT should be installed and configured on each PC in the network running Windows NT, and which has one or more COM ports to be shared.

To install the SAPS Server for Windows NT:

- Log onto Windows NT as an administrator.

- Insert the SAPS Server for Windows NT disk into floppy drive.

- Go into File Manager (NT-3.51) or Windows Explorer (NT-4.0)

- Click on Drive A: (or Drive B:).

- Double click on SETUP.

- Follow the self-explanatory prompts during installation.

NOTE: During installation you will be prompted to enter a serial number and key. This information is provided in the material packaged with SAPS.  If you do not provide this information, SAPS will run in evaluation mode.

Using the default installation settings, both the SAPS Server software and the Administration software will be installed.  The same SETUP program can be used to install the Administration software only. This will allow you to remotely maintain your SAPS Server(s) from any Windows NT or Windows 95 computer in the network.

## CONFIGURING SAPS SERVER for NT

After SAPS has been installed, you will need to configure SAPS to share the desired COM port(s) before other workstations can access the shared device(s).  To do this, you need to start the SAPS Server Manager:

- Go into the SAPS Program Group.

- Double-click on the SAPS Server Manager Icon.

- Click on the  icon.  The “Shares” window will display:

- Click on New.

- The "New Share" window will display:

- Enter the "Share Name" and optional "Password".  Click on Help if you need assistance.  The Share Name may identify a single port or a pool of ports.

- Select one or more ports from the list on the left and click on Add.  You can also click several times on Add to pool several ports.  Any time a SAPS Client requests a connection to the pool, the SAPS Server will use a round robin algorithm to select an available Port from the list of Shared Ports on the right.

- Click on OK

- Repeat steps 4 to 8 if you need to define several shared ports or pools.  Keep in mind that SAPS does not know what devices are connected to the Serial Ports and will assign any available port in the pool.  You should install the same type of modems or ISDN adapters in a given pool.

- Click on OK in the Shares window to save the changes.  You do NOT need to restart the SAPS Server to make the changes effective.

NOTE: SAPS can share a COM port that is assigned for use by RAS (Remote Access Service).  However, you must be running Windows NT version 3.51, or greater.

NOTE: To stop sharing a COM port, click on the  icon.  Select (high-light) the desired Shared port or pool and click on Remove.  Also, all COM port sharing on the server may be stopped by stopping the SAPS Server (Click on the Stop button).

You may also pause the SAPS Server, which will prevent the server from accepting new connections. Existing connections will not be aborted.

## SETTING SAPS OPTIONS

To set additional SAPS Server options, select Options... from the Configuration Menu.

The "Options" window will display:

To request that SAPS record significant events into the Windows NT application log, click on the “Write events in the applications log” button, and click on OK.

To request that SAPS is to initialize the COM port (modem) after each use, click on the Reinitialize Modem at Disconnect button.  SAPS will then issue the reset string "+++ATHATZ" to the COM port after each use.  Click on OK to save the change(s).

## STARTING SAPS SERVER FOR NT

The SAPS Server startup is automatic when starting the system.  However, the startup can be changed to Manual by using the Control Panel Services.

SAPS can be Started or Stopped anytime by clicking on the Start or Stop button.

## PAUSING SAPS SERVER FOR NT

To temporarily pause the SAPS server, click on the Pause button.  This will prevent SAPS from connecting any additional users to a shared resource, but will allow any existing connections to continue.

To resume the SAPS server, click on the CONTINUE button.

NOTE: The Pause button is displayed only when the SAPS server is started.

## VIEWING CURRENT SAPS USERS

To list the users currently connected to the SAPS server, click on the  icon.

The “User Sessions” window will display, showing the connected users and the shared resources in use.

The content of the window is updated every two seconds.

Statistics on the number of bytes Sent and Received are displayed.

Two buttons allow the Administrator to force disconnection of users.

For each resource, a LED displays the status of  the port:

- gray: port not open

- red: port open, Carrier Detect signal is off

- green: port open, Carrier Detect signal is on

The "Connected Users"  field will be empty if no SAPS group of users has been created on the server (see #3.9).

## SAPS EVENT VIEWING

The events recorded by SAPS can be viewed by using the Event Viewer utility provided by Windows NT.  The Event Viewer utility icon is located in the NT Administrative Tools group.

To view the SAPS events, do the following:

- Click on the Event Viewer icon.

- The Event Viewer window will display.

- Select the Log menu and click on Application.

- Exit the Log menu and select the View menu.

- Click on Filter Events... .

- Enter SPSRVNT in the Source dialog box.

- Click on OK.

- The SAPS events will be displayed in a scroll window.

- To view additional details on a selected event, click on Detail in the View menu.

The following information is displayed in the event scroll window:

| DATE | The date the event occurred |
| --- | --- |
| TIME | The time the event occurred |
| SOURCE | The application (SPSRVNT) that recorded the event |
| CATEGORY | Classification of the event as defined by SAPS ("none" or "communication") |
| EVENT | Event number that identifies the event (see below) |
| USER | User name provided on the Windows client station using the shared resource. This is valid only when the SAPS group exists (security). |
| COMPUTER | The name of the computer on which the event occurred. |

Event Numbers:

| 16 | Successful connection to a shared resource |
| --- | --- |
| 17 | Incorrect password supplied when attempting connection |
| 18 | Successful disconnection from a shared resource |
| 19 | Connection attempt by an unauthorized user |
| 256 | Successful "open" of a shared resource |
| 257 | Unsuccessful "open" of a shared resource |
| 258 | Successful "close" of a shared resource |
| 4096 | NetBIOS error |
| 8192 | Successful line connection (CD on) |
| 8193 | Unsuccessful line connection (CD off) |

The events in the NT event log can be imported into standard spreadsheet or database programs for summarizing or other processing.  Use the "Save As" facility of the Event viewer, and specify "Comma Delim. Text" format.

## USING SECURITY FOR SAPS SERVER FOR NT

SAPS allows the shared Ports to be limited to the users who are members of a specific local group.  This group must be named "SAPS".  The "SAPS" group can be defined by using the "User Manager for Domains" NT utility (located in the Administrative Tools group).  Both Users and Global users may be added to the SAPS group.

WARNING: the "SAPS" group will also contain the Windows NT Domain name of each authorized user or group of users.  SAPS will enforce full NT security.  For example, a user logged on LOCALLY on an NT Client workstation, but not on the Domain controller, will be denied access to shared resources.

NOTE:  The group GUESTS can be included as part of the SAPS group to allow all users to use the shared resources on the NT server.

NOTE:  It is NOT necessary to stop and restart the SAPS Server for the changes to the "SAPS" group to be effective.

## MANAGING REMOTE SAPS SERVERS FOR NT

You can run the "SAPS Server for NT Manager" software on any Windows NT or Windows 95 workstation.

Refer to section #3.2 for installing only the "SAPS Server for NT Manager" (check the right box in the first Setup window).

In the SAPS group, start the  "SAPS Server Manager".

Click on the  icon and select one of the installed SAPS Servers detected on your LAN.

Note: the search for SAPS servers on the network can take a long time if there are hundreds of computers on the LAN.

Note: you can also enter directly the name of your SAPS server.

## UNINSTALLING SAPS SERVER FOR NT

SAPS may be removed from the system using the SAPS Server Uninstall application.

## SAPS SERVER FOR NT and RAS SERVER

COM ports used by RAS Server can also be shared by SAPS.

When a SAPS Client requests one of the Server resources, the SAPS Server software

will ask RAS if the COM port is available.

If the COM port is not in use by RAS, SAPS will be allowed access to the requested COM port.  If the COM port is in use by a remote connection, SAPS will get an error.  Then, if the SAPS pooling mechanism is specified, SAPS will look for the next available port in the pool.

If RAS is installed after SAPS has been installed, you must reinstall SAPS, or use the Registry Editor (REGEDT32.EXE) to add the name "RasMan" to the "DependOnService" path in:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\SPSRVNT.

## SAPS and ZETAFAX SERVER

COM ports used by the ZETAFAX Server can also be shared by SAPS.

When a SAPS Client requests one of the ZETAFAX Server resources, the SAPS Server software will ask ZETAFAX if the COM port is available.

If the COM port is not in use by ZETAFAX, SAPS will be allowed access to the requested COM port.  If the COM port is in use, SAPS will get an error.  Then, if the SAPS pooling mechanism is specified, SAPS will look for the next available port.

If ZETAFAX is installed after SAPS has been installed, you must stop and restart the SAPS Server for NT.

If you are using SAPS, RAS and ZETAFAX simultaneously on your NT Server, please keep in mind that ZETAFAX and RAS are not compatible, i.e. they cannot share the same COM ports. You will need at least two COM ports :

- one used by SAPS and RAS

- one used by SAPS and ZETAFAX.

If you do not use ZETAFAX to receive incoming faxes, the above restriction does not apply.  You may install the SAPS Client for NT on your Server and configure ZETAFAX to use a SAPS COM port redirected to a shared RAS COM port.  Then, ZETAFAX will be able to send faxes using a RAS COM port.

## ADVANCED CONFIGURATION

Experienced Administrators using the Registry Editor (REGEDT32.EXE), can modify the SAPS Server advanced configuration parameters.

WARNING: erroneous modifications to the registry parameters may cause SAPS to fail. Any modifications should be performed with the utmost care.

The configuration parameters are stored in the registry under path: HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\SPSRVNT.

In order to make modifications you need to use REGEDT32.

DO NOT USE THE STANDARD REGISTRY EDITOR (REGEDIT).

For example, to add the ExcludeLanas string proceed as follows:

- select the menu 'Edit', 'Add Value'

- enter in the 'Value Name' field the following string : ExcludeLanas

- select 'REG_MULTI_SZ' in the Data Type field

- click on 'OK'

- in the 'Data' field enter all the Lana Numbers you want to exclude, for example : 3, 'Enter', and 4 'Enter'.  Note : press 'Enter' following each Lana number entered.

- click on 'OK'

- exit from the Registry Editor

- restart the SAPS Server

Spy		REG_DWORD	from 0 to 4096 bytes

Size of the buffer used to store the Hayes commands passed to the modem before

each connection (DCD going high). The dialed number is also stored in this buffer.

The content of the buffer is written to the Event Log if the Log parameter is set to 1.

Default = 256 bytes.

WriteDelay	REG_DWORD	from 1000 to 30000 milliseconds

Timeout value for Write requests on COM ports.

When the Windows NT COM Device Driver reports timeouts, the SAPS Server will emit short 'beeps'.  Do not change this parameter unless you encounter transmission

problems and hear 'beeps' on the Server.

Default = 15000.

RxSize		REG_DWORD	from 4096 to 32768 bytes

COM ports Receive Queue size.

Default = 8192.

TxSize		REG_DWORD	from 4096 to 32768 bytes

COM ports Transmit Queue size.

Default  = 8192.

ReinitModem 	REG_DWORD	0 or 1

This enables reset commands to be sent to the modem every time a COM port is

closed by SAPS. This parameter can be set using the SAPS Server Administration.

Default = 0.

Reset commands are:

- set DTR & RTS to high,

- wait 1 second,

- send the Hayes escape sequence "+++",

- wait 1 second,

- send "ATH"<cr>

- wait 1 second,

- send the Hayes reset sequence (see ResetModem parameter)

- wait 1 second,

- set DTR & RTS to low.

ResetModem		REG_SZ		ASCII string

This parameter contains the Hayes reset sequence. (e.g.: "AT&F S0=0 E1")

Default = "ATZ"

Mode			REG_DWORD	0 , 1 or 2

When set to 0, SAPS will allow any user who has the resource password to access

shared COM ports ("ShareLevel" mode).

When set to 1, SAPS will only allow users who are member of the group "SAPS"

to access shared COM ports ("UserLevel" mode). No Domain validation is performed.

When set to 2, SAPS will only allow users who are member of the group "SAPS", and logged on their workstation on the Domain specified in group "SAPS", to access shared COM ports ("UserLevel" mode).

This parameter is ignored if no group "SAPS" has been created.

Default : 2 since version 1.12, 1 for prior versions.

ExcludeLanas	REG_MULTI_SZ	Strings

This parameter can be used to prevent the SAPS Server from monitoring listed

Protocol Stacks for calling clients. For example, you could decide to accept SAPS Clients using only NetBIOS on TCP/IP protocol stack.

Default = (empty).

Please see the NetBIOS Interface properties in the Network control panel for information on the Lana Numbers:

- open the 'Control Panel' window

- double-click on 'Network'

- click on 'Services'

- select 'NetBIOS Interface'

- click on 'Properties'

- you should get a windows similar to the one below

KeepSession		REG_DWORD	from 0 to 127 seconds

Timeout period for NetBIOS receives.

(0 = infinite wait).

This is a watchdog system used to prevent failed SAPS Clients from locking up Server resources.  This timeout will free resources any time a SAPS Client locks up.

Default : 60

# SAPS SERVER for WINDOWS

## FEATURES

- Simultaneous sharing of up to 9 local communications ports (COMx)

- Compatible with Windows 95, Windows for Workgroups 3.11 and Windows 3.1

- Password protection (optional) incorporated for accessing each shared resource

- Hardware independent (multiport cards, internal/external modems, ISDN adapters, etc.).

## INSTALLATION

To install the SAPS Server for Windows:

- Insert the SAPS Server for Windows diskette into floppy drive.

- Go to the Windows Program Manager and select File menu.

- Select Run from the File menu.

- Type A:SETUP (or B:SETUP) and press Enter.

- Follow the self-explanatory prompts during installation.

- After the installation is complete, click on the INFO icon in the SAPS group window and review the document for additional information not included in this manual.  It is suggested that you print this document and keep it with the SAPS User Guide.

NOTE: During installation you will be prompted to enter a serial number and key.  This information is provided in the material packaged with SAPS. If you do not provide this information, SAPS will run in evaluation mode.

SAPS Server for Windows should be installed and configured on each station in the network which has one or more COM ports to be shared.

## CONFIGURING SAPS Server for Windows:

After SAPS has been installed, you will need to configure SAPS to share the desired COM port(s) before other workstations can access the shared device(s). To do this:

- Go into the Windows Control Panel.

- Select the SAPS Server icon (added during installation).

- If the SAPS Server is not started, answer Yes to start it.

- The "SAPS Server: Serial-Ports sharing" window will display.

- Under Ports, select (high light) the COM port to be shared.

- NOTE: SAPS does not support sharing COM ports to which a mouse is  attached.

- Click the Share as >>> button.

- The "Share a Serial-Port" window will display:

- Complete the "Share As:", "Comment:", and "Password:" dialog boxes.  Click on Help if you need assistance.

- NOTE: "Comment" and "Password" are optional.

- Click on Share at startup if the selected COM port is to be automatically shared each time the SAPS server is started.

- Click on OK.

- Repeat steps 5 - 10 for additional COM ports to be shared.

- Click on CLOSE to record the changes.

NOTE:  To stop sharing a COM port, select the desired COM	port in the "Serial-Ports sharing" window and click on the <<< Stop sharing button.  Also, all COM port sharing on a network station may be stopped by stopping the SAPS Server.  Click on the Stop server..." button.

## STARTING SAPS SERVER for WINDOWS

To start the SAPS Server, go into the Windows Control Panel.  Click on the SAPS Server icon in Control Panel.

NOTE:  SAPS can be started automatically whenever Windows is started by copying the SAPS server icon to the Windows Startup group.

## SAPS SERVER for WINDOWS ADMINISTRATION

SAPS Server for Windows provides current information on the shared local COM ports. This information includes:

- The number of shared local COM ports being used by other network stations.

- The name of the other stations connected to the local shared COM ports.

- Statistics on the shared COM port connections.

To display this information, click on the minimized SAPS Server icon.  The SAPS “About” window will be displayed.  Click on the Sessions and Statistics buttons to view the desired information.

## SAPS SERVER for WINDOWS PERFORMANCE CONSIDERATIONS

Using shared resources over a network is inherently slower than using local resources. However, because SAPS uses the local network's bandwidth to its maximum potential, the performance degradation is very slight (approximately 5%).  The following suggestions are provided for your consideration when attempting to optimize SAPS performance:

- Use as powerful a computer for the server as possible.

- Reduce the non-SAPS workload on the server.

- Choose an efficient and high speed network (Hardware and Software).

- Select a high value for block transfer size.

- Use a multiport card with co-processor.

- Use intelligent X.25 cards.

- Use serial port drivers that support high throughput.  For example, the standard Microsoft driver COMM.DRV that comes with Windows 3.1 does not use the UART FIFO and will fail to work properly with baud rates higher than 19.2Kbps.

Use shareware drivers like CYBERCOM.DRV, E-Mail at 100033.1723@compuserve.com.

## SAPS SERVER for WINDOWS Parameters

The SPSRV.INI file, [SERVER] section, contains the following parameters:

- WorkingDirectory	the directory where SAPS is installed

- Visible	controls SAPS Server Icon

0 = hidden (default)

1 = visible (value set by Setup)

The following parameters may be optionally added to the configuration file:

- Autoload	when opening the Control Panel, load SAPS Server

0 = do not load SAPS Server (default)

1 = load SAPS Server

- Polling	the SAPS Server may not operate properly with Windows 3.1 when DOS boxes are used concurrently with traffic on COM ports.  The system may hang or reset abruptly.  As a workaround, set the polling parameter to a non zero value.

0 (default) -  SAPS Server runs in event mode providing best performance.

50 to 500 - SAPS Server polling time (msec).  The computer will provide slower response times to LAN users (100 recommended value)

## UNINSTALLING SAPS sERVER FOR wINDOWS

- Quit Windows

- During the installation, SAPS copies the CONTROL.INI file into the working directory for SAPS as CTLOLD.INI.  If no additional updates to the CONTROL.INI file have occurred since the SAPS installation, restore this file from the SAPS working directory.

- NOTE: When you restore this file, you will lose any updates to the file that occurred since SAPS was installed.

As an alternative to restoring the CONTROL.INI file, do the following:

- In the [MMCPL] section of the CONTROL.INI file, suppress the line beginning with SAPSSERVER=

- Delete the following file from the \WINDOWS directory: SPSRV.INI.

- Delete the following file from the \WINDOWS\system directory:  SPSHARE.DLL.

- Delete all files from the SAPS working directory (specified during installation).  The SYSOLD.INI and CTLOLD.INI files are write protected and must be unprotected by entering  ATTRIB  -r  *.INI before attempting to delete them.

- Remove the SAPS working directory.

- Restart Windows.

- Delete the SAPS icon(s) that were added during installation.

# SAPS Client for Windows NT

## FEATURES

- Provides for automatic redirection of up to four (4) COM ports to shared COM ports on a Windows NT, Windows 95 or Windows 3.X SAPS server.

- Allows COM port pooling across different servers.  SAPS will connect to the first available port listed in the pool.

- May be setup to automatically disconnect when no COM port activity has occurred for a specified length of time.

- Compatible with 32 bit communications software using COM ports under Windows NT.

- Compatible with 16 bit Windows or DOS communications software using COM ports under Windows NT.

- Communication error logging is provided in the Windows NT Event log.

## INSTALLATION

To install SAPS Client for Windows NT:

- Log onto Windows NT as an ADMINISTRATOR

- Go into Control Panel, then select Network.

- Click on Add Software

- Note: When applying an update to an already installed SAPS Client for

- Windows NT, click on the Update button, instead of Add Software.

- Select <Other> Requires Disk From The Manufacturer from the Network Software list

- Insert the SAPS Client for Windows NT diskette

- Follow the instructions on subsequent screens to complete the installation

NOTE: During installation you will be prompted to enter a serial number and key.  This information is provided in the material packaged with SAPS.

SAPS Client for Windows NT should be installed on each Windows NT station in the network which will be allowed to redirect one or more COM ports to a shared COM port(s).  The new SAPS COM ports will be assigned to non-existing COM ports, for use by your applications.  Example: If the computer is equipped with COM1 and COM2,  the four (4) new ports will be assigned to COM3, COM4, COM5, and COM6.  However, you can change the COM port assignments made by the installation procedure.

The port assignments are changed using the SAPS Ports Configuration screen:

To accept the default COM port name assignments, click OK.

To change a default name assignment:

- Select (high-light) the port to be re-assigned

- Enter the new name (COMx) in the Port Naming section

- Click the Apply button

If all port assignments listed below the one being changed are to be reassigned sequentially, click on Automatically Enumerate Port Names.  Then, when you click the Apply button, the selected port and all ports below it will be assigned to sequential COM port names.

If you later desire to reassign the new ports:

- Go into Control Panel

- Click on Network

- Select (high-light) SAPS Port Redirector

- Click on Configure

- The SAPS Ports Configuration screen will be displayed

- Enter the new name(s) as described above

Before leaving the “Network Settings” screen, you can select the protocol stacks to be used with the SAPS Client, or change the protocol stacks binding order.  Click on  Bindings... and select SPNetNT Interface to display the following screen:

In this example, the SAPS Client does not use the RAS protocol stacks.

This is the recommended setting.

Note: Deactivating the Binding with RAS will allow a faster startup of the service.

Restart NT after completing the SAPS Client installation.

A new serial device driver, supporting four (4) COM ports, has been added to the NT system.  These four (4) ports can be redirected to a SAPS server.

## CONFIGURING SAPS Client for Windows NT

During installation, the SAPS Port Redirector icon was added in the Control Panel.  Double click on this icon to display the SAPS Port Redirector screen:

SAPS can be configured to redirect a COM port to a specific shared resource on a SAPS server, or can be configured to use COM port pools.  When using COM port pools, the SAPS Port Redirector will search for, and connect, the first available port in the pool.

Redirecting a COM port to a specific shared port.

- Select (high-light) one of the four (4) SAPS ports to be redirected.

- Click on Settings

- Enter the path to the shared COM port in the Network Path field.

- Example: \\server_name\resource_name

- Enter the password (optional) required to connect to the shared resource. Leave this field blank if you are running in User Security mode, i.e. you are a member of a SAPS group of users configured on the NT server.

- Click on OK.

- Click on OK.

- Exit Control Panel.

Redirecting a COM port to a pool of shared ports.

Pools of COM ports are usually setup on the SAPS Server for NT, but you may desire to use several SAPS servers. To do this, you simply define a list of pools located on different servers.

- From the SAPS Port Redirector screen, click on the Pools button.  The SAPS Pools and Resource Management screen will be displayed:

- Add the desired Pool name in the Pools section.

- Add the resource path for all shared ports to be included in the Pool Resources section.  Example: \\Server_Name\Resource_Name

- After all shared resources have been added, Click on OK.

- Click on the Settings button on the Saps Port Redirector screen.

- Click on Use Pools.

- Select (high-light) the desired pool name from the list of defined pools.

- Click on OK, twice.

- Exit Control Panel.

The redirected COM port can now be used by your applications.  Start the communications application and configure it to use the redirected COM port.  The connection to the shared COM port will be made when the redirected COM port is opened by the application.

## DISCONNECT ON INACTIVITY

SAPS Client for Windows NT can be configured to automatically disconnect from a shared COM port when no communications activity occurs for a specified length of time. On the SAPS Port Redirector screen, check the Disconnect on Inactivity button and enter the desired inactivity time in minutes.  Click on OK

## OPTIONS

SAPS Client for Windows NT can be configured to display the SAPS screens in normal or 3D mode.  On the SAPS Port Redirector screen, click on the Options button and select normal or 3D display mode.  Click on OK.

NOTE: This feature does not apply when running Windows NT 4.0.

## ADMINISTRATION

SAPS provides entries into the NT Event Log for communications errors.

To view the SAPS entries in the Event Log, select Event Viewer from the Administrative Tools Group, and specify a Source of SPNetNT or SPRDRNT in the View/Filter Events.

The recorded events will contain the following fields:

Date = Event date

Time = Event time

Category = “none”, NetBIOS”, “SMB”, or “System”

Event = Event number

When viewing the Event Log, double click on a selected event to display the detailed information for the entry.

## UNINSTALLING SAPS Client for Windows NT

Log on as an ADMINISTRATOR.

Select Network from Control Panel.

Select (high-light) SAPS Port Redirector in the Installed Network Software list.

Click on the Remove button.

# SAPS Client for Windows 95

## FEATURES

- Provides for automatic redirection of up to nine (9) COM ports to one or more shared COM ports on a Windows NT, Windows 95 or Windows 3.x SAPS Server.

- Allows COM port pooling. SAPS will connect to the first available port listed in the pool.

- May be setup to automatically disconnect when no COM port activity has occurred for a specified length of time.

- Compatible with 32 bit communications software using COM ports under Windows 95. Applications using TAPI and UNIMODEM are supported.

- Compatible with 16 bit Windows communications software using COM ports under Windows 95.

- Communication errors display in a SAPS pop-up Window.

## INSTALLATION

SAPS Client for Windows 95 should be installed on each Windows 95 workstation in the network, which will be allowed to redirect one or more COM ports to a shared COM port(s).

To install SAPS Client for Windows 95, or update an existing version, please use the diskette provided in the package.

The installation procedure will add one (1) COM port to the Windows 95 system. The new COM port will be assigned the first non-existing COM port number.  Example: if the computer is equipped with COM1 and COM2, the new port will be assigned to COM3.  For all programs that do not use UNIMODEM, the name of this new port will be "COMx".

For all programs that use UNIMODEM, the name of this new port will be the name of the redirection path : "\\SERVERNAME\RESOURCE (COMx)" or "POOLNAME (COMx)".

- Click on the Start icon, move the mouse pointer to Settings, and click on Control Panel.

- Double click on Add/Remove Programs.

- Click on Install and insert the SAPS Client for Windows 95 diskette.

- Click on Next.

- Click on Finish to start installation.

- Click on Next (on Welcome screen).

- During installation you will be prompted to enter a serial number and key.  This information is provided in the material packaged with SAPS.  If you do not provide this information, SAPS will run in evaluation mode.  Click on Next.

- In the 'Redirection type' window you will be prompted to select the type of redirection. You can either assign a specific shared port or a pool of shared ports to the new COM port. Select Use a Pool of Network Resources if you have several shared COM ports on your SAPS server(s) and you want to allow SAPS to connect to the first available port.

If you select Use a Network Resource:

- Click on Next>, and enter the path to the shared COM port in the Network Path field. Example : \\server_name\resource_name.

- Enter the password (optional) required to connect to the shared resource. This password must be the same as the one entered for the resource on the SAPS Server.

- SAPS Client for Windows 95 can be configured to automatically disconnect from a shared COM port when no communications activity occurs for a specified length of time. Check the Disconnect on inactivity box and enter the desired inactivity time in minutes.

- Click on Next> and follow the instructions on subsequent screens to complete the installation.

If you select Use a Pool of Network Resources:

Pools of COM ports are usually setup on the SAPS Server for NT, but you may desire to use several SAPS servers. To do this, you simply define a list of pools located on different servers.

- Add the desired Pool name in the Pools section.

- Add the shared resource path for all shared ports to be included in the pool in the Pool Resources section. Enter the required resource password (optional).

- After all shared resources have been added, click on Next>.

- SAPS Client for Windows 95 can be configured to automatically disconnect from a shared COM port when no communications activity occurs for a specified length of time.  Check the Disconnect on inactivity box and enter the desired inactivity time in minutes.

- Click on Next and follow the instructions on subsequent screens to complete the installation.

## MODEM DETECTION

Before using a shared modem, Windows 95 needs to detect it.

NOTE: You must have defined the COM port redirection (enter a network path to a shared port or pool) before running the modem detection.

To perform Modem detection :

- Go into Control Panel.

- Select Modems.

- Click on Add.

- Click on Next> to start the automatic detection on every port.

- If detection worked properly you will get a window similar to the one below, Click on Next>

- Click on Finish.

## CONFIGURING SAPS CLIENT FOR WIN 95

There are several ways to configure the SAPS Client for Windows 95 :

- from Modems in Control Panel

- from Modem Properties in any Win32 application (Hyperterminal, Dial-Up Networking, WinFax, etc.)

- from System in Control Panel

From the Control Panel  select Modem.

Select a modem from the list and click on Properties.

Select the Connection tab.

Click on Port Settings.

Select the SAPS Settings tab.

Proceed with modifications.

You can also change the configuration by selecting System from the Control Panel.

Click on the Device Manager tab.

Highlight Computer and click on Refresh.

Double click on Ports (COM & LPT). You should see a port in the port list with the name ‘SAPS Port Redirector (COMx)’.

Click on ‘SAPS Port Redirector (COMx).

Click on Properties.

Click on SAPS Settings tab.

Proceed with modifications.

## ERRORS

Connection errors are displayed in a window of Sapsctrl. This window will pop up anytime an error occurs.  You can force this window to pop up by double-clicking on the icon at the bottom right of your screen, in the Windows 95 taskbar.

If the mouse pointer is moved over the Sapsctrl icon, a message will indicate the status of the redirection.

## SAPSCTRL

To display the Sapsctrl window click on the Start icon, move your mouse to Run  and type SAPSCTRL.

Use Sapsctrl to display the current version of SAPS Client for Windows 95, change your licence number or other options.

## ADDING A PORT

To add another port to an existing SAPS installation:

- Go into Control Panel.

- Double click on Add/Remove Programs.

- Select SAPS Client for Windows 95 from the list of installed software.

- Click on Add/Remove...

- Select Add a SAPS Port in the SAPS Setup window.

- Click on Next.

- Proceed as described in Section 6.2.

## UNINSTALLING SAPS CLIENT FOR WIN95

To remove SAPS Client for Windows 95 :

- Click on the Start icon, move the mouse pointer to Settings and click on Control Panel.

- Double click on Add/Remove Programs.

- Select SAPS from the list of installed software.

- Click on Add/Remove...

- Select Uninstall SAPS Port Redirector in the SAPS Setup window and click on Next>.

- Click on OK.

Always restart Windows after updating an existing installation.

# SAPS Client for Windows

## FEATURES

- Provides for automatic or manual redirection of up to nine (9) COM ports to one or more shared COM ports on a Windows NT, Windows 95 or Windows 3.x SAPS server.

- Allows COM port pooling.  SAPS will connect to the first available port listed in the pool.

- May be setup to automatically disconnect when no COM port activity has occurred for a specified length of time.

- Compatible with most 16 bit Windows software using COM ports under Windows.

## INSTALLATION

To install SAPS Client for Windows:

- Insert the 'SAPS Client for Windows & DOS' diskette into floppy drive.

- Go to the Windows Program Manager and select File menu.

- Select Run from the File menu.

- Type A:\WIN3XCL\SETUP (or B:) and press Enter.

- Follow the self-explanatory prompts during installation.

- NOTE: During installation you will be prompted to enter a serial number and key. This information is provided in the material packaged with SAPS.  If you do not provide this information, SAPS will run in evaluation mode.

- When the installation is complete, you will be asked if  Windows may be restarted.  Click on "Restart Windows".

- The SAPS Group Window will display  (If not displayed, click on the SAPS Group icon which was added during installation).

- Click on the "Read me" icon and review the document for additional information not included in this manual.  It is suggested that you print this document and keep it with the SAPS Users Guide.

SAPS Client for Windows should be installed and configured on each station in the network which will be allowed to redirect one or more COM ports to a shared COM port(s).

## CONFIGURING SAPS Client for Windows

Once SAPS has been installed, you will need to configure SAPS to redirect one or more local COM ports to a shared port or pool before the redirected COM port can be used.

Click on the "Asynchronous Port Manager" icon in the SAPS Group Window.  The "Asynchronous Port Manager" window will display the available ports (COM1-9) and their current status.

A toolbar is provided just below the menu bar, and can be used to quickly access the functions of the Asynchronous Port Manager.

### TOOLBAR DESCRIPTION

Connect to a shared network COM port.

Disconnect a shared network COM port.

SAPS Server. Available only if the workstation has the SAPS Server installed.

Configure Port Pools.

Change SAPS setup options.

Update the "Asynchronous Port Manager" window.

Display Help screen

### SHARED COM PORT POOLS

SAPS allows shared COM port pools to be created and maintained.  A shared COM port pool is a user defined set of network ports with compatible attributes, that is given a generic global name.  When a connection is requested to a COM port pool, SAPS will connect the first available port in the pool.

Pools of COM ports are usually setup on the SAPS Server for NT, but you may want to use several servers. To do this, simply define a list of pools located on different servers.

To create or maintain a Shared COM port pool, do the following:

- Click on the "Manage Port Pools" icon.

- The "Manage Port Pools" window will display.

- Click on the "Help" button.

- The Help screens provide detailed instructions on maintaining port pools.  Follow the Help screen instructions.

- Click on OK to save the changes in the "Manage Port Pools" window.

### AUTOMATIC CONNECT/DISCONNECT

SAPS can be configured to provide automatic connection (transparent to the user) to a shared network COM port (or pool).  The connection will automatically be made when the communications application opens the local COM port.  Without the automatic connection, it is necessary to go into the Asynchronous Port Manager and manually establish the connection to a shared COM port before the local redirected COM port can be used.

SAPS also provides for automatic disconnection from a shared COM port (or pool) when an application closes the local COM port, or when no activity has occurred on the COM port for a specified length of time.  Inactivity means that no data has been received or transmitted.  Once the shared COM port has been disconnected, it becomes available for use by another station on the network.

If it is desirable to allow automatic connection and/or disconnection to the shared COM port, do the following:

- Click on the "Change SAPS setup options" icon in the "Asynchronous Port Manager" window.

- The "Connection/Disconnection" window will display:

- To allow the automatic reconnection of a local COM port to a shared network port (or pool):

  - Click on the "Reconnect automatically" button.

  - Click on the "Parameters" button.

  - The "Reconnect Automatically Parameters" window will display.

  - To allow automatic connection to the last shared COM port (or pool) successfully used, click on "Last network port used".

  - To allow automatic connection to a specific COM port:

    - Click on "Quiet connection to network port".

    - Click on the down arrow to the right of the "Network Path" dialog field, and the list of defined shared ports (and/or pools) will be displayed.

    - From the list, click on the desired port (or pool).

    - Enter the "User Name" (If required).

    - Enter the "Password" (If required).

    - Click on OK.

- To allow automatic disconnection from the shared COM port when the application closes the local COM port, click on "Disconnect automatically".

- To allow automatic disconnection from the shared COM port after a specified length of inactive time, click on "Disconnect on inactivity", and enter the inactivity time.

- Click on OK.

- NOTE 1 : SAPS does not support redirection of COM ports to which a serial mouse is  attached.

- NOTE 2 : Redirection of a locally shared port is not supported.

- NOTE 3 : Redirection to a locally shared port is not supported.

### SAPS SPECIAL KEYBOARD COMMANDS

| F1 | display on-line Help |
| --- | --- |
| ALT+C | display the Connection dialog window |
| ALT+D | display the Disconnection dialog window |
| F5 | refresh "Asynchronous Port Manager" window |
| ALT+F4 | quit SAPS, or quit Help |
| ALT+PrtScr | copy HELP screen to the clipboard |

## CONNECT TO A SHARED COM PORT

- Select (high-light) an available port (COM1-9) from the list in the Asynchronous Port Manager window.

- Click on the "Connect" icon.

- The "Connect to Network Port" window will display.

- If the "Use Port Pools" is checked, a list of defined port pools will be displayed.  Click on the desired port pool.  If there are also individual shared COM ports defined and you desire for them to be displayed, click on the “Use Port Pools” button to remove the port pool list.

- To re-display the port pools list, click on the “Use Port Pools” button again. If the "Use Port Pools" is NOT checked, click on the down arrow to the right of the "Network Path" dialog field, and a list of previously defined shared COM ports will be displayed.  Click on the desired shared port. If no shared COM ports have been previously defined or one is to be added, enter the "Network Path" of the shared COM port.  The correct syntax for the "Network Path" is as Follows: \\Server_Name\Share_Name

- Click on OK.

- If the shared COM port requires a password, the "Connection" window will display.  Enter the password for the shared COM port.  The password is either the password as entered when logging onto the Windows NT or OS/2 server, or the resource password as defined on a Windows SAPS server.

- Click on OK.

## DISCONNECT FROM A SHARED COM PORT

- Click on the "Disconnect a shared network COM port" icon in the “Asynchronous Port Manager” window.

- The "Disconnect from Network Port" window will display.

- Click on the port(s) to be disconnected in the Connected Network Port(s) list.

- Click on OK.

After completing the SAPS setup information, Exit (Quit) the Asynchronous Port Manager window.

## ADDITIONAL INSTALLATION NOTES

SAPS can be installed on a workstation from a shared directory on a network server.

  - Copy the SAPS installation diskette to a shared directory on a network server.

  - From the workstation, go into the Windows File Manager and log onto the shared directory.

  - Double click on the SETUP.EXE file.

  - Complete the installation as previously described for using the install diskette.

If multiple workstations are to be setup with the same SAPS configuration in the "Asynchronous Port Manager", the SAPS.INI file from the first installation can be copied to the \Windows directory of each of the other workstations.  This will eliminate the necessity of individually configuring SAPS on each workstation.

## SAPS ERROR MESSAGES

Note:  When a port is "In Error", you must manually disconnect it before attempting to connect it again.

| Error: | "COMn is redirected. To share it, stop the redirection" |
| --- | --- |
| The port you want to share is already redirected. |  |
| Error: | "NetBIOS Error xxx" |
| This indicates a problem with the Network. Error 05 may occur if you are trying to connect to a locally shared port. |  |
| Error: | "Network access is denied" |
| The server denied access to the resource. Check your access rights on the server. |  |
| Error: | "Redirection from port COMx to pool xxx did not complete successfully. See xxx.LOG file for more information" |
| SAPS probably could not find an available port in the pool. Additional information can be found in the xxx.LOG file in the SAPS working directory. (xxx represents the pool name). |  |
| Error: | "SMB_HRD Error xxx" |
| This indicates a problem with the server. Error 21 may occur if you are trying to connect to a port which does not exist on the server. |  |
| Error: | "The redirection is stopped" |
| The inactivity time-out stopped your redirection. |  |
| Error: | “SMB_DOS Error: code 67” |
| PRIOR TO SAPS V-2.11: The @ was omitted from the shared COM port path. Example: \\@Server_Name\Resource_Name SINCE SAPS V-2.11: The SAPS Server is NOT started. |  |
| Error: | "The resource is used on the server" |
| The network port you are trying to connect is already in use by an application running locally on the server. Stop the application on the server to free the resource. |  |

| Error: | "The resource isn't available" |
| --- | --- |
| The port you are trying to connect is no longer shared. |  |
| Error: | "The resource isn't currently available" |
| The network port you are trying to connect is already in use by an application running on another workstation. |  |
| Error: | "The selected port COMx is used by another application" |
| You are not allowed to connect or disconnect a port which is already in use by an application. |  |
| Error: | "Unable to reconnect automatically port COMx. Error xxx" |
| The pool used for reconnection is empty or could not be found. |  |

For other errors, please contact SpartaCom technical  support.

## SAPS CLIENT FOR WINDOWS PARAMETERS

### SAPS.INI FILE

The SAPS.INI file is stored in the \Windows directory and has several sections :

[SAPS]

- FastSpeed	indicates that line speeds higher than 19200bps will be used by the application.

0 = do not change the speed,

1 = change the speed according to the following conversion table:

110bps ---> 38400bps

300bps ---> 57600bps

600bps ---> 115200bps

- LanaBase	indicates which protocol stack should be used by SAPS.

Default = SAPS scans the installed protocol stacks

0 to 7 = use a specific protocol stack

Before changing the default configuration, make sure that the protocol stack you intend to use is defined in PROTOCOL.INI or SYSTEM.INI.

Other parameters such as the name of the communications driver to which SAPS directs messages sent by Asynchronous applications using a non-redirected port.  If you intend to control the local non-redirected serial ports with a driver other than the Windows driver (COMM.DRV), edit the COMM.DRV=MSCOMM.DRV statement, replacing MSCOMM.DRV with the appropriate driver name.  (SAPS gives COMM.DRV a new name of MSCOMM.DRV during installation.).

In the [COMx] sections, the configuration options for each of  the local serial ports.

In the [POOLS] section, a list of shared COM port pools and the resources included in each pool.

NOTE:  Be very careful when changing the contents of the SAPS.INI  file.  Any errors in the syntax will result in problems when running SAPS.

### SAPSUSER.INI FILE

The SAPSUSER.INI file is stored in the \Windows directory.

- In the [SAPS] section, the Serial number and key for this SAPS installation.

- In the [SPORTMAN] section, the SAPS server parameters, including the working directory.

- In the [CONNECTIONS] section, a list of the local COM port(s) that were successfully connected to a shared COM port and the path to the shared COM port.

- In the [COMn] sections, the parameters for each local port for which a Quiet mode connection has been activated.

NOTE:  This file is specific to the workstation and should not be duplicated on other network stations.

## UNINSTALLING SAPS CLIENT FOR wINDOWS

- Quit Windows

- During the installation, SAPS copies the SYSTEM.INI, and CONTROL.INI files into the working directory for SAPS as SYSOLD.INI and CTLOLD.INI.  If no updates to the original .INI files have occurred since the SAPS installation, restore these two files from the SAPS working directory.

- NOTE: When you restore these files, you will lose any updates to the files that occurred since SAPS was installed.

- As an alternative to restoring these two files, do the following:

- In the [BOOT] section of the SYSTEM.INI file, replace the line COMM.DRV=SPCOMM.DRV with COMM.DRV=COMM.DRV (or with the line saved in the SAPS.INI file)

- Delete the following files from the \WINDOWS directory:

- SAPS.INI and SAPSUSER.INI.

- Delete the following files from the \WINDOWS\SYSTEM directory:  SPCOMM.DRV and SPNET.DLL

- Delete all files from the SAPS working directory (specified during installation).  The SYSOLD.INI and CTLOLD.INI files are write protected and must be unprotected by entering  ATTRIB -r *.INI  before attempting to delete them.

- Remove the SAPS working directory

- Restart Windows

- Delete the SAPS icon(s) that were added during installation

# SAPS Client for DOS

## FEATURES / REQUIREMENTS

- The SAPS Client for DOS allows DOS applications to access shared resource on a SAPS server.

- The SAPS Client for DOS is built around a DOS TSR that redirects a local COM port.

- Requires NetBIOS to communicate with a SAPS server.  Works with applications that use the BIOS interrupt 14h interface (serial port interface).  Applications directly interfacing the hardware are not supported.

## INSTALLATION

To install the SAPS Client for DOS:

- Insert the SAPS Client for Windows/DOS diskette into your floppy drive.

- Make the floppy drive the default: Type A: and press Enter.

- Change to the directory DOSCL:  Type CD \DOSCL and press Enter.

- Type SETUP <path> and press Enter.  (<path> is the path on your hard disk where the SAPS Client for DOS will be installed.)

- Example:  SETUP C:\SAPSDOS

- After the installation is complete, add this path to your PATH environment variable.

- Note:You can edit your AUTOEXEC.BAT file and change your PATH setting.

- For example:  SET PATH=C:\SAPSDOS\;%PATH%

## CONFIGURING SAPS Client for DOS

After SAPS has been installed, you will need to configure SAPS to redirect one (1) COM port.

You first need to configure your SAPSDOS.INI file and optionally SAPSPOOL.INI file in your installation directory.  These are ASCII text files which can be easily edited using the DOS EDIT program.  Entries and keys are not case sensitive, but their associated value may be case sensitive, depending on the item.

Any time you change any setting in either file, you need to unload the TSR and reload it  to make the change effective.

Basic configuration parameters in SAPSDOS.INI

[SAPS]

| SerialNb= | Insert your product serial number after the equal sign. |
| --- | --- |
| Key= | Insert your product associated protection key. |
| Station= | Insert your workstation name as known on the Network. |
| User= | Insert your user name when you logon to your server. |
| [COM1], [COM2], [COM3], [COM4] | Specify the COM port you want to redirect. Enter either COM1, COM2, COM3 or COM4. Note: only one (1) may be specified. |
| CxAutoRes= | Specify the shared resource name on the server you want to redirect to. The syntax is \\ServerName\Resource (Ex: \\SERVER1\COM1). You can also specify a PoolName if you desire to use a pool of resources. |

Advanced configuration parameters in SAPSDOS.INI

[SAPSCTRL]

| SMBTempo= | You can adjust the SMB response time by changing its value. Specify a value in seconds. This is the maximum time that SAPSDOS will wait for a response to a SMB request. |
| --- | --- |
| TimeSlice= | This is the minimum delay between two consecutive SMBReadXs. Its value is specified in milliseconds. |
| FastSpeed= | If FastSpeed=1 (or different from 0) The low speed baud rates (110, 150, 300, 600) are mapped to a faster speed. Mapping values: |
|  | 110 is mapped to 19200 |
|  | 150 is mapped to 38400 |
|  | 300 is mapped to 57600 |
|  | 600 is mapped to 115200 |

[COM1], [COM2], [COM3] or [COM4]

| DxTimeOut= | Specifies that an automatic disconnection is to occur after a specified time of no activity. If DxTimeOut=1 (or different from 0), this feature is activated. Otherwise, this feature is not active. |
| --- | --- |
| TimeOut= | Associated with DxTimeOut. Specify this value in minutes. If DxTimeOut is active, and if the inactivity time is longer than this value, the resource will be disconnected. When this occurs, SAPSDOS should be unloaded and reloaded again, to reconnect to a shared resource. |

Configuration parameters in SAPSPOOL.INI

This file is required only when you use a pool of resources.  Any number of Pool Names may be specified.

[PoolName]

| Routex= | ResourceName. Where x equals 1,2,3 (Ex: Route1=, Route2=, Route3=), followed by ResourceName in the following format: \\ServerName\Resource Ex: Route1=\\SERVER1\COM1 |
| --- | --- |

Pool setup Example:

In SAPSDOS.INI:

[COM1]

CxAutoRes=Pool1

In SAPSPOOL.INI:

[Pool1]

Route1=\\SERVER1\COM1

Route2=\\SERVER1\COM2

## USING SAPS Client for DOS

Once configured, the SAPS Client for DOS is easy to use.

A batch file named “SPD.BAT” has been installed which allows you to start the COM port redirection, start your application, and then stop the redirection when the application terminates.

Enter:  SPD <application>    Example: SPD KERMIT

You will be prompted to enter a password, if required by the shared resource.  Enter the password at the prompt.

## UNINSTALLING SAPS Client for DOS

Delete all files located in the SAPSDOS install directory.

Delete the SAPSDOS directory itself.

Edit AUTOEXEC.BAT and remove all reference to SAPSDOS from the PATH parameter.

# Automated CONFIGURATION

An automated configuration capability is provided for fast and easy configuration of the SAPS Client for NT and the SAPS Client for Windows 95 products. The configuration data usually provided by the user on each workstation can be read from a text file.

## script file

The configuration script file is named setup.cfg and has the format of a standard Windows .INI file (no other file name can be specified).

The file must contain at least one section named [Port1], with a key ‘Path’ which is correctly formatted.  All other parameters are optional.

| Section | Key | Default value | Range / Format | Systems |
| --- | --- | --- | --- | --- |
| General | InstallPath (optional) | "Windows 95 Program Files directory"\saps95 | Must include the drive | W95 |
|  | RemovePorts (optional) | 1 | 0 = keep existing SAPS ports 1 = delete all SAPS ports before adding new ones | NT |
|  | FirstPort (optional) | COM1 | COM[1 to 256] | W95 & NT |
| Parameters | DisconnectTimeOut (optional) | 0 | 0 (Off) 1 (On) | W95 & NT |
|  | TimeOut (optional) | 5 | 1 to 999 (minutes) | W95 & NT |
| Port[1 to 9] | Path |  | 98 characters | W95 & NT |
|  | Password (optional) |  | 8 characters | W95 & NT |
|  | Mode (optional) | 9600,n,8,1,x | standard Win 3.1 mode string (BaudRate, Parity, Data, Stop, Retry) | W95 |
| Pool[1 to 9] | Name |  | 16 characters | W95 & NT |
|  | Comment (optional) |  | 128 characters | W95 & NT |
|  | Path [1 to 256] |  | 98 characters | W95 & NT |
|  | Password [1 to 256] |  | 8 characters | W95 & NT |

The parameter FirstPort identifies the point from which install will start looking for a free port name.  Example: FirstPort=COM9 and the ports COM1 and COM2 already exist on the system.  The new port will be named COM9.  On the same system, and if FirstPort=COM1, the new port will be COM3.  Future port enumerations will still be based on that number (when adding a port, even if this operation is not automatic).

Note: All existing pools are deleted by the automatic installation.

## LOG FILE

The file setup.log is created or appended in the same directory as the configuration file. It will inform about the installation result (success, or problems of disk space, etc.).  If errors occur, the automatic installation will abort.

## AUTOMATED CONFIGURATION FOR SAPS CLIENT FOR WIN 95

To run the automated configuration, the following switches can be used on the command line of SETUP.EXE the first time you install the product :

-a	  Run the installation Automatically, using the script file

-u	  Uninstall the product, without requesting the user to confirm.

-n          Do Not make any changes to the target system.  Use this option along with

-a in order to check if the script is correct.

-d"dir"  Read the script file from the "dir" Directory.

Note: If no -a option is indicated, but a valid script file setup.cfg is found in the current install directory, automatic installation will be performed as if option -a  had been specified.

Example :

setup -a -n -d"Long PathName"

setup -dShortPathName

setup -u

Possible switch combinations

| Automatic -a | Uninstall -u | do Nothing -n | Directory -d |
| --- | --- | --- | --- |
| x |  |  | (x) |
| x |  | x | (x) |
|  |  |  | (x) |
|  |  | x | (x) |
|  | x |  |  |
|  | x | x |  |

## AUTOMATED CONFIGURATION FOR SAPS CLIENT FOR NT

You can run the automated installation at the same time you install the SAPS Client for NT or at a later time.

- Running automated configuration during installation:

- Copy the script file SETUP.CFG on the installation diskette in the same directory as OEMSETUP.INF.  The installation software will detect that an automated configuration is requested and will perform all the required settings.

- Running automated configuration after installation:

Use the AUTOCONF.EXE software to perform the automated installation.

Usage is: AUTOCONF [<config_file>]

Default config_file is SETUP.CFG in the current directory.

## Sample script files

The minimal required file :

Sample 1

[Port1]

Path=\\MySAPSServer\MyNetworkPath

Sample file with one port :

Sample 2

[Parameters]

DisconnectTimeOut=0

TimeOut=5

[Port1]

Path=\\ServerName\PortName

Mode=115200,o,7,2,p

Several ports and pools :

Sample 3 for Windows 95

[General]

InstallPath=C:\Program Files\SAPS95

[Parameters]

DisconnectTimeOut=1

[Port1]

Path=\\ServerName\ProtectedPort

Password=toto4

[Port2]

Path=\\ServerName\PortName

Mode=1200,n,7,1

[Port3]

Path= Sample pool

[Pool1]

Name=MyPool

Path1=\\ServerName\PortName1

Path2=\\ServerName\PortName2

Path3=\\ServerName\ProtectedPort

Password3=toto4

[Pool2]

Name=Sample pool

Path1=\\ServerName\PortName1

Path2=\\ServerName\PortName2

Comment=My brand new speedy V.34 modems

# CONFIGURING OS/2 SERVERS

OS/2 servers do not require that SAPS be installed.  The shared COM ports on network servers running under OS/2 are handled by the standard Network Operating System.  To share COM ports on an OS/2 server, follow the instructions provided in the server Administrator's Manual.  The only noted difference for OS/2 is that when a workstation has redirected one of its local COM ports to a shared port on the OS/2 server, it appears twice on the server's administration screen (once with its name as redirected normally and again with its name preceded by ">>", which identifies it as having been redirected by SAPS).

The steps for sharing COM ports with Microsoft LAN Manager, are as follows:

- Logon the server as the Administrator.

- Start the Admin program.

- Select Shared Resources from the View menu.

- Select Add Share.

- Choose "Comm-device.

- Enter the requested Sharename, Devices, Remark.

- Change the User Limit from the default Unlimited to Max. Users = 1.

- Enter the requested Permissions for the resource.

Note: some OS/2 COM drivers do not support speeds higher than 19.2Kbps. Please contact your OS/2 supplier.

# Using SAPS with TCP/IP over a router

As we use NetBIOS which is a Name oriented protocol, some problems may occur,

most of the time related to the Name Þ IP address resolution.

Go through the following checklist when experiencing problems (most often SAPS Client cannot find the SAPS Server):

- try to 'Ping' the IP address of the SAPS Server from the Client workstation. If this fails, then the IP network is not running properly (not a SAPS problem).

- Open a DOS box and type : PING <Server IP address>

- if test 1 is successful, type NET DIAG /S at the command prompt and enter the name of the SAPS Server. If this fails, you have a problem with the Name resolution system or with the configuration of your routers.

- For example you may have configured your IP routers to filter broadcasts which may prevent NetBIOS names to be managed properly.

- if test 2 is successful, check that the name @<server> is listed in the Names Table displayed by NET DIAG. If not, then the SAPS Server is not started or not running on this TCP/IP LANA.

- make sure that your name resolution system is configured with @server .

- Depending on which name resolution system  you are using,  you will have to :

- - change the configuration of your NT WINS server

- or

- modify your local LMHOSTS or HOSTS file to add a line with

@server <Server IP address>

For more information about Name Resolution, WINS and LMHOSTS refer to the Microsoft documentation. For example the Windows NT Resource Kit 'Networking Guide' chapter 14 'WINS' and 15 'LMHOSTS'. Or have a look at Microsoft “TechNet” CD-ROMs.

# How to use SAPS with Novell Netware

SAPS cannot access COM ports on servers running Netware.  Any PC running Windows NT, Windows 95 or Windows 3.x can be used as a non dedicated SAPS server.

SAPS cannot be used directly with the IPX/SPX protocol.

If you are using SAPS with Windows 3.1, install the NetBIOS layer provided with every Netware package: NETBIOS.EXE.

NETBIOS.EXE is a DOS TSR which is loaded by typing “NETBIOS’ at the MS-DOS Prompt, before loading Windows.  Of course you can also put this command in your AUTOEXEC.BAT file.

Also, make sure NetBIOS is loaded on the PC acting as SAPS Server.

Check in the file NET.CFG that only one frame type is configured (either 802.2 or 802.3) on all PCs.

Under Windows 3.x some versions of Novell’s  NETBIOS.EXE do not operate properly. SpartaCom tested SAPS successfully with version 3.14

If you are using SAPS with Windows for Workgroups 3.11 (WFWG ) you could also use Microsoft’s “IPX/SPX Compatible Transport with NetBIOS” protocol, which is part of the WFWG package.  This protocol is installed using WFWG’s Network Setup.

Again, make sure the ‘Frame Type’ parameter is set to the same value, either 802.2 or 802.3 on all PCs.

You cannot use NETBIOS.EXE with the 32bit Netware protocol for Windows 95. Install Microsoft’s “IPX/SPX compatible protocol” and select the “I want to enable NetBIOS over IPX/SPX” option in the NetBIOS section of it’s properties.

Possible problems using SAPS with Netware:

On startup, SAPS Server for Windows & Windows 95 reports “Error # 0 (0000h)

On redirection, SAPS Client for Windows immediately reports “Miscellaneous network error.”

On redirection, SAPS Client for Windows 95 immediately reports “No network stack protocol”

All three messages mean no NetBIOS loaded. Check your network settings and make sure the IPX/SPX protocol has NetBIOS support.

When redirecting, the SAPS Client for Windows reports “The computer name specified in the network path cannot be found”.

When redirecting, the SAPS Client for Windows 95 reports “Name of server not found”.

These two error messages could mean:

No NetBIOS available on the PC running the SAPS Server, or the name of the SAPS Server is misspelled.

SAPS is not limited to Microsoft or Novell networking environments.  BaynanVINES, DEC PATHWORKS, LANtastic, etc., all provide NetBIOS support and should be compatible with SAPS.

# Questions and Answers

Does SAPS function on a Novell network?

Yes.  On computers running Windows 3.1, load the NetBIOS layer supplied by Novell as a DOS TSR (NETBIOS.EXE).  On PCs running Windows for Workgroups, install the 'IPX/SPX Compatible Transport with NetBIOS' protocol.  On PCs running Windows 95 install the 'IPX/SPX-compatible Protocol' and enable it's NetBIOS option.

SAPS cannot access COM ports on servers running Netware.  Any PC running Windows NT, Windows 95 or Windows 3.x can be used as a non dedicated

SAPS server.

Can I use SAPS with TCP/IP?

Yes.  Using NetBIOS on TCP/IP.  The NetBIOS layer is supplied as standard with most TCP/IP protocols (Microsoft, DEC, etc.).

Can I use a shared COM port from applications running in MS-DOS or in a DOS box in Windows?

Yes.  With no restrictions in the DOS box of Windows NT.

Yes, with applications that use the BIOS interrupt 14h interface (serial port interface). Applications directly interfacing the hardware are not supported under DOS, Windows and Windows 95.

Can I use the Microsoft RAS Client with SAPS?

Yes, with Windows NT (Remote Access) or Windows 95 (Dial-Up Networking).

No, with Windows for Workgroups.  The architecture of the 16 bit version of RAS is not compatible with SAPS.

Microsoft RAS Server is using modems to accept incoming calls, can SAPS use these modems as well?

Yes.  Starting with Windows NT 3.51 an interface between RAS and SAPS allows SAPS to query RAS for an available COM port.  Whenever no incoming call is

active, RAS will allow SAPS to get the COM port.  SAPS will return the COM port to RAS, when the application closes the COM port.

What is the maximum speed supported by SAPS?

SAPS supports a maximum speed of 115.2Kbps for Windows applications redirected to SAPS servers on Windows NT.

How does SAPS affect performance, compared to the use of a local COM port?

SAPS normally adds less than 5% to the system overhead.  However, it depends:

- on the application used (blocksize, protocol, etc.)

- on the SAPS client computer's hardware

- on the LAN (theoretical throughput, workload, etc.)

- on the SAPS server's hardware (CPU, operating system, workload, etc.)

- on the COM ports used on the SAPS server (UART, intelligent multiport card, etc.)

How is SAPS used with ISDN?

SAPS is compatible with any ISDN device which works the same as a standard COM port.  For example, you could use external ISDN adapters connected to a

standard COM port or to multiport cards.  Internal ISDN cards may not be compatible with SAPS if they are not provided with serial drivers.  SAPS does not support ISDN devices using CAPI.