FCC Radio Frequency Interference Statement

This device has been tested and has been found to comply with the standards for a Class B digital 	device, pursuant to Part 15 of FCC Rules.  These standards are designed to provide reasonable protection against harmful interference in a residential installation.  This equipment generates, uses and can radiate radio frequency energy, and if not installed in accordance with the instructions, may 	not occur in a particular installation.  In the event that interference is caused the user is encouraged to try to correct this by one or more of the following measures:

- Reorient the receiving antenna or increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- If necessary, consult your dealer or an experienced radio/TV technician.

NOTE:

Connecting this device to peripheral devices that do not comply with Class B requirements, or using 	an unshielded peripheral cable could also result in harmful interference to radio or television reception. The user is cautioned that any changes or modifications not expressly approved by the manufacturer could invalidate the user’s authority to operate the equipment.  This product requires 	the use of shielded cables and connectors. The use of a non-shielded interface cable is prohibited.

This PCB is approved under the following number reference: MCF168XD2M2LS

Trademark Acknowledgements:

- IBM PC, PC/XT and PC/AT all are registered trademarks of International Business Machines Corporation.

- Windows, MS DOS, and MS Word are registered trademarks of the Microsoft Corporation

- Auto CAD is a registered trademark of Autodesk, Inc.

- GEM is a trademark of Digital Research, Inc.

- Lotus 1-2-3 is a trademark of Lotus Development Corporation

- WordPerfect is a trademark of WordPerfect Corporation.

- Other product and brand names are trademarks and / or registered trademarks of their respective corporations.

Table of Contents

1. INTRODUCTION	1

2. HARDWARE INSTALLATION	2

2.1 Package Contents	2

2.2 System Compatibility	2

2.3 PT-580/E Board Layout	2

2.4 Jumper Setting	2

2.5 Memory Configuration	3

2.6 Installing the Card	3

2.7 Hardware Troubleshooting	3

3. SOFTWARE INSTALLATION	4

3.1 Software Driver Support	4

3.2 Installing PT-580/E Software Drivers	4

3.2.1 Windows 3.1 Driver Installation	5

3.2.2 Windows 95 Driver Installation	6

3.2.3 Windows NT Driver Installation	7

3.2.4 OS/2 2.1 & Warp Driver Installation	8

3.2.5 AutoCAD Driver Installation	8

3.2.6 PCAD 6.0 Driver Installation	9

3.2.7 WordPerfect Driver Installation	9

3.2.8 Lotus 1-2-3 Driver Installation	10

3.2.9 IGA Utilities	10

4. VIDEO MODES TABLE	11

1. Introduction

This menu documents the operation of the PT-580/E (EDO DRAM) PCI VGA board. The board is a PCI adapter designed to support the IGS IGA1680 single chip VGA controller.

This card has the following special features;

- High performance acceleration in PCI bus with 64-Bit graphics engine;

- High screen refresh rates up to 75MHz. Provides Flicker-Free environment for easier viewing ;

- High non-interlaced resolution modes including 1024 x 768 x 64K colours in 75Hz, and 1600 x 1200 x 256 colours in 60Hz at 2MB of memory on board;

- Supports EDO (Extended Data Output) and Fast page Mode DRAMs. Note: this depends on production;

- Bundled with choice applications such as Virtual desktop & Colour Calibration in order to offer a better working environment;

- 100% VGA Compatibility in both Hardware and Software;

- Good Support for all VGA, Super VGA and Multi Sync. Monitors;

- Easy installation for both Hardware & Software;

2.  Hardware Installation

2.1 Package Contents

Please check your PT-580 package to ensure that it contains the following items:

- 1 x PT-580/E VGA Card;

- 2 x  Software Driver Diskettes: Disk 1/1 for Windows 3.1 & 95.

Disk 1/1 for DOS & Utilities.

- 1 x this Manual.

2.2 System Compatibility

IBM PC Compatible . All systems require at least one PCI Bus slot.

2.3 PT- 580/E Board Layout

2.4 Jumper Settings

There is only one 2-pin jumper on the card, which is located at J6.

- J6 = Connect.  IRQ9 enable.

- J6 = Open.      IRQ9 disable.  (Default)

2.5 Memory Configuration

|  | U6 | U7 | U8 | U9 |
| --- | --- | --- | --- | --- |
| 1MB memory (default) | Install | Install | Empty | Empty |
| 2MB memory | Install | Install | Install | Install |

2.6 Installing the Card

1. Turn off your computer and remove the cover of  the system case.

2. Make sure the jumper settings and memory configuration are correct.

3. Install the card in one of the PCI slots.

4. Replace the cover.

5. Connect the CRT monitor connector to the 15 pin connector on the VGA card.

6. Power up your computer.

If the system successfully powers up and the normal DOS boot-up messages appear on the screen, the IGA1680 card is installed properly.  If the system does not boot up, there are a few things you can check in the following “Hardware Troubleshooting” section.

2.7 Hardware Trouble shooting

- Make sure the PCI bus connector gold-fingers are clean and that the card is installed in the PCI slot properly.

- After powering up the system, you may find that the system’s power supply vent does not work or you do not hear any sound from the system. If this occurs then it may be that there is a short bridge on the card, and you should turn off the system and take out the card.  Check the back of the card (the solder side) to see if there are any pieces of metal attached on the card that may cause the shortage.

- Check the BIOS at location U2.  Make sure all the pins are installed properly.

- Make sure you turn on the monitor when you power up the system.  Adjusting the screen contrast and brightness may help you see the display more clearly.

3. Software Installation

3.1 Software Driver Support

The 2 x “Software Installation Diskettes” supply drivers for the following operating 	systems and programs:

- Diskette 1/1 - Drivers for Windows 3.1 & 95:

Windows 3.1 under directory name “WIN31” &

Windows 95  under directory name “WIN95” &

Windows 95  under directory name “WIN95A”  for 256 colors only.

- Diskette 1/1 - Drivers for DOS & Utilities:

Windows NT

OS/2 2.1 & Warp

AutoCAD

PCAD 6.0

WordPerfect

Lotus 1-2-3

IGA DOS Utility Programs

3.2 Installing PT- 580/E Software Drivers

To prepare for installation, insert the respective “Software Installation Diskette” in drive 	A (or B) if you are installing drivers for Windows, Windows NT, AutoCAD, PCAD, 	WordPerfect, Lotus 1-2-3, or IGA1680 Utilities.

If you are installing drivers for Windows 95 and OS/2 & Warp, please make sure you 	insert their corresponding NT, Windows 95 and OS/2 & Warp programs. Also, please	make sure that you change your display setting back to VGA mode (640 x 480 x 16 	colours) before beginning installation.

You are now ready to install drivers for your application.

Please refer to the following sections for installation of specific drivers.

3.2.1 Windows 3.1 Driver installation

The installation disk ‘Disk 1/1 Windows 3.1 & 95’ provides two easy ways for installing 	the Windows 3.1 Driver. Please choose just one of the following methods - depending of 	course on which one you feel will be easiest for you to complete.

- Method #1:

1. Run Windows (type “c:\win” and press the Enter key).

2. Under your Windows “Program Manager”, select “Run” inside “File” (using your 	mouse, click on “run” and “file”).

3. Type “A:\WIN31\SETUPWIN” in the Command line prompt and press the Enter

key.

4. The IGA1680 Windows 3.1 driver installation menu will appear on the screen.  	Click on the “Start” icon to begin driver installation.  The installation program will 	automatically install the PT-580/E Windows 3.1 driver for you.

5. Select the “Restart Now” button (by clicking on it) to restart Windows.

- Method # 2:

1. Type “C:\WINDOWS\SETUP “ and press the Enter key.

2. The Microsoft Windows setup table will show on the display.  Select 	“DISPLAY” (by clicking on it with the mouse) and go all the way down to the end, 	which shows “OTHER” (“other” should be highlighted)  Hit  the enter key to choose 	“OTHER”.

3. Type “ A:\WIN31\WIN31” and press the Enter key.

4. The Windows 3.1 drivers menu will appear on screen as follows:

Select “IGS1680 Accelerated Driver” (by double clicking on it with your mouse)  The installation program will completely install the IGA1680 driver for you.

5. Select the “Restart Now” button (by clicking on it) to restart your Windows.

3.2.2  Windows ‘95 Driver Installation

1. In Windows ‘95, use the mouse to click on the “Start” command button to select 	“Run”.

2. Type “A:\WIN95\SETUPWIN”  or “A:\WIN95A\SETUPWIN” for 256 colors

only in the Command line prompt and press the Enter key

3. The IGA1680 Windows ‘95 driver installation menu will appear on the screen.  	Click on the “Start” icon to begin driver installation.  The installation program will 	automatically install the PT-580/E Windows ‘95 driver for you.

4. Select the “Restart Now” button (by clicking on it) to restart your Windows.

The software driver installations of the subsequent sections require Disk 1/1 ‘DOS & Utilities’. Before installing any of the drivers, you must first copy the respective drivers to your hard disk. Please choose one of the two following methods which are the best ways to copy the drivers into your hard disk.

- Method #1

1. Insert the software driver ‘DOS & Utilities diskette’ in drive A:

2. Go to drive A:

3. <Type>   A:\SETUPDOS   <Enter>

4. The IGA1680 installation manual will  appear on screen as follows:

Use the arrow keys or mouse to select the desired software. When you are prompted to specify the destination directory where the drivers will be put, please either accept the default directory or change to another directory if necessary.  Note. You should remember the directory for later installation.

5. After the drivers have been installed, you must “Exit” the installation 	program before you may install any software drivers.

- Method # 2

1. Insert the software driver diskette in drive A:

2. Go to drive A:

3. Type “pkunzip -d xxx\*.zip y:\zzz” where

xxx is the software in the “DOS & Utilities Diskette’ that you want to install.

y is the destination hard disk drive and

zzz is the directory in which you want the drivers to be put.

3.2.3 Windows  NT Driver Installation

- Windows NT 3.1:

1. Run Windows NT.

2. Run Windows NT SETUP program in MAIN window.

3. Select "Changing System Settings..." from OPTIONS pop-up menu.

4. Click on the down-arrow of Display and select "Other display (requires...)".

5. Follow the instructions to select a IGA1680 display driver which fits your monitor type.

6. Continue the setup and finally close the SETUP windows.

7. Shut down Windows NT and reboot the system.

- Windows NT 3.5:

1. Run Windows NT.

2. Double Click on the DISPLAY icon in Control Panel.

3. Select "Changing Display Type".

4. Select "Change".

5. Select "Other".

6. Select the directory where the Windows NT Drivers have been copied as in Page 7 i.e. y:\zzz.

7. Click on the “O.K.” button.

8. Select the mode and resolution you want.

9. Click on the “O.K.” button and wait until the driver is completely installed.

10. Click on the “Test” button to make sure that your monitor can support your selected resolution.  If the test fails, select a different resolution and try again.

11. If the test succeeds, shut down Windows NT and reboot the system.

3.2.4 OS2/2 2.1 and Warp Driver Installation

1. When in OS/2 open an OS/2 session (Window or full screen) Move your working directory to the directory where you have copied the OS/2 drivers. (default is "C:\OS2_DRV")

2. Run the OS/2 driver installation program by typing “OS2INST .  C:” and pressing the Enter key.  This command will install the OS/2 driver from the current working directory to the hard drive C.

3. Follow the instructions to proceed with installation.  When the “Display Driver Install” dialogue box appears, select 'Primary  Display' and then click on the “O.K.” button.

4. When the 'Primary Display Adapter Type' dialogue box appears, select "IGA1680" then click on the O.K. button.

5. When 'Select Display Resolution' dialogue box appears, select the resolution which best fits your needs, and then click on the O.K. button.

6. When 'Source Directory' dialogue box appears, click the button "Change" to set your source directory to C:\OS2_drv before you click the button "Install...".

7. After the installation is completed, shutdown and restart the system so that the changes can take effect.

Note: In case the display loses sync after the system restarts, please follow these 	procedures:

1. Reboot the system with a bootable floppy (DOS).

2. Run the IGA1680 utility - SETFREQ.EXE.  Select a lower refresh rate for your monitor (e.g. if you chose 75 Hz before, try 72 or 60 Hz) If you have an interlaced monitor and you are going to use it in 1024x768 resolution, you must select 1024x768 interlaced mode.  Then, choose not to save the selection into the Autoexec.bat file.

3. Change to directory C:\OS2.

4. Type "SVGA ON DOS" and Enter.  This will create the  SVGADATA.DOS file under the OS2 directory.

5. Type "Copy SVGADATA.DOS SVGADATA.PMI" and Enter.

6. Remove floppy diskette and reboot your system.

If you have a high quality monitor and you want to run it at a higher refresh rate, you 	can fix this by following the above procedures to select a higher refresh rate. But, 	remember to run the command "SVGA ON DOS" immediately after running the utility 	program SETFREQ.EXE.  By doing this, the SCGA.EXE will record the change you 	have made into the file SVGADATA.DOS. Then, after copying SVGADATA.DOS to 	SVGADATA.PMI, a new SVGADATA.PMI file with the new refresh rate information 	will be ready for OS/2.

3.2.5 AutoCAD Driver Installation

See README.TXT File.

3.2.6 PCAD 6.0 Driver Installation

1. Copy the files: PVGA800.DRV and PVGA1024.DRV from the directory in which you have kept the drivers from the ‘DOS & Utilities Diskette’ into the PCAD 6.0 directory.

2. Use any text editor to edit the file PCADDRV.SYS (usually located in the root directory, i.e. at c:\)  If you are using MS-DOS Version  .0 or higher, you may use the text editor and type “EDIT C:\PCADDRV.SYS” and press the Enter key.

3. Search for the line “PDISPLAY C:\PCAD\DRV\DIBMVGA.REX” and delete it.

4. Search for the line “DISPLAY C:\PCAD\DRV\DTEXT.DRV” and change it to: “DISPLAY C:\PCAD\DRV\PVGA1024.DRV” for 1024x768 resolution with 16 colours, or “DISPLAY C:\PCAD\DRV\PVGA800.DRV” for 800x600 resolution with 16 colours.

3.2.7 WordPerfect Driver Installation

See README.TXT File under the directory in which you have kept the Drivers from 	that you put the Drivers from the ‘Windows 3.1 Diskette’.

- Driver Files:

1. Graphics Drivers:

IGS1024.VRS - 1024x768 16 colours

IGS800.VRS - 800x600 16 colours

2. Text Drivers:

TIGS.VRS -

IGS 80x60 16 colours

IGS 132x25 16 colours

IGS 132x43 16 colours

IGS 132x50 16 colours

IGS 132x60 16 colours

- To install the drivers:

1. Directly copy IGS1024.VRS, IGS800.VRS and TIGS.VRS to WordPerfect 6.0 directory.

2. Run "WP" (WordPerfect 6.0)

3. Hit Alt+F, then select "setup".

- Another way to run WordPerfect at high resolution mode is to run the WordPerfect 6.0 Installation utility and select VESA.VRS driver (included in WordPerfect 6.0 package) as the graphics mode screen, since the ProSTAR 1000 board has built-in VESA BIOS support.

Note: you still need InteGraphics Text Driver TIGS.VRS to run WordPerfect 6.0 at 132 column Text mode.

3.2.8 Lotus 1-2-3 Driver Installation

See README.TXT File under the directory in which you kept the drivers from the ‘DOS & Utilities Diskette’.

12/22/1994  InteGraphics Lotus 1-2-3 v3.x Drivers

- Driver Files:

1. LIGS1024.DLD - 1024x768 16 colours

2. LIGS800.DLD - 800x600 16 colours

- Configuration Files:

a. VGA831CC.VBD

b. VGA843CC.VBD for 800x600x4

c. VGA875CC.VBD

d. VGA140CC.VBD

e. VGA154CC.VBD for 1024x768x4

f. VGA196CC.VBD

- To install the drivers:

1. Directly copy LIGS1024.VRS, LIGS800.VRS and *.VBD configuration 			files into Lotus 1-2-3 directory.

2. Run Install from Lotus 1-2-3 Directory.

3. Select "First time installation" or "Change selected equipment".

4. Select "modify current DCF"

5. Select "change selected display"

6. Select "IGS Super VGA 800" for 800x600 16 colours driver, or select "IGS Super VGA 1024" for 1024x768 16 colours driver, then follow the instructions to complete the installation.

3.2.9 IGA Utilities

The IGA utility files have the following functions:

SETMODE.EXE	: Set VGA and Super VGA mode utility.

SETFREQ.EXE	: Set monitor refresh rate utility.

FREQ.EXE		: Refresh rate grab (read back) utility.

IRQ2.EXE		: IRQ2 enable or disable utility.

4. Video Modes Table

| IGS Mode (Hex) | Vesa Mode (Hex) | Resolution | Colors | Refresh Rate (Hz) | Memory Size |
| --- | --- | --- | --- | --- | --- |
| 30 | 108 | 80x60 chars | 16 | 60 | 256KB |
| 31 | 109 | 132x25 char | 16 | 70 | 256KB |
| 32 | 10A | 132x43 char | 16 | 70 | 256KB |
| 33 | 10B | 132x50 char | 16 | 70 | 256KB |
| 34 | 10C | 132x60 char | 16 | 60 | 256KB |
| 43 | 106 | 1280x1024 | 16 | 60 | 1MB |
| 43 | 106 | 1280x1024 | 16 | 87 (I) | 1MB |
| 44 | 104 | 1024x768 | 16 | 60 | 512KB |
| 44 | 104 | 1024x768 | 16 | 72 | 512KB |
| 44 | 104 | 1024x768 | 16 | 75 | 512KB |
| 44 | 104 | 1024x768 | 16 | 87 (I) | 512KB |
| 45 | 102 | 800x600 | 16 | 60 | 512KB |
| 45 | 102 | 800x600 | 16 | 72 | 512KB |
| 45 | 102 | 800x600 | 16 | 75 | 512KB |
| 46 | 101 | 640x480 | 256 | 60 | 512KB |
| 46 | 101 | 640x480 | 256 | 72 | 512KB |
| 46 | 101 | 640x480 | 256 | 75 | 512KB |
| 47 | 103 | 800x600 | 256 | 60 | 512KB |
| 47 | 103 | 800x600 | 256 | 72 | 512KB |
| 47 | 103 | 800x600 | 256 | 75 | 512KB |
| 48 | 105 | 1024x768 | 256 | 60 | 1MB |
| 48 | 105 | 1024x768 | 256 | 72 | 1MB |
| 48 | 105 | 1024x768 | 256 | 75 | 1MB |
| 48 | 105 | 1024x768 | 256 | 87 (I) | 1MB |
| 49 | 107 | 1280x1024 | 256 | 60 | 2MB |
| 49 | 107 | 1280x1024 | 256 | 87 (I) | 2MB |
| 4A | NA | 1600x1200 | 256 | 47 | 2MB |
| 4A | NA | 1600x1200 | 256 | 87 (I) | 2MB |
| 50 | 111 | 640x480 | 64K | 60 | 1MB |
| 50 | 111 | 640x480 | 64K | 72 | 1MB |
| 50 | 111 | 640x480 | 64K | 75 | 1MB |
| 51 | 114 | 800x600 | 64K | 60 | 1MB |
| 51 | 114 | 800x600 | 64K | 72 | 1MB |
| 51 | 114 | 800x600 | 64K | 75 | 1MB |
| 52 | 117 | 1024x768 | 64K | 60 | 2MB |
| 52 | 117 | 1024x768 | 64K | 72 | 2MB |
| 52 | 117 | 1024x768 | 64K | 75 | 2MB |
| 52 | 117 | 1024x768 | 64K | 87 (I) | 2MB |
| 55 | 112 | 640x480 | 16.8M | 60 | 1MB |
| 55 | 112 | 640x480 | 16.8M | 72 | 1MB |
| 55 | 112 | 640x480 | 16.8M | 75 | 1MB |
| 56 | 115 | 800x600 | 16.8M | 60 | 2MB |
| 56 | 115 | 800x600 | 16.8M | 72 | 2MB |
| 56 | 115 | 800x600 | 16.8M | 75 | 2MB |