Component:				Generic Linux Driver

Engineering Release Date:		July 21, 2003

MegaRAID Products Supported:	648/649/680/3112/CSB6 MegaIDE controllers

| Version Numbers: |  |  |  |  |
| --- | --- | --- | --- | --- |
| ###### No | Component | Current Version | Previous Version | Date |
| 1 | Linux Driver | 5.06a | 5.06 | 07/21/2003 |

Release Note 5.06a

Bug Fixes:

1.  Symptom: 	During installation of Redhat AS 2.1 to a Stripe with 256 KB, when the partitions are being formatted ctrl-atl-f4 is pressed in this terminal window the following is seen:

>SCSI dis kerror host 0 channel 0 lun o return code 4000

>I/O error dev 08:00 sector 156279808

>get_last_setor ioctl bread returned null

Fix:	This is because the MegaIDE driver is reporting the logical drive size as a whole.  SCSI expects drive size in logical blocks that is 1 block less than the actual logical drive size.

How to validate:  While loading, make sure the driver checks drive size reported by driver and SCSI are the

same.

2.  Symptom: 	RedHat AS 21 update2 DUD installation is not working

Fix:	This is a known RedHat AS update 2 installation issue.  The issue is because of little available ramdisk space when booted off from CD. This does not leave enough space for our driver to load. The solution is to make a boot floppy and boot from it instead of CD. The boot floppy image can be found in the "images" directory in first CD of the install pack. Put the image "boot.img" on the floppy and boot from it.  Enter "linux dd" as normally and proceed with installation.

How to validate:  Try the above install method

3.  Symptom: 	INODE issue.

LSI noticed that there is remote chance of getting DMA interrupt while DMA engine is active. In this case, it is not advisable to stop DMA TRANSFER. This kind of interrupt happens during repeated reboot and the probability is very less. It may take even weeks of reboot sequence to reproduce the issue.

Fix:	 It is handled properly.

How to validate:  Test Inode issue with repeated reboot.

###### Component Features

Supports 648/649/680/3112/CSB6 MegaIDE controllers

8 LD support,  IOCTL, etc.

Known Restrictions

None

Known Bugs

None

###### Suggested Additional Areas to Test

None